# Introduction

## Overview

Brief overview.

## Scope

The scope of the document is to define detailed technical specification for SI-IF7 of the Billing & Provisioning Interface within the project “Decoupling of Roaming”.

## Definitions

| Term | Description |
| --- | --- |
| Calls | All call events defined for the applicable format (for example TAP, NRTRDE) |

## Abbreviations

| Term | Description |
| --- | --- |
| API | Application Protocol Interface |
| ARP | Alternative Roaming Provider |
| CRM | Client Relationship Management |
| DSP | Domestic Service Provider |
| ICCID | Integrated Circuit Card Identifier |
| IMSI | International Mobile Subscriber Identity |
| LBO | Local BreakOut |
| MNP | Mobile Number Portability |
| MSISDN | Mobile Station International Subscriber Directory Number |
| REST | REpresentational State Transfer |
| SOAP | Simple Object Access Protocol |
| UICC | Universal Integrated Circuit Card |

## References

| Ref | Doc Number | Title |
| --- | --- | --- |
|  | 531/2012 | Regulation (EU) No 531/2012 of the European Parliament and the Council of 13 June 2012 on roaming on public mobile communications networks within the Union |
|  | 1203/2012 | Regulations, Commission Implementing Regulation (EU) No 1203/2012 of 14 December 2012 on the separate sale of regulated roaming services within the Union |
|  | BoR (12) 68 | ROAMING REGULATION - CHOICE OF DECOUPLING METHOD: A consultation to assist BEREC in preparing advice to the Commission on its forthcoming Implementing Act, June 2012, 72 pages. |
|  | BoR (12) 109 | ROAMING REGULATION - CHOICE OF DECOUPLING METHOD, BEREC opinion on article 5 implementing act, 27 Sept 2012, 7 pages |
|  |  | High Level Technical specifications. V 0.8 |
|  |  | Processes draft 0.92 |
|  | RFC 2119 | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997. Available at http://www.ietf.org/rfc/rfc2119.txt |

## Conventions

The key words “must”, “must not”, “required”, “shall”, “shall not”, “should”, “should not”, “recommended”, “may”, and “optional” in this document are to be interpreted as described in RFC 2119 [7].

# Assumptions

It is possible that an individual subscriber may hold a roaming contract with more than one ARP for Single IMSI service at any one time.  However, a subscriber can only have one active ARP subscription at any time.

It is possible that an ARP would like to activate more than one subscription at a time. We assume that the ARP will send one activation request for each subscription separately.

In case of a batch activation request, it is possible that the DSP is not able to activate all of them within 1 working day. A maximum number of requests that can be activated within the standard timescale will be defined.

All messages are acknowledged with an implicit synchronous acknowledgement message. This message is not shown in diagrams but is described in 4.1.1.

IF7 APIs can be implemented using either REST or SOAP protocol. The choice of the one that will be used to connect a DSP with an ARP is a DSP option.

This means that ARP may have to implement both protocols in order to be able to connect to two or more DSPs (if they are not using the same option).

Red arrows on the following diagrams indicate the interactions that are in scope of this specification. Black arrows are out of the scope of SI-IF7.

# Processes with interaction between DSP and ARP

For a detailed process description refer to [6]

## Single IMSI

### Relationship between DSP and ARP

This process is out of scope of the document because it doesn’t require interactions between DSP and ARP.

### Service Activation by the Customer

The Activation Process could include a verification phase performed by DSP in order to understand whether the customer is eligible or not for an ARP Subscription. This verification is out of the scope of this document and can be different from a Country to another, depending on local regulation enforcement.

Scenario 1 - Activation: Activation with swap between two ARPs

This process applies when a customer requires an ARP subscription having another one active and the Donor ARP can’t stop the process

Diagram 1

Scenario 2 - Change from Domestic Service Provider to Alternative Roaming Provider

This process applies when a customer requires an ARP subscription without having another one  active.

Diagram 2

Scenario 3 - Activation: Change from Alternative Roaming Provider to Domestic Service Provider

This process applies when a customer wants to migrate back from an ARP subscription to a DSP offer.

This scenario is equivalent to a De-Activation.

Diagram 3

### Customer Service query responsabilities

This process is out of scope of the document because it doesn’t require interactions between DSP and ARP.

### Customer is issued a bill

This process is out of scope of the document because it doesn’t require interactions between DSP and ARP.

### Customer Changes Signalling Status (Customer Changes Billing Basis)

Scenario 1 – Change of Subscriber Contract at the ARP.

This process applies when an ARP customer wants to change its charge method (prepaid/postpaid)

Diagram 4

Scenario 2 – Change of Billing Interface used for a subscriber from DSP to ARP

This process applies when DSP changes the billing interface for an ARP customer.

Diagram 5

### Bill-shock measures

This process is out of scope of the document because it doesn’t require interactions between DSP and ARP.

### Fraud Management and Prevention

Scenario 1 – Process Flow with NRTRDE

Diagram 6

Scenario 2 – Process Flow with OnLine Interface

Diagram 7

### Service DeActivation initiated by the Customer

Diagram 8

### Service DeActivation initiated by the ARP

Diagram 9

### Service DeActivation initiated by the DSP

Diagram 10

#### Customer ports out via MNP during Single IMSI ARP contract

The MNP opt-out process implies that DSP will deactivate the ARP subscription during the MNP procedure.

This means that the “Service DeActivation initiated by DSP” process, described in 3.1.10 will be used.

#### Customer changes MSISDN and/or IMSI during Single IMSI ARP contract

The change of MSIDN/IMSI implies that DSP will deactivate the ARP subscription during the change procedure.

This means that the “Service DeActivation initiated by DSP” process, described in 3.1.10 will be used.

The Customer will start a new Activation Process in order to set up a new subscription with new identifier

### Suspension and Termination of contract between DSP and ARP

DSP and ARP can decide to suspend or deactivate their agreement, or one of the parties could will decide unilaterally to do it under conditions defined by the bilateral agreement.

If such a situation occurs all customer subscription will be deactivated and roaming service will fall back to being provided by the DSP.

A message, similar to the DeProvisioningAnnouncement should be sent by the ARP to all customers affected to inform them of the deactivation of their ARP contract.

# Operation Description

## Single IMSI

### AcK

This is the synchronous acknowledge

### PreProvisioningRequest

This service is used by ARP to ask DSP to activate an ARP subscription.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| ARP | TADIG Code of the ARP requesting the PreProvisioning operation |
| User Identifier | Array of parameters that identify the User (MSISDN, IMSI, ICCID) |
| ARP Signalling Status | Code that identifies the proposed signalling type of the ARP Subscription (OnLine/OffLine) |
| Authorization Information | Array of parameters used to transfer any information useful for Authorization Process |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service Activation by the customer – Scenario 1

Service Activation by the customer – Scenario 2

### PreProvisioningRequestAcknowledgement

This service is used by DSP to confirm to ARP that the PreProvisioningRequest has been taken on.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the customer subscription |
| Request Arrival Time | Date and Time of when the provisioning request arrived to DSP |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service Activation by the customer – Scenario 1

Service Activation by the customer – Scenario 2

### ProvisioningRequest

This service is used by ARP to confirm to DSP that internal activities have been concluded and the activation can start on DSP side.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service Activation by the customer – Scenario 1

Service Activation by the customer – Scenario 2

### ProvisioningCompletion

This service is used by DSP to notify the result of an activation request of an ARP subscription.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Provisioning Start Time | Date and Time of when the provisioning procedure started |
| Provisioning End Time | Date and Time of when the provisioning procedure ended |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Notification Code | Identification code of the result of the activation |
| Notification Description | Description of the result |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service Activation by the customer – Scenario 1

Service Activation by the customer – Scenario 2

### PreProvisioningCompletion

This service is used by DSP to notify ARP if the customer can be provisioned or not

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| InterfaceProvider | Array of parameters that identify the provider for each interface (IF1, IF2, IF3 …) |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| User Identifier | Array of parameters that identify the User (MSISDN, IMSI, ICCID) |
| Notification Code | Identification code of the result of the activation |
| Notification Description | Description of the result |
| ARP Signalling Status | Code that Identifies the Signalling (OnLine/OffLine) to be used by ARP |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service Activation by the customer – Scenario 1

Service Activation by the customer – Scenario 2

### ReProvisioningRequest

This service is used by ARP to request to DSP to change the customer charge type at ARP (online/offline)

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| OLD ARP Signalling Status | Code that Identifies the OLD Signalling Status of the ARP Subscription (OnLine/OffLine) |
| NEW ARP Signalling Status | Code that Identifies the NEW Signalling Status of the ARP Subscription (OnLine/OffLine) |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Customer changes Signalling Status

### ReProvisioningCompletion

This service is used by DSP to confirm to ARP that the customer charge type at ARP (online/offline) has been changed.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| InterfaceProvider | Array of parameters that identify the provider for each interface (IF1, IF2, IF3 …) |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Change Start Time | Date and Time of when the change procedure started |
| Change End Time | Date and Time of when the change procedure ended |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| ARP Signalling Status | Code that Identifies the Signalling (OnLine/OffLine) to be used by ARP |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Customer changes Signalling Status

### ReProvisioningNotification

This service is used by DSP to inform ARP that a ReProvisioning process has been started.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Change Type | Field that described the Change Type: From Online to Offline or from Offline to Online |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Customer changes Signalling Status

### ReProvisioningAcknowledgement

This service is used by DSP to confirm to ARP that the ReProvisioningRequest has been taken on or used by ARP to confirm to DSP that the ReProvisioningNotification has been received.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Arrival Timestamp | Date and Time of when the ReprovisioningRequest or the ReProvisioningNotification has been received, depending on involved scenario |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Customer changes Signalling Status

### SuspendRoaming

This service is used by ARP to ask DSP the suspension of the roaming service within a Fraud Management Process.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Fraud Management and Prevention – Scenario 1

Fraud Management and Prevention – Scenario 2

### RoamingSuspended

This service is used by DSP to notify ARP about the suspension of the roaming service.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Notification Code | Identification code of the result of the activation |
| Notification Description | Description of the result |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Fraud Management and Prevention – Scenario 1

Fraud Management and Prevention – Scenario 2

### UnSuspendRoaming

This service is used by ARP to ask DSP the unsuspension of the roaming service within a Fraud Management Process.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Fraud Management and Prevention – Scenario 1

### RoamingUnSuspended

This service is used by DSP to notify ARP about the unsuspension of the roaming service.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Notification Code | Identification code of the result of the activation |
| Notification Description | Description of the result |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Fraud Management and Prevention – Scenario 1

### DeProvisioningRequest

This service is used by ARP to ask DSP to cancel an ARP subscription.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Deactivation Reason | Identification of the reason why the subscription should be deactivated. |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service DeActivation initiated by of the customer

Service DeActivation initiated by ARP

### DeProvisioningAckowledgement

This service is used by DSP to confirm to ARP that the DeprovisioningRequest  has been taken on.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Notification Code | Identification code of the result of the deactivation |
| Notification Description | Description of the result |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service DeActivation initiated by of the customer

Service DeActivation initiated by ARP

### DeProvisioningCompletion

This service is used by DSP to notify the result of a cancellation request of an ARP subscription, either positive or negative.

Parameters:

| SENDER | TADIG Code of the message Sender |
| --- | --- |
| RECEIVER | TADIG Code of the message Receiver |
| Subscription Id | Unique Code that identifies the ARP Subscription |
| Deactivation Start Time | Date and Time of when the deprovisioning procedure started |
| Deactivation End Time | Date and Time of when the deprovisioning procedure ended |
| Deactivation Reason | Identification of the reason why the subscription has been deactivated |
| Notification Code | Identifiction code of the result of the deactivation |
| Notification Description | Description of the result |
| Transaction Id | Unique Code that identifies all messages exchanged within the same the process. The code is generated by ARP (or DSP in specific cases, when the process is started by DSP) and identifies all messages exchanged between ARP and DSP and related to the same process. For example, all messages exchanged for the same Activation Process for a specific subscription (PreProvisioningRequest, PreProvisioningAcknowledgement, PreProvisioningCompletion, ProvisioningRequest, ProvisioningCompletion) will contain the same TransactionId |
| Bilateral Information | Optional field bilaterally agreed that is beyond the scope of SI-IF7. His content is defined by bilateral agreement and may vary according to the context |

Processes:

Service DeActivation initiated by Customer

Service DeActivation initiated by ARP

Service DeActivation initiated by DSP

Service Activation by the customer – Scenario 1

Service Activation by the customer – Scenario 3

Customer ports out via MNP during Single IMSI ARP contract

# Parameters Definitions

| SERVICE | PARAMETERS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PreProvisioningRequest |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | ARP | Text – 5 Characters | NO | TADIG Code of the ARP requesting the PreProvisioning operation |
|  |  | UserIdentifier | Text | NO | Array of parameters used to communicate UserId. Each parameter is documented with the sintax “[UserIsType=UserId]” where: UserIdType can have one of the values: MSISDN IMSI ICCID MSISDN will be in international format Example: [MSISDN=393351234567] [IMSI=222011234567890] |
|  |  | ARP Signalling Status | OnLine OffLine | YES | Signalling status of ARP subscription |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TADIG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
|  |  | AuthorizationInformation | Text | YES | Array of Values. Each parameter is documented with the sintax “[Name=Value]” Parameters needed for this field are defined by National Regulator Example: [AuthorizationCode-00AFGH67] [FirstName-Francesco][LastName-Donato][DateOfBirth-21061970] |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
| PreProvisioningRequestAcknowledgement |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | RequestArrivalTimestamp | Date The date is specified in the following form YYYY-MM-DDThh:mm:ss[+/-]hh:mm where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO |  |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| ProvisioningRequest |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| ProvisioningCompletion |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
|  |  | Provisioning Start Timestamp | Date The date is specified in the following form YYYY-MM-DDThh:mm:ss[+/-]hh:mm where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO |  |
|  |  | Provisioning End Timestamp | Date The date is specified in the following form YYYY-MM-DDThh:mm:ss[+/-]hh:mm where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO |  |
|  |  | Notification Code | 0: Activated 1: Not Activated – Generic 2: Not Activated – DSP has not yet sent a PreProvisioningCompletion 3: Not Activated – DSP has sent a PreProvisioningCompletion(NOK) 4: Not Activated – invalid Transaction ID 5: Not Activated – valid Transaction ID but the MSISDN does not match with what was sent in the PreProvisioningRequest 100-199: reserved for local use 200-500: reserved for bilateral agreements | NO | This field will be populated accordingly with local privacy regulation. In case the sender has privacy restrictions for sending this information, “generic” codes must be used |
|  |  | Notification Description | Text [max 80 alphanumeric digits] | YES | Free text description of the Notification Code |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
| PreProvisioningCompletion |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | InterfaceProvider | Text | YES | Array of parameters that identify the provider for each interface (IF1, IF2, IF3 …) It will be populated only in case Notification Code=0 Each parameter is documented with the sintax “[Interface=Value]” where: Interface can be one of the values: IF1, IF2, IF3, IF4, IF5, IF6, IF7, IF8, IF9 and Value is the TAIDG Code of the MNO/MVNO providing the interface When an Interface is missing within the array, it is intended that it is provided by the SENDER Example: [IF1=ITASI][IF2=ITA01] |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | UserIdentifier | Text | YES | Array of parameters used to communicate UserId. Each parameter is documented with the sintax “[Name=Value]” where: Name can be one of the values: MSISDN IMSI ICCID MSISDN will be in international format Example: [MSISDN=393351234567] [IMSI=22011234567890] |
|  |  | Notification Code | 0: Activable 1: No Active Agreement 2: Not authorized - Generic 3: Not authorized – Not customer of this DSP 4: Not authorized – Incorrect Authorization Method 5: Not authorized – Incorrect Authorization Credentials 6: Not eligible - Generic 7: Not eligible – Recipient ARP doesn’t have an agreement with DSP 8: Not eligible – Customer not controlled by DSP 9: Not eligible – Subscriber’s domestic service has been suspended 10: Not eligible – Subscriber’s roaming service has been suspended 11: Not eligible – Subscriber has no contract to receive roaming service 12: Not eligible – The service requestor is not the legal responsible party 13: Not eligible - There is another ongoing provisioning or de-provisioning request for this UserId 14: Not eligible - Request is based on non-primary UserId for the subscrption 15: Not eligible – Customer requested MNP 100-199: reserved for local use 200-500: reserved for bilateral agreements | NO | This field will be populated accordingly with local privacy regulation. In case the sender has privacy restrictions for sending this information, “generic” codes must be used |
|  |  | Notification Description | Text [max 80 alphanumeric digits] | YES | Free text description of the Notification Code |
|  |  | ARPSignallingStatus | OnLine OffLine | NO | Signalling Status of ARP subscription after the ReProvisioning |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| ReProvisioningRequest |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | OLD ARP Signalling Status | OnLine OffLine | NO | Old Signalling Status of ARP subscription |
|  |  | NEW ARP Signalling Status | OnLine OffLine | NO | New Signalling Status of ARP subscription |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| ReProvisioningCompletion |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | InterfaceProvider | Text | YES | Array of parameters that identify the provider for each interface (IF1, IF2, IF3 …) It will be populated only in case Notification Code=0 Each parameter is documented with the sintax “[Interface=Value]” where: Interface can be one of the values: IF1, IF2, IF3, IF4, IF5, IF6, IF7, IF8, IF9 and Value is the TAIDG Code of the MNO/MVNO providing the interface. When an Interface is missing within the array, it is intended that it is provided by the SENDER Example: [IF1=ITASI][IF2=ITA01] |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Change Start Timestamp | Date The date is specified in the following form YYYY-MM-DDThh:mm:ss[+/-]hh:mm where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO |  |
|  |  | Change End Timestamp | Date The date is specified in the following form YYYY-MM-DDThh:mm:ss[+/-]hh:mm where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO |  |
|  |  | ARPSignallingStatus | OnLine OffLine | NO | Signalling Status of ARP subscription after the ReProvisioning |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| ReProvisioningNotification |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | ChangeType | 1: From OnLine to OffLine 2: From OffLine to OnLine | NO |  |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| ReProvisioningAcknowledgement |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Arrival Timestamp | Date The date is specified in the following form "YYYY-MM-DDThh:mm:ss[+/-]hh:mm" where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO | Date and Time of when the ReprovisioningRequest or the ReProvisioningNotification has been received, depending on involved scenario |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| DeProvisioningRequest |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Deactivation Reason | 1 digit, encoded as: 0: request by customer 1: MNP port out 2: swap to another ARP 3: customer deactivation by DSP 4: fraud management 5: termination of contract with the subscriber – bill payer initiated 6: termination of contract with the customer – operator initiated 7: Change in subscriber primary identifier 8: Subscription modification incompatible with ARP contract 10: Generic 50-99: Reserved for bilateral agreements | NO | In case the Deactivation has been requested for Fraud Management reason (i.e. Code Value 4), the DSP should treate the request as a priority and process it as soon as possible, Optionally, DSP and ARP can agree bilaterally on a specific Fraud Suspension Code |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | Unique Identifier of the Activate request. It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Must be the same code provided within the corresponding PreProvisioningRequest request Example: ITA0112345678912345678912 |
| DeProvisioningAckowledgement |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Notification Code | 0: Deactivated 1: Not authorized - Generic 2: Not authorized – Not customer of this DSP 3: Not eligible - Generic 4: Not Eligible - There is another ongoing provisioning or de-provisioning request for this UserId 100-199: Local 200-500: reserved for bilateral agreements | NO |  |
|  |  | Notification Description | Text [max 80 alphanumeric digits] | YES | Free text description of the Notification Code |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| DeProvisioningCompletion |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 When the message is exchanged within an “Activation with swap between two ARPs” process, the SubscriptionId contains the NEW ARP TADIG CODE, even if the message is sent from DSP to the OLD ARP to communicate the swap. |
|  |  | Deactivation Reason | 0: request by customer 1: MNP port out 2: swap to another ARP 3: customer deactivation by DSP 4: fraud management 5: termination of contract with the subscriber – bill payer initiated 6: termination of contract with the customer – operator initiated 7: Change in subscriber primary identifier 8: Subscription modification incompatible with ARP contract 10: Generic 50-99: Reserved for bilateral agreements | YES | In case the Deactivation has been requested for Fraud Management reason (i.e. Code Value 4), the DSP should treat the request as a priority and process it as soon as possible, Optionally, DSP and ARP can agree bilaterally on a specific Fraud Suspension Code In case the DeProvisioningCompletion is sent as part of a process that doesn’t contain a DeProvisioningRequest sent by ARP (Service Deactivation Initiated by the DSP or Swap to another ARP, or MNP port-out, Change of UserId, etc.) this field must be populated |
|  |  | DeProvisioning Start Timestamp | Date The date is specified in the following form "YYYY-MM-DDThh:mm:ss[+/-]hh:mm" where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO |  |
|  |  | DeProvisioning End Timestamp | Date The date is specified in the following form "YYYY-MM-DDThh:mm:ss[+/-]hh:mm" where: - YYYY indicates the year - MM indicates the month - DD indicates the day - T indicates the start of the required time section - hh indicates the hour - mm indicates the minute - ss indicates the second - [+/-]hh:mm indicates time zones Example: 2013-05-01T19:50:00+01:00 | NO |  |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TADIG Code and a unique code usually generated by ARP, except for the process “Service Deactivation Initiated by DSP” where the parameter is generated by DSP Example: ITA0112345678912345678912 |
|  |  | Notification Code | 0: Deactivate 1: Not authorized - Generic 2: Not authorized – Not customer of this DSP 3: Not eligible - Generic 4: NoteEligible - There is another ongoing provisioning or de-provisioning request for this UserId 100-199: Local 200-500: reserved for bilateral agreements | NO |  |
|  |  | Notification Description | Text [max 80 alphanumeric digits] | YES | Free text description of the Notification Code |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
| SuspendRoaming |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| RoamingSuspended |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Notification Code | 0: Activated 1: Not Activated – Generic 2: Not Activated – invalid Transaction ID 3: Not Activated – Invalid SubscriptionId 100-199: reserved for local use 200-500: reserved for bilateral agreements | NO | This field will be populated accordingly with local privacy regulation. In case the sender has privacy restrictions for sending this information, “generic” codes must be used |
|  |  | Notification Description | Text [max 80 alphanumeric digits] | YES | Free text description of the Notification Code |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| UnSuspendRoaming |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |
| RoamingUnSuspended |  | Name | Type/Values | Optional | Description |
|  |  | SENDER | Text – 5 Characters | NO | TADIG Code of the Sender |
|  |  | RECEIVER | Text – 5 Characters | NO | TADIG Code of the Receiver |
|  |  | SubscriptionId | Text [30 alphanumeric digits] | NO | It’s obtained by the concatenation of DSP TADIG CODE, ARP TAIDG Code and a unique code generated by DSP Example: ITASIITA0112345678912345678912 |
|  |  | Notification Code | 0: Activated 1: Not Activated – Generic 2: Not Activated – invalid Transaction ID 3: Not Activated – Invalid SubscriptionId 100-199: reserved for local use 200-500: reserved for bilateral agreements | NO | This field will be populated accordingly with local privacy regulation. In case the sender has privacy restrictions for sending this information, “generic” codes must be used |
|  |  | Notification Description | Text [max 80 alphanumeric digits] | YES | Free text description of the Notification Code |
|  |  | Bilateral Information | Text [max 80 alphanumeric digits] | YES | Bilaterally agreed content |
|  |  | TransactionId | Text [25 alphanumeric digits] | NO | It’s obtained by the concatenation of ARP TAIDG Code and a unique code generated by ARP Example: ITA0112345678912345678912 |

# Testing Procedure

TBD

# Release Management

TBD

# Security

Security requirements between DSP and ARP are are subject to further study and will be included in a separate document

- Document Management

  - Document History

| Version | Date | Brief Description of Change | Approval Authority | Editor / Company |
| --- | --- | --- | --- | --- |
| 0.1 | 27/02/2013 | Skeleton with first inputs | B&P | Francesco Donato (Telecom Italia) |
| 0.2 | 04/03/2013 | Definition of Activation ad DeActivation Precesses. | B&P | Francesco Donato (Telecom Italia) |
| 0,3 | 15/03/2013 | Modified as agreed during Paris f2f meeting and alignment to version 0.4 of Process Specs [6] | B&P | Francesco Donato (Telecom Italia) |
| 0.4 | 22/03/2013 | Modified as agreed during conference call held on 18th March | B&P | Francesco Donato (Telecom Italia) |
| 0.5 | 04/04/2013 | Modified as agreed during Bonn f2f meeting Aligned to version 0.5 of Process Specs [6] Aligned with answers from Process Group | B&P | Francesco Donato (Telecom Italia) |
| 0.6 | 19/04/2013 | Aligned to version 0.71 of Process Specs [6] | B&P | Francesco Donato (Telecom Italia) |
| 0.7 | 02/05/2013 | Modified as agreed during Stockholm f2f meeting | B&P | Francesco Donato (Telecom Italia) |
| 0.8 | 03/05/2013 | Modified as agreed during conference call held on 2nd of May | B&P | Francesco Donato (Telecom Italia) |
| 0.9 | 21/05/2013 | Aligned to version 0.9 of Process Specs [6] | B&P | Francesco Donato (Telecom Italia) |
| 0.91 | 07/06/2013 | Aligned to version 0.92 of Process Specs [6] | B&P | Francesco Donato (Telecom Italia) |
| 0.92 | 25/06/2013 | Modified as agreed during conference call held on June 18th. | B&P | Francesco Donato (Telecom Italia) |
| 0.93 | 26/06/2013 | Modified as agreed during conference call held on June 26th. | B&P | Francesco Donato (Telecom Italia) |
| 0.94 | 04/07/2013 | Modified as agreed during conference call held on June 28th. | B&P | Francesco Donato (Telecom Italia) |
| 0.95 | 11/07/2013 | Modified as agreed during conference call held on July 5th. | B&P | Francesco Donato (Telecom Italia) |
| 0.96 | 15/07/2013 | Included CRs 01, 02 and 03 and other modifications agreed during conference call held on July 12th. | B&P | Francesco Donato (Telecom Italia) |
| 1.0 | 18/07/2013 | First official version | B&P | Francesco Donato (Telecom Italia) |