Change Request

| Title: | Flow of PN Establishment and Registration | Public OMA Confidential |
| --- | --- | --- |
| To: | CD-CPNS SWG |  |
| Doc to Change: | OMA-AD-CPNS-V1_0-20100111-D |  |
| Submission Date: | 2nd Feb 2010 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Takayuki Warabino, KDDI, ta-warabino@kddi.com Takashi Matsunaka, KDDI, ta-matsunaka@kddi.com Yuichi Ishikawa, KDDI, yi-ishikawa@kddi.com Tet Chiba, KDDI, chiba@kddi.com |  |
| Replaces: | n/a |  |

Reason for Change

The current AD does not clearly describe how a secure session is established when PN is being established. This CR intends to clarify the message flow of a secure session establishment especially when Entity-User Key is used.

R01: During the Sorrento meeting (including off-line discussion), there were following comments for previous version of this CR:

- Terminology “CPNS Server/PN GW initiated assignment” should be reworded to “CPNS Server/PN GW generated assignment”

R01 changes the terminology according to this comment

- CPNS SWG has not yet decided specific identity model, which decide whether CPNS Server generates PNE’s ID or PN GW does so.

R01 changes the figure to accommodate both two cases

- If we use IMS control-plane to send and receive CPNS signalling messages, additional secure-session establishment between PNGW and CPNS Server is not necessary.

R01 added a sentence to mention the above in description of 1st step (secure session establishment between CPNS Server and PNGW)

- Since connection setup request is the very first message, which triggers PN Establishment and Registration, ID and Key assignment to PNE should also be triggered by the request.

R01 changes the figure and description according to the above comment.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

CPNS SWG is recommended to discuss the contribution and agree with the detailed proposal and include in the CPNS AD as described below.

Detailed Change Proposal

Add Flow of PN establishment and registration in Appendix

PN establishment and registration

Figure 1 shows a flow of PN establishment and registration.

Figure 1 Flow of PN establishment and registration

As the figure shows, the flow of PN establishment and registration consists of 5 steps:

- Secure session establishment between PN GW and CPNS Server

Secure session should be established before PN GW and CPNS Server start communication with each other. If the session already exists, this step is skipped. In addition, if Secure WAN provides a security mechanism which ensures security of the communication, this step is skipped.

  - Secure session establishment is triggered by either of two events

Although the figure shows that the session establishment is performed before 2nd step, it can be during 2nd or later step.

    - When user manually triggers via PN GW’s UI

    - When PN GW sends message to CPNS Server triggered by message from PNE

  - Secure session is established by either underlying network mechanism or OMA SEC_CF.

- Physical pairing

In this step, physical connection between PN GW device and PNE device is established by underlying PN technology, after which PN GW and PNE discover each other.

(Editor note: More explanation or reference to other section may be added.)

- Entity discovery

In this step, PN GW and PNE discover each other

(Editor note: More explanation or reference to other section may be added.)

- Connection setup initiation

  - PNE sends PN GW a connection setup request.

  - By using underlying network security mechanism, a device which has PNE is authenticated and authorized by a device which has PN GW

  - PNE checks if it already has ID and Entity-User Key. If it does, subsequent procedures in the 5th step is skipped. Otherwise, it continues processing 5th and later steps.

- ID & Entity-User Key assignment for PNE

- PN GW or CPNS Server generates ID to be assigned PNE. Depending on the ID schema, CPNS Entity which generates ID differs.

- Entity-User Key is assigned to PNE by either CPNS server generated assignment or PN GW generated assignment as described in 5.3.1.12.5 “Entity-User Key Assignment”. Example flows are shown in Appendix B.1 “Entity-User Key assignment”.

- Secure session establishment between PNE and PN GW

Secure session is established after authentication and authorization.

  - PN GW asks CPNS Server if the PNE is allowed to access its PN by sending request to CPNS Server

  - CPNS Server authenticates PNE. PNE also authenticates CPNS Server. Detailed flow for this is described in TS.

c), d) and e) CPNS Server performs authorization of the connection setup request by checking policy.

- PNE and PN GW authenticate each other and share a session key, which is used to ensure security of subsequent communication between PNE and PN GW.

- PN inventory update

  - PN inventory is updated.

(Editor note: More explanation or reference to other section may be added.)

  - After PN inventory update, PN GW notifies PNE of connection setup completion.