# Change Request

| Title: | Status Management | Public OMA Confidential |
| --- | --- | --- |
| To: | CD CPNS |  |
| Doc to Change: | OMA-TS-CPNS_System_Description-V1_0-20100617-D |  |
| Submission Date: | 30 Septmber 2010 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Clerical |  |
| Source: | Jean-Francois Deprun jf.deprun@lge.com Christophe Ochem, christophe.ochem@lge.com |  |
| Replaces: | n/a |  |

Reason for Change

This CR will add the diagram flows of the Status Management Request.

In the AD:

“This function allows following status management functions.

- Status information report

  - In response to status information retrieval request, status information can be reported to requester.

  - Under certain circumstances triggered by a specific event (threshold hit, etc.), status information can be reported to target CPNS entity.

    - Setting the policy based on request from CPNS entities, who want to be notified of specific event occurrence

    - In occurrence of the event, status information is reported to the CPNS entity who requests the policy based status information report

This function provides information about:

- The presence of PNE and PN GW

- Their status (e.g. Online, Offline, Busy, etc)

“

This CR specifies the different process to check the status of PNE, GW and No-CPNS Entity.

R01: new version after the F2F meeting in Las Vegas.

R02: new version after comments about Budapest.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Review and approve the proposed change in this CR for incorporation into the CPNS TS Sd.

Detailed Change Proposal

Proposal of the flow on

## 5.x Statuts Management

### 5.x .1 General

The Status Management provides a mechanism to publish, collect, subscribe and notify the status of PNE and PN GW.

Each message in Status Management SHALL be formatted as CPNS Messages. The format of each message can be found in section xxx.

The Status Management can be performed after CPNS Entity Discovery, PN Management and PN Registration. In case of status management of content provider (i.e., PNE), the Status Management should be performed after Service/Content delivery.

Two different processes allow to collect the status of the PNE and PN GW:

- The PNE or PN GW will send it own status without any external request:

The PNE or PN GW will update and send its status to the CPNS Server. The decision to send the status to the server will be done by one or several applications embedded in the PNE or PN GW.

The CPNS server will hold the status information for collection and notification when the status changes.

The CPNS server could forward the status information to other Entities (External Entities or CPNS Entities).

- An Entity (External Entity or CPNS Entity) will send a request to have status of other CPNS Entities

The status information function in the other CPNS Entities is initialized by the CPNS server.

The CPNS server will send status requests to PNE and/or PN GW. These status requests could specify details or information to update status or to have specific information about status.

The decision to request the status of the PNE or PN GW will be done by applications embedded in other the PNE, PN GW or External Entity.

The CPNS server could authorize or not these status requests.

The CPNS server will hold the status information for collection and notification when the status changes.

The CPNS server could forward the status information to other Entities (external Entities or CPNS Entities).

5.x.1.1  PNE or PN GW send it own status without any external request

## […]

5.x.1.2 Status requested by Entities

The status information of PNE or PN GW is requested by Entities (External Entities or CPNS Entities).

These requests are forwarded by the CPNS Server to PNE and PN GW. The CPNS Server manages these requests:

- The CPNS Server receives Status Requests from Entities (External Entities or CPNS Entities).

- The CPNS Server controls the initialization of the Status function and the modification of the Status in the PNE and/or PN GW in function of the Status Requests sent by Entities (External Entities or CPNS Entities).

- The CPNS Server collects and stores the status information coming from PNE and PN GW.

- The CPNS Server forwards if needed the status information or a part of the status information to other Entities (External Entities or CPNS Entities).

## 6. High level Flows (informative)

6.x  Statuts Management

6.x.1 Introduction

[…]

The Status Management provides a mechanism to publish, collect, subscribe and notify the status of PNE and PN GW.

Each message in Status Management is formatted as CPNS Messages. The format of each message can be found in section xxx.

Following status management procedures are identified.

Publishing status information

Subscribe and notify

Entity Status request

Update Status

6.x.2 Publishing status information

[…]

6.x.3 Subscribe and Notify

[…]

### 6.x.4 Entity Status Request

The Entity (External Entity or CPNS Entity) which wants to have the status of one or several target Entities sends a request to the CPNS Server.  This request includes the ID(s) of the target Entities and the Status Parameters.

The Status parameters allow to specify how the status feedback has to be done.

The Status Parameters could be:

- A time: the Status is sent periodically; this time defines the period. The parameter is the Status Period.

- The event that the status is modified. The new status is sent after each modification.

- The request to have the current status.

- An Entity (External Entity or CPNS Entity) sends a request to the CPNS Server to have the status of one or several CPNS Entities. The Status Parameters is sent in the request.

- The CPNS Server will check if this request is allowed in function of a status Policy. If yes, the CPNS server sends Status Request.  If not, the CPNS server sends a negative answer to the Entity.

- The Status request is send to the CPNS Entities.

- The Entities store the Status Parameters.

- The Entities answers with their status if needed.

- The CPNS server controls the forward.

- The CPNS forward the status to the Entity.

### 6.x.5 Update Status

The PNE or PN GW has already received a request from an Entity (see 5.x.4 Entity Status Request) and has stored Status Parameters.

In function of this Status Parameters, the PNE/ PN GW will send it status to the Entity.

- The PNE/ PN GW has the Status Parameters to manage the status sending. Following an event (time out or modification  of the status) the PNE/ PN GW decides to send its status

- The PNE/ PN GW sends its status with its ID and the ID of Entity which asks it.

- The CPNS Server could forward or not this answer if function of Status Policy.  If the Status Policy bans to forward the status, the StatusAnswer is not forwarded.

- If the Status Policy allows the forward of the status, the Status is forwarded.