Change Request

| Title: | Update the data structure of Thermometer APIs | Public OMA Confidential |
| --- | --- | --- |
| To: | Content Delivery (DWAPI/GotAPI) WG |  |
| Doc to Change: | Thermometer APIs Draft Version 1.0 – 17 August 2015 |  |
| Submission Date: | 15 September 2015 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Max Hata, NTT DOCOMO, masato.hata.uf@s1.nttdocomo.com Takafumi Yamazoe, NTT DOCOMO, yamazoet@nttdocomo.com |  |
| Replaces: |  |  |

Reason for Change

This CR proposes to change the data structure of Thermometer APIs for consistency with other PCH APIs such as Pulse Oximeter APIs.

For example, the current Thermometer APIs defines the data structure sent from a GotAPI Server to an application on the GotAPI-1 Interface as follows (One-shot API):

{

...

"thermometer" : {

"deviceProductName"      : "ABC Thermo Pro",

"deviceManufacturerName" : "ABC Inc.",

...,

"temperature"            : 36.4,

"temperatureMderFloat"   : "FFFFC8E",

...

},

...

}

In the above structure, all data are in the same layer. This flat structure will cause confusion if the number of key-value pairs increase. Besides, the names of keys tend to be long.

This CR proposes the data structure as follows:

{

...

"thermometer"  : {

"device": {

"productName"      : "ABC Thermo Pro",

"manufacturerName" : "ABC Inc.",

...

},

"temperature": {

"value"            : 36.4,

"mderFloat"        : "FFFFC8E",

...

}

},

...

}

It introduces one additional layer to organize the structure, which makes it easier to organize many different values. This is the same data structure that we plan on using consistently for other devices such as pulse oximeters.

Impact on Backward Compatibility

None

Impact on Other Specifications

none

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Recommend to incorporate this CR to the Device WebAPI specification.

Detailed Change Proposal

5.2.3 Response for one-shot measuring on the GotAPI-4 Interface

Replace "Definition of the data object for the response" as below (The changed parts are represented in red):

Definition of the data object for the response

| Name |  |  | Type | Definition of value | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| method |  |  | String | This value SHALL be "RESPONSE". | Mandatory if the OS is not Android. Otherwise, optional. If the OS is Android, the "Action" value SHALL include this information as described below. |
| requestCode |  |  | int | The request code coming from the GotAPI Server. | Mandatory |
| result |  |  | int | If success, the value is 0, otherwise an integer greater than 0, which indicates an error code. This specification doesn't define error codes. | Mandatory |
| thermometer |  |  |  |  |  |
|  | device |  | Object |  | Mandatory |
|  |  | productName | String | The product name of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, it SHALL create a name for the thermometer using an arbitrary algorithm. The algorithm is up to the Plug-In implementation, and this specification does not define any algorithms. | Mandatory |
|  |  | manufacturerName | String | The manufacturer name of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | modelNumber | String | The model number of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | firmwareRevision | String | The firmware revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | serialNumber | String | The serial number of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | softwareRevision | String | The software revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | hardwareRevision | String | The hardware revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | partNumber | String | The part number of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | protocolRevision | String | The protocol revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | systemId | String | The system id of the targeted thermometer. This value SHALL be a 16-character HEX string without a '0x' prefix (e.g. "ABCDEF0123456789"). If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be "0000000000000000" (a string of 16 '0' characters). | Mandatory |
|  |  | batteryLevel | Float | The battery level of the targeted thermometer. This value must be a float number in a range from 0.0 to 1.0. The value 0.0 represents that the targeted thermometer is completely out of charge. The value 1.0 represents that the targeted thermometer is fully charged. Even if the targeted thermometer reports this value in percent in a range from 1 to 100, the Plug-In SHALL convert it to a float number in a range from 0.0 to 1.0. If the Plug-In can't obtain battery level from the targeted thermometer, this value SHALL be -1.0. | Mandatory |
|  | temperature |  | Object |  | Mandatory |
|  |  | value | Float | This value represents the temperature measured by the targeted thermometer. | Mandatory |
|  |  | mderFloat | String | This value represents the temperature measured by the targetd thermometer, which is a hexadecimal string of a MDER FLOAT, such as "FFFFC8E". | Mandatory |
|  |  | type | String | This value represents the body part where the Thermometer is measuring, which is expressed by a human readable string such as "Oral body temperature". If the Plug-In can't obtein the type, this value SHALL be an empty string. | Mandatory |
|  |  | typeCode | String | This value represents the body part where the Thermometer is measuring, which is expressed by a code such as "188424" (This code means "Oral body temperature"). If the Plug-In can't obtein the type, this value SHALL be an empty string. | Mandatory |
|  |  | unit | String | This value represents the unit of the reported temperature, which is expressed by a human readable string such as "deg C". | Mandatory |
|  |  | unitCode | String | This value represents the unit of the reported temperature, which is expressed by a code such as "268192" (This code means "deg C"). | Mandatory |
|  |  | timeStamp | int | This value represents the measurement time when the temperature was measured. If the measurement time is reported from the targeted thermometer, the Plug-In SHALL convert it to a unix time stamp in millisecond. Otherwise, the Plug-In set this value to the unix time when the Plug-In receives the measurement value from the Plug-In based on the clock of the underlying operating system. | Mandatory |
|  |  | timeStampString | String | This value represents the same time stamp as "timeStamp". The format is "YYYYMMDDHHMMSS.sss+/-HHMM", such as "20150504135813.220-0400" | Mandatory |

Replace "Example of the data object of the Android Intents" as below (The changed parts are represented in red):

Example of the data object of the Android Intents

| Name |  |  |  | Example of value | Note |
| --- | --- | --- | --- | --- | --- |
| Action |  |  |  | org.deviceconnect.action.RESPONSE | This value is defined by the GotAPI Server application. But the last part SHALL be "RESPONSE". |
| Component |  |  |  | org.deviceconnect | This value is the package name of the GotAPI Server application. |
| Extra |  |  |  |  |  |
|  | requestCode |  |  | 10 |  |
|  | result |  |  | 0 |  |
|  | thermometer |  |  |  |  |
|  |  | device |  |  |  |
|  |  |  | productName | ABC Thermo Pro |  |
|  |  |  | manufacturerName | ABC Inc. |  |
|  |  |  | modelNumber | TP-001 |  |
|  |  |  | firmwareRevision | rev.1.001.003 |  |
|  |  |  | serialNumber | 01234-5678-9ABCD-EF01 |  |
|  |  |  | softwareRevision | rev.2.000.000 |  |
|  |  |  | hardwareRevision | rev.1.0 |  |
|  |  |  | partNumber | 002 |  |
|  |  |  | protocolRevision | rev.3.1 |  |
|  |  |  | systemId | ABCDEF0123456789 |  |
|  |  |  | batteryLevel | 0.5 |  |
|  |  | temperature |  |  |  |
|  |  |  | value | 36.0 |  |
|  |  |  | mderFloat | FFFFC8E |  |
|  |  |  | type | Oral body temperature |  |
|  |  |  | typeCode | 188424 |  |
|  |  |  | unit | deg C |  |
|  |  |  | unitCode | 268192 |  |
|  |  |  | timeStamp | 1431856940275 | The sample value in the left cell represents "Sun, 17 May 2015 10:02:20 GMT". |
|  |  |  | timeStampString | 20150517100220.000-0000 |  |

5.2.4 Response for one-shot measuring on the GotAPI-1 Interface

Replace "Definition of the data object for the response" as below (The changed parts are represented in red):

Definition of the data object for the response

| Name |  |  | Type | Definition of value | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| product |  |  | String | The name of the GotAPI Server (e.g. "ABConnect") | Mandatory |
| version |  |  | String | The version of the GotAPI Server (e.g. "1.0"). | Mandatory |
| result |  |  | Number | If success, the value is 0, otherwise an integer greater than 0, which indicates an error code. This specification doesn't define error codes. | Mandatory |
| thermometer |  |  |  |  |  |
|  | device |  | Object |  | Mandatory |
|  |  | productName | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | manufacturerName | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | modelNumber | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | firmwareRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | serialNumber | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | softwareRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | hardwareRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | partNumber | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | protocolRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | systemId | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | batteryLevel | Number | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  | temperature |  | Object |  | Mandatory |
|  |  | value | Number | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | mderFloat | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | type | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | typeCode | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | unit | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | unitCode | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | timeStamp | Number | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | timeStampString | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
| hmac |  |  | String | An HMAC generated for the counter measure against the GotAPI Server spoofing attack. If the application includes a key for HMAC calculation in the API request, the GotAPI Server adds this value in the API response. Evaluating whether the HMAC is identical to the result of calculation of HMAC from the key, the application can ensure that the response is genuine. | Mandatory if the application provide a key to the GotAPI Server |

Replace "Example of the response" as below (The changed parts are represented in red):

Example of the response

{ "product" : "ABCConnect", "version" : "1.0", "requestCode" : 10, "result" : 0, "thermometer" : { "device" : { "productName" : "ABC Thermo Pro", "manufacturerName" : "ABC Inc.", "modelNumber" : "TP-001", "firmwareRevision" : "rev.1.001.003", "serialNumber" : "01234-5678-9ABCD-EF01", "softwareRevision" : "rev.2.000.000", "hardwareRevision" : "rev.1.0", "partNumber" : "002", "protocolRevision" : "rev.3.1", "systemId" : "ABCDEF0123456789", "batteryLevel" : 0.5 }, "temperature": { "value" : 36.4, "mderFloat" : "FFFFC8E", "type" : "Oral body temperature", "typeCode" : "188424", "unit" : "deg C", "unitCode" : "268192", "timeStamp" : 1431856940275, "timeStampString" : "20150517100220.000-0000" } }, "hmac" : "0123456789" }

5.3.3 Response for asynchronous messaging on the GotAPI-4 Interface

Replace "Definition of the data object for the response" as below (The changed parts are represented in red):

Definition of the data object for the response

| Name |  |  | Type | Definition of value | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| method |  |  | String | This value SHALL be "RESPONSE". | Mandatory if the OS is not Android. Otherwise, optional. If the OS is Android, the "Action" value SHALL include this information as described below. |
| requestCode |  |  | Number | The request code coming from the GotAPI Server. | Mandatory |
| result |  |  | Number | If success, the value is 0, otherwise an integer greater than 0, which indicates an error code. This specification doesn't define error codes. | Mandatory |
| thermometer |  |  | Object |  | Mandatory |
|  | device |  | Object |  | Mandatory |
|  |  | productName | String | The product name of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, it SHALL create a name for the thermometer using an arbitrary algorithm. The algorithm is up to the Plug-In implementation, and this specification does not define any algorithms. | Mandatory |
|  |  | manufacturerName | String | The manufacturer name of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | modelNumber | String | The model number of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | firmwareRevision | String | The firmware revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | serialNumber | String | The serial number of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | softwareRevision | String | The software revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | hardwareRevision | String | The hardware revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | partNumber | String | The part number of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | protocolRevision | String | The protocol revision of the targeted thermometer. If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be an empty string. | Mandatory |
|  |  | systemId | String | The system id of the targeted thermometer. This value SHALL be a 16-character HEX string without a '0x' prefix (e.g. "ABCDEF0123456789"). If the Plug-In cannot obtain this information from the targeted thermometer, this value SHALL be "0000000000000000" (a string of 16 '0' characters). | Mandatory |

Replace "Example of the data object of the Android Intents" as below (The changed parts are represented in red):

Example of the data object of the Android Intents

| Name |  |  |  | Example of value | Note |
| --- | --- | --- | --- | --- | --- |
| Action |  |  |  | org.deviceconnect.action.RESPONSE | This value is defined by the GotAPI Server application. But the last part SHALL be "RESPONSE". |
| Component |  |  |  | org.deviceconnect | This value is the package name of the GotAPI Server application. |
| Extra |  |  |  |  |  |
|  | requestCode |  |  | 10 |  |
|  | result |  |  | 0 |  |
|  | thermometer |  |  |  |  |
|  |  | device |  |  |  |
|  |  |  | productName | ABC Thermo Pro |  |
|  |  |  | manufacturerName | ABC Inc. |  |
|  |  |  | modelNumber | TP-001 |  |
|  |  |  | firmwareRevision | rev.1.001.003 |  |
|  |  |  | serialNumber | 01234-5678-9ABCD-EF01 |  |
|  |  |  | softwareRevision | rev.2.000.000 |  |
|  |  |  | hardwareRevision | rev.1.0 |  |
|  |  |  | partNumber | 002 |  |
|  |  |  | protocolRevision | rev.3.1 |  |
|  |  |  | systemId | ABCDEF0123456789 |  |

5.3.4 Response for asynchronous messaging on the GotAPI-1 Interface

Replace "Definition of the data object for the response" as below (The changed parts are represented in red):

Definition of the data object for the response

| Name |  |  | Type | Definition of value | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| product |  |  | String | The name of the GotAPI Server (e.g. "ABConnect") | Mandatory |
| version |  |  | String | The version of the GotAPI Server (e.g. "1.0"). | Mandatory |
| result |  |  | Number | If success, the value is 0, otherwise an integer greater than 0, which indicates an error code. This specification doesn't define error codes. | Mandatory |
| thermometer |  |  | Object |  | Mandatory |
|  | device |  | Object |  | Mandatory |
|  |  | productName | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | manufacturerName | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | modelNumber | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | firmwareRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | serialNumber | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | softwareRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | hardwareRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | partNumber | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | protocolRevision | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | systemId | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
| hmac |  |  | String | An HMAC generated for the counter measure against the GotAPI Server spoofing attack. If the application includes a key for HMAC calculation in the API request, the GotAPI Server adds this value in the API response. Evaluating whether the HMAC is identical to the result of calculation of HMAC from the key, the application can ensure that the response is genuine. | Mandatory if the application provide a key to the GotAPI Server |

Replace "Example of the response" as below (The changed parts are represented in red):

Example of the response

{ "product" : "ABCConnect", "version" : "1.0", "requestCode" : 10, "result" : 0, "thermometer" : { "device" : { "productName" : "ABC Thermo Pro", "manufacturerName" : "ABC Inc.", "modelNumber" : "TP-001", "firmwareRevision" : "rev.1.001.003", "serialNumber" : "01234-5678-9ABCD-EF01", "softwareRevision" : "rev.2.000.000", "hardwareRevision" : "rev.1.0", "partNumber" : "002", "protocolRevision" : "rev.3.1", "systemId" : "ABCDEF0123456789" } }, "hmac" : "0123456789" }

5.3.5 Asynchronous message from the Plug-In to the GotAPI Server on the GotAPI-4 Interafce

Replace "Definition of the data object for request" as below (The changed parts are represented in red):

Definition of the data object for request

| Name | Sub name |  | Type | Definition of value | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| method |  |  | String | This value SHALL be "EVENT". | Mandatory if the OS is not Android. Otherwise, optional. If the OS is Android, the "Action" value SHALL include this information as described below. |
| requestCode |  |  | int | The request code coming from the GotAPI Server. | Mandatory |
| result |  |  | Number | If success, the value is 0, otherwise an integer greater than 0, which indicates an error code. This specification doesn't define error codes. | Mandatory |
| thermometer |  |  | Object |  | Mandatory |
|  | device |  | Object |  | Mandatory |
|  |  | batteryLevel | Float | The battery level of the targeted thermometer. This value must be a float number in a range from 0.0 to 1.0. The value 0.0 represents that the targeted thermometer is completely out of charge. The value 1.0 represents that the targeted thermometer is fully charged. Even if the targeted thermometer reports this value in percent in a range from 1 to 100, the Plug-In SHALL convert it to a float number in a range from 0.0 to 1.0. If the Plug-In can't obtain battery level from the targeted thermometer, this value SHALL be -1.0. | Mandatory |
|  | temperature |  | Object |  | Mandatory |
|  |  | value | Float | This value represents the temperature measured by the targeted thermometer. | Mandatory |
|  |  | mderFloat | String | This value represents the temperature measured by the targetd thermometer, which is a hexadecimal string of a MDER FLOAT, such as "FFFFC8E". | Mandatory |
|  |  | type | String | This value represents the body part where the Thermometer is measuring, which is expressed by a human readable string such as "Oral body temperature". If the Plug-In can't obtein the type, this value SHALL be an empty string. | Mandatory |
|  |  | typeCode | String | This value represents the body part where the Thermometer is measuring, which is expressed by a code such as "188424" (This code means "Oral body temperature"). If the Plug-In can't obtein the type, this value SHALL be an empty string. | Mandatory |
|  |  | unit | String | This value represents the unit of the reported temperature, which is expressed by a human readable string such as "deg C". | Mandatory |
|  |  | unitCode | String | This value represents the unit of the reported temperature, which is expressed by a code such as "268192" (This code means "deg C"). | Mandatory |
|  |  | timeStamp | int | This value represents the measurement time when the temperature was measured. If the measurement time is reported from the targeted thermometer, the Plug-In SHALL convert it to a unix time stamp in millisecond. Otherwise, the Plug-In set this value to the unix time when the Plug-In receives the measurement value from the Plug-In based on the clock of the underlying operating system. | Mandatory |
|  |  | timeStampString | String | This value represents the same time stamp as "timeStamp". The format is "YYYYMMDDHHMMSS.sss+/-HHMM", such as "20150504135813.220-0400" | Mandatory |

Replace "Example of the data object of the Android Explicit Intents" as below (The changed parts are represented in red):

Example of the data object of the Android Explicit Intents

| Name | Extra key name |  |  | Example of value | Note |
| --- | --- | --- | --- | --- | --- |
| Action |  |  |  | org.deviceconnect.action.EVENT | This value is defined by the GotAPI Server application. But the last part SHALL be "EVENT". |
| Component |  |  |  | org.example.plugin | This value is the package name of the Plug-In application. |
| Extra |  |  |  |  |  |
|  | requestCode |  |  | 10 |  |
|  | result |  |  | 0 |  |
|  | thermometer |  |  |  |  |
|  |  | device |  |  |  |
|  |  |  | batteryLevel | 0.5 |  |
|  |  | temperature |  |  |  |
|  |  |  | value | 36.0 |  |
|  |  |  | mderFloat | FFFFC8E |  |
|  |  |  | type | Oral body temperature |  |
|  |  |  | typeCode | 188424 |  |
|  |  |  | unit | deg C |  |
|  |  |  | unitCode | 268192 |  |
|  |  |  | timeStamp | 1431856940275 | The sample value in the left cell represents "Sun, 17 May 2015 10:02:20 GMT". |
|  |  |  | timeStampString | 20150517100220.000-0000 |  |

5.3.6 Asynchronous message from the GotAPI Server to the application on the GotAPI-5 Interface

Replace "Definition of the data object" as below (The changed parts are represented in red):

Definition of the data object

| Name | Sub name |  | Type | Definition of value | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| serviceId |  |  | String | The identifier of the targeted Service. This value is provided by the application when the application send the originated API request on the GotAPI-1 Interface. | Mandatory |
| thermometer |  |  |  |  | Mandatory |
|  | device |  | Object |  | Mandatory |
|  |  | batteryLevel | Number | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  | temperature |  | Object |  | Mandatory |
|  |  | value | Number | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | mderFloat | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | type | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | typeCode | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | unit | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | unitCode | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | timeStamp | Number | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
|  |  | timeStampString | String | This value SHALL be the same as that which the GotAPI Server received from the Plug-In. | Mandatory |
| hmac |  |  | String | An HMAC generated for the counter measure against the GotAPI Server spoofing attack. If the application includes a key for HMAC calculation in the API request, the GotAPI Server adds this value in the API response. Evaluating whether the HMAC is identical to the result of calculation of HMAC from the key, the application can ensure that the response is genuine. | Mandatory if the application provide a key to the GotAPI Server |

Replace "Example of the JSON string" as below (The changed parts are represented in red):

Example of the JSON string

{ "serviceId" : 0, "thermometer": { "device" : { "batteryLevel" : 0.5 } "temperature" : { "value": : 36.4, "mderFloat" : "FFFFC8E", "type" : "Oral body temperature", "typeCode" : "188424", "unit" : "deg C", "unitCode" : "268192", "timeStamp" : 1431856940275, "timeStampString" : "20150517100220.000-0000" } } }