Change Request

| Title: | Clarification to requirement PO-HSF-01.2 | Public OMA Confidential |
| --- | --- | --- |
| To: | Content Delivery (DWAPI/GotAPI) WG |  |
| Doc to Change: | Device WebAPI Draft Version 1.0 |  |
| Submission Date: | January 14 2016 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Alan Hameed, Fujitsu, alan.hameed@us.fujitsu.com |  |
| Replaces: |  |  |

Reason for Change

Update requirement PO-HSF-01.2 based on TS work.

Clarified the RD to articulate that two formats are needed, (1) a usual float and (2) a string that represents an MDER float in a string format. The latter is to ensure preserving the original precision provided by the MDER FLOAT.

Impact on Backward Compatibility

None

Impact on Other Specifications

none

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Recommend to incorporate this CR to the Device WebAPI specification.

Detailed Change Proposal

Update to section 6.4 Pulse Oximeter Specific Functional Requirements

6.4  Pulse Oximeter Specific Functional Requirements Current text

| # PO-HSF-01.1 | # The Pulse Oximeter Plug-In SHALL report the value of the Supplemental types attribute as a human readable string and as its 32-bit MDC code (combine the 16-bit bit partition code with the 16-bit code partition:code) if a supplemental types exists. # Example: String: “Spot (average) measurement” # Code: “150588” [MDC_MODALITY_SPOT] | # 1.0 |
| --- | --- | --- |
| # PO-HSF-01.2 | # The Pulse Oximeter Plug-In SHALL report the value reported from the appropriate *-Nu-Observed-Val attribute in two formats, i.e., float and string: - Float that represents the Pulse Oximeter report data measured by the targeted Pulse Oximeter and represented in a float format. This is for general use of application programming. - String that represents the Pulse Oximeter report data measured by the targeted Pulse Oximeter and reported to the Plug-In in an MDER FLOAT format. Since there is no way to transfer an MDER FLOAT from Plug-Ins to applications, an MDER FLOAT is represented as a hexadecimal string, such as "FFFFC8E". In this way, the MDER FLOAT can preserve the exact precision as derived from the MDER encoded FLOAT. (Note that IEEE devices will report this value typically as an MDER SFLOAT which the Temperature Plug-In maps to an MDER FLOAT.) MDER FLOAT may be sent to the cloud for other applications while preserving the original precision provided by the MDER FLOAT. # Example: String: “98” # MDER FLOAT: “00000062’ | # 1.0 |