Change Request

| Title: | Appendix F fixes | Public OMA Confidential |
| --- | --- | --- |
| To: | CD |  |
| Doc to Change: | OMA-ER-MobSocNet-V1_0-20120425-D |  |
| Submission Date: | 27 Apr 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Laurent-Walter Goix, Telecom Italia, laurentwalter.goix@telecomitalia.it |  |
| Replaces: | n/a |  |

Reason for Change

As per MobSocNet-2012-A005 this CR checks potential conflicts with the IETF ENUM ACCT draft. It also updates the wording to reflect the informative nature of this section.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Accept the proposed changes and close MobSocNet-2012-A005.

Detailed Change Proposal

Update appendix F

- (Informative): DNS-ENUM recipient MSISDN address resolution

For requests asking to interact with a recipient user identified through a tel: URI (e.g. MSISDN address), the MSN Server willing to use the DNS-ENUM recipient MSISDN address resolution procedure performs as in the following example:

- The MSN Server verifies that the recipient address (MSISDN) complies with the E.164 address format and includes the ‘+’ character. In the case of national or local addressing scheme (e.g. only operator code followed by a number), the MSN Server converts the national or local number to an E.164 address format.

# EXAMPLE:	UK phone number 01632 960123

# The UK phone number 01632 960123 (country code 44) is converted to +4401632960123

- The MSN Server applies the ENUM First Well Known Rule (see [RFC6116] section 3.2) to create the associated FQDN.

EXAMPLE:	3.2.1.0.6.9.2.3.6.1.0.4.4.e164.arpa (public top level domain)

- The MSN Server requests NAPTR records using this FQDN (see [RFC3403] and [RFC6116] section 3)

EXAMPLE:	The following is an example of NAPTR Resource Records associated with the FQDN derived from the recipient MSISDN address (+4401632960123)

IN NAPTR 100 10 "u" "E2U+sip" "!^.*$!sip:John.Doe@sip.example.com!"		.

IN NAPTR 100 11 "u" "E2U+sn" "!^.*$!acct:John.Doe@example.com!"			.

IN NAPTR 101 10 "u" "E2U+sn" "!^.*$!acct:Johnny@doe.example.com!"		.

IN NAPTR 102 10 "u" "E2U+email:mailto" "!^.*$!mailto:Johnny@doe.example.com!"	.

Note: The ENUM service identifiers "E2U+sip" and "E2U+email:mailto" are defined in [RFC6118].

- If the DNS ENUM service is unavailable, or if no NAPTR record exist for that number, the MSN Server invokes an appropriate address resolution exception handling procedure (e.g. reporting the error condition to the requesting MSN entity).

- Otherwise if NAPTR record(s) exist for that number, the MSN Server proceeds as follows:

  - If “acct:” NAPTR record(s) exist for that number

    - The MSN Server sorts the ” acct:” NAPTR record(s) according to the Order and Preference fields as described in [RFC3403] and [RFC6116]

    - The MSN Server considers the “acct:” URI of the highest precedence ” acct:” NAPTR record as the resolved recipient address.

  - Otherwise, the MSN Server follows the procedures defined in section 9.1.1.2 and in [ENUM-ACCT] section 4.

EXAMPLE:	The highest precedence “acct:” URI for +4401632960123is acct:John.Doe@example.com