Change Request

| Title: | CPM Session History Storage SD | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_System_Description-V2_0-20120905-D |  |
| Submission Date: | 5 September 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com |  |
| Replaces: |  |  |

Reason for Change

The purpose of this CR is to improve the CPM Storage capability when dealing with CPM Session History Object during CPM Session and incorporate Group State History for long-live chat.

The current CPM Storage structure is not efficient and poor performance when storing chat session with large media content.

The proposal is to add an option to support session as folder and have each message stored as separated message in that folder.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM System Description TS.

Detailed Change Proposal

Section 4.2

## CPM Message Storage

### Storage Model

The CPM Message Storage SHALL allow for the storage of CPM Standalone Messages, CPM File Transfer Histories, CPM Session Histories, CPM Conversation Histories, and any potential Media objects either stand-alone or attached to CPM Standalone Messages and CPM Session Histories. A Group State Object is stored in relation to the CPM Session Histories and it contains the last information related to the CPM Group Session. The storage model of the CPM Message Storage closely resembles the storage models that are exposed by IMAP mailboxes, and containing folders and objects stored as IMAP messages.

Figure 1 shows a schematic overview of the storage model exposed by the CPM Message Storage. The dark-red shaded items in the model are new for this version of the Enabler.

Conv Hist 1

Default Folder

Conv Hist 2

Conv Hist 3

File xfer 1

Msg 1

Sess

Hist 1

Msg 3

User Folder 1

Sub Folder 1

Sub Folder 2

Conv Hist 4

Conv Hist 5

Conv Hist 6

Sess

Hist 2

Msg 4

Sess

Hist 3

File xfer 1

Msg 1

Conv Hist 1

Default Folder

Conv Hist 5

Conv Hist 6

User Folder 1

Sub Folder 1

Sub Folder 2

Conv Hist 2

Conv Hist 3

Conv Hist 4

Sess

Hist 2

Msg 2

Sess

Hist 3

Msg 4

Msg 5

Group State 4

Msg 3

Session

Hist 4

Sess

Hist 1

Sess

Info 4

File xfer 1

Figure 1: Storage Model of the CPM 2.0 Message Storage

      - Folders

Within this storage model the following types of folders can be distinguished:

- the “default” system folder, which is the root folder for all CPM Message Store folders and objects. There SHALL be only one such folder per CPM User. This folder SHALL only contain conversation history folders.

- user folders, which themselves further can contain any number of sub-folders. User folders MAY contain conversation history folders which have been moved from the “default” system folder by the CPM User. Once they have been moved, conversation history folders SHALL NOT be moved back to the “default” system folder. There MAY be any number of such folders per CPM User,

- conversation history folder, which is used to store all items related to a single CPM Conversation. The conversation history objects SHALL be represented towards the end-user as separate threads in a folder and NOT as folders. Conversation history folders MAY be moved, in their entirety, to a user folder in the user folders hierarchy. Conversation history folders SHALL contain zero or more of the following items:

  - message object

  - file transfer object

  - session history object

  - stand-alone media object

  - session history folder

- session history folder, which contain one or more of the following items:

  - chat-message object

  - file transfer object

  - stand-alone media object

  - group state object

  - session info object

      - Objects

Within the folders of the CPM Message Storage the following types of stored objects can be distinguished:

- A message object, which is a container for a CPM Standalone Message (including any Media objects attached to the CPM Standalone Message).

- A file transfer object, which is a container for a CPM File Transfer History (including the file transferred in the CPM File Transfer).

- A session history object, which is a container for a CPM Session History. The session history object SHALL include all information to be stored for a CPM Session as described in section Error! Reference source not found. in a single object

- A stand-alone media object, which is stand-alone media wrapped as a MIME formatted object with no requirement to be [RFC2822] conforming, but it may include an indication to its content type.

- A chat-message object, which is a container for a CPM Message exchanged within a CPM Group Chat. This object may only appear in a session history folder.

- A group state object, which is a container for the CPM Group State information, as defined in section 5.2.1.2. This object may only appear in a session history folder.

- A session info object, which is similar to the session history object, but contains only the session information as stated in section 5.2.1.2 when recording the session history. The other parts from the session history object are instead recorded and stored as separate objects, such as chat-message object, file transfer object, and group state object.

NOTE 1: 	In this release of the CPM Enabler only complete conversation history objects can be moved between folder objects. The individual components of conversation histories such as threaded message objects, file transfer objects or session history objects cannot be moved separately.

NOTE 2:	Message objects, file transfer objects and session history objects can only be so-called “leaf-objects”, meaning that they SHALL NOT contain other message objects, file transfer objects, session history objects, conversation history objects or folder objects as their sub-objects within the storage hierarchy.

NOTE 3: For this release of the CPM Enabler, the session history object SHALL NOT be used for recording or storing CPM Sessions.  Instead the session history folder SHALL be used to record or store CPM Session. However, for backward compatibility this release of the CPM Enabler SHALL support all other operations for managing the session history object when requested to do so.

Metadata is associated with the message objects, the file transfer objects, the session history objects and the folder objects. This metadata is used to store data that is associated with the content / folder, but is not part of the real contents of the content / folder. The following are examples of the metadata that is supported:

Flags associated with the message object, file transfer object or session object (e.g. read-flag, answered-flag, forwarded-flag)

Access control lists to determine who has access to a folder and who can modify the folder.

NOTE 3:	Access control lists SHALL be only associated with folder objects, not with individual conversation history objects, message objects, file transfer objects or session history objects.

Note: in practice, since the CPM Message Store is based on IMAP message storage principles, the folders shown in the model in [Figure 1] are mapped to IMAP mailboxes. The IMAP specification dictates that before any operation can be performed on objects in the message store, a mailbox must be selected using the IMAP SELECT operation, which is analogous to “changing directory” in a computer file system. Strictly speaking, there is no “default” system folder in IMAP, but by convention the “inbox” is often accepted as the default system folder. However, mapping of the “default” system folder to “inbox” or anything else is implementation specific, and out of scope for CPM.