# Change Request

| Title: | CPM_Session_Clarification | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20130123-D |  |
| Submission Date: | 18 Feb. 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com Bernhard Meier, bernhard.meier@ericsson.com Thomas Scheerbarth, Thomas.Scheerbarth@telekom.de |  |
| Replaces: | n/a |  |

Reason for Change

| D020 | 2013.01.23 | T | 5.1 | Source: NSN, Bernie (Ericsson), Thomas (DT) Form: CONRR CPM2.0 Comment: Clarify how ad-hoc and predefine groups are used with CPM Standalone Message. Proposed Change: See CR16 | Status: CLOSED by CR16 |
| --- | --- | --- | --- | --- | --- |

| D021 | 2013.01.23 | T | 5.2 | Source: NSN, Bernie (Ericsson), Thomas (DT) Form: CONRR CPM2.0 Comment: Clarify how ad-hoc, predefine group, and close group related in CPM Session Proposed Change: See CR16 | Status: CLOSED by CR16 |
| --- | --- | --- | --- | --- | --- |

| D022 | 2013.02.13 | T | 5.3 | Source: NSN, Bernie (Ericsson), Thomas (DT) Form: CONRR CPM2.0 Comment: It is not clear of how to use these IDs in each of CPM conversation, such as CPM Standalone, CPM Session, CPM File Transfer. Also, there is no mention of CPM Group Session-ID. Proposed Change: See CR16 | Status: CLOSED by CR16 |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Conversation Function TS and close the CONRR comments addressed in section 1 of the CR.

Detailed Change Proposal

## Comment D020 &D021

## CPM Standalone Message

A CPM Standalone Message is either a Pager Mode CPM Standalone Message carried in a SIP MESSAGE request as described in [RFC3428] or a Large Message Mode CPM Standalone Message. The size of a Pager Mode CPM Standalone Message should not exceed 1300 bytes. If a CPM Standalone Message is larger than 1300 bytes, the contents of the message are not inserted into the SIP MESSAGE request but are carried via MSRP as defined in [RFC4975]. In that case, a SIP session is established between the interested parties (sender and all receivers) with MSRP as the Media Stream. When addressing a CPM Standalone Message to an Ad-hoc Group, the concept of open or close group is not applicable. The CPM Standalone Message contents are then transmitted via MSRP, using chunking if necessary. This SIP session should not be confused with a CPM Session as no CPM Session is established. The SIP session is only used to transmit exactly one Large Message Mode CPM Standalone Message after which the SIP session is torn down. The CPM Standalone Message contents are contained in a CPIM wrapper as defined in [RFC3862].

## CPM Session

A CPM Session uses SIP session functionality as defined by [RFC3261] to exchange multimedia content as well as CPM Chat Messages between two or more CPM Principals or between CPM Principals and non-CPM Principals. A CPM Session can be established between two Principals as a CPM 1-1 Session or between several Principals as a CPM Group Session. In the latter case, a CPM Controlling Function acts as a conference focus as defined by [RFC4353].

A CPM 1-1 Session can be extended by one of the Participants to a CPM Group Session as described in section 7.3.3 “Extending a CPM 1-1 Session to a CPM Group Session”.

A CPM User can invite other CPM and non-CPM Principals to a CPM Session. The invitees are either members of a CPM Pre-defined Group or a number of ad-hoc recipients that the CPM User selects dynamically. A CPM Pre-defined Group is a “Shared Group” as described in [OMA-XDM-Group]. Ad-hoc recipients are carried as a 'recipient-list' body in the SIP INVITE request as described in [RFC5366].

The duration of a CPM Session depends on whether it is a CPM 1-1 Session or a CPM Group Session. In the case of a CPM 1-1 Session, the session is terminated when one of the two Participants leaves the session. In the case of a CPM Group Session for a CPM Ad-hoc Group, the session usually ends when the session initiator ends the session. In the case of a CPM Group Session for a CPM Pre-defined Group, the ending of the session depends on the conditions set in the policy of that group as described in [OMA-XDM-Group].  However, any CPM Group Session can be handled as CPM Long-lived Group Session. CPM Long-lived Group Session differ from CPM Group Session in the fact that CPM Session can be restarted by anyone of the Participants in the most recent Participants list.

Following notifications are provided for CPM Chat Messages:

- “isComposing”,

- delivery reports and,

- read reports notifications

to the CPM User’s device that has sent the CPM Chat Message.

The delivery reports and read reports are stored also as part of the Conversation History that includes the CPM Session.

The delivery notifications and read reports can be sent within the CPM Session, or outside of the CPM session if they are generated after the CPM Session was terminated.

The “isComposing” notifications are generated during the CPM Session according to the rules and procedures of [RFC3994]. A CPM Client receiving a MSRP SEND message holding an “isComposing” status SHOULD change the “isComposing” status of the sender according to the rules and procedures of [RFC 3994].

In a CPM Session, “isComposing” status SHALL be sent without CPIM wrapper and therefore no delivery and/or display notification shall be requested.

- The terminating CPM Participating Function SHALL always set the CPIM “DateTime” header value in the CPM Chat Messages it receives. The CPM Participating Function SHALL also set the CPIM “DateTime” header value and IMDN “DateTime” element in notifications. In both cases, the CPM Participating Function SHALL overwrite any “DateTime” information provided by the CPM Client. A CPM Client receiving these CPM Chat Messages SHOULD therefore rely on these headers fields to contain the correct time, and can for example use the provided information to ensure correct ordering of the sent and received CPM Chat Messages and CPM notifications (i.e. disposition notifications and/or “isComposing”). The CPM Client SHOULD perform any timezone conversions, if necessary.

## Comment D022

## CPM Conversation Identification

In order to support the concept of CPM Conversations and a threaded view of a CPM Conversation History, three new SIP headers have been defined:

- Conversation-ID: this is a header that identifies the CPM Conversation Identity that is associated with CPM Standalone Messages, CPM File Transfers and CPM Sessions. All CPM Standalone Messages, CPM File Transfers and CPM Sessions belonging to the same CPM Conversation carry the same value for the Conversation-ID header. All CPM notifications, such as delivery and read reports, that are sent for CPM Chat Messages after the CPM Session was terminated, SHALL carry the CPM Conversation Identity of the respective CPM Session.

- Contribution-ID: this is a header that identifies the CPM Contribution Identity of an individual CPM Standalone Message, CPM File Transfer or CPM Session and disposition notification that is part of a CPM Conversation.

- InReplyTo-Contribution-ID: this is a header that, in case of a reply to an earlier received CPM Standalone Message, CPM File Transfer or CPM Session, identifies the Contribution-ID of the CPM Standalone Message, CPM File Transfer or CPM Session that is being replied to.

Appendix C “CPM-defined SIP Headers” contains a formal definition of these three SIP headers.

These three SIP headers are end-to-end SIP headers. They SHALL NOT be modified or deleted by any intermediate node in any CPM communication.

The three headers SHALL be carried in Pager Mode CPM Standalone Messages and disposition notifications (realized over SIP MESSAGE request) and the session invitations of Large Message Mode CPM Standalone Messages, CPM File Transfers and CPM Sessions (SIP INVITE request). All CPM functional components SHALL support these SIP headers.

In addition the above identifications, the CPM Group Session uses CPM Group Session Identity, as described in [RFC4579] to identify the CPM Controller Function handling the CPM Group Session.