| # Enhanced Visual Call Object |  |  |  |
| --- | --- | --- | --- |
| Approved Version 1.0 – 21 Jun 2018 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-EVC_MO-V1_0-20180621-A |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DEVCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2018 Open Mobile Alliance All Rights Reserved.
Used with the permission of the Open Mobile Alliance under the terms set forth above.

Contents

1.	Scope	4

2.	References	5

2.1	Normative References	5

2.2	Informative References	5

3.	Terminology and Conventions	6

3.1	Conventions	6

3.2	Definitions	6

3.3	Abbreviations	6

4.	Introduction	7

4.1	Version 1.0	7

5.	OMA EVC Management Object	8

5.1	Management Object Tree	8

5.2	Management Object Parameters	8

Appendix A.	Change History (Informative)	11

A.1	Approved Version History	11

Figures

Figure 1: The OMA EVC Management Object tree	8

# Scope

This document defines the OMA Enhanced Visual Call (EVC) Management Object (MO). The MO is defined using the OMA Device Description Framework.

# References

## Normative References

| [OMA DM] |  | “OMA Device Management”, Open Mobile Alliance™, OMA-ERP-DM-V1_3, URL:http://www.openmobilealliance.org/ |  |
| --- | --- | --- | --- |
| [OMA EVC ER] |  |  | “OMA Enhanced Visual Call”, Open Mobile Alliance™, OMA-ER-EVC-V1_0, URL:http://www.openmobilealliance.org/ |  |
| [OMA XDM MO] |  | “OMA Management Object for XML Document Management”, Version 2.1, Open Mobile Alliance™, OMA-TS-XDM_MO-V2_1, URL:http://www.openmobilealliance.org/ |  |
| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |  |
| [RFC4234] |  | “Augmented BNF for Syntax Specifications: ABNF”. D. Crocker, Ed., P. Overell. October 2005, URL:http://www.ietf.org/rfc/rfc4234.txt |  |

## Informative References

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

| EVC Client | See [OMA EVC ER] |
| --- | --- |
| EVC Server | See [OMA EVC ER] |
| Management Object | See [OMA DM] |

## Abbreviations

| DM | Device Management |
| --- | --- |
| EVC | Enhanced Visual Call |
| MO | Management Object |
| OMA | Open Mobile Alliance |

# Introduction

This document describes the configuration parameters and associated management object syntax that allows configuration of OMA EVC Client(s).

## Version 1.0

The EVC MO Version 1.0 aims to provide the configuration parameters for the EVC Client.

The EVC MO Version 1.0 can be used for EVC 1.0 Enabler.

# OMA EVC Management Object

This subclause defines the Management Object (MO) for OMA EVC. The MO SHOULD be used for initial provisioning of parameters when the OMA DM is to be used, and the MO SHOULD be used for continuous provisioning, which allows the service provider to update any parameter defined in the MO tree for service configurations during service deployment  [OMA DM].

The OMA EVC Management Object consists of relevant parameters for configuration of EVC Client(s). It is defined using the OMA DM Device Description Framework as described in [OMA DM].

The Management Object Identifier is: urn:oma:mo:oma-EVC:1.0

Protocol compatibility: This MO is compatible with OMA DM 1.3 (see [OMA DM]).

Management object name: oma-EVC

## Management Object Tree

This section describes the Management Object tree (i.e. nodes and leaf objects) for provisioning of OMA EVC Client(s).

Figure 1 shows the interior nodes and leaf objects defined under the OMA-EVC node, which is used for configuration of EVC Clients.

Figure 1: The OMA EVC Management Object tree

## Management Object Parameters

This section describes the parameters for OMA EVC MO.

| …/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node represents the unique object id of an Enhanced Visual Call Management Object, or EVC MO. The purpose of this interior node is to group together the parameters of a single EVC MO object. The ancestor elements of this node define the position in the management tree of the EVC MO. The Management Object Identifier is: “urn:oma:mo:oma-EVC:1.0” |  |  |  |  |

| APPID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This parameter/leaf node provides an identifier for the application characteristics name for this application, to be used by DM Client to uniquely identify the application. |  |  |  |  |

| NAME |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This parameter/leaf node provides an application name which is displayed in equipment and is specific for each service provider. |  |  |  |  |

| PROVIDER-ID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This parameter/leaf node provides an identity of the EVC service provider provides an identifier for the application service access point described by an APPLICATION characteristic. |  |  |  |  |

| EVC-SERVER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node represents the information to connect the EVC Server, such as Hostname and port and release version. |  |  |  |  |

| EVC-SERVER /HostName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This parameter/leaf node indicates the host name of the EVC Server. |  |  |  |  |

| EVC-SERVER /Port |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This parameter/leaf node indicates the port number of the EVC Server. |  |  |  |  |

| EVC-SERVER /Version |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | This parameter/leaf node indicates the release number of the EVC Server. |  |  |  |  |

| EVC-XDMS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node represents a container for configuring EVC XDMS related properties. |  |  |  |  |

| EVC-XDMS/ToXDMRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This parameter/leaf node specifies a reference to XDM MO as defined in [XDM MO] that is used to configure the XDM Client associated with the EVC Client. The referenced XDM management object is either an existing instance on the client device or an instance specific to this EVC MO. The value SHALL be a valid reference to a XDM Management Object. |  |  |  |  |

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Tree Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | This interior node designates a branch of the <x> sub-tree into which vendor extensions MAY be added, permanently or dynamically. Ext sub tree provides flexible points of extension for vendor-specific parameters. However, vendor extensions MUST NOT be defined outside of Ext sub-tree. This elment can contain information about external content servers. |  |  |  |  |

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| OMA-TS-EVC_MO-V1_0-20180621-A | 21 Jun 2018 | Status changed to Approved by COM Doc Ref # OMA-COM-2018-0004-INP_EVC_V1_0_ERP_for_final_Approval |