Change Request

| Title: | Message_Storage_Client_removal | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA EVVM |  |
| Doc to Change: | OMA-ER-EVVM-V1_0-20110921-D |  |
| Submission Date: | September 27, 2011 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | jindrich.kubelik@t-mobile.cz |  |
| Replaces: | n/a |  |

Reason for Change

Because all the EVVM relevant IMAP4 communication is done only between the EVVM Client and the EVVM Server (Scenario A), the EVVM Enabler will not specify nor reuse any functionality the Message Storage Client (MSC). Therefore this CR recommends removing the Message Storage Client (MSC) from Architectural Diagram and relevant sections of ER.

Impact on Backward Compatibility

None.

Impact on Other Specifications

None.

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Review and agree the proposed changes.

Detailed Change Proposal

Changes in chapter 6.2

## 6.2 Architectural Diagram

Editor’s note: if the CPM-MSG is found to be sufficient to support all requirements this interface and corresponding components will be changed to dash line.

Figure 2: OMA EVVM logical architecture

Figure 2 presents the EVVM logical architecture model, depicting the EVVM Enabler’s functional components and their interactions with each other and with external entities such as the Supporting Enablers and the Remote EVVM Server.

The EVVM Enabler consists of the following EVVM functional components:

- The EVVM Client, which resides in the user’s Device and allows the EVVM User to use the EVVM-based services by interacting with the EVVM Server and the components in the Device.

- The EVVM Server, which resides in the network’s Server Domain and interacts with the EVVM Client and other network components such as the Forwarding Gateway.

- The Message Storage Server (MSS) as defined in [OMA-CPM] provides storage functions for EVVM user’s voicemails, personalised greetings and voice signatures including their attachments (e.g., audio, display of logos, advertisements).

  - Note: When the EVVM Server provides storage only internally, the Message Storage Server and the CPM-MSG interface exposed by the MSS are omitted.

- The Forwarding Gateway, which forwards voicemails to external non-VM services such as Email, MMS and SMS.

- The Transcoder, which resides in the network’s Server Domain and serves transcoding requests from other  EVVM components.

- The Telephony User Interface (TUI), which allows the user to access his/her EVVM VM box via traditional telephony systems.

The EVVM functional components, if and when needed, interact with the following external functional components:

- The following supporting Enablers, which are other OMA Enablers used to support the EVVM Enabler:

  - The CPM Enabler (MSS), which provides external message storage capabilities to support the VM SS functionalities.

- The XDM Enabler (XDM Client), which provides XML document management capabilities to support, creating, storing and managing EVVM user preferences, group lists for greetings and black-list for blocking incoming voicemails.The Remote EVVM Server, which is another EVVM server residing in another (remote) server domain.

- The Push Enabler (Push Client and Push Proxy Gateway), which provides push notifications to support informing multiple devices about server-side events..

- The DM Enabler (DM Client and DM Server), which provides device management to support remote management of EVVM-specific device parameters.

- The Remote EVVM Server, which is another EVVM server residing in another (remote) server domain.

- The Underlying Network Infrastructure, which provides Telephony-based and IP-based functionalities that are needed for the EVVM Enabler to provide the required services.

- Figure 3 shows the details of the interactions between the functional components of the EVVM Enabler and the supporting Enablers. A more detailed informational figure is available in.

Figure 3 - Interactions between the EVVM Enabler functional components and the supporting Enablers.

Editor’s Notes:

the CPM-MSG line may later be changed to a dash line, similarly to Figure 2.

Changes in chapter 6.3