Change Request

| Title: | CR_TS_Control_Plane_RFC3265_Updates_Pt1a | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA-PCPS-TS_Control_Plane-V1_0-20140525-D |  |
| Submission Date: | 27 July 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | n/a |  |

Reason for Change

This change request updates the Control Plane TS with regard to references to obsolete RFC3265.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Updating obsolete normative reference RFC3265 to replacement RFC6665

From section 2.1, Normative References:

2.1	Normative References

.

.

| [RFC5373] | IETF RFC 5373 (November 2008): "Requesting Answering Modes for the Session Initiation Protocol (SIP)", URL: http://www.ietf.org/rfc/rfc5373.txt |
| --- | --- |
| [RFC5627] | IETF RFC 5627 (October 2009): "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)", URL: http://www.ietf.org/rfc/rfc5627.txt |

.

.

From 3.2, definitions:

3.2	Definitions

.

.

| Full Duplex Call Follow-on Proceed | A feature which allows PoC Session Participant to request the other PoC Session Participants to set up another independent full duplex voice call (either a circuit switched voice call or voice-over-IP call, subject to Service Provider Policy and configuration). The full duplex voice call set up is out of the scope of this specification. |
| --- | --- |
| Globally Routable User Agent URI | From RFC5627: “... a unique useragent identifier which is still globally routable. This identifier is called a Globally Routable User Agent (UA) URI (GRUU). |

.

.

From 3.3, Abbreviations:

3.3	Abbreviations

.

.

| FDCFO | Full Duplex Call Follow-On Proceed |
| --- | --- |
| GRUU | Globally Routable User Agent URI |

.

.

From 6.1.3.2.2, Pg. 63:

Upon receiving a request from a PoC User to establish an Ad-hoc PoC Group Session or a 1-1 PoC Session within a Pre-established Session the PoC Client:

1. SHALL generate an initial SIP REFER request as specified in subclause 6.1.3.1 "General" and according to rules and procedures of [RFC3515], if the SIP REFER request will be sent in a new dialog; otherwise generate a SIP REFER request according to rules and procedures of [RFC3515];

2. SHALL set the Request-URI of the SIP REFER request to the conference URI that identifies the Pre-established Session;

3. SHALL include the PoC feature tag '+g.poc.discretemedia' in the Contact header, if Discrete Media is supported and acceptable to the PoC User during the PoC Session, and if it was included in the Pre-established Session;

4. SHALL in case of a 1-1 PoC Session either set the Refer-To header of the SIP REFER request to the PoC Address of the Invited PoC User according to rules and procedures of [RFC3515] and continue with the following steps; or continue in the step 5;

- a) MAY in case of a 1-1 PoC Session and if the PoC Address is included in the Refer-To header (as specified in the step 4 above) include a URI Usage Type uri-parameter according to subclause E.5.4 "URI Usage Type uri-parameter"; and,

- b) skip the step 5.

5. SHALL either modify the SIP REFER request according to rules and procedures of [RFC5368] with the following clarifications:

- a) include in the Refer-To header of the SIP REFER request a URL identifying the body part containing the MIME resource-lists body with the Invited PoC Users;

- b) include a MIME resource-lists body according to rules and procedures of [RFC5368] with the list of the Invited PoC Users and invited Pre-arranged PoC Groups. 
The PoC Client MAY, for each URI in the list,

- i. set the "copyControl" attribute to 'to', and set the "anonymize" attribute to 'true' if the URI is requested to be anonymous and not to be presented to Invited PoC Users, according to rules and procedures of [RFC5364]; and,

- ii. include a URI Usage Type uri-parameter according to subclause E.5.4 "URI Usage Type uri-parameter".

- and,

- include option tag 'multiple-refer' to the Require header.

- or, modify the SIP REFER request as follows:

- include in the Refer-To header of the SIP REFER request a URL identifying a body part containing a MIME "application/resource-lists-bitmap" body with the Invited PoC Users;

- include a MIME "application/resource-lists-bitmap" body, as defined in Annex E.6.2 "Bit Map", with the Bit Map based list of Invited PoC Users; and,

- include option tag 'multiple-refer' to the Require header.

6. SHALL include the following according to rules and procedures of [RFC4488], when more than one PoC User is invited:

- a) the option tag 'norefersub' in the Require header; and,

- b) the value 'false' in the Refer-Sub header.

7. MAY include the following according to rules and procedures of [RFC4488], when only one PoC User is invited:

- a) the option tag 'norefersub' in the Require header; and,

- b) the value 'false' in the Refer-Sub header.

8. SHALL include value 'id' in the Privacy header according to rules and procedures of [RFC3325], if anonymity is requested;

9. SHALL include in the Refer-To URI a Priv-Answer-Mode header with the value 'Auto' according to rules and procedures of [RFC5373], in case the PoC User has requested manual answer override;

10. SHALL include in the Refer-To URI an Answer-Mode header with the value 'Manual;Require' according to rules and procedures of [RFC5373], in case the PoC User has requested that Manual Answer Mode be required at the Invited PoC Client;

11. SHALL include in the Refer-To URI a Reject-Contact header with the feature tags 'sip.automata' and 'sip.actor' with the value of 'msg-taker' along with 'require' and 'explicit' and another Reject-Contact header the feature tags  'sip.automata' and 'sip.actor' with the value of 'principal' and 'sip.description' with the value "poc recording device" along with 'require' and 'explicit', if the PoC Client doesn't want to be routed to a PoC Box;

12. SHALL include in the Refer-To URI an Accept-Contact header the feature tags '+g.poc.talkburst', 'sip.automata' and 'sip.actor' with the value of 'msg-taker' along with 'require' and 'explicit' and another Accept-Contact header the feature tags '+g.poc.talkburst', 'sip.automata' and 'sip.actor' with the value of 'principal' and 'sip.description' with the value "poc recording device" along with 'require' and 'explicit', if the PoC User explicitly requests that only a PoC Box is to accept the invitation, otherwise SHALL include in the Refer-To URI an Accept-Contact header with the PoC feature tag '+g.poc.talkburst' along with 'require' and 'explicit' parameters according to rules and procedures of [RFC3841];

NOTE 1:	The handling of any other non-PoC specific feature tags in Accept-Contact headers or Reject-Contact headers is specified in the subclause 5.13 "Handling of non-PoC specific feature tags".

13. SHOULD include a Resource-Priority header according to rules and procedures of [RFC4412], if the QoE Profile assigned to the Pre-established Session is 'Official Government Use' QoE Profile. If included, the value of the Resource-Priority header SHALL be equal to the level of priority assigned to the PoC User, as specified in subclause 5.8 "QoE Profiles"; the Resource-Priority header is included as a header of the SIP REFER request as well as a Refer-to uri parameter;

NOTE 2:	When using Pre-established Sessions, QoE Profile assigned to the Pre-established Session is the same than the Local QoE Profile assigned to the PoC User  for any established PoC Session.

- 14. MAY include media content in one or more MIME bodies as specified in [RFC2046] with a total size equal to or less than the maximum size indicated in "INCLUDED-MEDIA-CONTENT-SIZE". For each included media content, the PoC Client

NOTE 3:	If "INCLUDED-MEDIA-CONTENT-SIZE" is not provisioned or if "INCLUDED-MEDIA-CONTENT-SIZE" is provisioned and set to zero, a MIME body with a Media Type specified in [RFC2046] can not be included.

- a) SHALL include a Content-Disposition header as specified in [RFC2046]  with  disposition type set to "render", "attachment", "icon" or "alert" depending on the purpose of the Included Media Content and with "handling" parameter set to "handling=optional".

NOTE 4:	The disposition type "icon" indicates that the body part contains an image suitable as an iconic representation of the Inviting PoC User. The value "alert" indicates that the MIME body contains information, such as an audio clip, that should be rendered by the PoC Client in an attempt to alert the PoC User when receiving the invitation to the PoC Session.

- 15. MAY include text content in Subject header in the URI of the Refer-To header according to rules and procedures of [RFC3261] with a total size equal to or less than the maximum size indicated in "INCLUDED-TEXT-CONTENT-SIZE";

NOTE 5:	If "INCLUDED-TEXT-CONTENT-SIZE" is not provisioned or if "INCLUDED-TEXT-CONTENT-SIZE" is provisioned and set to zero text content as specified in [RFC3261] can not be included.

- 16. MAY include a reference to media content in the Alert-Info header or in the Call-Info header or both in the URI of the Refer-To header  according to rules and procedures in [RFC3261];

- 17. MAY in case of an Ad-hoc PoC Group Session and if Dynamic PoC Groups is supported:

- a) include in the SIP REFER request a Content-Type header with the value "multipart/mixed" as specified in [RFC2046];

- b) include in the SIP REFER request a MIME body containing Dynamic PoC Group rules as specified in [draft-cbus-event]; and,

- c) include in the SIP REFER request Condition Re-evaluation parameters as specified in [draft-cbus-event], if Condition Re-evaluation is supported by the PoC Client.

18. SHALL, if the conference URI is a GRUU, include a Target-Dialog header as specified in [RFC4538] identifying the Pre-established Session.

19. SHALL, if the conference URI is a GRUU, send the SIP REFER request towards the PoC Server in a new SIP dialog according to rules and procedures of the SIP/IP Core; otherwise, send the SIP REFER request towards the PoC Server within the SIP dialog of the Pre-established Session according to rules and procedures of the SIP/IP Core.

NOTE 6:	If PoC User wants to cancel the PoC Session initiation, the PoC Client can send SIP BYE request and release the Pre-established Session as specified in subclause 6.1.3.2.4 "PoC Client releases a Pre-established Session" and re-establish the Pre-established Session as specified in 6.1.3.2.1 "PoC Client initiates a Pre-established Session".

Upon receiving a SIP 2xx final response to the SIP REFER request the PoC Client SHALL interact with the User Plane as specified in [OMA-PoC-UP] "PoC Client procedures at PoC Session initialization".

Upon receiving an incoming SIP NOTIFY request that is part of the same dialog as the previously sent SIP REFER request the PoC Client:

1. SHALL handle the request according to rules and procedures of [RFC3515] and [RFC6665];

2. MAY display information to the PoC User based on the information in the SIP NOTIFY body; and,

NOTE 7:	The PoC Server does not send any SIP NOTIFY request in case the value 'false' was included in the Refer-Sub header of the SIP REFER request.

3. SHOULD inform the PoC Server performing the Controlling PoC Function, as specified in section 6.1.4.4 "User Plane Adaptation" of the achieved QoE Profile if a QoE Profile was negotiated and if the PoC Client is unable to obtain that negotiated QoE Profile according to the parameters provisioned to the PoC Client, as specified in subclause 5.8 "QoE Profiles".

In addition to this, the PoC Client MAY subscribe to the conference state event package as specified in subclause 6.1.10 "PoC Client subscription to the conference state event package".

.

.

From 6.1.3.2.3, pg. 64:

Upon receiving a request from a PoC User to establish a Pre-arranged or Chat PoC Group Session within the Pre-established Session the PoC Client:

1. SHALL generate an initial SIP REFER request as specified in subclause 6.1.3.1 "General" and according to rules and procedures of [RFC3515], if the SIP REFER request will be sent in a new dialog; otherwise generate a SIP REFER request according to rules and procedures of [RFC3515];

2. SHALL set the Request URI of the SIP REFER request to the conference URI that identifies the Pre-established Session;

3. SHALL include the PoC feature tag '+g.poc.discretemedia' in the Contact header, if Discrete Media is supported and acceptable to the PoC User during the PoC Session, and if it was included in the Pre-established Session;

4. SHALL set the Refer-To header of the SIP REFER request to the URI of the Pre-arranged PoC Group or Chat PoC Group according to rules and procedures of [RFC3515] and include Session Type uri-parameter "session=prearranged" or "session=chat", respectively as specified in E.5.1 "Session Type uri-parameter";

5. SHALL, if the PoC Dispatcher capability is supported and if the PoC User requested to initiate a Dispatch PoC Session as PoC Dispatcher, proceed as specified in subclause 6.1.3.2.3.1 "Dispatch PoC Session initiation as PoC Dispatcher" before continuing with the rest of the steps;

6. MAY include the following according to rules and procedures of [RFC4488]:

- a) the option tag 'norefersub' in the Require header; and,

- b) the value 'false' in the Refer-Sub header.

7. SHALL include in the Refer-To URI a Priv-Answer-Mode header with the value 'Auto' according to rules and procedures of [RFC5373], in case the PoC User has requested manual answer override;

8. SHALL include in the Refer-To URI an Answer-Mode header with the value 'Manual;Require' according to rules and procedures of [RFC5373], in case the PoC User has requested that Manual Answer Mode be required at the Invited PoC Client;

9. SHALL include in the Refer-To URI a Reject-Contact header with the feature tags 'sip.automata' and 'sip.actor' with the value of 'msg-taker' along with 'require' and 'explicit' and in another Reject-Contact header the feature tags 'sip.automata' and 'sip.actor' with the value of 'principal' and 'sip.description' with the value "poc recording device" along with 'require' and 'explicit' if the PoC Client doesn't want to be routed to a PoC Box;

10. SHALL include in the Refer-To URI in an Accept-Contact header the feature tags '+g.poc.talkburst', 'sip.automata' and 'sip.actor' with the value of 'msg-taker' along with 'require' and 'explicit' and in another Accept-Contact header the feature tags '+g.poc.talkburst', 'sip.automata' and 'sip.actor' with the value of 'principal' and 'sip.description' with the value "poc recording device" along with 'require' and 'explicit' if the PoC User explicitly requests that only a PoC Box is to accept the invitation, otherwise SHALL include in the Refer-To URI an Accept-Contact header with the PoC feature tag '+g.poc.talkburst' along with 'require' and 'explicit' parameters according to rules and procedures of [RFC3841];

NOTE 1:	The handling of any other non-PoC specific feature tags in Accept-Contact headers or Reject-Contact headers is specified in the subclause 5.13 "Handling of non-PoC specific feature tags".

11. SHALL include value 'id' in the Privacy header according to rules and procedures of [RFC3325], if anonymity is requested;

12. SHOULD include a Resource-Priority header according to rules and procedures of [RFC4412], if the QoE Profile assigned to the Pre-established Session is 'Official Government Use' QoE Profile. If included, the value of the Resource-Priority header SHALL be equal to the level of priority assigned to the PoC User, as specified in subclause 5.8 "QoE Profiles"; the Resource-Priority header is included as a header of the SIP REFER request as well as a Refer-to URI parameter;

- 13. MAY include media content in one or more MIME bodies as specified in [RFC2046] with a total size equal to or less than the maximum size indicated in "INCLUDED-MEDIA-CONTENT-SIZE". For each included media content, the PoC Client:

NOTE 2:	If "INCLUDED-MEDIA-CONTENT-SIZE" is not provisioned or if "INCLUDED-MEDIA-CONTENT-SIZE" is provisioned and set to zero, a MIME body with a Media Type specified in [RFC2046] can not be included.

- a) SHALL include a Content-Disposition header as specified in [RFC2046]  set to "render", "attachment", "icon" or "alert depending on the purpose of the Included Media Content and with "handling" parameter set to "handling=optional".

NOTE 3:	The disposition type "icon" indicates that the body part contains an image suitable as an iconic representation of the Inviting PoC User. The value "alert" indicates that the MIME body contains information, such as an audio clip, that should be rendered by the PoC Client in an attempt to alert the PoC User when receiving the invitation to the PoC Session.

- 14. MAY include text content in Subject header in the URI of the Refer-To header according to rules and procedures of [RFC3261] with a total size equal to or less than the maximum size indicated in "INCLUDED-TEXT-CONTENT-SIZE";

NOTE 4:	If "INCLUDED-TEXT-CONTENT-SIZE" is not provisioned or if "INCLUDED-TEXT-CONTENT-SIZE" is provisioned and set to zero text content as specified in [RFC3261] can not be included.

- 15. MAY include a reference to media content in the Alert-Info header or in the Call-Info header or both in the URI of the Refer-To header according to rules and procedures of [RFC3261];

NOTE 5:	Included Media Content is not applicable, when establishing a Chat PoC Group Session.

- 16. MAY, in case of an Pre-arranged PoC Group Session and if Dynamic PoC Groups is supported:

- include in the SIP REFER request a Content-Type header with the value "multipart/mixed" as specified in [RFC2046];

- include in the SIP REFER request a MIME body containing Dynamic PoC Group rules as specified in [draft-cbus-event]; and,

- include in the SIP REFER request Condition Re-evaluation parameters as specified in [draft-cbus-event], if Condition Re-evaluation is supported by the PoC Client.

17. SHALL, if the conference URI is a GRUU, include a Target-Dialog header as specified in [RFC4538] identifying the Pre-established Session.

18. SHALL, if the conference URI is a GRUU, send the SIP REFER request towards the PoC Server in a new SIP dialog according to rules and procedures of the SIP/IP Core; otherwise, send the SIP REFER request towards the PoC Server within the SIP dialog of the Pre-established Session according to rules and procedures of the SIP/IP Core.

Upon receiving a SIP 2xx final response to the SIP REFER request the PoC Client SHALL interact with the User Plane as specified in [OMA-PoC-UP] "PoC Client procedures at PoC Session initialization".

NOTE 6:	If PoC User wants to cancel the PoC Session initiation, the PoC Client can send SIP BYE request and release the Pre-established Session as specified in subclause 6.1.3.2.4 "PoC Client releases a Pre-established Session" and re-establish the Pre-established Session as specified in 6.1.3.2.1 "PoC Client initiates a Pre-established Session".

Upon receiving an incoming SIP NOTIFY request that is part of the same dialog as the previously sent SIP REFER request the PoC Client:

1. SHALL handle the request according to rules and procedures of [RFC3515] and [RFC6665];

2. MAY display information to the PoC User based on the information in the SIP NOTIFY body; and,

3. SHOULD inform the PoC Server performing the Controlling PoC Function, as specified in section 6.1.4.4 "User Plane Adaptation" of the achieved QoE Profile if a QoE Profile was negotiated and if the PoC Client is unable to obtain that negotiated QoE Profile according to the parameters provisioned to the PoC Client, as specified in subclause 5.8 "QoE Profiles".

In addition to this, the PoC Client MAY subscribe to the conference state event package as specified in subclause 6.1.10 "PoC Client subscription to the conference state event package".

.

.

- From 6.1.5.2, Pg. 77:

- .

- .

Upon receiving a request from a PoC User to re-join a PoC Session within a Pre-established Session that has been established as specified in subclause 6.1.3.2.1 "PoC Client initiates a Pre-established Session", the PoC Client:

1. SHALL generate an initial SIP REFER request as specified in subclause 6.1.3.1 "General" and according to rules and procedures of [RFC3515], if the SIP REFER request will be sent in a new dialog; otherwise generate a SIP REFER request according to rules and procedures of [RFC3515];

2. SHALL set the Request-URI of the SIP REFER request to the conference URI that identifies the Pre-established Session;

3. SHALL include the PoC feature tag '+g.poc.discretemedia' in the Contact header, if Discrete Media is supported and acceptable to the PoC User during the PoC Session, and if it was included in the Pre-established Session;

4. SHALL set the Refer-To header of the SIP REFER request to the PoC Session Identity of the PoC Session to be re-joined with the Session Type uri-parameter indicating the PoC Session type, e.g. "session=prearranged", "session=chat" or "session=adhoc" as defined in E.5.1 "Session Type uri-parameter";

NOTE:	The PoC Session Identity of the PoC Session to be re-joined may have been received in the TBCP Connect message.

5. MAY include the following according to rules and procedures of [RFC4488]:

- a) the option tag 'norefersub' in the Require header; and,

- b) the value 'false' in the Refer-Sub header.

6. SHALL include value 'id' in the Privacy header according to rules and procedures of [RFC3325] , if anonymity is requested;

7. SHOULD include a Resource-Priority header according to rules and procedures of [RFC4412], if the QoE Profile assigned to the Pre-established Session is 'Official Government Use' QoE Profile. If included, the value of the Resource-Priority header SHALL be equal to the level of priority assigned to the PoC User, as specified in subclause 5.8 "QoE Profiles"; the Resource-Priority header is included as a header of the SIP REFER request as well as a Refer-to URI parameter; and,

9. SHALL, if the conference URI is a GRUU, include a Target-Dialog header as specified in [RFC4538] identifying the Pre-established Session.

10. SHALL, if the conference URI is a GRUU, send the SIP REFER request towards the PoC Server in a new SIP dialog according to rules and procedures of the SIP/IP Core; otherwise, send the SIP REFER request towards the PoC Server via SIP/IP Core within the SIP dialog of the Pre-established Session according to rules and procedures of the SIP/IP Core.

Upon receiving a SIP 2xx response to the SIP REFER request the PoC Client SHALL interact with the User Plane as specified in the [OMA-PoC-UP] "PoC Client procedures at PoC Session initialization".

Upon receiving a SIP NOTIFY request that is part of the same dialog as the previously sent SIP REFER request the PoC Client:

1. SHALL handle the request according to rules and procedures of [RFC3515] and [RFC6665]; and,

2. MAY display information to the PoC User based on the information in the SIP NOTIFY body.

In addition to this, the PoC Client MAY subscribe to the conference state event package as specified in subclause 6.1.10 "PoC Client subscription to the conference state event package".

- When the SIP/IP Core corresponds to 3GPP/3GPP2 IMS, the PoC Client SHALL use 3GPP/3GPP2 IMS mechanisms according to rules and procedures of [3GPP TS 24.229] / [3GPP2 X.S0013.4] with the clarifications given in this subclause.

- .

- .

- From 6.1.6.2 “Leaving a PoC Session – Pre-established Session case”, pg. 78-79:

.

.

Upon receiving a request from a PoC User to leave a PoC Session, the PoC Client:

1. SHALL interact with the User Plane as specified in [OMA-PoC-UP] "PoC Client procedures at PoC Session release";

2. SHALL generate an initial SIP REFER request as specified in subclause 6.1.3.1 "General" and according to rules and procedures of [RFC3515], if the SIP REFER request will be sent in a new dialog; otherwise generate a SIP REFER request according to rules and procedures of [RFC3515];

3. SHALL set the Request-URI of the SIP REFER request to the conference URI that identifies the Pre-established Session;

4. SHALL set the Refer-To header of the SIP REFER request to the PoC Session Identity to leave;

NOTE 1:	The PoC Session Identity of the PoC Session to leave may have been received in the TBCP Connect message or in the SIP NOTIFY request associated with the SIP REFER request.

5. MAY include the following according to rules and procedures of [RFC4488]:

- a) the option tag 'norefersub' in the Require header; and,

- b) the value 'false' in the Refer-Sub header.

6. SHALL include the "method" parameter with the value "BYE" in the Refer-To header;

7. SHALL include value 'id' in the Privacy header according to rules and procedures of [RFC3325], if anonymity is requested;

8. SHOULD include a Resource-Priority header according to rules and procedures of [RFC4412], if the PoC Client is allowed to use the 'Official Government Use' QoE Profile and the PoC User requests that QoE Profile. If included, the value of the Resource-Priority header SHALL be equal to the level of priority assigned to the PoC User, as specified in subclause 5.8 "QoE Profiles"; the Resource-Priority header is included as a header of the REFER request as well as a Refer-to URI parameter; and,

9. SHALL, if the conference URI is a GRUU, include a Target-Dialog header as specified in [RFC4538] identifying the Pre-established Session.

10. SHALL, if the conference URI is a GRUU, send the SIP REFER request towards the PoC Server in a new SIP dialog according to rules and procedures of the SIP/IP Core; otherwise, send the SIP REFER request towards the PoC Server via SIP/IP Core within the SIP dialog of the Pre-established Session according to rules and procedures of the SIP/IP Core.

Upon receiving a SIP 2xx response to the SIP REFER request, the PoC Client:

1. SHALL handle it according to rules and procedures of [RFC3515] and [RFC6665]; and,

2. SHALL interact with the User Plane as specified in [OMA-PoC-UP] "PoC Client procedures at PoC Session release".

NOTE 2: 	If the Media Streams of the Pre-established Session were modified by the PoC Session initiation or a PoC Session modification, the PoC Client can initiate the Pre-established Session modification in 6.1.4.8 "Pre-established Session modification" to restore the Media Streams used before the association of the PoC Session with the Pre-established Session.

Upon receiving an incoming SIP NOTIFY request that is part of the same dialog as the previously sent SIP REFER request the PoC Client:

1. SHALL handle the request according to rules and procedures of [RFC3515] and [RFC6665]; and,

2. MAY display information to the PoC User based on the information in the SIP NOTIFY body.

NOTE 3:	Depending on the release policy described in subclause 7.2.1.16 "PoC Session release policy" this procedure either removes the release initiator from the PoC Session or releases the whole PoC Session.

.

.

From 6.1.7, pg. 80:

Upon receiving a request from the PoC User to add one or more PoC Users to an ongoing PoC Session, the PoC Client:

1. SHALL generate an initial SIP REFER request as specified in subclause 6.1.3.1 "General" and according to rules and procedures of [RFC3515], if the SIP REFER request will be sent in a new dialog; otherwise generate a SIP REFER request according to rules and procedures of [RFC3515];

2. SHALL set the Request-URI of the SIP REFER request to the PoC Session Identity of an ongoing PoC Session;

3. SHALL include an Accept-Contact header with the PoC feature tag '+g.poc.talkburst' along with 'require' and 'explicit' parameters according to rules and procedures of [RFC3841] if the SIP REFER request will be sent in a new dialog ;

NOTE 1:	The handling of any other non-PoC specific feature tags in Accept-Contact headers or Reject-Contact headers is specified in the subclause 5.13 "Handling of non-PoC specific feature tags".

NOTE 2:	If the ongoing PoC Session is established using Pre-established Session but is not controlled by the same PoC Server, which hosts the Pre-established Session, the PoC Session Identity of the on-going PoC Session has been received in the  MBCP Connect message.

4. SHALL perform the following actions, if only one PoC User is invited, either

- a) set the Refer-To header of the SIP REFER request to the PoC Address of the Invited PoC User according to rules and procedures of [RFC3515] if the "copyControl" and the "anonymize" attributes are not used, and skip the next step; or

- b) continue to the next step.

NOTE 3:	If a PoC User wants to add a PoC User using a TEL URI the PoC Client can convert TEL URI to SIP URI according to [RFC3261], if needed.

5. SHALL perform the following actions:

- a) include a Refer-To header with a content-ID URL pointing to an URI-list in a body part containing the MIME resource-lists body according to rules and procedures of [RFC5368];

- b) include a MIME resource-lists body with the list of the PoC Users to be added according to rules and procedures of [RFC5368];

- c) optionally, for each URI in the list, set the "copyControl" attribute to 'to', and set the "anonymize" attribute to 'true' if the URI is requested to be anonymous and not to be presented to Invited PoC Users, according to rules and procedures of [RFC5364]; and,

- d) include option tag 'multiple-refer' to the Require header according to rules and procedures of [RFC5368].

6. SHALL include the following according to rules and procedures of [RFC4488], when more than one PoC User is added:

- a) the option tag 'norefersub' in the Require header; and,

- b) the value 'false' in the Refer-Sub header.

7. MAY include the following according to rules and procedures of [RFC4488], when only one PoC User is added:

- a) the option tag 'norefersub' in the Require header; and,

- b) the value 'false' in the Refer-Sub header.

8. SHALL include value 'id' in a Privacy header according to rules and procedures of [RFC3325], if anonymity is requested;

NOTE 4:	If anonymity is not allowed for the PoC Group indicated with the Request-URI of the SIP REFER request based on rules specified in the [OMA-PoC-Document-Mgmt] the PoC User(s) will not be added to the PoC Session by the PoC Server hosting the PoC Group.

9. SHALL include in the Refer-To URI a Priv-Answer-Mode header with the value 'Auto' according to rules and procedures of [RFC5373] if the PoC User has requested manual answer override; NOTE 4: The Refer-To URI can be a PoC Address or a content-ID URL.

10. SHALL include in the Refer-To URI an Answer-Mode header with the value 'Manual;Require' according to rules and procedures of [RFC5373] if the PoC User has requested that Manual Answer Mode be required at the Invited PoC Client;

11. SHOULD include a Resource-Priority header according to rules and procedures of [RFC4412], if the PoC Client is using 'Official Government Use' as the Local QoE Profile for the on-going PoC Session. If included, the value of the Resource-Priority header SHALL be equal to the level of priority assigned to the PoC User, as specified in subclause 5.8 "QoE Profiles"; the Resource-Priority header is included as a header of the REFER request as well as a Refer-to URI parameter; and,

12. SHALL include a Target-Dialog header as specified in [RFC4538] identifying the existing dialog from which the PoC Session Identity of the ongoing PoC Session was received in the Contact header of the SIP final response, if such a dialog exists and the PoC Session Identity is a GRUU.

13. SHALL send the SIP REFER request towards the PoC Server within the existing dialog from which the PoC Session Identity of the ongoing PoC Session was received in the Contact header of the SIP final response if such a dialog exists and the PoC Session Identity is not a GRUU; otherwise, send the SIP REFER request towards the PoC Server using a new SIP dialog according to rules and procedures of the SIP/IP Core.

NOTE 5:	The SIP REFER request is sent using a new SIP dialog in case of Pre-established Session if the PoC Session Identity is different from Pre-established Session identity.

Upon receiving an incoming SIP NOTIFY request that is part of the same dialog as the previously sent SIP REFER request the PoC Client:

1. SHALL handle the request according to rules and procedures of [RFC3515] and [RFC6665]; and,

2. MAY display information to the PoC User based on the information in the SIP NOTIFY body.

NOTE 6:	The PoC Server does not send any SIP NOTIFY request if the PoC Client inserted the Refer-Sub header with value 'false' in the SIP REFER request.