Change Request

| Title: | OMA-COM-PCPS-2014-0161-CR_TS_Multicast_Section_5_6_Mcast_Channel | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA-PCPS-TS_Multicast-V1_0-20140817-D |  |
| Submission Date: | 25 August 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | n/a |  |

Reason for Change

Portions of the Multicast TS are specific to 3GPP UTRAN. For PCPS 1.0, updates are required to adapt this specification for 3GPP eUTRAN. This CR updates the description of 5.6, “Sending and receiving Media over the Multicast PoC Channel” accordingly, as well as other supporting material such as References, Abbreviations and Definitions.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Updated the Multicast TS section 5.6 and supporting material for eUTRAN transport

# References

## Normative References

.

.

.

| [3GPP TS 23.003] | 3GPP TS 23.003 "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Numbering, addressing and identification", URL: http://www.3gpp.org/ |
| --- | --- |
| [3GPP TS 23.246] | 3GPP TS 23.246 "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description". URL: http://www.3gpp.org/ |
| [3GPP TS 23.401] | 3GPP TS 23.401 “3rd Generation Partnership Project;Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network”. (E-UTRAN) access URL: http://www.3gpp.org/ |
| [3GPP TS 24.008] | 3GPP TS 24.008 "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Mobile radio interface Layer 3 specification; Core network protocols; Stage 3". URL: http://www.3gpp.org/ |
| [3GPP TS 25.331] | 3GPP TS 25.331 "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Radio Resource Control (RRC); Protocol Specification". URL: http://www.3gpp.org/ |
| [3GPP TS 25.413] | 3GPP TS 25.413 "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; UTRAN Iu interface RANAP signaling". URL: http://www.3gpp.org/ |
| [3GPP TS 26.346] | 3GPP TS 26.346, "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs", 3rd Generation Partnership Project (3GPP). URL: http://www.3gpp.org/ |
| [3GPP TS 29.061] | 3GPP TS 29.061 "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)". URL: http://www.3gpp.org/ |
| [3GPP TS 29.281] | 3GPP TS 29.281 "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)". URL: http://www.3gpp.org/ |
| [3GPP TS 33.220] | 3GPP TS 33.220 "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Generic Authentication Architecture (GAA); Generic bootstrapping architecture", URL: http://www.3gpp.org/ |
| [3GPP TS 33.246] | 3GPP TS 33.246 "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Security of Multimedia Broadcast/Multicast Service (MBMS)", URL: http://www.3gpp.org/ |

.

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

For the purposes of the PoC specifications, the terms and definitions given in [OMADICT] and the following terms and definitions apply.

| Broadcast Multicast Service Centre | A function in the MBMS providing the functionality for interacting with a Multicast Access Network for MBMS user service provisioning and delivery as defined in [3GPP TS 23.246]. |
| --- | --- |
| Common Terminal Endpoint Identifier | Identifies the endpoint of a tunnel in the receiving GTP-U entity for a specific UDP/IP endpoint (see [3gpp TS 29.281]). |
| Continuous Media | Media with an inherent notion of time (e.g., PoC Speech, audio, and video). |
| Control Plane | The specification of the signalling between PoC Client and PoC Server, between PoC Box and PoC Server and between PoC Servers for the Push to talk over Cellular (PoC) service. |
| Core Network | An architectural term relating to the part of 3GPP/3GPP2 System which is independent of the connection technology of the terminal. |
| Discrete Media | Media that itself does not contain an element of time (e.g. images, text). |
| File Delivery Table | See, [BCAST Distribution] |
| Forward Error Correction | See, [BCAST Distribution] |
| Home PoC Network | A network comprising of a Home PoC Server and SIP/IP Core operated by the PoC User's PoC Service Provider. The Home PoC Network is the same as the Home Network defined in 3GPP/3GPP2 IMS specifications. |
| Home PoC Server | The PoC Server of the PoC Service Provider that provides PoC service to the PoC User. |
| MBMS Client | A function in the PoC Client capable of interacting with a Multicast Access Network based on the MBMS technology. |
| MBMS Service Area Identity | An identifier that identifies a group of cells within a PLMN independent of the associated Location/Routing/Service Areas and the physical location of the cell(s) [3GPP TS 23.003]. |
| Media Burst Control | Media Burst Control is a control mechanism that arbitrates requests from the PoC Clients, for the right to send Media and Multimedia. |
| Media Parameters | Media Parameters are SIP/SDP based information exchanged between the PoC Server and the PoC Client specify the characteristics of the Media for a PoC Session being established or that already exists. |
| Media Streaming Control | A Media Type allowing Participants to control a Continuous Media stream using RTSP in a PoC Session. |

.

.

## Abbreviations

| ALC | Asynchronous Layered Coding NOTE:Asynchronous Layered Coding is defined in [RFC3450]. |
| --- | --- |
| AVP | Attribute-Value Pair |
| BCMCS | Broadcast and Multicast Service |
| BM-SC | Broadcast Multicast Service Centre |
| BSS | Base Station System |
| B-TID | Bootstrapping Transaction Identifier, see [3GPP TS 33.220] |
| CEA | Capability-Exchange-Answer, see [RFC3588]. |
| CER | Capability-Exchange-Request, see [RFC3588]. |
| C-TEID | Common Terminal Endpoint Identifier, see [3GPP TS 29.281] |
| DVB-H | Digital Video Broadcast - Handheld |
| eNB | Evolved Node B |
| FDT | File Delivery Table |
| FEC | Forward Error Correction |
| FLUTE | File Delivery over Unidirectional Transport, see [RFC3926]. |
| GGSN | Gateway GPRS Service Node |
| GPRS | General Packet Radio System |
| GTP | GPRS Tunneling Protocol |
| GTPv1 | GPRS Tunneling Protocol, version 1, see [3GPP TS 29.281] |
| GTP-C | GPRS Tunneling Protocol, Control Part |
| GTP-U | GPRS Tunneling Protocol, User Part |
| GSNs | SGSN/GGSN nodes |
| LCT | Layered Coding Transport NOTE: Layered Coding Transport is defined in [RFC3451]. |
| MBCP | Media Burst Control Protocol, see [OMA-PoC-UP]. |
| MBMS | Multimedia Broadcast/Multicast Service |
| MBMS-GW | Multimedia Broadcast/Multicast Service Gateway |
| MCCH | MBMS point-to-multipoint Control Channel |
| MICH | MBMS Indication Channel |
| MSCH | MBMS point-to-multipoint Scheduling Channel |
| MME | Mobility Management Entity, see [3GPP TS 23.401] |
| OMA | Open Mobile Alliance |
| PCF | Packet Control Function |
| PDN | Packet Data Network |
| PDSN | Packet Data Service Node |
| PGW | PDN Gateway |
| PLMN | Public Land Mobile Network |
| PoC | Push to talk Over Cellular |
| RAA | Re-Auth-Answer, see [3GPP TS 29.061]. |
| RAN | Radio Access Network |
| RANAP | Radio Access Network Application Protocol specified in [3GPP TS 25.413] |
| RAR | Re-Auth-Request, see [RFC3588]. |
| RNC | Radio Network Controller |
| RRC | Radio Resource Control, see [3GPP TS 25.331], [3GPP TS 36.331] |
| RRM | Radio Resource Management |
| SGSN | Service GPRS Support Node |
| SGW | Serving Gateway |
| TBCP | Talk Burst Control Protocol, see [OMA-PoC-UP]. |
| TCP | Transmission Control Protocol |
| TMGI | Temporary Mobile Group Identifier, see [3GPP TS 26.346]. |
| TOI | Transport Object Identifier |
| UE | User Equipment |
| 3GPP | 3rd Generation Partnership Project |

.

.

.

## 5.6 Sending and receiving Media over the Multicast PoC Channel

During a PoC Session TBCP messages, MBCP messages, Continuous Media and Discrete Media can be sent over the Multicast PoC Channel.

The Multicast PoC Channel MAY be shared by several PoC Sessions according to the Service provider’s local policy.

NOTE 1: One example of a local policy can be that all PoC Sessions where the PoC Group Identity is part of the same sub-domain shares one Multicast PoC Channel. Another local policy could be that the PoC Server is configured with the PoC Group Identities of the PoC Groups sharing the same Multicast PoC Channel.

In this section and sections 5.6a and 5.6b below, a description of the multicast path through the access network is provided. In subsequent sections, only the interfaces between the BM-SC and the entry point of the Multicast Access Network, the GGSN node (UTRAN) or MBMS GW (for eUTRAN), is described. The detailed interfaces description for the User Plane data transport using a Multicast Access Network is transparent towards the PoC server and is out of the scope of this TS.  For detailed information, please refer to [3GPP TS 29.061] and [3GPP TS 23.246].

In the Multicast Access Network system when MBMS is used, the Media and TBCP/MBCP messages are delivered from the PoC Server along the path of downstream nodes, i.e. GGSN, SGSN, RNC for UTRAN, MBMS-GW, eNB for eUTRAN and finally down to the PoC client, in MBMS bearer context which is created during Multicast PoC Channel bearer initiation as specified in section 7.2.1 "Starting a Multicast Bearer".

NOTE 2: The exact protocols and interfaces used for MBMS bearer service are specified in [3GPP TS 29.061] “Interface to Packet Domain Bearer Services”.

The path for the Media and TBCP/MBCP messages is different depending upon whether the access network is UTRAN or eUTRAN as is described below.

## 5.6a Multicast PoC Channel Path (UTRAN)

- The User Plane data is first transported towards GGSN from BM-SC in the Participating PoC Server as specified in 5.6.4  “Multicast PoC Channel user data transport mode” via Gi reference point ;

- User Plane data is further tunneled from GGSN to RNC using GTP, more specifically the transport of User Plane data along the MBMS bearer path is realized by the GTP-U protocol, while GTP-C protocol is used for the MBMS control plane between GGSN and SGSN. MBMS control plane signaling between the SGSN and the RNC is carried out via RANAP. GTP-U supports the transport of data packets from a variety of different User Plane protocols such as IPv4, IPv6 or PPP.

- When GGSN receives the IP data packets from the BM-SC, it duplicates the User Plane data packet, encapsulates the data into GTP for each SGSN.

NOTE 1: Besides the unicast delivery of User Plane data within GSNs, IP multicast procedures similar to those used for multicast between BM-SC and GGSN can also be used within the IP infrastructure of GSNs backbone, depending on the realization.

- When SGSN receives an incoming MBMS User Plane data packet, it delivers it to the respective RNCs as in the list of downstream nodes for GGSN, via the corresponding GTP-U tunnels.

- RAN finally delivers the Multicast PoC User Plane data over MBMS radio bearer towards the PoC client in the air interface.

## 5.6b Multicast PoC Channel Path (eUTRAN)

## 1. The User Plane data is first transported towards MBMS-GW from the BM-SC in the Participating PoC Server as specified in 5.6.4  “Multicast PoC Channel user data transport mode” via the SGi-mb reference point.

## 2. User Plane data is further tunneled from the MBMS-GW to the eNBs via the M1 reference point using GTPv1-U, identified by a C-TEID. GTPv2-C is used for the MBMS control plane between the MBMS-GW and the MME. MBMS control plane signaling between the MME and eNBs communicates the C-TEID to the multicast listeners to allow them to identify the User Plane data. See [3GPP TS 29.281] for more information.

## 3. The MBMS-GW performs multicast distribution of the User Plane data to the eNBs.

## 4. The eNBs deliver the Multicast PoC User Plane data over MBMS radio bearers towards the PoC client via the air interface.

.

.

.

.