Change Request

| Title: | CR_TS_Multicast_Ext_Sec_Numbers | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA-PCPS-TS-Multicast-V1_0-20140817-D |  |
| Submission Date: | 15 September 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | n/a |  |

Reason for Change

This change request updates the Multicast specification regarding removing references to section numbers in external documents.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Removing referenc to section numbers in external documents for consistency.

### .

.

.

### From 5.6.1 “Sending and receiving of MBCP Messages”:

During a PoC Session PoC Server performing the Controlling PoC Functions sends MBCP messages according to rules and procedures of [OMA-PoC-UP] towards all PoC Clients in the PoC Session.

On receipt of a MBCP Media Burst Taken message, a TBCP Talk Burst Taken message, a MBCP Media Burst Idle message or a TBCP Talk Burst Idle message from the Controlling PoC Server towards a PoC Client in a PoC Session where a Multicast PoC Channel, controlled by the PoC Server, is used, the PoC Server performing the Participating function:

NOTE:	Other MBCP/TBCP messages are sent as specified by [OMA-PoC-UP] independent on if a Multicast PoC Channel is used or not.

- SHALL check the value of MBCP-sequence-number field if it is included in the received MBCP/TBCP message. If the value is bigger than the cached value, then SHALL cache it and SHALL  send the MBCP/TBCP message over the Multicast PoC Channel with the content encrypted as specified in the subclause 5.1.4 "Media encryption and decryption" with the multicast IP address and the same port number for Media-floor Control Entity to which corresponding Multicast Media Streams are bound. The MBCP/TBCP message SHALL NOT include a request for a MBCP Acknowledgment message.

- SHALL send the MBCP/TBCP message unencrypted toward the PoC Client if the PoC Client is not listening to the Multicast PoC Channel according to rules and procedures of [OMA-PoC-UP] "Procedures at the PoC Server performing the Participating PoC Function".

On receipt of the MBCP Media Burst Taken, the TBCP Talk Burst Taken, the MBCP Media Burst Idle or the TBCP Talk Burst Idle message from the PoC Server Performing the Participating PoC Function over the Multicast PoC Function the PoC Client:

- SHALL check that the TBCP/MBCP message is intended for this PoC Session by means of the port number in the destination address and if that is not the case do not continue with the rest of the steps; and,

- SHALL act according to rules and procedures of [OMA-POC-UP] "Procedures at the PoC Client" as if the MBCP/TBCP message was received over the normal unicast MBCP session.

.

.

.

### From 5.6.2 “Sending and receiving Continuous Media”:

The PoC Server performing the Controlling PoC Functions sends Continuous Media in the form of RTP Media packets according to rules and procedures of the [OMA-PoC-UP] towards all PoC Clients that has negotiated Continuous Media in the PoC Session unless the PoC Server Participating PoC Function are using PoC Media Traffic Optimisation as specified in [OMA-PoC-UP] "PoC Media Traffic Optimisation".

NOTE 1: If the PoC Media Traffic Optimisation is used the PoC Server performing the Controlling PoC Function only sends one RTP Media packets to the PoC Server performing the Participating PoC Function instead of sending one RTP Media packet to each PoC User handled by the PoC Server performing the Participating PoC Function.

On receipt of a RTP Media packet from the Controlling PoC Server towards a PoC Client in a PoC Session where a Multicast PoC Channel, controlled by the PoC Server, is used, the PoC Server performing the Participating PoC Function:

- SHALL, if not already sent, send the RTP Media packet over the Multicast PoC Channel with the content encrypted as specified in the subclause 5.1.4 "Media encryption and decryption" towards the multicast IP address and port negotiated for the Continuous Media;

NOTE 2:	The PoC Server is using RTP header information to determine whether the Packet is already sent or not.

- SHALL send the RTP Media packet unencrypted towards the PoC Client if the PoC Client is not listening to the Multicast PoC Channel according to rules and procedures of [OMA-PoC-UP] "Procedures at the PoC Server performing the Participating PoC Function" if PoC Media Traffic Optimisation is not used; and,

- SHALL send the RTP Media packet unencrypted towards all PoC Clients not listening to the Multicast PoC Channel according to rules and procedures of [OMA-PoC-UP] "PoC Media Traffic Optimisation" and [OMA-PoC-UP] "Procedures at the PoC Server performing the Participating PoC Function" if PoC Media Traffic Optimisation is used..

On receipt of a RTP Media packet from the PoC Server performing the Participating PoC Function over the Multicast PoC Function the PoC Client:

- SHALL check that the RTP Media packet is intended for this PoC Session by means of the port number in the destination address and if that is not the case do not continue with the rest of the steps; and,

- SHALL act according to rules and procedures of [OMA-POC-UP] "Procedures at the PoC Client" as if the RTP Media packet was received over the normal unicast RTP Session.

- SHALL by itself discard Media for keeping conversation uninterrupted if a PoC Client is listening to more than one Multicast PoC Channels or unicast Media Stream.

NOTE 3: The Simultaneous PoC Session is not supported in the Multicast PoC.

.

.

.

### From 5.6.3 “Sending and receiving of Discrete Media”

During the PoC Server performing the Controlling PoC Functions sends Discrete Media using MSRP SEND requests according to rules and procedures of the [OMA-PoC-UP] towards each PoC Client that has negotiated Discrete Media.

On receipt of an MSRP SEND request from the PoC Server performing the Controlling PoC Function towards a PoC Client in a PoC Session where a Multicast PoC Channel controlled by the PoC Server is used, the PoC Server performing the Participating PoC Function:

- SHALL if the target PoC Client is a PoC Client listening to the Multicast PoC Channel:

  - generate and send a MSRP 200 "OK" response according to rules and procedures of [OMA-PoC-IM];

  - generate a Discrete Media Transfer Progress Report as specified in subclause 5.5.2 "Discrete Media Transfer Progress Report" if Discrete Media Transfer Progress Report is requested; and,

  - generate a Discrete Media Transfer Final Report as specified in subclause 5.5.1 "Discrete Media Transfer Final Report" if Discrete Media Transfer Final Report is requested;

- SHALL send the file contained in the SEND request over the Multicast PoC Channel using the procedures described in subclause 5.4 "File distribution".

NOTE: If the MSRP SEND request contains only a part of the final file the PoC Server collects all parts prior to sending the file over the Multicast PoC Channel. When sending the file, the PoC Server needs to control the sending rate in order not to cause congestion in the access network.

- SHALL, if the target PoC Client is a PoC Client not listening to the Multicast PoC Channel send the MSRP SEND request according to the rules and procedures of [OMA-PoC-UP] "Procedures at the PoC Server performing the Participating PoC Function".

On receipt of a file as specified in subclause 5.4 "File distribution" from the PoC Server performing the Participating PoC Function over the Multicast PoC Channel, the PoC Client:

- SHALL check that the Discrete Media is intended for this PoC Session by means of the port number in the destination address and if that is not the case do not continue with the rest of the steps;

- SHALL act as if a MSRP SEND request with a complete file is received and follow the rules and procedures of [OMA-POC-UP] "Procedures at the PoC Client";

- SHALL NOT generate any Discrete Media Transfer Progress Report; and,

- SHALL NOT generate any Discrete Media Transfer Final Report.

.

.

.

#### From 6.1.3.1 “Handoff”:

When the PoC Client detects that the PoC Client moves from Multicast service area to non Multicast service area or vice versa, the PoC Client:

- SHALL generate a SIP UPDATE request according to rules and procedures of [RFC3311] as specified in [OMA-PoC-CP] subclause "General".

- SHALL include in the SIP request a MIME SDP body as an SDP offer as specified in subclause 6.1.5.1 "SDP offer generation";

.

.

.

#### From 6.1.5.1 “SDP offer generation for a Multicast PoC Channel”

The SDP offer SHALL be composed according to rules and procedures of [RFC3264] and [RFC4566].

When composing an SDP offer the PoC Client:

- SHALL include a MIME SDP body as an SDP offer for the unicast Media Streams and Media-Floor Control Entities as specified in [OMA-PoC-CP] "SDP offer generation" with the clarification in the following steps:

.

.

.

#### From 6.1.5.3 “PoC Client PoC Session modification”

The PoC Client SHALL modify the PoC Session as specified in [OMA-PoC-CP] "PoC Client PoC Session modification". The PoC Client SHALL NOT modify the media-level section for each multicast Media Stream received in the SIP INFO request for the Multicast PoC Channel announcements.

.

.

.

##### From 6.2.3.1.1 “SDP for stopping a Multicast PoC Channel”

This subclause describes the SDP that the PoC Server performing the Participating PoC Function uses when stopping an entire Multicast PoC Channel.

The SDP in the SIP INFO request SHALL be composed as an SDP answer according to rules and procedures of [RFC3264] and [RFC4566].

When composing the SDP announcement for stopping the Multicast PoC Channel PoC Server:

- SHALL include a MIME SDP body for the unicast Media Parameters as specified in [OMA-PoC-CP] "SDP answer generation" with the clarification in the following step;

.

.

.

##### From 6.2.3.1.2 “SDP for removing multicast Media Streams”

This subclause describes the SDP that the PoC Server performing the Participating PoC Function uses when closing an individual Media.

The SDP in the SIP INFO request SHALL be composed as an SDP answer according to rules and procedures of [RFC3264] and [RFC4566].

When composing the SDP announcement for closing individual multicast Media Streams the Multicast PoC Channel PoC Server:

- SHALL include a MIME SDP body for the unicast Media Parameters as specified in [OMA-PoC-CP] "SDP answer generation" with the clarification in the following step;

.

.

.

#### From 6.2.4.1 “SDP in SIP INFO request”

.

.

.

When composing the SDP announcement the PoC Server:

- SHALL include a MIME SDP body as an SDP offer for the unicast Media Parameters as specified in [OMA-PoC-CP] " SDP offer generation in case of On-demand session" with the clarification in the following steps:

- SHALL include in the media-level section for each offered unicast Media-level description where the Media Stream is sent over the Multicast PoC Channel and received as unicast the "a=recvonly";

- SHALL include in the media-level section for each offered multicast Continuous Media Stream consisting of:

- the port number for the multicast Continuous Media Stream as specified in as specified in [3GPP TS26.346] "SDP Parameters for MBMS streaming session";

- .

- .

- .

#### From 6.2.4.2 “SDP answer generation”:

When composing an SDP answer according to rules and procedures of [RFC3264] and [RFC4566] the PoC Server:

- SHALL include a MIME SDP body as an SDP answer for the unicast Media Parameters as specified in [OMA-PoC-CP] "SDP answer generation" with the clarification in the following steps:

- SHALL include in the SDP session level the "a=key-mgmt" attribute as described in the subclause 5.1 "User Plane security";

- SHALL include in the media-level section for each accepted unicast Media-level description where the Media Stream is sent over the Multicast PoC Channel and received as unicast the "a=recvonly";