Ad-hoc and 1-1 PoC Session establishment On-demand Session

Originating flow – Unconfirmed Indication

The signaling sequences in this subclause show the establishment of the Ad-hoc PoC Session using on-demand signaling in the originating side. As a prerequisite for the on-demand signaling, PoC Client needs to be registered.

This flow shows the signaling sequence for the unconfirmed case. In this case a SIP 183 "Session Progress" response is received from the terminating side before a SIP 200 "OK" response is sent, PoC Server performing the Controlling PoC Function sends a SIP 200 "OK" response to the PoC Client. In this flow the media buffering is supported and the MBCP Media Burst Granted is sent after the SIP 183 "Session Progress" is received.  If buffering is not supported the MBCP Media Burst Granted is sent after the SIP 200 "OK" is received.

QoE Profiles feature is used in this flow. PoC Client A requests Premium QoE Profile for the PoC Session and for his Local QoE Profile.

The flow is as shown in Figure 8 "Ad-hoc PoC Session establishment on the originating side using on-demand signaling, unconfirmed case".

Figure 8: Ad-hoc PoC Session establishment on the originating side using on-demand signaling, unconfirmed case

The steps of the flow are as follows:

1.	SIP INVITE request (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP INVITE request to the SIP/IP Core A.

| Request-URI | sip:PoCConferenceFactoryURI.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Preferred-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Supported: | timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Content-Type: | multipart/mixed |
| Require: | recipient-list-invite |
| Priv-Answer-Mode: | Auto |
| Accept-Language: | en |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:premium |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 5678 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |
| XML MIME BODY |  |
| Content-Type: | application/resource-lists+xml |
| Content-Disposition: | recipient-list |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserC@networkC.com;uriusage=user" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> </list> </resource-lists> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

2.	SIP 100 "Trying" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A returns a SIP 100 "Trying" response.

3.	SIP INVITE request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP INVITE request to the PoC Server A.

| Request-URI | sip:PoCConferenceFactoryURI.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Supported: | timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Content-Type: | multipart/mixed |
| Require: | recipient-list-invite |
| Priv-Answer-Mode: | Auto |
| Accept-Language: | en |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:premium |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 5678 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |
| XML MIME BODY |  |
| Content-Type: | application/resource-lists+xml |
| Content-Disposition: | recipient-list |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserC@networkC.com;uriusage=user" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> </list> </resource-lists> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

4.	SIP 100 "Trying" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 100 "Trying" response to the SIP/IP Core A.

5.	PoC Server A sends SIP INVITE requests towards the Invited PoC Clients

The PoC Server A sends SIP INVITE requests to the Invited PoC Clients as shown in Figure 11 "PoC Session invitation with automatic answer".

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=adhoc>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,timer,norefersub |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Priv-Answer-Mode: | Auto |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserC@networkC.com;uriusage=user" cc:copyControl="to" cc:anonymize="true"/> <entry uri="sip:PoC-UserD@networkD.com" cc:copyControl="to" cc:anonymize="false"/> </list> </resource-lists> |

6.	PoC Server A receives SIP 183 "Session Progress" response

The PoC Server A receives the first SIP 183 "Session Progress" response as shown in Figure 11 "PoC Session invitation with automatic answer".

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-ServerB@networkBPoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>;+g.poc.talkburst |
| P-Answer-State: | Unconfirmed |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,MESSAGE,OPTIONS |
| Require: | 100rel |

A.	MBCP Media Burst Granted (PoC Server A to PoC Client A)

The PoC Server A sends MBCP Media Burst Granted message to PoC Client A. This step may occur before SIP ACK request or SIP 200 "OK,", because the SIP 200 "OK" response travels through the SIP IP/Core, and the MBCP packet is sent directly from the PoC Server to the PoC Client.

7.	SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 200 "OK" to SIP/IP Core A. (Optionally PoC Server A inserts Media Burst granted indication into the SIP 200 "OK" response).

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net> |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=adhoc>;+g.poc.talkburst;isfocus |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| P-Answer-State: | Unconfirmed |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Supported: | norefersub |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 57787 RTP/AVP 97 |
| i= | speech |
| a= | label:xxx |
| a= | rtpmap:97 AMR |
| a= | rtcp:57000 |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:xxx yyy |
| m= | video 23456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:yyy |
| m= | message 34567 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[57777::eee:fff:aaa:bbb]:12345/jshA7weztas;tcp |

8.	SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the 200 "OK" response to PoC Client A

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net > |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=adhoc>;+g.poc.talkburst;isfocus |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| P-Answer-State: | Unconfirmed |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Supported: | norefersub |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 57787 RTP/AVP 97 |
| i= | speech |
| a= | label:xxx |
| a= | rtpmap:97 AMR |
| a= | rtcp:57000 |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:xxx yyy |
| m= | video 23456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:yyy |
| m= | message 34567 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[57777::eee:fff:aaa:bbb]:12345/jshA7weztas;tcp |

9.	SIP ACK request (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP ACK request to SIP/IP Core A

10.	SIP ACK request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP ACK request to PoC Server A

B.	PoC Client sends RTP Media to PoC Server A

The PoC Client A sends RTP Media packets to PoC Server A

Originating flow – Confirmed Indication with manual answer

The signaling sequences in this subclause show the establishment of the 1-1 PoC Session using on-demand signaling in the originating side. As a prerequisite for the on-demand signaling, PoC Client needs to be registered.

This flow shows the signaling sequence for the Confirmed indication case when the terminating side answers manually and SIP 200 "OK" is not sent before the SIP 200 "OK" from the terminating side is received.

QoE Profiles feature is used in this flow. PoC Client A requests Professional QoE Profile for the PoC Session and for his Local QoE Profile.

The flow is as shown in Figure 9 "Ad-hoc PoC Session establishment on the originating side using on-demand signaling, confirmed case manual answer ".

Figure 9: Ad-hoc PoC Session establishment on the originating side using on-demand signaling, confirmed case manual answer

The steps of the flow are as follows:

1.	SIP INVITE request (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP INVITE request to SIP/IP Core A.

| Request-URI | sip:PoCConferenceFactoryURI.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Preferred-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Supported: | Timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Require: | recipient-list-invite |
| Content-Type: | multipart/mixed |
| Accept-Language: | en |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| m= | audio 3456 RTP/AVP 97 |
| a= | poc-qoe: professional |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 5678 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |
| XML MIME BODY |  |
| Content-Type: | application/resource-lists+xml |
| Content-Disposition: | recipient-list |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

2.	SIP 100 "Trying" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A sends a SIP 100 "Trying" response to PoC Client A.

3.	SIP INVITE request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP INVITE request to the PoC Server A.

| Request-URI | sip:PoCConferenceFactoryURI.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst;+g.poc.fdcfo;+g.poc.discretemedia |
| Supported: | timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Require: | recipient-list-invite |
| Content-Type: | multipart/mixed |
| Accept-Language: | en |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:professional |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 5678 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |
| XML MIME BODY |  |
| Content-Type: | application/resource-lists+xml |
| Content-Disposition: | recipient-list |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |

4.	SIP 100 "Trying" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 100 "Trying" response to the SIP/IP Core A.

5.	PoC Server A sends SIP INVITE requests towards the Invited PoC Clients

The PoC Server A sends SIP INVITE requests to the Invited PoC Client.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,timer,norefersub |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| Reject-Contact: | sip.automata;sip.actor=msg-taker;require;explicit |
| Reject-Contact: | sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

6.	PoC Server A receives SIP 180 "Ringing" response

The PoC Server A receives the SIP 180 "Ringing" response.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>;+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK, REFER,UPDATE,MESSAGE,OPTIONS |

7.	SIP 180 "Ringing" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends the SIP 180 "Ringing" response to the SIP/IP Core A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net> |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |

8.	SIP 180 "Ringing" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP 180 "Ringing" response to PoC Client A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net> |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |

9.	PoC Server A receives SIP 200 "OK" response

The first SIP 200 "OK" response is received from the terminating PoC Network.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |
| Accept-Language: | en |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | Rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2;timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:uu vv |
| m= | video 17566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:vv |
| a= | a=upcc:0 |
| m= | message 43821 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[99055::fff:aaa:eee:bbb]:43821/xk83a45;tcp |
| a= | max-size:10000 |

10.	SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 200 "OK" response to SIP/IP Core A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net> |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uac |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE, REFER,MESSAGE,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 57787 RTP/AVP 97 |
| i= | speech |
| a= | label:xxx |
| a= | rtpmap:97 AMR |
| a= | Rtcp:57000 |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:xxx yyy |
| m= | video 23456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:yyy |
| m= | message 34567 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[57777::eee:fff:aaa:bbb]:12345/jshA7weztas;tcp |

A.	MBCP Media Burst Granted (PoC Server A to PoC Client A)

The PoC Server A sends MBCP Media Burst Granted message to PoC Client A.

11.	SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP 200 "OK" response to PoC Client A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net> |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uac |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE, REFER,MESSAGE,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 57787 RTP/AVP 97 |
| i= | speech |
| a= | label:xxx |
| a= | rtpmap:97 AMR |
| a= | Rtcp:57000 |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1;imp_mb_req=1 |
| a= | floorid:0 mstrm:xxx yyy |
| m= | video 23456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:yyy |
| m= | message 34567 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[57777::eee:fff:aaa:bbb]:12345/jshA7weztas;tcp |

12.	SIP ACK request (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP ACK request to SIP/IP Core A.

B.	PoC Client sends RTP Media to PoC Server A

The PoC Client A sends RTP Media packets to PoC Server A.

13.	SIP ACK request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP ACK request to PoC Server A.