Joining a Chat PoC Group Session

Using on-demand signaling

This subclause describes the successful joining to an ongoing Chat PoC Group PoC Session using On-demand signaling.

The PoC Media Traffic Optimisation is used between the PoC Server A and PoC Server X.

The QoE Profile assigned to the Chat PoC Group is the 'Premium' QoE Profile. The PoC Client A requests 'Official Government Use' as the Local QoE Profile.

NOTE:	The signaling flow is valid for rejoining a Chat PoC Group Session.

Figure 13: "Joining a Chat PoC Group Session" shows the signaling flow for the scenario.

Figure 13: Joining a Chat PoC Group Session.

The steps of the flows are as follows:

1.	SIP INVITE request (from PoC Client A to SIP/IP Core A)

The PoC Client receives a request from the PoC User to join a Chat Group PoC Session and sends a SIP INVITE request to the SIP/IP Core A. The SIP INVITE request includes the Chat PoC Group Identity in the Request-URI.

| Request-URI | sip:OMA-Golf-buddies@networkX.net;gr;session=chat |
| --- | --- |
| SIP HEADERS |  |
| P-Preferred-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA@networkA.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst +g.poc.fdcfo; +g.poc.discretemedia |
| Supported: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Accept-Language | en |
| Resource-Priority | wps.2 |
| SDP PARAMETERS |  |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:government-use |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 3456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |

2.	SIP 100 "Trying" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A sends a SIP 100 "Trying" response to the PoC Client A.

3.	SIP INVITE request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A sends the SIP INVITE request to the PoC Server A.

| Request-URI | sip:OMA-Golf-buddies@networkX.net;gr;session=chat |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA@networkA.net;gr= urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="<urn:uuid:00000000-0000-1000-8000-000A95A0E128>";+g.poc.talkburst; +g.poc.fdcfo; +g.poc.discretemedia |
| Supported: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Accept-Language | en |
| Resource-Priority | wps.2 |
| SDP PARAMETERS |  |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:government-use |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 3456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| m= | message 12345 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[5555::aaa:bbb:ccc:ddd]:12345/kjhd37s2s20w2a;tcp |
| a= | max-size:10000 |

4.	SIP 100 "Trying" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 100 "Trying" response to the SIP/IP Core A.

5.	SIP INVITE request (from PoC Server A to SIP/IP Core A)

Using the PoC Group Identity the PoC Server A determines that the Chat PoC Group is owned by another PoC Server and sends the SIP INVITE request to the SIP/IP Core A. The SIP INVITE request includes the Chat PoC Group Identity in the Request-URI.

| Request-URI | sip:OMA-Golf-buddies@networkX.net;gr;session=chat |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr>;+g.poc.talkburst; +g.poc.discretemedia;+g.poc.fdcfo; b2bua |
| Supported: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Accept-Language | en |
| Resource-Priority | wps.2 |
| SDP PARAMETERS |  |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:government-use |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| a= | upcc:0 |
| a= | sendonly |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 53456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| a= | upcc:0 |
| a= | sendonly |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555:: ccc:ddd:aaa:bbb]:54321/kjhd37p4z789w1;tcp |
| a= | max-size:10000 |

6.	SIP 100 "Trying" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A sends a SIP 100 "Trying" response to the PoC Server A.

7.	SIP INVITE request (from SIP/IP Core A to SIP/IP Core X)

Based on the Chat PoC Group Identity in the Request-URI the SIP/IP Core A sends the SIP INVITE request to the SIP/IP Core X.

| Request-URI | sip:OMA-Golf-buddies@networkX.net;gr;session=chat |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr>;+g.poc.talkburst; +g.poc.fdcfo;+g.poc.discretemedia; b2bua |
| Supported: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Accept-Language | en |
| Resource-Priority | wps.2 |
| SDP PARAMETERS |  |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:government-use |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| a= | upcc:0 |
| a= | sendonly |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 53456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| a= | upcc:0 |
| a= | sendonly |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555:: ccc:ddd:aaa:bbb]:54321/kjhd37p4z789w1;tcp |
| a= | max-size:10000 |

8.	SIP 100 "Trying" response (from SIP/IP Core X to SIP/IP Core A)

The SIP/IP Core X sends a SIP 100 "Trying" response to the SIP/IP Core A.

9.	SIP INVITE request (from SIP/IP Core X to PoC Server X)

The SIP/IP Core X sends the SIP INVITE request to the PoC Server X.

| Request-URI | sip:OMA-Golf-buddies@networkX.net;gr;session=chat |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr>;+g.poc.talkburst; +g.poc.fdcfo;+g.poc.discretemedia; b2bua |
| Supported: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Accept-Language | en |
| Resource-Priority | wps.2 |
| SDP PARAMETERS |  |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:government-use |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| a= | sendonly |
| a= | upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 53456 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| a= | upcc:0 |
| a= | sendonly |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555:: ccc:ddd:aaa:bbb]:54321/kjhd37p4z789w1;tcp |
| a= | max-size:10000 |

10.	SIP 100 "Trying" response (from PoC Server X to SIP/IP Core X)

The PoC Server X sends the SIP 100 "Trying" response to the SIP/IP Core X.

11.	SIP 200 "OK" response (from PoC Server X to SIP/IP Core X)

The PoC Server X authorizes the PoC Client A to rejoin to the Chat PoC Group Session and sends the SIP 200 "OK" response to the SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:OMA-Golf-buddies@networkX.net;session=chat> |
| Contact | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=chat>;+g.poc.talkburst;isfocus |
| Server: | PoC-serv/OMA2.0 |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK, UPDATE, REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 9666::eee: aaa:bbb:fff |
| a= | poc-qoe:premium |
| m= | audio 9652 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:9600 |
| a= | upcc:0 |
| a= | recvonly |
| i= | speech |
| a= | label:aa |
| m= | application 9600 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 9652 RTP/AVP 93 |
| a= | rtpmap:93 MP4V-ES |
| a= | label:bb |
| a= | upcc:0 |
| a= | recvonly |
| m= | message 18912 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[9666:: eee:aaa:bbb:fff]:18912/zrtopxW45;tcp |
| a= | max-size:64000 |

12.	SIP 200 "OK" response (from SIP/IP Core X to SIP/IP Core A)

The SIP/IP Core X sends the SIP 200 "OK" response to the SIP/IP Core A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:OMA-Golf-buddies@networkX.net;session=chat> |
| Contact | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=chat>;+g.poc.talkburst;isfocus |
| Server: | PoC-serv/OMA2.0 |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK, UPDATE, REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 9666::eee: aaa:bbb:fff |
| a= | poc-qoe:premium |
| m= | audio 9652 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:9600 |
| a= | upcc:0 |
| i= | speech |
| a= | recvonly |
| a= | label:aa |
| m= | application 9600 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 9652 RTP/AVP 93 |
| a= | rtpmap:93 MP4V-ES |
| b= | AS:75 |
| a= | label:bb |
| a= | upcc:0 |
| a= | recvonly |
| m= | message 18912 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[9666:: eee:aaa:bbb:fff]:18912/zrtopxW45;tcp |
| a= | max-size:64000 |

13.	SIP 200 "OK" response (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Server A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:OMA-Golf-buddies@networkX.net;session=chat> |
| Contact | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=chat>;+g.poc.talkburst;isfocus |
| Server: | PoC-serv/OMA2.0 |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK, UPDATE, REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 9666::eee: aaa:bbb:fff |
| a= | poc-qoe:premium |
| m= | audio 96525 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:9600 |
| a= | upcc:0 |
| i= | speech |
| a= | label:aa |
| a= | recvonly |
| m= | application 9600 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 9652 RTP/AVP 93 |
| a= | rtpmap:93 MP4V-ES |
| a= | label:bb |
| a= | upcc:0 |
| a= | recvonly |
| m= | message 18912 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[9666:: eee:aaa:bbb:fff]:18912/zrtopxW45;tcp |
| a= | max-size:64000 |

14.	SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends the SIP 200 "OK" response to SIP/IP Core A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:OMA-Golf-buddies@networkX.net;session=chat> |
| Contact | <sip:PoC-SessionABCDEF@ PoC-ServerA.networkA.net ;gr;session=chat>;+g.poc.talkburst;isFocus |
| Server: | PoC-serv/OMA2.0 |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK, UPDATE, REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 57787 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:57000 |
| a= | upcc:0 |
| i= | speech |
| a= | label:aa |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 57787 RTP/AVP 93 |
| a= | rtpmap:93 MP4V-ES |
| a= | label:bb |
| m= | message 23456 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[ 57777::eee:fff:aaa:bbb]:23456/l2wzz1K92w5;tcp |
| a= | max-size:64000 |

15.	SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A sends the SIP 200 "OK" response to the PoC Client A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:OMA-Golf-buddies@networkX.net;session=chat> |
| Contact | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net ;gr;session=chat>;+g.poc.talkburst;isFocus |
| Server: | PoC-serv/OMA2.0 |
| Require: | timer |
| Session-Expires: | 1800;refresher=uac |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK, UPDATE, REFER,MESSAGE,SUBSCRIBE, NOTIFY,PUBLISH,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:premium |
| m= | audio 57787 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:57000 |
| a= | upcc:0 |
| i= | speech |
| a= | label:aa |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1; multimedia=1; mbc_scheme="random"; tb_txbw=64; poc_sess_priority=1;local_grant=1; imp_mb_req=0 |
| a= | floorid:0 mstrm:aa bb |
| m= | video 57787 RTP/AVP 93 |
| a= | rtpmap:93 MP4V-ES |
| a= | label:bb |
| m= | message 23456 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[ 57777::eee:fff:aaa:bbb]:23456/l2wzz1K92w5;tcp |
| a= | max-size:64000 |

A.	MBCP Media Burst Taken message (from PoC Server X to PoC Server A)

The PoC Server X sends a MBCP Media Burst Taken message to the PoC Server A.

B.	MBCP Media Burst Taken message (from PoC Server A to PoC Client A)

The PoC Server A sends the MBCP Media Burst Taken message to the PoC Client A.

16.	SIP ACK request (PoC Client A to SIP/IP Core A)

The PoC Client A acknowledges the SIP 200 "OK" response with a SIP ACK request sent to the SIP/IP Core A.

17.	SIP ACK request (SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP ACK request to the PoC Server A.

18.	SIP ACK request (PoC Server A to SIP/IP Core A)

PoC Server A forwards the SIP ACK request to SIP/IP Core A.

19.	SIP ACK request (SIP/IP Core A to SIP/IP Core X)

The SIP/IP Core A forwards the SIP ACK request to the SIP/IP Core X.

20.	SIP ACK request (SIP/IP Core X to PoC Server X)

The SIP/IP Core X forwards the SIP ACK request to the PoC Server X.