Change Request

| Title: | Presentation related requirements to the MMS client | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA-MWG-MMSG |  |
| Doc to Change: | MMS conformance v 1.3 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Clerical |  |
| Source: | Sony Ericsson Mobile Communications AB T-Mobile Nokia Orange (ansgar_bergmann@yahoo.co.uk) |  |
| Replaces: | n/a |  |

- Reason for Change

The technical/physical capabilities of the MMS client and associated terminal are not unlimited. In order to have predictable presentations for content messaging, the limitations of MMS clients and associated terminals should be harmonized.

- Impact on Backward Compatibility

None

- Impact on Other Specifications

None

- Intellectual Property Rights Considerations

Not known

- Recommendation

Update MMS conformance v 1.3 with the proposal below (new section).

- Detailed Change Proposal

XX Presentation related Requirements to the Client	(Normative)

For content messaging it is particularly important that MM are presented to the user as intended, i.e., without modifications. This might be achieved by creating – for different target terminals/clients – different versions (that is, different MM) for a given content; however this is a cumbersome method.

A main necessity to enable content messaging from content provider to user is the following:

- For a given content to be sent via MMS, only a limited number of versions (in form of different MM) is provided (maybe one, two, or three)

- The presentation of MM needs to be predictable for the content provider.

Note: Of course the content provider can always tailor the content to different target clients/terminals.

For content messaging, MMS supports the PSS SMIL language definition fully as defined in [TS 26.234 and 26.246 Release 6]. However, necessarily, there are technical limits to present MM:

(a) Technical/physical limitations of the MMS client (and the hardware on that it resides)

(b) Technical/physical limitations of the terminal associated with the MMS client (e.g., screen size).

In order to achieve predictable presentations

- certain limits for the technical presentation of MM are defined in section XX.1

- rules how the client has to handle the case where the technical capabilities of the associated terminal are exceeded are defined in section XX.2.

## XX.1 Limits for the physical presentation of MM

The restrictions of this section apply for the usage of 3GPP PSS6 SMIL Language Profile (as defined in [3GPP TS 26.234 and 26.246 Release 6) for MMS. If these restrictions are exceeded, the exact presentation can not be guaranteed.

Restrictions

General

The MMS client SHALL recognize the complete 3GPP PSS6 SMIL Language Profile (as defined in 3GPP TS 26.234 Release 6). This means that the client SHALL be able to recognise a presentation part of this profile as a valid document and to handle it within the limits of this section. Attributes and values that are not handled by the client SHALL be ignored by the client. The client SHOULD display as many media as possible (best effort) - even if some SMIL elements received are not fully supported. If technical limitations of the client are exceeded, priority SHOULD be given to video contents over other contents to be displayed simultaneously.

Nesting

The MMS client SHALL be able to handle at least a nesting depth of timecontainers up to and including five. Deeper nestings MAY be discarded by the MMS client, but SHALL be correctly parsed.

Basic Layout

The MMS client and associated terminal SHALL be able to display up to at least 20 regions and/or images at the same time. The MMS client and associated terminal is not required to be able to display more than 20 regions and/or images at the same time. The MMS client and associated terminal SHALL be able to render up to at least 4 media objects starting at the same time. The MMS client and associated terminal is not required to be able to render more than 4 media objects starting at the same time.

The MMS client and associated terminal are not required to be able to present an object on top of video or animation

The MMS client and associated terminal are not required to display at the same time regions with more that 4 different z index values.

Media

The MMS client is not required to simultaneously play two audio sources (this includes audio associated to video).

The MMS client SHALL be able to handle Fit value scrollStatic for text media elements; the MMS client is not required to handle Fit values different from scrollStatic for text media elements. The MMS client SHALL be able to handle Fit values hidden, meet, fill for image media elements; the MMS client is not required to handle Fit values different from hidden, meet, fill for image media elements. The MMS client SHALL be able to handle Fit values hidden for non-static media elements; the MMS client is not required to handle Fit values different from hidden for non-static media elements.

Basic Inline Timing

The MMS client and associated terminal shall execute time related SMIL parts with an accuracy of at least 100 ms.

The MMS client is not required to correctly handle negative values of begin and end attributes.

Basic Linking

If the MMS client resides in a terminal with keypad, the client SHALL be able to handle 0-9, '#' and '*' for access key.

The MMS client is not required to support the target attribute.

The MMS client is not required to handle more than one volume control. The terminal associated to the MMS client is not required to support more than one volume control.

The MMS client is not required to handle attributes show, sourceplaystate and sourceLevel.

The MMS client is not required to handle audio and video element as child nodes of the <a> tag.

Basic Content control

The client SHALL be able to handle the following switch attributes: -systemLanguage -systemScreenSize -systemScreenDepth The MMS client is not required to handle switch attributes different from those listed above.

Media Clipping

The MMS client is not required to handle ClipBegin for local media. The MMS client is not required to handle attributes clipBegin, clipEnd for local media. The MMS client is not required to handle attributes clipBegin, clipEnd for remote media with non-seekable streaming protocol. An MMS client residing in a terminal that does not support streaming is not required to handle clipBegin, clipEnd

Meta Data/Media Description/Media Accessibility

BasicTransitions

The MMS client is only required to handle transition applied to static media.

If the MMS client does support the semantics of BasicTransition, the MMS client shall support, at minimum, following transition types and their default subtypes: barWipe, irisWipe, clockWipe, snakeWipe, pushWipe, slideWipe and fade.

PrefetchControl

The MMS client is not required to handle prefetch.

EventTiming

The MMS client is not required to support a cursor.

The MMS client SHALL at least handle the types of events listed below: - ID.activateEvent (to handle user interaction); the support is not required for audio and video objects - ID.beginEvent, ID.endEvent (fluidity between media)

## XX.2 Requirements to the client when technical limits of the associated terminal are exceeded

Content should be designed according to a targeted screen size.

The MMS client shall alter a SMIL presentation layout only if the technical limits of the associated terminal are exceeded.

The MMS client SHOULD adapt the SMIL layout to the screen

- by rotating the presentation

  - by +90 degree (rotation to the left) from a landscape format to a portrait format, if the display of the associated terminal has a portrait format

  - by -90 degree (rotation to the right) from a portrait format to a landscape format, if  the display of the associated terminal has a landscape format

- by zooming the layout to best fit the screen size without clipping parts and keeping the aspect ratio.

- by covering the unused parts of the screen by an appropriate background colour.