Change Request

| Title: | IP Voice Call | Public OMA Confidential |
| --- | --- | --- |
| To: | IOP |  |
| Doc to Change: | OMA-ETS-RCS_CON-V5_2-20141103-D |  |
| Submission Date: | 04 Nov 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Markus Hanhisalo, markus.hanhisalo@microsoft.com |  |
| Replaces: | n/a |  |

Reason for Change

New RCS 5.2 RCS IP Voice Call test case.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

IOP WG approve this CR and update RCS 5.2 ETS.

Detailed Change Proposal

## Call Establishment and Termination

### RCS-con-400 Mobile Originated IP Voice Call Set Up – Mobile Originated IP Voice Call Termination (Includes Optional Features)

| Test Case Id | RCS-con-400 |
| --- | --- |
| Test Object | RCS Client |
| Test Case Description | Mobile Originated IP Voice call set-up– Mobile originated IP Voice call termination |
| Specification Reference | [RCS 5.2]: RCS 3.8 |
| SCR Reference |  |
| Test code | Verified TTCN-3 code |
| Preconditions | User A’s RCS device is provisioned for RCS User A is IMS registered for RCS WiFi/WLAN connection only - Applicability: - ics_IPVoiceCall |
| Test Procedure | - User A establishes an IP Voice call with User B - Execute “Mobile Originated IP Voice Call Establishment” (C.2.1)” - User A ends the IP Voice call - Execute “Mobile Originated IP Voice Call termination” (C.3.1) |
| Pass-Criteria | 1. At step 2 The IP voice call is established. |

B3

Applicability

Client ICS

| ICS | Description | Reference(s) | Supported (yes/no) |
| --- | --- | --- | --- |
| ics_fileTransfer | Support of file transfer | [RCS 1.2.2] 1.2.2 |  |
| ics_imageShare | Support of in-call image share | [RCS 1.2.2] 1.2.2 |  |
| ics_videoShare | Support of in-call video share | [RCS 1.2.2] 1.2.2 |  |
| ics_request_displayNotifications | Support of requesting display notifications | [RCS 1.2.2] 3.2.2.2 |  |
| ics_stun_udp | Support of STUN when using SIP/UDP | [RCS 1.2.2] 2.8 |  |
| ics_IPVoiceCall | Support of RCS IP Voice Call | [RCS 5.2] 3.8 |  |

Table 2

B4

Client ICS to test case mapping

# According to the Client ICS described above the applicable optional Client test cases can be derived from the following table.

| Applicability | Client Test Cases |
| --- | --- |
| ics_imageShare OR ics_videoShare | RCS-con-050 Capability update during a MO call for video /image sharing RCS-con-052 Capability update during a MO voice call fails as other end has no capability |
| ics_request_displayNotifications | RCS-con-102 Mobile Originated 1-to-1 Chat– display notification |
| ics_fileTransfer | RCS-con-200 Mobile Originated File Transfer RCS-con-201 Mobile Originated File Transfer – Rejected RCS-con-210 Mobile Terminated File Transfer RCS-con-211 Mobile Terminated File Transfer – Rejected RCS-con-212 Mobile Terminated File Transfer - File Size Limit (receiver) RCS-con-213 Mobile Terminated File Transfer - File Size Warn Limit (receiver) |
| ics_imageShare | RCS-con-250 Mobile Originated Image Sharing - Session Establishment – Mobile Originated Session Termination RCS-con-251 Mobile Originated Image Sharing - Session Establishment – rejected RCS-con-270 Mobile Terminated Image Sharing - Session Establishment – Mobile Terminated Session Termination RCS-con-271 Mobile Terminated Image sharing - Session Establishment - rejected |
| ics_videoShare | RCS-con-300 Mobile Originated Video Sharing Session Establishment – Mobile Originated Session Termination RCS-con-301 Mobile Originated Video Sharing Session Establishment – Mobile Terminated Session Termination RCS-con-302 Mobile Originated Video Sharing Session Establishment – Rejected RCS-con-320 Mobile Terminated Video Sharing Session Establishment – Mobile Terminated Session Termination RCS-con-321 Mobile Terminated Video Sharing Session Establishment – Mobile Originated Session Termination RCS-con-322 Mobile Terminated Video Sharing Session Establishment - Rejected |
| ics_stun_udp | RCS-con-020 Keep-Alive for SIPoUDP (STUN) |
| ics_IPVoiceCall | RCS-con-400 Mobile Originated IP Voice Call Set Up – Mobile Originated IP Voice Call Termination |

C.2

Session Establishment

This section SHALL serve with default procedures to establish one of the following session types:

Video Sharing

Image Sharing

File Sharing

Group Chat

IM 1-1 Chat

RCS IP Voice Call

Mobile Originated Session Establishment: Video share, Image share, File share, basic 1-to-1 Chat, RCS IP Voice Call

NOTE: For Video share IMS mode or RCS IP Voice Call (according to pc_precondition) execute C.2.1a otherwise for IETF mode execute the following steps.

1. User A’s RCS client sends a SIP INVITE message (D.1.1) towards User B to start the session.

2. The test tool responds with a SIP 183 SESSION PROGRESS towards User A’s RCS client.

3. The test tool responds with a 180 RINGING towards User A’s RCS client.

4. (1-to-1 chat only) The test tool responds with a SIP MESSAGE (D.1.13) (message delivered) towards User A’s RCS client.

5. (1-to-1 chat only) User A’s RCS client responds with a 200 OK (D.1.14) towards User B.

6. The test tool responds with a 200 OK (D.1.2) towards User A’s RCS client.

7. User A’s RCS client sends a SIP ACK message towards User B.

D1.1

INVITE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | B1 | +g.3gpp.cs-voice |  |  |
|  | B2 | +g.3gpp.iari-ref="urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-is" |  |  |
|  | B3 | +g.oma.sip-im |  |  |
|  | B4, B5 | +g.oma.sip-im |  |  |
|  | B6 | +g.gsma.rcs.ipcall |  |  |
| Contact |  |  |  |  |
| feature-param | B1 | +g.3gpp.cs-voice+ |  |  |
|  | B2 | +g.3gpp.iari-ref="urn%3Aurn-7%3A3gpp-application.ims.iari.gsma-is" |  |  |
|  | B3 | +g.oma.sip-im |  |  |
|  | B4, B5 | +g.oma.sip-im |  |  |
|  | B6 | +g.gsma.rcs.ipcall |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type | B1, B2, B3, B6 | application/sdp |  |  |
|  | B4, B5 | multipart/mixed;boundary="boundary1" |  |  |
| Supported |  |  |  | RFC 3261 [15] |
|  |  | Precondition is not included |  |  |
| Require |  |  |  | RFC 3261 [15] |
|  |  | Precondition is not included |  |  |
| Message-body | B1 | m=video port RTP/AVP (rtp payload types) a=sendonly a=rtpmap: payload type H264/90000 a=fmtp: payload type profile-level-id=42C00D; packetization-mode=0 a=rtpmap: payload type H263-2000/90000 a=framesize: payload type 176-144 a=framerate:8 a=fmtp: payload type profile=0; level=45 |  | RFC 4119 [99] |
|  | B2 | m=message port TCP/MSRP * a=sendonly a=accept-types:message/cpim a=accept-wrapped-types:* a=path:msrp://( IP address):port/jshA7we;tcp a=file-selector: type:image/jpeg size:(size of file) a=file-transfer-id:Q6LMoGymJdh0IKIgD6wD0jkcfgva4xvE |  |  |
|  | B3 | m=message port TCP/MSRP * a=sendonly a=accept-types:message/cpim a=accept-wrapped-types:* a=path:msrp://( IP address):port/jshA7we;tcp a=file-selector: type:image/jpeg size:(size of file) a=file-transfer-id:Q6LMoGymJdh0IKIgD6wD0jkcfgva4xvE |  |  |
|  | B4 | --boundary1 Content-Type: application/sdp m=message port TCP/MSRP * a=accept-types: message/cpim text/plain a=max-size: 7665 a=path:msrp://( IP address):port/jshA7we;tcp --boundary1 Content-Type: message/cpim From: <sip:anonymous@anonymous.invalid> To: <sip:anonymous@anonymous.invalid> DateTime: 2014-01-15T10:17:49.727+01:00 NS: imdn <urn:ietf:params:imdn> imdn.Message-ID: 7QsgkAl8QZvr9XPFtiiCtLj3sls0oNok imdn.Disposition-Notification: positive-delivery, display Content-type: text/plain;charset=UTF-8 Initial message --boundary1-- |  |  |
|  | B5 | --boundary1 Content-Type: application/sdp <B4 content> --boundary1 Content-Type: application/resource-lists+xml Content-Disposition: recipient-list <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cp="urn:ietf:params:xml:ns:copycontrol"> <list> <entry uri="<SIP or TEL URI of User 1>" cp:copyControl="to"/> … (NOTE: Add further users as necessary) </list> </resource-lists> --boundary1-- |  |  |
|  | B6 | m=audio (transport port) RTP/AVP (fmt) b=AS: (bandwidth-value) b=RS:0 b=RR:0 a=rtpmap:(payload type) AMR/8000/1 a=fmtp:(format) mode-change-capability=2; max-red=220 a=rtpmap:(payload type) telephone-event a=ptime:20 a=maxptime:240 |  |  |

| Condition | Explanation |
| --- | --- |
| B1 | Video Sharing (ics_videoShare) |
| B2 | Image Sharing (ics_imageShare) |
| B3 | File Sharing (ics_fileTransfer) |
| B4 | IM Chat (1 to 1 session) |
| B5 | IM Group chat |
| B6 | RCS IP Voice Call |

C3.1

Session Termination

Mobile Originated Session Termination: Video share, Image share, File share, 1-to-1 sessions, RCS IP Voice Call

1. User A’s RCS client sends a SIP BYE message towards User B to terminate the session.

2. The test tool responds with 200 OK  (D.1.14) towards User A’s RCS client.

Applicability

Mapping to RCS profiles

The following table specifies the applicability of each conformance test case for previous RCS profiles.

| Section | Test Case Title | RCS 1.2.2 | RCS 5.1 | RCS 5.2 |
| --- | --- | --- | --- | --- |
| 5.2.1 | RCS-con-001 First-time unsuccessful configuration - Subscriber unauthorized (Auto-Configuration Server) | Applies |  |  |
| 5.2.4 | RCS-con-004 Configuration - RCS re-configuration successful | Applies |  |  |
| 5.3.1 | RCS-con-020 Keep-Alive for SIPoUDP (STUN) | Applies |  |  |
| 5.4.1 | RCS-con-030 Mobile Originated Capability Discovery – Successful | Applies |  |  |
| 5.4.2 | RCS-con-031 Mobile Originated Capability Discovery – Unsuccessful – 480 Temporarily Unavailable | Applies |  |  |
| 5.4.3 | RCS-con-032 Mobile Originated Capability Discovery – Unsuccessful –408 Request Timeout | Applies |  |  |
| 5.4.4 | RCS-con-033 Mobile Originated Capability Discovery – Unsuccessful – 404 Not Found | Applies |  |  |
| 5.4.5 | RCS-con-034 Mobile Originated Capability Discovery – Successful – Multiple Identities | Applies |  |  |
| 5.4.6 | RCS-con-035 Mobile Originated Capability Discovery – Successful (IMS registered, does not support RCS) | Applies |  |  |
| 5.5.1 | RCS-con-040 Mobile Terminated Capability Discovery – Correct Presentation of Remote Capabilities | Applies |  |  |
| 5.5.2 | RCS-con-041 Mobile Terminated Capability Discovery – Learning Unknown New Users RCS Capabilities | Applies |  |  |
| 5.6.1 | RCS-con-050 Capability update during a MO call for video /image sharing | Applies |  |  |
| 5.6.2 | RCS-con-051 Capability update during a MT call for video /image sharing | Applies |  |  |
| 5.6.3 | RCS-con-052 Capability update during initiation of an MO voice call - fails as other end has no capability | Applies |  |  |
| 5.6.4 | RCS-con-053 Capability Exchange Optimization during a Call | Applies |  |  |
| 5.7.1 | RCS-con-100 Mobile Originated 1-to-1 Chat - session establishment – Mobile Originated session termination | Applies |  |  |
| 5.7.2 | RCS-con-101 Mobile Originated 1-to-1 Chat - session establishment – Mobile Terminated session termination | Applies |  |  |
| 5.7.3 | RCS-con-102 Mobile Originated 1-to-1 Chat – display notification | Applies |  |  |
| 5.7.4 | RCS-con-103 Mobile Originated 1-to-1 Chat– duplicate session | Applies |  |  |
| 5.7.5 | RCS-con-104 Mobile Originated 1-to-1 Chat - race condition | Applies |  |  |
| 5.7.6 | RCS-con-105 Mobile Originated 1-to-1 Chat – inactivity timeout | Applies |  |  |
| 5.7.7 | RCS-con-106 Mobile Originated 1-to-1 Chat - store and forward – sender still in active IM session | Applies |  |  |
| 5.7.8 | RCS-con-107 Mobile Originated 1-to-1 Chat - store and forward – sender still online | Applies |  |  |
| 5.8.1 | RCS-con-120 Mobile Terminated 1-to-1 Chat - session establishment – Mobile Terminated session | Applies |  |  |
| 5.8.2 | RCS-con-121 Mobile Terminated 1-to-1 Chat - session establishment – Mobile Originated session | Applies |  |  |
| 5.8.3 | RCS-con-122 Mobile Terminated 1-to-1 Chat – spam/blacklist | Applies |  |  |
| 5.8.4 | RCS-con-123 Mobile Terminated 1-to-1 Chat – display notification | Applies |  |  |
| 5.8.5 | RCS-con-124 Mobile Terminated 1-to-1 Chat– duplicate session | Applies |  |  |
| 5.8.6 | RCS-con-125 Mobile Terminated 1-to-1 Chat - race condition | Applies |  |  |
| 5.8.7 | RCS-con-126 Mobile Terminated 1-to-1 Chat - store and forward | Applies |  |  |
| 5.9.1 | RCS-con-130 Mobile Originated Group Chat - establishment – Originating User leaves the session | Applies |  |  |
| 5.9.2 | RCS-con-131 Mobile Originated Group Chat - non-originating Users leave the session | Applies |  |  |
| 5.9.3 | RCS-con-132 Mobile Originated Group Chat - adding new User to the session | Applies |  |  |
| 5.10.1 | RCS-con-140 Mobile Terminated Group Chat - establishment – Terminating User leaves the session | Applies |  |  |
| 5.11.1 | RCS-con-145 Group Chat automatic re-join | Applies |  |  |
| 5.11.2 | RCS-con-146 Group Chat re-start: 404 (Not Found) | Applies |  |  |
| 5.11.3 | RCS-con-147 Group Chat re-start: 403 (Forbidden) | Applies |  |  |
| 5.11.4 | RCS-con-148 Group Chat: Concurrent sessions | Applies |  |  |
| 5.12.1 | RCS-con-200 Mobile Originated File Transfer | Applies |  |  |
| 5.12.2 | RCS-con-201 Mobile Originated File Transfer – Rejected | Applies |  |  |
| 5.13.1 | RCS-con-210 Mobile Terminated File Transfer | Applies |  |  |
| 5.13.2 | RCS-con-211 Mobile Terminated File Transfer – Rejected | Applies |  |  |
| 5.13.3 | RCS-con-212 Mobile Terminated File Transfer – File Size Limit | Applies |  |  |
| 5.13.4 | RCS-con-213 Mobile Terminated File Transfer – File Size Warn Limit | Applies |  |  |
| 5.14.1 | RCS-con-250 Mobile Originated Image Sharing Session Establishment – Mobile Originated Session Termination | Applies |  |  |
| 5.14.2 | RCS-con-251 Mobile Originated Image Sharing - Session Establishment - rejected | Applies |  |  |
| 5.15.1 | RCS-con-270 Mobile Terminated Image Sharing - Session Establishment – Mobile Terminated Session | Applies |  |  |
| 5.15.2 | RCS-con-271 Mobile Terminated Image Sharing - Session Establishment - rejected | Applies |  |  |
| 5.16.1 | RCS-con-300 Mobile Originated Video Sharing - Session Establishment – Mobile Originated Session Termination | Applies |  |  |
| 5.16.2 | RCS-con-301 Mobile Originated Video Sharing - Session Establishment – Mobile Terminated Session Termination | Applies |  |  |
| 5.16.3 | RCS-con-302 Mobile Originated Video Sharing - Session Establishment - Rejected | Applies |  |  |
| 5.17.1 | RCS-con-320 Mobile Terminated Video Sharing - Session Establishment – Mobile Terminated Session Termination | Applies |  |  |
| 5.17.2 | RCS-con-321 Mobile Terminated Video Sharing - Session Establishment – Mobile Originated Session Termination | Applies |  |  |
| 5.17.3 | RCS-con-322 Mobile Terminated Video Sharing - Session Establishment - Rejected | Applies |  |  |
| 5.18.1 | RCS-con-400 Mobile Originated IP Voice Call Set Up – Mobile Originated IP Voice Call Termination |  | Applies | Applies |