Change Request

| Title: | Use Case for Virtual Observe Notify Object | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA IPSO |  |
| Doc to Change: | OMA-TS-LwM2M_VirtualObserveNotify-V1_0-20180829-D |  |
| Submission Date: | 4 Sep 2018 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Hongna Chang, Huawei, changhongna@huawei.com Bei Xu, Huawei, echo.xubei@huawei.com |  |
| Replaces: | n/a |  |

Reason for Change

The purpose of this CR is to introduce use cases of Virtual Observe Notify Object.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The DM WG should review and accept this CR.

Detailed Change Proposal

Add Virtual Observe Notify use cases

- Virtual Observe Notify Use cases

  - Overview

The Virtual Observe Notify Object provides a main function to notify multiple resources to a LwM2M Server in one notification message whether the resources are in one object/object instance or across multiple objects.

The Virtual Observe Notify Object also provides some functions to implement more efficient multi-resource report. For example, the notification message send by the LwM2M Client just includes changed resources or the resources which are the LwM2M Server wants to monitor. For more complex monitoring scenario, the LwM2M Client may need to monitor multiple resources, and when multiple resource meet these observed conditions at the same time, the LwM2M Client sends the notification message to the LwM2M Server.

  - Observe Multiple Resources

The Virtual Observe Notify Object provides a function to observe multiple resources which are across multiple objects or in the same object instance. This object is special in that its Observe operations on multiple resources are implemented by using the Write operation.

For example, the LwM2M Server wants to observe the Radio Signal Strength (/4/0/2) and Batter Level(/3/0/9) resources. The conditions of the observation are set that the Radio Signal Strength is less than -120dBm and the Battery Level is less than 50%.

Figure 1 shows an example flow where the LwM2M Server observe multiple resources

Figure 1: LwM2M Server Observe Multiple Resources

Step 1: The LwM2M Server sends the Write request to LwM2M Client to change the value of the Resource /22/0/0 (ObserveLinks). The request payload is a array of application/link-format CoRE Links where each element points to a resource or object instance to be observed and its observation criteria. In this example, the payload is the resources where are Radio Signal Strength (/4/0/2) and Batter Level (/3/0/9) with their “lt” Attributes.

Step 2: The LwM2M Client sends the ACK to the LwM2M Server.

Step 3: The LwM2M Server sends the Write-attribute request to the LwM2M Client to update the Attributes attached to the Resource /22/0/1 (Report). This resource is populated by the LwM2M Client each time an observed resource from ObserveLinks meets its observation criteria.

Step 4: The LwM2M Client sends the ACK to the LwM2M Server.

  - Notify Multiple Resources

After multiple resources across different objects or in one object instance are observed by the LwM2M Server, the LwM2M Server wants to receive these resources in one notification each time. So the LwM2M Server obtain these resources through observing the Resource /22/0/1 (Report).

Figure 2 shows an example flow where the LwM2M Client reports multiple resources in a notification message.

Figure 2: LwM2M Client Reports Multiple Resources in a Notification Message

Note: The LwM2M Server has observed multiple resource through Write operation to the Resource /22/0/0.

Step 1: The LwM2M Server initiates an observation request for changes of the Resource /22/0/1 within the LwM2M Client.

Step 2: The LwM2M Client sends the ACK to the LwM2M Server.

Step 3: The LwM2M Client sends the Notify message to the LwM2M Server. The Report Resource is populated by the LwM2M Client each time an observed resource from ObserveLinks Resource meets its observation criteria, thereby resulting in a notification to the LwM2M Server that is observing Report Resource.

  - Observe Multiple Resources and Notify the Same Resources

Multiple resources can be send to the LwM2M Server in one notification message. No matter whether these resource are across multiple objects or in the same object instance, there are no difference in its implementation.

Figure 3 shows an example flow where the Server observe multiple resources and the Client reports the same resources.

Figure 3: LwM2M Server Observe Multiple Resources and LwM2M Client Reports the Same Resources

Step 1: The LwM2M Server sends the Write request to LwM2M Client to change the value of the Resource /22/0/0 (ObserveLinks). The request payload is the array of resources where are Radio Signal Strength (/4/0/2) and Batter Level (/3/0/9) with their “lt” Attributes.

Step 2: The LwM2M Client sends the ACK to the LwM2M Server.

Step 3: The LwM2M Server sends the Write-attribute request to the LwM2M Client to update the Attributes attached to the Resource /22/0/1 (Report).

Step 4: The LwM2M Client sends the ACK to the LwM2M Server.

Step 5: The LwM2M Server initiates an observation request for the Resource /22/0/1 within the LwM2M Client.

Step 6: The LwM2M Client sends the ACK to the LwM2M Server.

Step 7: The LwM2M Client sends the Notify message to the LwM2M Server. The Report Resource is populated by the LwM2M Client each time the Radio Signal Strength (/4/0/2) and Batter Level (/3/0/9) meets its observation criteria.

  - Observe Multiple Resources and Notify the Changed Resource

The LwM2M Server can monitor the LwM2M Client multiple resource, which are in one object instance or across multiple objects, by the Virtual Observe Notify Object. It is not necessary for the LwM2M Client to report all resources observed by the LwM2M Server each time, because some resources may not change or change very little during the observation period. The Virtual Observe Notify Object provides a function to notify resources which are be changed compared to last notification message.

For example, the LwM2M Server wants to monitor the status of the LwM2M Client, the LwM2M Client needs to report the information of the device and the status of the network connectivity to the LwM2M Server periodically. The LwM2M Server observes the Device Object instance 0 and Connectivity Monitoring Object instance 0 by the Virtual Observe Notify Object. The notification message just includes changed Resources of the observed Object instance.

Figure 4 shows an example flow where the Server observe multiple resources and the Client reports changed resources.

Figure 4: LwM2M Server Observe Multiple Resources and LwM2M Client Reports Changed Resources

Step 1: The LwM2M Server sends the Write request to LwM2M Client to change the value of the Resource /22/0/0 (ObserveLinks). The request payload is a list of application/link-format CoRE Links for targeted Objects, Object Instances, or Resources, along with their Attributes. In this example, the payload is the ID of Device Object Instance 0 and Connectivity Monitoring Object Instance 0 and their values.

Step 2: The LwM2M Client sends the ACK to the LwM2M Server.

Step 3: The LwM2M Server sends the Write request to the LwM2M Client to update the Resource /22/0/2 (ResourceFiltering). This resource is used by the LwM2M Server to indicate the LwM2M Client whether report all resources observed in ObserveLinks Resource or changed resources in ObserveLinks. 0 represents reporting all resources, 1 represents reporting only changed resources compared with last notification.

Note: The LwM2M Server can send Write-Composite request to LwM2M Client to change the values of Resource /22/0/0 and Resource /22/0/2 simultaneously. In other words, step 1 and step 3 can be combined into one request message including Resource /22/0/0 and Resource /22/0/2.

Step 4: The LwM2M Client sends the ACK to the LwM2M Server.

Step 5: The LwM2M Server sends the Write-attribute request to the LwM2M Client to update the Attributes’ values of Resource /22/0/1 (Report).

Step 6: The LwM2M Client sends the ACK to the LwM2M Server.

Step 7: The LwM2M Server initiates an observation request for changes of the Resource /22/0/1 within the LwM2M Client.

Step 8: The LwM2M Client sends the ACK to the LwM2M Server.

Step 9: The LwM2M Client sends the Notify message to the LwM2M Server. The Report Resource is populated by the LwM2M Client each time observed resources from ObserveLinks Resource meet their observation criteria and are changed compared with last notification, thereby resulting in a notification to the LwM2M Server that is observing Report Resource.

  - Observe Multiple Resources and Notify the Partial Resources

Although the LwM2M Server observes multiple resources, the LwM2M Server just needs to receive partial resources, and the other partial resources are only used as the observation conditions.

  - Observe Multiple Resources and Notify Different Resources

Although the LwM2M Server observes multiple resources, the LwM2M Server wants to receive other resources, and the observed resources are only used as the observation conditions.

Add resources of the Observe Notify Object

- LWM2M Object: Virtual Observe Notify

Description

The Virtual Observe Notify Object provides a main function to notify multiple resources to a LwM2M Server in one notification message whether the resources are in one object/object instance or across multiple objects. The Virtual Observe Notify Object also provides some functions to implement more efficient multi-resource report.

Object definition

Name; Object ID; Instances; Mandatory; Object URN. Virtual Observe Notify; 22; Multiple; Optional; urn:oma:lwm2m:oma:22:1.0.

Resource definitions

ID; Name; Operations; Instances; Mandatory; Type; Range or Enumeration; Units; Description. 0; ObserveLinks; RW; Single; Mandatory; Corelnk; Array of Core Links where each element points to a resource or object instance to be observed and its observation criteria (lt, gt, st, pmin, pmax).. 1; Report; R; Single; Mandatory; Opaque; This resource is populated by the LwM2M Client each time an observed resource from ObserveLinks meets its observation criteria, thereby resulting in a notification to a LwM2M Server that is observing Report resource. 2; ResourceFiltering; RW; Single; Optional; Boolean; Used by the LwM2M Server to indicate the LwM2M Client whether report all instances of ObserveLinks or changed instances of ObserveLinks. 0 represents reporting all resources, 1 represents reporting only changed resources compared with last notification.. 3; ReportLinks; RW; Single; Optional; Corelnk; Array of resource Links where each element identifies the resource to be included in the notification. If ReportLinks resource is absent, the notification will include the resources that are observed. If the ReportLinks resource is present, the notification will only include the resources that are listed in ReportLinks. To provide maximum flexibility to the server the Notifications will not comprise the union of resources in ObserveLinks and ReportLinks when both present.. 4; ObserveRelation; RW; Single; Optional; String; Used by the server to indicate the AND/OR relationship between or among instances of ObserveLinks.