Change Request

| Title: | Bug Fix for Virtual Observe Notify Object | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA IPSO |  |
| Doc to Change: | OMA-TS-LwM2M_VirtualObserveNotify-V1_0-20181022-D |  |
| Submission Date: | 31 Oct 2018 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Hongna Chang, Huawei, changhongna@huawei.com Bei Xu, Huawei, echo.xubei@huawei.com |  |
| Replaces: | n/a |  |

Reason for Change

The purpose of this CR is to fix R&A comments and some comments from our application developers.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The DM WG should review and accept this CR.

Detailed Change Proposal

Clarify the report rule when ResourceFilter is set to 1

## Observe Multiple Resources and Notify the Changed Resource

This use case descripts the procedure when the LwM2M Server wants to get the changed Resources in the multiple Resources. It specifies the usage of Resource “ResourceFilter”.

The LwM2M Server initiates a Write request for changes of the Object Instant (/22/0), the payload includes ObserveLinks (/22/0/0) and ResourceFilter (/22/0/2).

For example, the DM server (LwM2M Server) needs changed Resources of the Device Object (/3/0) and Connectivity Monitoring Object (/4/0) of the water meter (LwM2M Client) via NB-IoT to monitor the device.

Figure 2: Example flow for Observing Multiple Resources and Notifying Changed Resources

Step 1: The LwM2M Server sends the Write request to LwM2M Client to change the value of the Object instance /22/0. The request payload includes the Resource ObserveLinks (/22/0/0) and Resource ResourceFilter (/22/0/2). In this example, the value of the Resource ObserveLinks are Device Object Instance 0 and Connectivity Monitoring Object Instance 0; the value of the Resource ResourceFilter is 1.

Note: The LwM2M Server can send Write-Composite request to LwM2M Client to change the values of Resource /22/0/0 and Resource /22/0/2 simultaneously.

Step 2: The LwM2M Client sends the ACK to the LwM2M Server and starts to observe the two object instance. When any Resource of two Objects is changed compared with the last notification message and meets its condition set by the LwM2M Server using R-Attributes, the LwM2M Client writes the changed Resources into the Resource Report (/22/0/1).

Note: If any Resource of two Objects is not changed which is compared with the value in the last notification message, the LwM2M Client doesn’t write any Resource into the Resource Report (/22/0/1).

Step 3: The LwM2M Server sends the Write-attribute request to the LwM2M Client to update the Attributes’ values of Resource /22/0/1 (Report).

Step 4: The LwM2M Client sends the ACK to the LwM2M Server.

Step 5: The LwM2M Server initiates an observation request for changes of the Resource /22/0/1 within the LwM2M Client.

Step 6: The LwM2M Client sends the ACK to the LwM2M Server.

Step 7: The LwM2M Client sends the Notify message to the LwM2M Server including the changed resources in the Resource ObserveLink.

## Observe Multiple Resources and Notify Different Resources

This use case descripts the procedure when the LwM2M Server wants to get the different Resources from observed the multiple Resources. It specifies the usage of Resource “ReportLinks” and “ObserveRelation”.

The LwM2M Server initiates a Write request for changes of the Object Instant (/22/0), the payload includes ObserveLinks (/22/0/0), ReportLinks (/22/0/3) and ObserveRelation (/22/0/4).

For example, the shared bike operator need protect the shared bike security. The LwM2M Server needs location (/6/0/1 and /6/0/2) of Device Object (/6/0) when the state of the lock is switched-off and the speed of the client is more than 0. The notification may trigger the LwM2M Server to trace the client and or do other processing.

Figure 3: Example flow for Observing Multiple Resources and Notifying Different Resources

Step 1: The LwM2M Server sends the Write request to LwM2M Client to change the value of the Object instance /22/0. The request payload includes the Resource ObserveLinks (/22/0/0), ReportLinks (/22/0/3) and ObserveRelation (/22/0/4). In this example, the value of the Resource ObserveLinks are the Resource Speed (/6/0/6) of Location Object (/6/0) with the “gt” attribute, and the Digital Input State resource (/3342/0/5500) of On/Off switch Object (/3342/0) with the “lt” attribute. The value of the Resource ReportLinks are the Resource Latitude (/6/0/0) and the Resource Longitude (/6/0/1). The value of ObserveRelation is 1.

Note: The LwM2M Server can send Write-Composite request to LwM2M Client to change the values of Resource /22/0/0, Resource /22/0/4, and Resource /22/0/3 simultaneously.

Step 2: The LwM2M Client sends the ACK to the LwM2M Server and starts to observe the two resources. When the Digital Input State resource is OFF and the Resource Speed is more than 0, the LwM2M Client writes the value of the Resource Latitude (/6/0/0) and Resource Longitude (/6/0/1) into the Resource Report (/22/0/1).

Step 3: The LwM2M Server sends the Write-attribute request to the LwM2M Client to update the Attributes’ values of Resource /22/0/1 (Report).

Step 4: The LwM2M Client sends the ACK to the LwM2M Server.

Step 5: The LwM2M Server initiates an observation request for changes of the Resource /22/0/1 within the LwM2M Client.

Step 6: The LwM2M Client sends the ACK to the LwM2M Server.

Step 7: The LwM2M Client sends the Notify message to the LwM2M Server including the resources in the Resource ReportLinks..

Resource definitions

| ID | Name | Operations | Instances | Mandatory | Type | Range or Enumeration | Units | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | ObserveLinks | RW | Multiple | Mandatory | Corelnk |  |  | Array of Core Links where each element points to a resource or object instance to be observed and its observation criteria (lt, gt, st, pmin, pmax). When the LwM2M Server write the resources or object instances to this Resource, the LwM2M Client starts to observe these resource. When the LwM2M Server write empty string to this Resource, the LwM2M Client stop to observe resources. |  |  |
| 1 | Report | R | Single | Mandatory | String |  |  | This resource is populated by the LwM2M Client each time an observed resource from ObserveLinks meets its observation criteria, thereby resulting in a notification to a LwM2M Server that is observing Report resource. The string format MUST be SenML JSON |  |  |
| 2 | ResourceFilter | RW | Single | Optional | Boolean |  |  | Used by the LwM2M Server to indicate that the LwM2M Client send all resources or changed resources in the ObserverLinks. •false: report all Resources, •true: report only changed Resources. When it is absent, the LwM2M Client shall report all Resources as default. |  |  |
| 3 | ReportLinks | RW | Multiple | Optional | Corelnk |  |  | Array of resource Links where each element identifies the resource to be included in the notification. If ReportLinks Resource is absent, the notification will include the resources in ObserveLinks.. If the ReportLinks Resource is present, the notification will only include the resources that are listed in ReportLinks. To provide maximum flexibility to the server the Notifications will not comprise the union of resources in ObserveLinks and ReportLinks when both present. |  |  |
| 4 | ObserveRelation | RW | Single | Optional | Integer |  |  | Used by the server to indicate the AND/OR relationship between or among instances of ObserveLinks. •0: OR •1: AND When it is absent, the relationship among the instances of ObserveLinks is OR as default. |  |  |