Change Request

| Title: | Bug Fix for Virtual Observe Notify Object | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA IPSO |  |
| Doc to Change: | OMA-TS-LwM2M_VirtualObserveNotify-V1_0-20190203-D |  |
| Submission Date: | 22 Mar. 2019 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Hongna Chang, Huawei, changhongna@huawei.com |  |
| Replaces: | n/a |  |

Reason for Change

The purpose of this CR is bug fix.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The IPSO WG should review and accept this CR.

Detailed Change Proposal

Modify JSON format

  - Observe and Notify Multiple Resources

This use case descripts the procedure when the LwM2M Server wants to get the report of multiple Resources of different Objects. It specifies the usage of Resource “ObserveLinks” and “Report”.

The LwM2M Server initiates a Write request for changes of the Resource ObserveLinks (/22/0/0), which includes the Resources within an Object or different Objects within the LwM2M Client.

For example, the LwM2M Server wants to get the report of the Radio Signal Strength (/4/0/2) and Battery Level (/3/0/9) resources when the Radio Signal Strength is less than -120dBm or the Battery Level is less than 50%.

Figure 1 - Example flow for observing and notifying multiple Resources

Step 1: The LwM2M Server sends the Write-Composite request to LwM2M Client to observe multiple resources which are included in the Resource ObserveLinks (/22/0/0). The payload in the request is an array of application/link-format CoRE Links. In this example, the payload includes Radio Signal Strength (/4/0/2) and Batter Level (/3/0/9) with their “lt” Attributes.

…...

  - Observe Multiple Resources and Notify Changed Resources

This use case descripts the procedure when the LwM2M Server wants to get the changed Resources in the multiple Resources. It specifies the usage of Resource “ResourceFilter”.

The LwM2M Server initiates a Write request for changes of the Object Instant (/22/0), the payload includes ObserveLinks (/22/0/0) and ResourceFilter (/22/0/2).

For example, the DM server (LwM2M Server) needs changed Resources of the Device Object (/3/0) and Connectivity Monitoring Object (/4/0) of the water meter (LwM2M Client) via NB-IoT to monitor the device.

Figure 2 - Example flow for Observing Multiple Resources and Notifying Changed Resources

Step 1: The LwM2M Server sends the Write-Composite request to LwM2M Client to observe multiple resources which are included in the Resource ObserveLinks (/22/0/0). The request payload includes the Resource ObserveLinks (/22/0/0) and Resource ResourceFilter (/22/0/2). In this example, the value of the Resource ObserveLinks are Device Object Instance 0 and Connectivity Monitoring Object Instance 0; the value of the Resource ResourceFilter is 1.

…...

  - Observe Multiple Resources and Notify Different Resources

This use case descripts the procedure when the LwM2M Server wants to get the different Resources from observed the multiple Resources. It specifies the usage of Resource “ReportLinks” and “ObserveRelation”.

The LwM2M Server initiates a Write request for changes of the Object Instant (/22/0), the payload includes ObserveLinks (/22/0/0),  ReportLinks (/22/0/3) and ObserveRelation (/22/0/4).

For example, the shared bike operator need protect the shared bike security. The LwM2M Server needs location (/6/0/1 and /6/0/2) of Device Object (/6/0) when the state of the lock is switched-off and the speed of the client is more than 1. The notification may trigger the LwM2M Server to trace the client and or do other processing.

Figure 3 - Example flow for Observing Multiple Resources and Notifying Different Resources

Step 1: The LwM2M Server sends the Write-Composite request to LwM2M Client to observe multiple resources which are included in the Resource ObserveLinks (/22/0/0). The request payload includes the Resource ObserveLinks (/22/0/0), ReportLinks (/22/0/3) and ObserveRelation (/22/0/4). In this example, the value of the Resource ObserveLinks are the Resource Speed (/6/0/6) of Location Object (/6/0) with the “gt” attribute, and the Digital Input State resource (/3342/0/5500) of On/Off switch Object (/3342/0) with the “lt” attribute. The value of the Resource ReportLinks are the Resource Latitude (/6/0/0) and the Resource Longitude (/6/0/1). The value of ObserveRelation is 1.

…...

  - Observe Multiple Resources and Notify Partial Resources

This use case descripts the procedure when the LwM2M Server wants to get the partial Resources in the multiple Resources. It specifies the usage of Resource “ReportLinks” and “ObserveRelation”.

The LwM2M Server initiates a Write request for changes of the Object Instant (/22/0), the payload includes ObserveLinks (/22/0/0), ReportLinks (/22/0/3) and ObserveRelation (/22/0/4).

For example, the LwM2M Server want to check the NB-IoT network coverage, and ask the LwM2M Client to report Radio Signal Strength when Network Bearer is NB-IoT and Radio Signal Strength is less than one threshold.

Figure 4 - Example flow for Observing Multiple Resources and Notifying Partial Resources

Step 1: The LwM2M Server sends the Write-Composite request to LwM2M Client to observe multiple resources which are included in the Resource ObserveLinks (/22/0/0). The request payload includes the Resource ObserveLinks (/22/0/0), ReportLinks (/22/0/3) and ObserveRelation (/22/0/4). In this example, the value of the Resource ObserveLinks are the Resource Network Bearer (/4/0/0) of Connectivity Monitoring (/4/0) with the “gt” and “lt” attribute, and the Resource Radio Signal Strength (/4/0/2) of Connectivity Monitoring (/4/0) with the “lt” attribute . The value of the Resource ReportLinks are the Resource Radio Signal Strength (/4/0/2). The value of ObserveRelation is 1.

…...