Change Request

| Title: | ConvTS_Section7 | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20130123-D |  |
| Submission Date: | 29 January 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com |  |
| Replaces: | N/A |  |

Reason for Change

Per CONRR comments

- Lacking deferred delivery and rejoin statement

- Adding clarification to the CPM Client to support deferred delivery when receiving Feature Tag ‘3gpp-service.ims-icsi.oma.cpm.deferred’.

R01 – online edits during Budapest mtg

| D023 | 2013.02.13 | T | 7 | Source: NSN Form: CONRR CPM2.0 Comment: Lacking deferred delivery and rejoin statement Adding clarification to the CPM Client to support deferred delivery when receiving Feature Tag ‘3gpp-service.ims-icsi.oma.cpm.deferred’ Proposed Change: See CR11 | Status: CLOSED by CR#11R01 |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Conversation Function TS

Detailed Change Proposal

Section 7

# Procedures at CPM Client

A CPM Client provides the CPM User with an interface to access the functionalities of the CPM Enabler.

Before a CPM Client can provide CPM functionalities to the CPM User it has to register to the SIP/IP core as described in section 7.1 “Registering at the SIP/IP Core”.

A CPM Client can initiate new communications in a number of ways, as follows:

- When a CPM Client needs to send a Pager Mode CPM Standalone Message, it SHALL follow the procedures described in section 7.2.1.1 “Sending a Pager Mode CPM Standalone Message”.

- When a CPM Client needs to send a Large Message Mode CPM Standalone Message, it SHALL follow the procedures described in section 7.2.1.2 “Sending a Large Message Mode CPM Standalone Message”.

- When a CPM Client needs to initiate a new CPM Session or rejoin a previous CPM Session, it SHALL follow one of the procedures described in section 7.3.1 “Initiating New CPM Sessions”, depending on what kind of CPM Session is to be initiated.

- When a CPM Client needs to initiate a new CPM File Transfer, it SHALL follow the procedures defined in section 7.4.1 “CPM File Transfer Session Initiation”.

- When a CPM Client needs to handle Deferred CPM Message(s), it SHALL follow the procedures defined in section 7.2.3.2 “Handling Deferred CPM Message(s)”.

- When a CPM Client needs to handle Deferred CPM 1-1 Session Message(s) delivery, it SHALL follow the procedures defined in section 7.3.12 “Handling Deferred CPM 1-1 Session Message Delivery”.

- When a CPM Client needs to handle Deferred CPM Group Session Message(s) delivery, it SHALL follow the procedures defined in section 7.3.11 “Handling Deferred CPM Group Session Message Delivery”.

A CPM Client handles incoming CPM communications in the following manners:

- Upon receiving a SIP MESSAGE request with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.msg’ or with CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.deferred’ (defined in Appendix H) included in the Accept-Contact header, the CPM Client SHALL handle this SIP MESSAGE request as described in section 7.2.2.1 “Receiving a Pager Mode CPM Standalone Message”,

- Upon receiving a SIP INVITE request with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.largemsg’ or with CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.deferred’ (defined in Appendix H) included in the Accept-Contact header, the CPM Client SHALL handle this SIP INVITE request as described in section 7.2.2.2 “Receiving a Large Message Mode CPM Standalone Message”,

- Upon receiving a SIP INVITE request with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.session’ or with CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.deferred’ (defined in Appendix H) included in the Accept-Contact header, the CPM Client SHALL handle this SIP INVITE request as described in section 7.3.2 ”Receiving a CPM Session Invitation”,  or section 7.3.11 “Handling Deferred CPM Group Session Message Delivery or section 7.3.12 “Handling Deferred CPM 1-1 Session Message Delivery”,

- Upon receiving a SIP INVITE request with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.filetransfer’ (defined in Appendix H) included in the Accept-Contact header, the CPM Client SHALL handle this SIP INVITE request as described in section 7.4.2 “Receiving a CPM File Transfer Request”,

The CPM Client SHALL include a User-Agent header to indicate the OMA CPM release version of the CPM Client as specified in Appendix D “Release Version in User-agent and Server headers”.