# Change Request

| Title: | D034 & D099 Comment_ConvTS | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20130225-D |  |
| Submission Date: | 7 MAR. 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com |  |
| Replaces: | n/a |  |

Reason for Change

| D034 | 2013.02.13 | T | App B1. B.2 B.3 | Source: NSN Form: CONRR CPM2.0 Comment: Add SCR tables for CPM Client, CPM Participant Function, and CPM Controlling Function. Proposed Change: See CR13 | Status: CLOSED BY CR13 |
| --- | --- | --- | --- | --- | --- |

| D099 | 2013.02.13 | T | Appendix B | Source: Ericsson Form: OMA-CONR-2013-0002 Comment: update SCR tables Proposed Change: update SCR tables | Status: CLOSED BY CR13 |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Conversation Function TS and close the CONRR comments addressed in section 1 of the CR.

Detailed Change Proposal

## D34 and D99 comments

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [OMA-SCRRULES].

SCR for CPM Client

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| - CPM-CF-C-001-M | Registering | 7.1 |  |
| CPM-CF-C-002-M | Sending a Pager Mode CPM Standalone Message to one recipient | 7.2.1.1 |  |
| CPM-CF-C-003-M | Sending a Pager Mode CPM Standalone Message to a CPM Ad-hoc Group | 7.2.1.1 |  |
| CPM-CF-C-004-M | Sending a Pager Mode CPM Standalone Message to a CPM Pre-defined Group | 7.2.1.1 |  |
| CPM-CF-C-005-M | Sending a Large Message Mode CPM Standalone Message to one recipient | 7.2.1.2 |  |
| CPM-CF-C-006-M | Sending a Large Message Mode CPM Standalone Message to a CPM Ad-hoc Group | 7.2.1.2 |  |
| CPM-CF-C-007-M | Sending a Large Message Mode CPM Standalone Message to a CPM Pre-defined Group | 7.2.1.2 |  |
| CPM-CF-C-008-O | Forwarding/Including Stored Data without Downloading to the CPM Client – Pager Mode | 7.2.1.4 |  |
| CPM-CF-C-009-O | Forwarding/Including Stored Data without Downloading to the CPM Client – Large Message Mode | 7.2.1.4 |  |
| CPM-CF-C-010-M | Receiving Pager Mode CPM Standalone Message | 7.2.2.1 |  |
| CPM-CF-C-011-M | Receiving Large Message Mode CPM Standalone Message | 7.2.2.2 |  |
| CPM-CF-C-012-M | Subscribing to Deferred CPM Message Information | 7.2.3.1 |  |
| CPM-CF-C-013-M | Handling selected Deferred CPM Messages before expiry | 7.2.3.2.1 |  |
| CPM-CF-C-014-M | Retrieving all Deferred CPM Messages before expiry | 7.2.3.2.1 |  |
| CPM-CF-C-015-O | Retrieving CPM Standalone Messages after expiry | 7.2.3.2.2 |  |
| CPM-CF-C-016-O | Handling expiry notifications for Deferred CPM Messages | 7.2.3.3 |  |
| CPM-CF-C-017-M | Out-of-band notification handling | 7.2.3.4.1 |  |
| CPM-CF-C-018-M | In-band notification handling | 7.2.3.4.2 |  |
| CPM-CF-C-019-M | Generate Delivery Notification | 5.4.1 |  |
| CPM-CF-C-020-M | Generate Read Report | 5.4.2 |  |
| CPM-CF-C-021-M | Receive Delivery Notification | 5.4.3 |  |
| CPM-CF-C-022-M | Receive Read Report | 5.4.4 |  |
| CPM-CF-C-023-M | Initiating a CPM 1-1 Session | 7.3.1.1 |  |
| CPM-CF-C-021-M | Initiating a CPM Group Session for a CPM Ad-hoc Group | 7.3.1.2 |  |
| CPM-CF-C-022-M | Initiating a CPM Group Session for a CPM Pre-defined Group | 7.3.1.3 |  |
| CPM-CF-C-023-M | Join a CPM Group Session for a Join-in Group | 7.3.1.4 |  |
| CPM-CF-C-024-M | Receiving a CPM Session Initiation | 7.3.2 |  |
| CPM-CF-C-025-M | Extending a CPM 1-1 Session to a CPM Group Session | 7.3.3 |  |
| CPM-CF-C-026-M | Sending a CPM 1-1 Session Closure | 7.3.4.1 |  |
| CPM-CF-C-027-M | Leaving a CPM Group Session | 7.3.4.2 |  |
| CPM-CF-C-028-M | Receiving a CPM Session Closure | 7.3.4.3 |  |
| CPM-CF-C-029-M | Inviting one User to existing CPM Group Session | 7.3.5 |  |
| CPM-CF-C-030-M | Inviting more than one User to existing CPM Group Session | 7.3.5 |  |
| CPM-CF-C-031-M | Removing one Participant from a CPM Group Session | 7.3.6 |  |
| CPM-CF-C-032-O | Removing more than one Participant from a CPM Group Session | 7.3.6 |  |
| CPM-CF-C-033-M | Modifying a CPM Session | 7.3.7 |  |
| CPM-CF-C-034-M | CPM Client receiving an CPM Session modification request | 7.3.8 |  |
| CPM-CF-C-035-M | CPM Session Media Plane Handling – MSRP | 7.3.9.1 |  |
| CPM-CF-C-036-M | CPM Session Media Plane Handling – RTP/RTCP | 7.3.9.2 |  |
| CPM-CF-C-037-M | Subscribe to Receiving Participant Information | 7.3.10.1 |  |
| CPM-CF-C-038-M | Receive Participant Information Notification | 7.3.10.2 |  |
| CPM-CF-C-039-M | Initiating a CPM File Transfer to a single recipient | 7.4.1 |  |
| CPM-CF-C-040-M | Initiating a CPM File Transfer to a CPM Ad-hoc Group | 7.4.1 |  |
| CPM-CF-C-041-M | - Initiating a CPM File Transfer to a CPM Pre-defined Group | 7.4.1 |  |
| CPM-CF-C-042-M | Receiving a CPM File Transfer | 7.4.2 |  |
| CPM-CF-C-043-M | Releasing a CPM File Transfer | 7.4.3 |  |
| CPM-CF-C-044-M | Fetching Deferred CPM File Transfer file(s) | 7.4.4 |  |
| CPM-CF-C-045-M | Resuming an interrupted CPM File Transfer | 7.4.5 |  |
| CPM-CF-C-046-M | Handling “isComposing” | 5.5 |  |
| CPM-CF-C-047-M | Handling Deferred CPM Group Session message delivery | 7.3.11 |  |
| CPM-CF-C-048-M | Handling Deferred CPM 1-1 Session message delivery | 7.3.12 |  |
| CPM-CF-C-049-M | Handling Re-joining CPM Long-live Group Session | 7.3.1.5 |  |
| CPM-CF-C-050-M | Handle CPM Session Closed Group Chat | 7.3.1 | CPM-CF-C-021-M AND CPM-CF-C-030-M |

SCR for CPM Participating Function

| Item | Function |  | Reference |  | Requirement |  |
| --- | --- | --- | --- | --- | --- | --- |
| CPM-CF-PF-S-001-M | Handle registration events |  | 8.1 |  |  |  |
| CPM-CF-PF-S-002-M | Handle Pager Mode CPM Standalone Message in originating network |  | 8.2.1.1 |  |  |  |
| CPM-CF-PF-S-003-O | Fetch and include objects from Message Storage Server |  | 8.2.1.1 |  |  |  |
| CPM-CF-PF-S-004-O | Extend Pager Mode to Large Message Mode |  | 8.2.1.1 |  |  |  |
| CPM-CF-PF-S-005-M | Handle Large Message Mode CPM Standalone Message in the originating network |  | 8.2.1.2 |  |  |  |
| CPM-CF-PF-S-006-O | Fetch and include objects from Message Storage Server |  | 8.2.1.2 |  |  |  |
| CPM-CF-PF-S-007-M | Handle Disposition Notification in the originating network |  | 8.2.4 & 8.2.5 |  |  |  |
| CPM-CF-PF-S-008-M | Handle a CPM Session Invitation in the originating network |  | 8.2.2.1 |  |  |  |
| CPM-CF-PF-S-009-M | Handle a SIP CANCEL request in the originating network |  | 8.2.2.2 |  |  |  |
| CPM-CF-PF-S-010-M | Handle a SIP BYE request in the originating network |  | 8.2.2.3 |  |  |  |
| CPM-CF-PF-S-011-M | Handle SIP session expiry in the originating network |  | 8.2.2.4 |  |  |  |
| CPM-CF-PF-S-012-M | Handle CPM Session modification request in the originating network |  | 8.2.2.5 |  |  |  |
| CPM-CF-PF-S-013-M | Handle CPM File Transfer initiation in the originating network |  | 8.2.3.1 |  |  |  |
| CPM-CF-PF-S-014-M | Handle CPM File Transfer closing in the originating network |  | 8.2.3.2 |  |  |  |
| CPM-CF-PF-S-015-M | Handle Pager Mode CPM Standalone Message in the terminating network |  | 8.3.1.1 |  |  |  |
| CPM-CF-PF-S-016-M | Handle Large Message Mode CPM Standalone Message in the terminating network |  | 8.3.1.2 |  |  |  |
| CPM-CF-PF-S-017-O | Replacing Media with a reference |  | 8.3.1.3 |  |  |  |
| CPM-CF-PF-S-018-M | Sending disposition notifications |  | 8.3.1.5 |  |  |  |
| CPM-CF-PF-S-019-M | Defer CPM Standalone Messages |  | 8.3.1.6 |  | CPM-CF-PF-S-020-O OR CPM-CF-PF-S-021-O OR CPM-CF-PF-S-022-O |  |
| CPM-CF-PF-S-020-O | Pushing Deferred CPM Messages |  | 8.3.1.6.2 |  |  |  |
| CPM-CF-PF-S-021-O | Sending notifications and awaiting CPM Client action |  | 8.3.1.6.3 |  |  |  |
| CPM-CF-PF-S-022-O | Delivering Deferred CPM Message to the Message Storage Server |  | 8.3.1.6.4 |  |  |  |
| CPM-CF-PF-S-023-M | Handle Deferred Message Information Request |  | 8.3.1.6.5 |  |  |  |
| CPM-CF-PF-S-024-M | Handling Deferred CPM Messages on expiry time |  | 8.3.1.6.8 |  |  |  |
| CPM-CF-PF-S-025-O | Sending expiry notifications |  | 8.3.1.6.8 |  |  |  |
| CPM-CF-PF-S-026-M | Handle a CPM Session Invitation in the terminating network |  | 8.3.2.1 |  |  |  |
| CPM-CF-PF-S-027-M | Handle a SIP CANCEL request in the terminating network |  | 8.3.2.2 |  |  |  |
| CPM-CF-PF-S-028-M | Handle a SIP BYE request in the terminating network |  | 8.3.2.3 |  |  |  |
| CPM-CF-PF-S-029-M | Handle SIP session expiry in the terminating network |  | 8.3.2.4 |  |  |  |
| CPM-CF-PF-S-030-M | Handle CPM Session modification request in the terminating network |  | 8.3.2.5 |  |  |  |
| CPM-CF-PF-S-031-M | Handle CPM File Transfer initiation in the terminating network |  | 8.3.3.1 |  |  |  |
| CPM-CF-PF-S-032-M | Handle CPM File Transfer closing in the terminating network |  | 8.3.3.2 |  |  |  |
| CPM-CF-PF-S-033-O | Record CPM Standalone Message |  | 8.5.1 |  |  |  |
| CPM-CF-PF-S-034-O | Record CPM Session |  | 8.5.2 |  |  |  |
| CPM-CF-PF-S-035-O | Record CPM File Transfer |  | 8.5.3 |  |  |  |
| CPM-CF-PF-S-036-M |  | Generate Delivery Notification |  | 5.4.1 |  |  |  |
| CPM-CF-PF-S-037-M |  | Handling CPM Long-lived Group Chat |  | 8.2.2.7.2 |  | CPM-CF-PF-S-010-M AND CPM-CF-PF-S-038-O AND CPM-CF-PF-S-039-M |  |
| CPM-CF-PF-S-038-O |  | Handle Participant Information |  | 8.2.2.6 |  |  |  |
| CPM-CF-PF-S-039-M |  | Handle CPM Group Session Re-join Request |  | 8.2.2.7 |  |  |  |
| CPM-CF-PF-S-040-M | Handle Deferred delivery CPM Group Session |  | 8.3.2.9 |  | CPM-CF-PF-S-038-O |  |
| CPM-CF-PF-S-041-M | Handle Deferred CPM File Transfer Files(s) |  | 8.3.3.3 |  | CPM-CF-PF-S-042-O AND CPM-CF-PF-S-043-O AND CPM-CF-PF-S-044-O AND CPM-CF-PF-S-045-O AND CPM-CF-PF-S-046-O |  |
| CPM-CF-PF-S-042-O | Delivering Deferred CPM File Transfer file(s) |  | 8.3.3.4 |  |  |  |
| CPM-CF-PF-S-043-O | Generate request for Deferred delivery of CPM File Transfer File(s) |  | 8.3.3.5 |  |  |  |
| CPM-CF-PF-S-044-O | Receive request for Deferred delivery of CPM File Transfer File(s) |  | 8.3.3.6 |  |  |  |
| CPM-CF-PF-S-045-O | Storing CPM File Transfer Deferred Information |  | 8.3.3.7 |  |  |  |
| CPM-CF-PF-S-046-O | Media Plane handling for MSRP Session |  | 8.3.3.9 |  |  |  |

SCR for CPM Controlling Function

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| CPM-CF-CF-S-001-M | Pager Mode CPM Message to CPM Ad-Hoc Group | 9.1.1 |  |
| CPM-CF-CF-S-002-M | Pager Mode CPM Message to CPM Pre-defined Group | 9.1.1 |  |
| CPM-CF-CF-S-003-M | Large Message Mode CPM Message to CPM Ad-Hoc Group | 9.1.2 |  |
| CPM-CF-CF-S-004-M | Large Message Mode CPM Message to CPM Pre-defined Group | 9.1.2 |  |
| CPM-CF-CF-S-005-M | Disposition Notification Handling | 9.1.3 |  |
| CPM-CF-CF-S-006-M | Ad-hoc Session Invitation Request | 9.2.1 |  |
| CPM-CF-CF-S-007-M | Pre-defined Group Session Invitation Request | 9.2.1 |  |
| CPM-CF-CF-S-008-M | Session Cancellation Request | 9.2.2 |  |
| CPM-CF-CF-S-009-M | Joining Pre-defined Group Session Request | 9.2.3 |  |
| CPM-CF-CF-S-010-M | Rejoining Group Session Request | 9.2.4 |  |
| CPM-CF-CF-S-011-M | Adding Participants Request | 9.2.5 |  |
| CPM-CF-CF-S-012-M | Removing Participant Request | 9.2.6 |  |
| CPM-CF-CF-S-013-M | Session Leaving Request | 9.2.7 |  |
| CPM-CF-CF-S-014-M | Group Session Modification | 9.2.8 |  |
| CPM-CF-CF-S-015-M | Group Session Ending Request | 9.2.9 |  |
| CPM-CF-CF-S-016-M | Group Session Media Plane Handling – MSRP | 9.2.12.1 |  |
| CPM-CF-CF-S-017-M | Group Session Media Plane Handling – RTP/RTCP | 9.2.12.2 |  |
| CPM-CF-CF-S-018-O | Pseudonyms in a CPM Group Session | 9.2.13 |  |
| CPM-CF-CF-S-019-M | CPM Group Session Information Request | 9.2.14.1 |  |
| CPM-CF-CF-S-020-M | Sending Participant Information notifications | 9.2.14.2 |  |
| CPM-CF-CF-S-021-M | Terminating the subscription | 9.2.14.3 |  |
| CPM-CF-CF-S-022-M | CPM File Transfer Handling | 9.3 |  |
| CPM-CF-CF-S-023-M | Handling CPM Long-lived Group Chat | 9.2.4 | CPM-CF-CF-S-010-M AND CPM-CF-CF-S-015-M AND CPM-CF-CF-S-024-M |
| CPM-CF-CF-S-024-M | Participant Removing Request | 9.2.11 |  |
| CPM-CF-CF-S-025-M | Handling CPM Session Closed Group Chat | 8.2.1 | CPM-CF-CF-S-006-M AND CPM-CF-CF-S-007-M AND CPM-CF-CF-S-008-M AND CPM-CF-CF-S-009-M AND CPM-CF-CF-S-010-M AND CPM-CF-CF-S-011-M AND CPM-CF-CF-S-013-M |