# Change Request

| Title: | Deferred delivery 1-1 Chat client side | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-AD-CPM-V2_0-20120926-D |  |
| Submission Date: | 18 Feb. 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com |  |
| Replaces: | n/a |  |

Reason for Change

R01 – online edits during Budapest mtg

| B002 | 2013.02.13 | T | 4.2 | Source: NSN Form: CONRR CPM2.0 Comment: The above bullet gave examples of unavailable CPM Client. Editor’s Note: “CPM Client is unavailable”, Need to define the “unavailable” term is FFS. Proposed Change: Remove editor note | Status: CLOSED by CR#19R01 |
| --- | --- | --- | --- | --- | --- |

| B004 | 2013.02.13 | T | 5.3.1.3 | Source: NSN Form: CONRR CPM2.0 Comment: Missing participant list information support during deferred delivery. Proposed Change: add the following bullets OLD TEXT: Temporarily keeping CPM Messages, CPM File Transfers files and associated metadata when the messages and/or files cannot be delivered. NEW TEXT: Temporarily keeping CPM Messages, CPM File Transfers files, participant information, and associated metadata when the messages and/or files cannot be delivered. | Status: CLOSED by CR#19R01 |
| --- | --- | --- | --- | --- | --- |

| B005 | 2013.02.13 | T | 5.3.1.7 | Source: NSN Form: CONRR CPM2.0 Comment: Add support for Session Info Object and Group State Object. Proposed Change: add the following bullets OLD TEXT: - CPM Session Histories including any Media Objects attached to them, and any disposition notifications exchanged during the CPM Session, NEW TEXT: - CPM Session Histories including any Media Objects attached to them, and any disposition notifications exchanged during the CPM Session, Session Info Object, and Group State Object, | Status: CLOSED with No ACTION (CR#19R01). |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Architecture TS and close the CONRR comments addressed in section 1 of the CR.

Detailed Change Proposal

Comment B002

## Version 2.0

Additional features are provided in this release, as follows:

- Chat enhancements:

  - Support for notifications within the chat such as “isComposing” and disposition notifications (i.e. delivered, displayed), with a seamless delivery for interworking with SIMPLE IM V2.0

  - Support for storing chat messages and associated notifications while a chat participant is not available (e.g. lost coverage, not registered in IMS) or missed chat

  - Support for delivery of stored chat messages and disposition notifications once the recipient becomes available

  - Additional types of group chat, closed group chat and long-lived group chat

  - Enhanced multi-device functionality for chat (forking in case of auto-answer)

  - Support for live recording of chat messages

  - Seamless chat experience (backward compatibility with SIMPLE IM V2.0)

  - Interworking over the NNI with SIMPLE IM V2.0 chat

- CPM File Transfer enhancements:

  - Support CPM File Transfer while having an ongoing CPM Session.

  - Support deferring a CPM File Transfer and delivery notification when CPM Client is unavailable, (such as temporarily lost connection during CPM File Transfer, CPM User offline, CPM File Transfer invitation expired).

  - Support delivery of the deferred CPM File Transfer file(s) to the CPM Client, or the CPM User’s permanent network based storage.

  - Support of CPM File Transfer deferring functions on either originating or terminating side.

  - Support for resuming CPM File transfer delivery when the CPM File Transfer is interrupted.

  - Support CPM File Transfer termination independently without impact an ongoing CPM Session, or vice versa.

  - Support CPM File Transfer termination by either sender or recipient

  - Support for file size, file name, and type in a CPM File Transfer request.

  - Define a maximum file size policy.

  - Support multi device CPM File Transfer request and termination, with cancellation to all remaining devices after one of the devices accepts.

  - Support thumbnail image of the file to be transferred in CPM File Transfer request message.

- Enhanced CPM Standalone Message support:

  - Enhanced multi-device functionality for sending delivery notifications and read reports towards the device that originated the CPM Standalone Message

Comment B004

#### CPM Participating Function

The CPM Participating Function is a user-specific functional component. Its functionalities are performed in a CPM User’s home network and are invoked when the CPM User initiates an originating request or receives a terminating request. The CPM Participating Function handles CPM Sessions, CPM File Transfers, CPM Standalone Messages, and Media on behalf of the Participant it serves and enforces CPM User Preferences and service provider policies. Additionally, it performs deferring functionality by storing CPM Messages and CPM File Transfer file(s) temporarily and deferring CPM Messages and CPM File Transfer file(s) for later delivery depending on user preferences, availability and willingness of the CPM User. The CPM Participating Function can also provide conversation history functionality by recording a Participant’s CPM Conversations and storing these recorded CPM Conversations into the Message Storage Server.

The CPM Participating Function SHALL support the following functionalities:

- Enforcing policies when handling originating and terminating CPM Session requests, CPM File Transfers and CPM Messages according to the CPM User preferences/service provider policies as follows:

Validating the service subscription of the CPM User.

Giving precedence to service provider policies over CPM User preferences, when applicable.

Providing support for Content Screening based on CPM User preferences and service provider policies.

- For CPM Session handling:

Managing CPM Session initiation, modification and termination for originating or terminating CPM Users.

Providing SIP session handling, such as SIP session origination, release, etc, on behalf of the involved CPM Client.

Providing SDP negotiation support.

Providing support for multiple SIP sessions handling in a single CPM Conversation.

Providing support for multiple CPM Sessions handling for multiple simultaneous CPM Conversations.

- For CPM File Transfer handling:

Managing CPM File Transfer initiation, resumption and termination for originating and terminating CPM Users.

Providing SIP session handling, such as SIP session origination, release, etc, on behalf of the involved CPM Client.

Supporting CPM File Transfer request containing thumbnail.

Supporting deferring of CPM File Transfers and delivery notifications on behalf of either originating or terminating CPM Client.

Providing support for resuming file transfer when the CPM File Transfer is interrupted, if requested by the CPM User.

- For CPM Message handling:

Providing sending and receiving of CPM Messages for originating and terminating CPM Users.

Providing support for delivery notifications and read reports.

Deferring CPM Messages when the recipient is offline or not willing to receive them, based on the preferences of the CPM User.

Interacting with the Message Storage Server to handle a request from a CPM Client to send CPM Messages with referenced stored resources without prior download of the resource to the Device.

Interacting with the Message Storage Server to handle a request from a CPM Client to forward the referenced stored CPM Messages or CPM Session Histories without prior download.

- For  performing deferred messaging and CPM File Transfer functionalities:

Providing policy enforcement according to the CPM User preferences and service provider policies.

Temporarily keeping CPM Messages, CPM File Transfers files, Participants information, and associated metadata when the messages and/or files cannot be delivered.

Providing support for message delivery when the recipient becomes available and reachable. Both Push and Pull methods are supported as follows:

- The Push method directly delivers the message to all or a subset of the Devices bound to the recipient CPM User’s address(es).

- The Pull method provides notifications with message information (e.g. message size, message sender’s address, message sending time) to all or a subset of the Devices bound to the recipient CPM User’s address(es), and then the recipient can retrieve the message when needed.

Managing the expiry time of a deferred message.

- Overriding the expiry time provided by the sending CPM Client with the operator defined maximum expiry time when applicable.

- Handling expired CPM Messages and CPM File Transfers files in accordance with the preferences of the recipient and the service provider policies (i.e. discard the message or store in the Message Storage Server).

- Sending a non-delivery delivery notification in case of a discarded expired CPM Message and/or CPM File Transfer files.

- For performing store and forward file transfer functionalities:

  - Temporarily keeping CPM File Transfer file or by delivery the file to the CPM User’s Message Store, based on the preference of the CPM user, when CPM User who was invited is either temporarily become unavailable or has not accepted the CPM File Transfer invitation.

  - Providing support for CPM File transfer delivery when the CPM User becomes available.

  - Providing support for resuming CPM File Transfer delivery when the CPM User becomes available.

- For multiple Devices handling:

Supporting multiple Devices handling based on Media characteristics, Communication Capabilities, CPM User preferences and/or service provider policies:

Supporting delivery of CPM Session Invitations to more than one Device for a CPM User until the CPM Session is handled by the CPM user on one of the Devices. The determination of the Device to be used is subject to Service Provider policies.

Supporting delivery of CPM File Transfer initiation requests to more than one Device for a CPM User.

Supporting delivery of CPM Standalone Messages to more than one Device for a CPM User.

Supporting cancellation of CPM File Transfer invitations on other user’s Devices, once one Device has accepted the request.

Supporting the notification of a deferred CPM Standalone Message to more than one Device, when notifications are enabled by the CPM User or service provider policies.

Ensuring that only one delivery notification or read report is sent to the originator of the CPM Standalone Message in case the CPM Standalone Message was delivered to multiple Devices.

Ensuring that only one delivery notification or read report is sent back to the originator of the CPM Chat Message in case the CPM Chat Message was delivered to multiple Devices.

Terminating the session with remaining Devices that are not used in the CPM Session.

Supporting that a requested delivery notification and/or a read report of a CPM Standalone Message is sent to the device of the CPM User that originated the message.

- Involving the Interworking Selection Function:

for terminating CPM Session Invitations and terminating CPM Standalone Messages that are to be routed towards Non-CPM Communication Services based on the preferences of the recipient and service provider policies;

for originating CPM Session Invitations and originating CPM Standalone Messages that are to be routed towards Non-CPM Communication Services based on service provider policies.

- For performing conversation history functionalities, i.e. recording of CPM Conversations persistently (based on either a CPM Client’s request or CPM user preferences):

Storing conversation history into the Message Storage Server

- Storing CPM Standalone Messages and Media Objects that are part of CPM Standalone Messages

- Storing CPM Sessions including the disposition notifications exchanged during the Session, as CPM Session Histories, when activated by the CPM User and provided that:

  - the CPM User is a participant in the CPM Session; or

  - the CPM User goes temporarily offline (e.g. connection loss) during an established CPM Session; or

  - the CPM User has missed a CPM Session invitation.

- Storing CPM File Transfers as CPM File Transfer Histories

- Storing CPM Conversations as CPM Conversation Histories

Providing metadata associated with items to be stored to the Message Storage Server, e.g. identifier linking CPM Standalone Messages to the thread they belong to.

The CPM Participating Function interacts with the Message Storage Server for the storage of recorded CPM Conversations in the CPM User’s own message storage area, and to store expired CPM Messages in the recipient’s network storage, and to store CPM File Transfer files as determined by the preferences of the recipient and service provider policies.

The CPM Participating Function interacts with the following supporting Enablers:

- The XDM Enabler for the retrieval of user preferences, User Preferences Profiles, and user access policies.

- The Push Enabler to provide notification of deferred messages.