# Change Request

| Title: | SD Comments | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_System_Description-V2_0-20130111-D |  |
| Submission Date: | 18 Feb. 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com |  |
| Replaces: | n/a |  |

Reason for Change

| C003 | 2013.02.13 | T | 2.1 | Source: NSN Form: CONRR CPM2.0 Comment: Add reference [RFC4579] Proposed Change: [RFC4579] “Session Initiation Protocol (SIP) Call Control - Conferencing for User Agents”, A. Johnston et al, August 2006, URL:http://www.ietf.org/rfc/rfc4579.txt | Status: CLOSED by CR20R01 |
| --- | --- | --- | --- | --- | --- |

| C004 | 2013.02.13 | T | 5.1.1 | Source: NSN Form: CONRR CPM2.0 Comment: The current text only define the format, but it does not specify the value of the CPM Group Session ID. Per Conversation TS, the CPM Group Session ID is based on the value of Conference URI, as per RFC4579. CPM Group Session A CPM Group Session is uniquely identified by a CPM Group Session Identity. A CPM Group Session Identity SHALL be in the format of a SIP URI as specified in [RFC3261]. Proposed Change: OLD TEXT: CPM Group Session A CPM Group Session is uniquely identified by a CPM Group Session Identity. A CPM Group Session Identity SHALL be in the format of a SIP URI as specified in [RFC3261]. NEW TEXT: CPM Group Session A CPM Group Session is uniquely identified by a CPM Group Session Identity. A CPM Group Session Identity SHALL be in the format of a SIP URI as specified in [RFC3261], and SHALL be based on Conference URI as specified in [RFC4579] | Status: CLOSED by CR20R01 |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM System Description TS, and close the CONRR comments addressed in section 1 of the CR.

Detailed Change Proposal

Comment C003

## Normative References

| [3GPP TS24.229] | “Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3”, 3GPP, TS24.229, URL:http://www.3gpp.org/ |  |  |
| --- | --- | --- | --- |
| [3GPP TS33.203] | “3G security; Access security for IP-based services”, 3GPP, TS33.203, URL:http://www.3gpp.org/ |  |  |
| [3GPP2 S.R0086-0] | “IMS Security Framework”, 3GPP2, S.R0086-0, URL:http://www.3gpp2.org/ |  |  |
| [3GPP2 X.S0013-004] |  | “All-IP Core Network Multimedia Domain - IP Multimedia Call Control Protocol Based on SIP and SDP Stage 3”, 3GPP2, X.S0013-004-A, URL:http://www.3gpp2.org/ |  |
| [IETF-DRAFT-Chat] |  |  | Multi-party Chat Using the Message Session Relay Protocol, Version 14, March 2, 2012, http://tools.ietf.org/html/draft-ietf-simple-chat-14 |
| [OMA-CPM-AD] |  |  | “Converged IP Messaging Architecture”, Open Mobile Alliance™, OMA-AD-CPM-V2_0, URL:http://www.openmobilealliance.org/ |
| [OMA-CPM-RD] |  |  | “Converged IP Messaging Requirements”, Open Mobile Alliance™, OMA-RD-CPM-V2_0, URL:http://www.openmobilealliance.org/ |
| [OMA-IMPS_CSP-TS] |  | “OMA IMPS Client-Server Protocol Transport Bindings”, Open Mobile Alliance™, OMA-TS-IMPS_CSP_Transport-V1_3, URL:http://www.openmobilealliance.org/ |  |
| [OMA SEC_CF-AD] |  | “Security Common Functions Architecture”, Open Mobile Alliance™, OMA-AD-SEC_CF-V1_0, URL:http://www.openmobilealliance.org/ |  |
| [OMA-XDM-AD] |  | “XML Document Management (XDM) Architecture”, Open Mobile Alliance™, OMA-AD-XDM-V2_1, URL:http://www.openmobilealliance.org/ |  |
| [OMA-XDM_Core-TS] |  | “XML Document Management (XDM) Specification”, Open Mobile Alliance™, OMA-TS-XDM_Core-V2_1, URL:http://www.openmobilealliance.org/ |  |
| [OMA-XDM_Group-TS] |  |  | “Group XDM Specification”, Open Mobile Alliance™, OMA-TS-XDM_Group-V1_1, URL:http://www.openmobilealliance.org/ |
| [OMA-XDM-Policy] |  | “Policy XDM Specification”; Open Mobile Alliance™, OMA-TS-XDM _Policy-V1_1, URL:http://www.openmobilealliance.org/ |  |
| [OMA-XDM-UPP] |  | “UPP Directory XDM Specification”; Open Mobile Alliance™, OMA-TS-XDM_UPP_Directory-V1_0, URL:http://www.openmobilealliance.org/ |  |
| [RFC2119] |  |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| [RFC2141] |  |  | "URN Syntax", R. Moats, May 1997, URL:http://www.ietf.org/rfc/rfc2141.txt |  |
| [RFC2246] |  | "The TLS Protocol, version 1.0", T. Dierks et al, January 1999, URL:http://www.ietf.org/rfc/rfc2246.txt |  |
| [RFC2368] |  | "The mailto URL scheme", P. Hoffman et al, July 1998, URL:http://www.ietf.org/rfc/rfc2368.txt |  |
| [RFC2396] |  | "Uniform Resource Identifiers (URI): Generic Syntax", T. Berners-Lee et al, August 1998, URL:http://www.ietf.org/rfc/rfc2396.txt |  |
| [RFC3261] |  | "SIP: Session Initiation Protocol", U. Rosenberg et al, June 2002, URL:http://www.ietf.org/rfc/rfc3261.txt |  |
| [RFC3264] |  | “An Offer/Answer Model with the Session Description Protocol (SDP)”, J. Rosenberg et al, June 2002, URL:http://www.ietf.org/rfc/rfc3264.txt |  |
| [RFC3320] |  | "Signalling Compression(SigComp)", R. Price et al, January 2003, URL:http://www.ietf.org/rfc/rfc3320.txt |  |
| [RFC3321] |  | "Signaling Compression (SigComp) - Extended Operations", H. Hannu et al, January 2003, URL:http://www.ietf.org/rfc/rfc3321.txt |  |
| [RFC3485] |  | "The Session Initiation Protocol (SIP) and Session Description Protocol (SDP) Static Dictionary for Signaling Compression (SigComp)", M. Garcia-Martin et al, February 2003, URL:http://www.ietf.org/rfc/rfc3485.txt |  |
| [RFC3486] |  | "Compressing the Session Initiation Protocol (SIP)", G. Camarillo, February 2003, URL:http://www.ietf.org/rfc/rfc3486.txt |  |
| [RFC3711] |  | "The Secure Real-time Transport Protocol (SRTP)", M. Baugher et al, March 2004, URL:http://www.ietf.org/rfc/rfc3711.txt |  |
| [RFC3840] |  | " Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)", J. Rosenberg et al, August 2004, URL:http://www.ietf.org/rfc/rfc3840.txt |  |
| [RFC3966] |  | "The tel URI for Telephone Numbers", H. Schulzrinne, December 2004, URL:http://www.ietf.org/rfc/rfc3966.txt |  |
| [RFC4122] |  |  | “A Universally Unique IDentifier (UUID) URN Namespace”, P. Leach et al, July 2005, URL:http://www.ietf.org/rfc/rfc4122.txt |
| [RFC4279] |  | “Pre-Shared Key Ciphersuites for Transport Layer Security (TLS)”, P. Eronen et al, December 2005, URL:http://www.ietf.org/rfc/rfc4279.txt |  |  |
| [RFC4483] |  | “A Mechanism for Content Indirection in Session Initiation Protocol(SIP) Messages”, E. Burger, May 2006, URL:http://www.ietf.org/rfc/rfc4483.txt |  |  |
| [RFC4566] [RFC 4579] |  | “SDP: Session Description Protocol”, M. Handley et al, July 2006, URL:http://www.ietf.org/rfc/rfc4566.txt “Session Initiation Protocol (SIP) Call Control - Conferencing for User Agents”, A. Johnston et al, August 2006, URL:http://www.ietf.org/rfc/rfc4579.txt |  |
| [RFC5547] |  |  | “A Session Description Protocol (SDP) Offer/Answer Mechanism to Enable File Transfer,” M. Garcia-Martin et al, May 2009, URL: http://tools.ietf.org/html/rfc5547 |
| [RFC5627] |  |  | “Obtaining and Using Globally Routable User Agent (UA) URIs (GRUU) in the Session Initiation Protocol (SIP)”, J. Rosenberg, October 2009, URL:http://www.ietf.org/rfc/rfc5627.txt |
| [SCRRULES] |  | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL:http://www.openmobilealliance.org/ |  |

Comment C004

### Identification

The objects described in this section can be addressed in the CPM Enabler.

#### CPM User

A CPM User is identified by a CPM User address.

NOTE:	A CPM User can have multiple CPM User addresses.

A CPM User address SHALL be in the format of

a SIP URI as specified in [RFC3261]; or,

a TEL URI as specified in [RFC3966].

#### Non-CPM Principal

A non-CPM Principal is identified inside the CPM Enabler by a non-CPM address. When a non-CPM address is transported inside the CPM Enabler, the non-CPM address SHALL be in the URI format as specified in [RFC2396].

Examples of the possible formats are:

a SIP URI as specified in [RFC3261];

a TEL URI as specified in [RFC3966];

a WV URI as specified in [OMA-IMPS_CSP-TS];

a MAILTO URI as specified in [RFC2368].

CPM requests from non-CPM Users MAY contain a Non-CPM Communication Service Identifier.

#### CPM Client

A CPM Client is uniquely identified by a Unique User Agent Identifier. A Unique User Agent Identifier distinguishes the CPM Client on a Device from other CPM Clients on other Devices used by the same CPM User.

The Unique User Agent Identifier SHALL be in the format of

UUID URN as specified in [RFC4122].

The SIP/IP core assigns, during registration, a unique globally routable user agent URI to the device hosting the CPM Client and returns this GRUU in the registration response as specified in [RFC5627]. The GRUU allows the CPM Enabler to uniquely address each CPM Client residing on a different device.

#### CPM Pre-defined Group

A CPM Pre-defined Group is uniquely identified by a CPM Pre-defined Group address. A CPM Pre-defined Group address SHALL be in the format of a SIP URI as specified in [RFC3261].

#### CPM Group Session

A CPM Group Session is uniquely identified by a CPM Group Session Identity.

A CPM Group Session Identity SHALL be in the format of a SIP URI as specified in [RFC3261] , and SHALL be based on Conference URI as specified in [RFC4579].

#### CPM-based Service

A CPM-based Service is uniquely identified by a CPM-based Service Identifier.

A CPM-based Service Identifier SHALL be in the format of a URN as specified in [RFC2141].

#### CPM Conversation Identity

A CPM Conversation is uniquely identified by a CPM Conversation Identity.

#### CPM Contribution Identity

A CPM Standalone Message, CPM File Transfer or CPM Session Invitation is uniquely identified by a CPM Contribution Identity.