# Change Request

| Title: | D130_Comment_ConvTS | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20130225-D |  |
| Submission Date: | 26 Feb, 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Ning-Chia Yeh, Verizon Wireless, ning-chia.yeh@verizonwireless.com |  |
| Replaces: | N/A |  |

Reason for Change

For CONRR comments D130 in section 8.3.2.9 of Conversation Function,  the trigger to store  session messages  should include both MSRP disconnect and SIP INVITE timeout cases for Full Store and Forward environment

This CR closes the following CONRR comment:

| D130 | 2013.02.13 | T | 8.3.2.9 | Source: Verizon Form: OMA-CONR-2013-0007-VZ_cmmts_CPM2.0_CONR.doc Comment: the trigger to store session messages should include MSRP disconnect and SIP INVITE timeout case for full s&f environment Proposed Change: change as follows: If the SIP BYE was received on the session dialog with the CPM Client with a Reason header field with the reason-protocol set to SIP and a reason-cause set to a value that is different than 200, indicating that the CPM User has not left the session voluntarily, then Tthe CPM Participating Function SHALL stay/remain in the media path of the CPM Group Session with the CPM Controlling Function, in view of a deferred delivery to the CPM User of all further CPM Messages exchanged within the CPM Group Session in the following cases: 1. The SIP BYE was received on the session dialog with the CPM Client with a Reason header field with the reason-protocol set to SIP and a reason-cause set to a value that is different than 200, indicating that the CPM User has not left the session voluntarily. 2. The SIP INVITE invitation to the CPM user timeout 3. The MSRP connection setup fails or message delivery on existing connection fails | Status: CLOSED by CR0035 |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Conversation Function TS

Detailed Change Proposal

Section 8.3.2.9

#### 8.3.2.9 Deferred CPM Group Session handling

Upon receiving a SIP BYE request associated with an existing CPM Session, the CPM Participating Function SHALL proceed as described in sect. 8.2.2.3. “Handle a SIP BYE Request“.

The CPM Participating Function SHALL stay/remain in the media path of the CPM Group Session with the CPM Controlling Function, in view of a deferred delivery to the CPM User of all further CPM Messages exchanged within the CPM Group Session in the following cases:

1. The SIP BYE was received on the session dialog with the CPM Client with a Reason header field with the reason-protocol set to SIP and a reason-cause set to a value that is different than 200, indicating that the CPM User has not left the session voluntarily.

2. The SIP INVITE invitation to the CPM user timeout

3. The MSRP connection setup fails or message delivery on existing connection fails

The CPM Participating Function SHALL also subscribe to the focus of the CPM Group Session as described 8.2.2.6.2 “CPM Long-lived Group Session“, to obtain the latest Participant Information that will be delivered together with the CPM Group Session data during the deferred delivery.

The deferred delivery of a CPM Group Session is triggered in two ways:

- CPM Participating Function initiates it when the CPM Client becomes available, i.e. upon receiving 3rd party register;

- The CPM Client sends a re-join request as soon as it comes back online, and the CPM Participating Function receives the re-join before reacting on the 3rd party register and issuing an invitation to the CPM Client, then the CPM Participating Function:

  - SHALL not send any SIP INVITE to the CPM Client for the deferred delivery of the CPM Group Session,

  - it SHALL use the session setup following the received re-join request also for deferred delivery purposes. The deferred delivery of the missed CPM Group Session data is performed over MSRP.