# Change Request

| Title: | OMA-COM-CPM-2013-0055-CR_ETR_H01_comment | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-ETR-CPM-V2_0_0-20121218-D |  |
| Submission Date: | 25 MAR. 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Thinh Nguyenphu, NSN; thinh.nguyenphu@nsn.com |  |
| Replaces: | n/a |  |

Reason for Change

| ID | Open Date | Type | Section | Description | Status |
| --- | --- | --- | --- | --- | --- |
| H001 | 2013.02.13 | T | Global | Source: NSN Form: CONRR CPM2.0 Comment: Miss all of test requirements for CPM2.0 Proposed Change: | Status: CLOSED BY CR55 |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Enabler Test Requirements and close the CONRR comments addressed in section 1 of the CR.

Detailed Change Proposal

## H001 comment

#### CPM Session

|  | Feature Key | Feature Description | Feature Test Requirements |
| --- | --- | --- | --- |
| Normal Flow | CS-001 | Establishing CPM 1-1 Session [OMA-CPM-TS-CONV] 7.3.1, 7.3.2 | Verify that the inviting CPM Client is able to establish a 1-1 CPM Session with the invited client and media is exchanged between the clients. |
|  | CS-002 | Establishing CPM Ad-hoc Group Session [OMA-CPM-TS-CONV] 7.4.1, 7.4.3 | Verify that group session among the participants specified in the URI-List is established and invited participants are able to exchange media. |
|  | CS-003 | Establishing CPM Pre-defined Group Session [OMA-CPM-TS-CONV] 7.4.2, 7.4.3 | Verify that group session among the participants specified in the CPM Pre-defined Group is established and participants are able to exchange media. |
|  | CS-004 | Chat alias in a CPM Group session [OMA-CPM-TS-CONV] 9.4 | Verify that the chat alias is unique in the CPM Group session when joining a CPM Group session with a chat alias. |
|  | CS-005 | Invite other Users to existing CPM Group Session [OMA-CPM-TS-CONV] 7.4.5 | Verify that the CPM Client is able to add users to the existing CPM Group session. |
|  | CS-006 | Extending a CPM 1-1 Session to a CPM Group Session [OMA-CPM-TS-CONV] 7.3.5 | Verify that the CPM Client is able to add additional users to start or join a CPM Group session by extending the existing CPM 1-1 Session. And that the initial CPM 1-1 Session is released. |
|  | CS007 | Remove Participants from a CPM Session [OMA-CPM-TS-CONV] 7.4.6 | Verify that the CPM CF is able to remove a user from a CPM Group session. |
|  | CS-008 | Leave CPM Group Session [OMA-CPM-TS-CONV] 7.4.4 | Verify that the participant is able to leave the CPM Group session. |
|  | CS-009 | Handling CPM Session Closed Group Chat [OMA-CPM-TS-CONV] 7.3.1, 8.2.2.7.2, 8.2.1 | Verify that Closed group session among the participants is established and participants are able to exchange media. |
|  | CS-010 | Handling Re-joinging CPM Long-Live Group Session [OMA-CPM-TS-CONV] 7.3.1.5, 8.2.2.6, 8.2.2.7, | Verify that the CPM Client is able to rejoin CPM Long-Live Group Session |
|  | CS-011 | Handling Deferred CPM 1-1 and CPM Group Session message delivery [OMA-CPM-TS-CONV] 7.3.11, 7.3.12, 8.3.3.4, 8.3.3.5, 8.3.3.6, 9.2.4 | Verify that CPM Client is able to establish and receive deferred message delivery. |
| Error Flow | N/A |  |  |

Table 3: Mandatory Test Requirements for CPM Session

#### CPM Message Handling

|  | Feature Key | Feature Description | Feature Test Requirements |
| --- | --- | --- | --- |
| Normal Flow | CMH-001 | Deliver the CPM Message [OMA-CPM-TS-CONV] 8.3.2.1.1, 8.3.2.1.2 | Verify that CPM Message is delivered to user if allowed by user preferences, device capabilities, device connectivity and server provider policies. |
|  | CMH-002 | Reject the CPM Message [OMA-CPM-TS-CONV] 8.3.2.1.1, 8.3.2.1.2 | Verify that CPM Message is rejected and error response is sent if recipient’s preferences are to reject the CPM Message. |
|  | CMH-003 | Defer the CPM Message [OMA-CPM-TS-CONV] 8.3.2.1.1, 8.3.2.1.2, 8.3.2.3 | Verify that CPM Message is put in the deferred message queue if recipient’s preferences are to defer the CPM Message. |
|  | CMH-004 | Deliver the CPM Message to CPM Message Storage Server [OMA-CPM-TS-CONV] 8.3.2.1.1, 8.3.2.1.2 | Verify that CPM Message is stored in the CPM Message Storage Server if recipient’s preferences are to have the CPM Message delivered to his server-side storage area. |
|  | CMH-005 | Deliver the message as a non-CPM Message [OMA-CPM-TS-CONV] 8.3.2.1.1, 8.3.2.1.2 | Verify that CPM Message is sent to Interworking Selection Function for interworking selection if recipient’s preferences are to interwork the CPM Message. |
|  | CMH-006 | Deferred CPM Messages push to a CPM Client [OMA-CPM-TS-CONV] 8.3.2.4.1 | Verify that deferred CPM Messages are automatically pushed to CPM Client when the CPM Client comes back online. Also verify that the CPM Client has its settings for deferred CPM Messages to be pushed to it. |
|  | CMH-007 | CPM Client retrieves deferred CPM Message(s) [OMA-CPM-TS-CONV] 7.2.6.2 | Verify that CPM Client is able to retrieve one deferred CPM Message, some selected deferred CPM Messages and all deferred CPM Messages |
|  | CMH-008 | Managing deferred CPM Message [OMA-CPM-TS-CONV] 8.3.2.4.2, 8.3.2.4.3 | Verify that CPM Client is able to apply management operaton (i.e. deliver to the CPM Client, store in the network storage, interwork) on one deferred message, some deferred messages and all deferred messages. Deferred message metadata is updated when each deferred message management function procedure occurs. |
| Error Flow | N/A |  |  |

Table 4: Mandatory Test Requirements CPM Message Handling

#### File Transfer

|  | Feature Key | Feature Description | Feature Test Requirements |
| --- | --- | --- | --- |
| Normal Flow | FT-001 | Transfering file to a user [OMA-CPM-TS-CONV] 7.6.1, 7.6.3 | Verify that a client can send a file transfer request to another user that consists of single file |
|  | FT-002 | Transferring file to multiple users [OMA-CPM-TS-CONV] 7.6.1, 7.6.3 | Verify that a client can send a single file to multiple users. |
|  | FT-003 | Receiving a file [OMA-CPM-TS-CONV] 7.6.1, 7.6.3 | Verify that a client is able to receive a file transfer request correctly |
|  | FT-004 | Fetching deferred CPM File Transfer file(s) [OMA-CPM-TS-CONV] 7.4.4 | Verify that a client can fetch a deferred CPM File Transfer file(s) |
|  | FT-005 | Resuming an interrupted CPM File Transfer [OMA-CPM-TS-CONV] 7.4.5 | Verify that a client can resume CPM File Transfer file(s) |
| Error Flow | N/A |  |  |

Table 5: Mandatory Test Requirements for File Transfer