# Change Request

| Title: | OMA-COM-CPM-2013-0060-CR_C026 | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_System_Description-V2_0-20130225-D |  |
| Submission Date: | 25 March. 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Jerry Shih, AT&T, jerry.shih@att.com |  |
| Replaces: | n/a |  |

Reason for Change

This CR addresses the following SD CONR comment:

| C026 | 2013.02.13 | Q | 5.2.3.4.2 | Source: Jerry Shih Form: OMA-CONR-2013-0005-INP_CPM2_ATT_CONR_Comments.doc Comment: Need clarification of the following text (why a non-CPM Principal can be an inviting user?) The CPM Controlling Function SHALL include into the CPM Session Invitations it sends: - the address of the inviting CPM User or the inviting non-CPM Principal; and, - the address of the invited CPM User or the invited non-CPM Principal; and, . Proposed Change: | Status: Closed with CR060 |
| --- | --- | --- | --- | --- | --- |

Not sure how a non-CPM Principal can initiate a group conversation.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to discuss and agree the proposed change.

Detailed Change Proposal

Change the following text in section 5.2.3.4..2 as marked

###### 5.2.3.4..2 Network Hosting the CPM Group Session

The SIP/IP core of the network hosting the CPM Group Session routes the CPM Session Invitation to the CPM Controlling Function hosting the CPM Group Session based on the included CPM Feature Tag.

When the CPM Controlling Function receives a CPM Session Invitation targeted to a CPM Pre-defined Group, the CPM Controlling Function SHALL fetch the CPM Pre-defined Group members as well as the policies from the Group XDMS and SHALL determine whether to allow the request or not based on the policies stored as part of the CPM Pre-defined Group definition such as whether the service requested is allowed for the group, whether the requesting CPM User is allowed to initiate the service, etc.

When the CPM Controlling Function receives a CPM Session Invitation targeted to a CPM Ad-hoc Group, the CPM Controlling Function SHALL determine whether to allow the request or not based on the service provider policies related to CPM Group Sessions such as the type of CPM Ad-hoc Group Session (e.g. Closed), media types offered, etc

If determined to continue the CPM Group Session establishment, the CPM Controlling Function SHALL send a CPM Session Invitation to the invited CPM Users via the SIP/IP core from which the CPM Session Invitation was received. In case of a CPM Ad-hoc Group, the invited CPM Users are determined from the received CPM Session Invitation, in case of a CPM Pre-defined Group which is a Pre-arranged Group, the invited CPM Users are the members of the CPM Pre-defined Group indicated in the received CPM Session Invitation.

NOTE: No CPM Session Invitations are sent for a CPM Pre-defined Group which is a Join-in Group.

The CPM Controlling Function SHALL indicate the CPM Feature Tag in the CPM Session Invitations it sends.

The CPM Controlling Function SHALL include into the CPM Session Invitations it sends:

- the address of the inviting CPM User; and,

- the address of the invited CPM User or the invited non-CPM Principal; and,

- the addresses of the target CPM Users and the target non-CPM Principals if the CPM Session Invitation is for a CPM Ad-hoc Group; and,

- the CPM Pre-defined Group address if the CPM Session Invitation is for a CPM Pre-defined Group; and,

- the CPM Group Session Identity, and

- The CPM Conversation Identity, the CPM Contribution Identity and the CPM Contribution Identity being replied to received in the CPM Session Invitation.

If the received CPM Session Invitation included a CPM Conversation Identity and a CPM Contribution Identity, then the CPM Controlling Function SHALL store the CPM Conversation Identity and the CPM Contribution Identity for the duration of the CPM Group Session. If the received Session Invitation also included an indication of a CPM Contribution Identity being replied to, then the CPM Controlling Function SHALL store this CPM Contribution Identity being replied to as well for the duration of the CPM Group Session. The CPM Controlling Function SHALL include those values in the response to the CPM Session Invitation.

The CPM Controlling Function SHALL include the CPM Group Session Identity into the CPM Session Invitation response.

If interworking is to occur as described in section 5.3.1.1, the CPM Controlling Function SHALL send the CPM Session Invitation to the Interworking Selection Function.