# Change Request

| Title: | OMA-COM-CPM-2013-0070-CR_ D124_Comment_ConvTS | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20130313-D |  |
| Submission Date: | 26 Mar, 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Ning-Chia Yeh, Verizon Wireless, ning-chia.yeh@verizonwireless.com |  |
| Replaces: | N/A |  |

Reason for Change

For CONRR comments D124 in section 7.3.1.5 and 9.2.4 of Conversation Function.

This CR closes the following CONRR comments:

| D124 | 2013.02.13 | Q | 7.3.1.5 | Source: Verizon Form: OMA-CONR-2013-0007-VZ_cmmts_CPM2.0_CONR.doc Comment: page 39, in the paragraph starting: On receiving a SIP 200 "OK" response to the SIP INVITE request…. List #3 in what circumstance where the 200 OK response to the INVITE request with a different Converation-ID will be received? Proposed Change: Please clarify | Status: CLOSED by CR70 Need CR to remove the Conv-Id storing by Client in step 3) since Conv-Id must be the same. Need change in CF section to add a check on Conv-Id in the re-join request to make sure it matches the one stored in CF for the conf focus id, otherwise return 403. |
| --- | --- | --- | --- | --- | --- |

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to agree on the changes proposed in this CR and implement it accordingly in the CPM Conversation Function TS

Detailed Change Proposal

Section 7.3.1.5

#### Re-joining a CPM Long-lived Group Session

The CPM Client SHALL generate a SIP INVITE request according to the rules and procedures of [RFC3261] . In this SIP INVITE request the CPM Client:

- SHALL set the Request-URI to the address of the target CPM Group Session of CPM Long-live Group Session;

- SHALL include an Accept-Contact header with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.session’ percent encoded as per [3GPP TS 24.229] section 7.2A.8.2 “Coding of the ICSI“ in a g.3gpp.icsi-ref media feature tag;

- SHALL include a Contact header with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.session’ percent encoded as per [3GPP TS 24.229] section 7.2A.8.2 “Coding of the ICSI“ in a g.3gpp.icsi-ref media feature tag;;

- SHALL set the P-Preferred-Service header field with the value of the CPM Feature Tag ‘urn:urn-7:3gpp-service.ims.icsi.oma.cpm.session’;

- SHALL include the CPM Address of the CPM User as the authenticated originator's CPM Address;

- SHALL include  a User-Agent header to indicate the OMA CPM release version of the CPM Client as specified in Appendix D “Release Version in User-agent and Server headers”

- If anonymity is requested, SHALL include value ''id'' in a Privacy header according to the rules and procedures of [RFC3325].

NOTE: If anonymity is not allowed for the CPM Group indicated with the Request-URI of the SIP INVITE based on the rules specified in the [OMA-XDM-Group] the CPM Session Invitation will not be allowed by the CPM Controlling Function hosting the CPM Group.

- SHALL include the Conversation-ID header of the requested CPM Group Session;

- SHALL include the Contribution-ID header of the requested CPM Group Session;

- SHALL include the MIME SDP body of the requested CPM Group Session as a SDP offer as described in section 5.2.1.1 “SDP Contents when Initiating or Modifying a CPM Session”.

- SHALL send the SIP INVITE request according to the rules and procedures of the SIP/IP core.

On receiving a SIP 200 "OK" response to the SIP INVITE request the CPM Client SHALL handle the response according to the rules and procedures of [RFC3261], with the following clarifications:

- The CPM Client SHALL generate and send a SIP ACK request as an acknowledgement of the final response towards the CPM Controlling Function;

- The CPM Client SHALL store the contents of the Contact header as the CPM Group Session Identity, as described in [RFC4579];

- The CPM Client SHALL initiate the Media Plane as in section 7.3.9 “Media Plane Handling for CPM Sessions”.

==========================End of Change 1======================================

Change 2 Section 9.2.4

### 9.2.4  Rejoining CPM Group Session Request

Upon receipt of a SIP INVITE request that includes an Accept-Contact header with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.session’ and a CPM Group Session Identity in the Request-URI, the CPM Controlling Function:

- SHALL check whether this CPM Group Session Identity exists for an active CPM Group Session, or for an inactive CPM Group Session that was previously handled by the CPM Controlling Function (i.e. a CPM Long-lived Group Session). If it does not exist, the CPM Controlling Function SHALL return a SIP 404 “Not Found” response and SHALL include a Warning header with the warning text set to “123 Session does not exist” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL check whether the Conversation-ID in the INVITE request matches with the one stored for the CPM Group Session of the existing  CPM Group Session Identity, either active or inactive. If it does not match, the CPM Controlling Function SHALL return a SIP 403 “Forbidden” response and SHALL include a Warning header with the warning text set to “122 Function not allowed” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL check CPM Group rules as specified in [OMA-XDM-Group] whether the authenticated originator's CPM Address of the joining CPM User is allowed to join. If it is not allowed to join the CPM Controlling Function SHALL return a SIP 403 ”Forbidden” response and SHALL include a Warning header with the warning text set to “122 Function not allowed” in the response according to the rules and procedures of [RFC3261].

- SHALL check the CPM Group Session rules (e.g. metadata and policies) for a Long-lived CPM Group Session and/or for Closed CPM Group Session, whether the authenticated originator's CPM Address of the joining CPM User is allowed to join (by checking the service provider policies for the CPM User, and by checking the last participants list to ensure that the CPM user is among the participants for the case of inactive CPM Long-lived Group Session or for the case of CPM Closed Group Session). If it is not allowed to join the CPM Controlling Function SHALL return a SIP 403 “Forbidden” response and SHALL include a Warning header with the warning text set to “122 Function not allowed” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL, in the case of a CPM Group Session for a CPM Pre-defined Group, check if the <max-participant-count> as specified in [OMA-XDM-Group] is already reached.  If reached, the CPM Controlling Function SHALL return a SIP 486 “Busy Here” response and SHALL include a Warning header with the text set to “102 Too many participants” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL use the display name, if a display name is included, according to the priority specified in section 6.3 “Display Name and Anonymity”;

- SHALL accept the SIP request and generate a SIP 200 "OK" response;

- SHALL include the stored CPM Conversation Identity, CPM Contribution Identity and InReplyTo-Contribution-ID  in the SIP 200 “OK” response;

- SHALL send the SIP 200 "OK" response according to the rules and procedures of SIP/IP core;

- If the re-join request re-starts an inactive CPM Long-lived Group Session, the CPM Controlling Function SHALL use the CPM Group Session rules (e.g. metadata and policies) for a Long-lived CPM Group Session and/or for Closed CPM Group Session, to determine which participants have to be invited  to the re-started CPM Long-lived Group Session and the type of CPM Group Session, then send out the invitations as per step 8 in sect. 9.2.10 “Create Session With a Participant “.

- SHALL generate and send a SIP NOTIFY request to the CPM Clients and/or CPM Participating Functions, which have subscribed to the Participant Information as specified in section 9.2.14.2 “Sending Participant Information Notifications”.

===========================End of Change 2===================================