# Change Request

| Title: | OMA-COM-CPM-2013-0096-CR_Close_AI_A011 | Public OMA Confidential |
| --- | --- | --- |
| To: | COM CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20130405-D |  |
| Submission Date: | 19 Feb. 13 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Jerry Shih, AT&T, jerry.shih@att.com |  |
| Replaces: | n/a |  |

Reason for Change

This CR closes the following Action Item:

| Open | CPM-2013-A011 | Jerry Shih (AT&T) |  | Submit CR to Conversation TS to add the private message to the a=chatroom SDP |
| --- | --- | --- | --- | --- |

To align the changes agreed in CR059R01.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

N/A

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM CPM WA is recommended to discuss and agree the proposed change.

Detailed Change Proposal

Change the following text in section 7.3.1.2 as marked

#### 7.3.1.2 Initiating a CPM Group Session for a CPM Ad-hoc Group

The CPM Client SHALL generate an initial SIP INVITE request according to the rules and procedures of [RFC3261]. In this SIP INVITE request the CPM Client:

- SHALL set the Request-URI of the SIP INVITE request to the CPM Controlling Function URI provisioned in the device;

- SHALL include an Accept-Contact header with the CPM feature-tag ‘3gpp-service.ims.icsi.oma.cpm.session’, percent encoded as per [3GPP TS 24.229] section 7.2A.8.2 “Coding of the ICSI“ in a g.3gpp.icsi-ref media feature tag;

- SHALL include a Contact header with the CPM feature-tag ‘3gpp-service.ims.icsi.oma.cpm.session’, percent encoded as per [3GPP TS 24.229] section 7.2A.8.2 “Coding of the ICSI“ in a g.3gpp.icsi-ref media feature tag;

- SHALL set the P-Preferred-Service header field with the value of the CPM Feature Tag ‘urn:urn-7:3gpp-service.ims.icsi.oma.cpm.session’;

- SHALL include a MIME resource-list body with the invited CPM Users as specified in [RFC5366];

- SHALL check that the number of Invited CPM Users on the URI-list does not exceed the maximum number of Participants allowed in a CPM Group Session for a CPM Ad-hoc Group, as provisioned to the device. If exceeded, the CPM Client SHALL notify the CPM User.

Otherwise, continue with the rest of the steps;

- If anonymity is requested by the CPM User, SHALL include value "id" in the Privacy header according to the rules and procedures of [RFC3325];

- If the CPM Session starts a new CPM Conversation, the CPM Client SHALL include a Conversation-ID header set to a newly generated value as specified in Appendix C “CPM-defined SIP Headers”; Otherwise include the Conversation-ID of the existing CPM Conversation;

- The CPM Client SHALL include a Contribution-ID header with a newly generated Contribution-ID value as specified in Appendix C “CPM-defined SIP Headers”;

- If the CPM Session is in reply to a previously received CPM Standalone Message, CPM File Transfer or CPM Session Invitation, the CPM Client SHALL include the InReplyTo-Contribution-ID header with the value of Contribution-ID in the previously received CPM Standalone Message, CPM File Transfer or CPM Session Invitation;

- SHALL include a MIME SDP body as a SDP offer as described in section 5.2.1.1 “SDP Contents when Initiating or Modifying a CPM Session”;

- If the CPM User requested the CPM Group Session to be a Closed CPM Group Session, then the CPM Client SHALL populate the SDP attribute a=chatroom defined in [IETF-DRAFT-Chat] with the CPM reserved chat-token value of ‘org.oma.groupchat.closed’ to indicate the CPM Closed Group Session in the SIP INVITE; The SDP attribute value SHALL be: “a=chatroom:org.oma.groupchat.closed”.

- SHALL send the SIP INVITE request according to the rules and procedures of the SIP/IP core.

On receiving a SIP 200 "OK" response to the SIP INVITE request, the CPM Client SHALL handle the response according to the rules and procedures of [RFC3261], with the following clarifications:

- The CPM Client SHALL generate and send a SIP ACK request as an acknowledgement of the final response according to the rules and procedures of [RFC3261];

- The CPM Client SHALL store the contents of the Contact header as the CPM Group Session Identity, as described in [RFC4579];

The CPM Client SHALL initiate the Media Plane as in section 7.3.9 “Media Plane Handling for CPM Sessions

Change the following text in section 9.2.1 as marked

### 9.2.1 CPM Group Session Initiation

Upon receiving an initial SIP INVITE request with the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.session’ included in the Accept-Contact header, the CPM Controlling Function:

- SHALL check whether the authenticated originator’s CPM Address is of a CPM User that is allowed to send the request and if not, the CPM Controlling Function SHALL returna SIP 403 “Forbidden” response and SHALL include a Warning header with the warning text set to “127 Service not authorized” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with rest of the steps;

- SHALL check the presence of the "isfocus" feature parameter in the URI of the Contact header and if it is present then the CPM Controlling Function SHALL reject the request with a SIP 403 "Forbidden" response and SHALL include a Warning header with the warning text set to “105 isfocus already assigned” in the response according to the rules and procedures of [RFC3261].

- SHALL check if the SDP parameters in the SDP offer are allowed and supported by the CPM Controlling Function according to service provider policy. If the received SDP parameters are not allowed, the CPM Controlling Function SHALL return a SIP 488 “Not Acceptable here” with a description of the supported SDP parameters by the CPM Controlling Function.

    - SHALL check if the SDP attribute a=chatroom defined in [IETF-DRAFT-Chat] with the CPM reserved chat-token value of ‘org.oma.groupchat.closed’ to indicate that a CPM Closed Group Session is present; i.e. the SDP attribute value “a=chatroom:org.oma.groupchat.closed”. If set, or even if not set but the service provider policies mandate that the CPM Group Session is Closed, the CPM Controlling Function SHALL handle it as a Closed CPM Group Session.

- Otherwise continue with the rest of the steps;

- If the Request-URI of the SIP INVITE request is set to the address of the CPM Controlling Function, the CPM Controlling Function:

- SHALL check if the SIP INVITE request received for a CPM Ad-hoc Group has anonymity requested and whether anonymity is allowed by service provider policy. If not allowed, the CPM Controlling Function SHALL return a SIP 403 "Forbidden" response and SHALL include a Warning header with the warning text set to “119 Anonymity not allowed” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL check from the MIME resource-lists body that the maximum number of Participants allowed in a CPM Ad-hoc Group is not exceeded. If exceeded, the CPM Controlling Function SHALL return a SIP 486 “Busy Here”  response and SHALL include a Warning header with the warning text set to “102 Too many participants” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL check if the MIME resource-list body includes an empty URI list. If the URI list is empty, the CPM Controlling Function SHALL return a SIP 403 "Forbidden" response and SHALL include a Warning header with the warning text set to“129 No destinations” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL fetch the member list contained in MIME <resource-lists> body according to procedures specified in [RFC5366]. This list is used as the set of CPM Group members that have to be invited to the CPM Group Session;

- Otherwise, the CPM Controlling Function:

- SHALL check that the CPM Pre-defined Group address in the Request-URI, identifying a CPM Pre-defined Group as defined in [OMA-XDM-Group], is specified to support the CPM service by evaluating if the <supported-services> element indicates support for the CPM service. If the CPM service is not supported then the CPM Controlling Function SHALL return a SIP 404 "Not found" response" according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL check if the SIP INVITE request received for a CPM Pre-defined Group has anonymity requested and whether privacy is allowed for the authenticated originator’s CPM Address. Allowing privacy for a specific authenticated originator’s CPM Address is defined using <allow-anonymity> element of the CPM Group’s authorization rules as specified in [OMA-XDM-Group]. If not allowed, the CPM Controlling Function SHALL return a SIP 403 "Forbidden" response and SHALL include a Warning header with the warning text set to “Anonymity not allowed” in the response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL determine if the authenticated originator’s CPM Address is allowed the action <allow-initiate-conference> as specified in [OMA-XDM-Group]. If the policy does not allow session initiation, then the CPM Controlling Function SHALL return a SIP 403 “Forbidden” response and SHALL include a Warning header with the warning text set to “122 Function not allowed” in the response according to the rules and procedures of [RFC3261];

Otherwise, continue with the rest of the steps.

- SHALL validate that the media parameters are acceptable for the CPM Pre-defined Group. If not acceptable, then the CPM Controlling Function SHALL reject the request with a SIP 488 "Not Acceptable Here" response according to the rules and procedures of [RFC3261].

Otherwise, continue with the rest of the steps;

- SHALL check if a CPM Group Session is active for the CPM Pre-defined Group. If there is already an active CPM Group Session, then the CPM Controlling Function SHALL process the request as described in section 9.2.3 “Participant Joining a CPM Group Session”.

Otherwise, continue with the rest of the steps;

- SHALL determine if the members of the CPM Pre-defined Group have to be invited to the CPM Group Session by checking if the value of the <invite-members> element is ‘true’. If the value is set to ‘true’, then use the members of the CPM Pre-defined Group as the set of CPM Group members that have to be invited to the CPM Group Session.

- SHALL use the display name described in section 6.3 “Display Name”;

- SHALL establish a CPM Group Session and allocate a CPM Group Session Identity as described in [RFC4353] and [RFC4579];

- SHALL invite each of the users listed as the CPM Group members that have to be invited to the CPM Group Session as specified in section 9.2.10 “Create Session with a Participant”. If a Session-Replaces header (as defined in Appendix C “CPM-defined SIP Headers”) is included in one of the entries in the MIME resource-lists body (when the CPM Group Session is set up for a CPM Ad-hoc Group), the CPM Controlling Function SHALL ensure that the header is added into the SIP INVITE request sent to that entry.

In the case that the CPM Group did not have an active CPM Group Session yet, the CPM Controlling Function SHALL handle the responses from the invited group members as follows:

Upon receiving a SIP final response other than 2xx from an invited CPM Group member, the CPM Controlling Function SHALL remove the inviting CPM Group member from the CPM Group Session as specified in section 9.2.11 "Participant Removing Request".

When all invited group members have responded with a SIP final response other than 2xx, the CPM Controlling Function SHALL send an error response along the incoming signalling path.

Upon receiving the first SIP 200 “OK” response for the SIP INVITE request from one of the invited group members, the CPM Controlling Function:

- SHALL generate a SIP 200 “OK” response to the SIP INVITE request according to the rules and procedures of [RFC3261] with the following clarifications:

  - SHALL include the Session-Expires header with the refresher parameter set to 'uac' and start supervising the SIP Session according to the rules and procedures of [RFC4028];

  - SHALL include the CPM Group Session Identity with the feature-tag 'isfocus' as described in [RFC4579] and the CPM Feature Tag ‘3gpp-service.ims.icsi.oma.cpm.session’ corresponding to CPM Session in the Contact header;

  - SHALL include a User-Agent header to indicate the OMA CPM release version of the CPM Controlling Function as specified in Appendix D “Release Version in User-agent and Server headers”;

  - SHALL include the authenticated originator’s CPM Address as specified in section 6.1 “Authenticated Originator’s CPM Address” set to the CPM Controlling Function;

- SHALL include in the SIP 200 “OK” response a SDP body as a SDP answer based on the received SDP answer in the SIP 200 “OK” response, as described in section 5.2.1.2 “SDP Handling at Intermediate Nodes”.

NOTE: 	Service provider policy determines how to handle the sessions when different participants accept different media types

- SHALL send a SIP 200 “OK” response along the incoming signalling path.

- SHALL NOT forward any further responses from the remaining group members that had not responded yet.

Upon receiving a SIP ACK request, the CPM Controlling Function:

- SHALL forward the SIP ACK request towards all Participants that sent a SIP 200 “OK” response.

- SHALL initiate the Media Plane as in section 9.2.12 “Media Plane Handling”.