Change Request

| Title: | Conversation TS using Contribution ID for Session History | Public OMA Confidential |
| --- | --- | --- |
| To: | COM-CPM WA |  |
| Doc to Change: | OMA-TS-CPM_Conversation_Function-V2_0-20130723-D.doc |  |
| Submission Date: | 22 May 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bernhard Meier/Ericsson; bernhard.meier@ericsson.com |  |
| Replaces: | n/a |  |

Reason for Change

This CR proposes fix a minor bug with respect to using Contribution-ID as the name of the Session History Folder in section 8.3.2.1 “Handle a CPM Session Invitation,” in the PF of the terminating network.

The procedure text in Change 1 was copied from the existing steps in corresponding section 8.2.2.1 for the PF in the originating network.

Change 3 corrects the use of “session-id” to be CPM Session Identity.

Additionally, this CR adds some minor editorial corrections in change 2 and change 4.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The COM-CPM WA is recommended to agree on the proposed changes in this CR and incorporate them in the named document(s).

Detailed Change Proposal

Change in section 8.3.2.1 to use Contribution ID for the Session History folder name:

… [prior text from section 8.3.2.1 omitted] …

Upon receiving the first SIP 200 "OK" response from one of those appropriate CPM Clients, if the CPM Participating Function is acting as a B2BUA, the CPM Participating Function:

- SHALL generate a SIP 200 "OK" response;

- SHALL include an SDP body as a SDP answer based on the SDP answer received in the SIP 200 “OK” response as described in as described in section 5.2.1.2 “SDP Handling at Intermediate Nodes";

- SHALL include a SIP URI in the Contact header field that can resolve back to the original SIP URI in the received Contact header;

- SHALL send the SIP 200 "OK" response according to the rules and procedures of SIP/IP core; and

- SHALL check the <actions> element <allow-offline-storage> retrieved from XDMS as described in 8.4.1 “Retrieving User Preferences”, and if set to ”true”, it SHALL execute the processing described in 8.5 ”Record CPM Conversation History”.

  - In the case where the CPM Participating Function uses a Session History Folder for the CPM Session, the CPM Participating Function SHALL verify if a Session History Folder whose name matches the Contribution-ID exists in the Message Storage Server, and then:

    - If one does exist, SHALL skip the creation of the session info object;

    - If one does NOT exist, SHALL first create a Session History Folder with the folder name set to the Contribution-ID value and then create a session info object for this session in this folder and then retrieve a UID from the Message Storage Server for the newly created session info object.

Upon receiving the first SIP 200 "OK" response from one of those appropriate CPM Clients, and when the CPM Participating Function is acting as a SIP proxy, the CPM Participating Function:

- SHALL forward the SIP 200 "OK" response according to the rules and procedures of [RFC3261] and SIP/IP core;

- SHALL continue to act as a SIP proxy for the duration of the CPM Session.

Upon receiving a SIP final response other than a SIP 200 “OK” response from all the appropriate CPM Clients, the CPM Participating Function SHALL

… [further text from section 8.3.2.1 omitted] …

Editorial: Change in section 8.3.3.4 to remove the colon “:” from the heading:

#### 8.3.3.4 	Delivering deferred CPM File Transfer file(s)

Change in section 9.2.8 to replace the term “session-id” with CPM Session Identity:

### 9.2.8 	CPM Group Session Modification

Upon receiving a SIP re-INVITE request (i.e. an INVITE request with an existing CPM Session Identity in the Request-URI, including a new SDP offer) the CPM Controlling Function:

… [further text from section 9.2.8 omitted] …

Editorial: replace semi-colon “;” with perid “.” at end of last sentence in section 9.2.8:

Upon receiving a SIP ACK request, the CPM Controlling Function SHALL modify the Media Plane for all CPM Group Session Participants who accepted the CPM Session modification as specified in section 9.2.12 “Media Plane Handling”.

### 9.2.9 	Group Session Ending Request

… [further text from section 9.2.9 omitted] …

- End of changes