Change Request

| Title: | Adding Access Control Example | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0-20130412-D |  |
| Submission Date: | 21 May 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Seongyoon Kim, LGE, seongyoon.kim@lge.com Seungkyu Park, LGE, seungk.park@lge.com |  |
| Replaces: | n/a |  |

Reason for Change

This CR proposes to resolve editor’s note in Appendix C.2.1

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR

Detailed Change Proposal

Change C.1

- C.2 LWM2M Object: Access Control

Description: Access Control Object is used to check whether the LWM2M Server has access right for performing an operation. Each Access Control Object Instance contains ACL for a certain Object Instance.

Object Info:

| Object | Object ID | Object URN | Multiple Instances? |
| --- | --- | --- | --- |
| Access Control | 2 |  | Yes |

Resource Info:

| Resource Name | Resource ID | Access Type | Multiple Instances? | Mandatory? | Type | Range or Enumeration | Units | Descriptions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Object ID | 0 | R, W | NO |  | Unsigned Integer | 16-bit | - | See Table 3. |
| Object Instance ID | 1 | R, W | NO |  | Unsigned Integer | 8-bit | - | See Table 3. |
| ACL | 2 | R, W | YES |  | Binary | 8-bit | - | Resource Instance ID MUST be the same with Short Server ID of a certain LWM2M Server which has an access right. Resource Instance ID 0 is for default ACL entry. Value corresponding to the Resource Instance ID is 1 byte access right value specified as below. 1st lsb: Read 2nd lsb: Write 3rd lsb: Execute 4th lsb: Create 5th lsb: Delete Other bits are reserved for future use |
| Access Control Owner | 3 | R, W | NO |  | Unsigned Integer | 8-bit | - | Short Server ID of a certain LWM2M Server. This LWM2M Server only can manage these Resources of the Object Instance. |

- C.2.1 Object Instance Consideration

This Object MUST be instantiated in two cases:

| Case 1: Object Instance for authorizing Read, Write, Execute and Delete operation | Case 2: Object Instance for authorizing Create operation |
| --- | --- |
| The Object Instance MUST be used for authorizing Read, Write, Execute and Delete operation. In this case, Object ID, Object Instance ID, ACL, and Access Control Owner Resource MUST be specified. Access right for Create operation in the Object Instance MUST be ignored. This Object Instance MUST be managed by Access Control Owner. | The Object Instance MUST be used for authorizing Create operation. In this case, Only Object ID and ACL MUST be specified and the other Resources MUST NOT be specified. Access right except for Create operation MUST be ignored. This Object Instance MUST be managed by Bootstrap interface. |

Modify Appendix B

- Appendix B. Example LWM2M Client (Informative)

This section defines an example LWM2M Client for a simple imaginary device with a Cellular interface including instantiated Objects and their values, which is used throughout this specification in examples. The example device has two Server Objects (it is configured to register with two different LWM2M Servers), two accompanying ACL Objects for those servers, a Device Object and a Connectivity Monitoring Object for a Cellular interface. The first Server controls the access control rights for both servers.

Table 3 Object instances of the example.

| Object | Object ID | Object Instance ID |
| --- | --- | --- |
| LWM2M Server Object [1] | 1 | 1 |
| LWM2M Server Object [2] | 1 | 2 |
| Access Control Object [0] | 2 | 0 |
| Access Control Object [1] | 2 | 1 |
| Access Control Object [2] | 2 | 2 |
| Device Object | 3 | - |
| Connectivity Monitoring Object | 4 | - |

Table 4 LWM2M Server Object [1]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| LWM2M Server URI | 0 |  | coap://server1.example.com | Example LWM2M server |
| Lifetime | 1 |  | 86400 |  |
| Security Mode | 2 |  | 0 | PSK mode |
| Security Key | 3 |  | - |  |
| Short Server ID | 4 |  | 1 |  |
| Default Minimum Period | 5 |  | 300 |  |
| Default Maximum Period | 6 |  | 6000 |  |
| DisableTimeout | 8 |  | 86400 |  |
| Notification Storing When Disabled or Offline | 9 |  | True |  |
| Binding Preference | 10 |  | 0 | UDP binding preference |

Table 5 LWM2M Server Object [2]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| LWM2M Server URI | 0 |  | coap://server2.example.com | Example LWM2M server |
| Lifetime | 1 |  | 86400 |  |
| Security Mode | 2 |  | 0 | PSK mode |
| Security Key | 3 |  | - |  |
| Short Server ID | 4 |  | 2 |  |
| Default Minimum Period | 5 |  | 60 |  |
| Default Maximum Period | 6 |  | 6000 |  |
| DisableTimeout | 8 |  | 86400 |  |
| Notification Storing When Disabled or Offline | 9 |  | False |  |
| Binding Preference | 10 |  | 1 | UDP with Queuing binding preference |

Table 6 Access Control Object [0] (for the Device Object)

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Object ID | 0 |  | 3 | Device Object |
| Object Instance ID | 1 |  | - |  |
| ACL | 2 | 1 | R. W, E | Server 1 has all access rights. Note that the Resource Instance ID indicates the Server ID. |
| ACL | 2 | 2 | R | Server 2 has read-only access rights. Note that the Resource Instance ID indicates the Server ID. |
| Access Control Owner | 3 |  | 1 | Server 1 controls this Object’s access rights. |

Table 7 Access Control Object [1] (for the Connectivity Monitoring Object)

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Object ID | 0 |  | 4 | Connectivity Monitoring Object |
| Object Instance ID | 1 |  | - |  |
| ACL | 2 | 1 | R | Server 1 has read-only access rights. Note that the Resource Instance ID indicates the Server ID. |
| ACL | 2 | 0 | R | The other Servers except Server 1 have read-only access rights. Note that this Resource Instance ID indicates default Server ID. |
| Access Control Owner | 3 |  | 1 | Server 1 controls this Object’s access rights. |

Table 6 Access Control Object [2] (for the Firmware Object)

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Object ID | 0 |  | 5 | Firmware Object |
| Object Instance ID | 1 |  | - |  |
| ACL | 2 | 1 | C | Server 1 can create Firmware Object Instance |
| Access Control Owner | 3 |  | - | This Object Instance must be managed by Bootstrap Interface |

Table 8 Device Object

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Manufacturer | 0 |  | Open Mobile Alliance |  |
| Model Number | 1 |  | Lightweight M2M Client |  |
| Serial Number | 2 |  | 345000123 |  |
| Firmware version | 7 |  | 1.0 |  |
| Power source status | 10 |  | 0 | Line power |
| Battery level | 11 |  | 100 |  |
| Memory free | 16 |  | 15 | 15 kB of free memory |
| Error code | 18 |  | 0 | No errors |
| Current Time | 20 |  | 1367491215 | May 2nd, 2013 at 11:42 AM GMT |
| Time Zone | 21 |  | 2 | GMT+2 (CET) |

Table 9 Connectivity Monitoring Object

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Network Bearer | 0 |  | 0 | GSM Bearer |
| Available Network Bearer | 1 |  | 0 | GSM Bearer |
| Radio signal strength | 2 |  | 92 | RSSI in dBm |
| Link Quality | 3 |  | 2 | RxQual Downlink |
| IP Addresses | 4 |  | 192.168.0.100 |  |
| Parent IP Addresses | 5 |  | 192.168.1.1 |  |
| Link Utilization | 6 |  | 5 | % |
| APN | 7 |  | internet |  |