Change Request

| Title: | Clarify observation lifetime over registrations | Public OMA Confidential |
| --- | --- | --- |
| To: | DM LightweightM2M WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0-20160308-D |  |
| Submission Date: |  |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Julien Vermillard – Sierra Wireless |  |
| Replaces: | n/a |  |

Reason for Change

- From github: https://github.com/OpenMobileAlliance/OMA-LwM2M-Public-Review/issues/71

- It’s not clear how long an observation requests should be persisted by the client and it doesn’t seem reasonable to force the device to persist forever (like over reboot) because it enforce flash write and other non-very lightweight operations.

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and agree this CR

Detailed Change Proposal

### Observe

The LWM2M Server initiates an observation request for changes of a specific Resource, Resources within an Object Instance or for all the Object Instances of an Object within the LWM2M Client.

Related parameters for “Observe” operation are described in 5.4.4 Write Attributes and those parameters are configured by “Write Attributes” operation.

The Observe operation includes the following parameters:

| Parameter | Required | Default Value | Notes |
| --- | --- | --- | --- |
| Object ID | Yes | - | Indicates the Object. |
| Object Instance ID | No | - | Indicates the Object Instance to observe. If no Object Instance ID is indicated, then all the Object Instances of Objects are observed and Resource ID MUST NOT be specified. |
| Resource ID | No | - | Indicates the Resource to observe. If no Resource ID is indicated, then the whole Object Instance is observed. |

Table 16: Observe parameters

When “Observe” operation contains only Object ID, the “Notify” operation MUST be done per Object Instance.

### Until the client sends a new registration, the server expects the client to remember the observation requests.  If a client forgets the observation requests (e.g. reboot, power cycle), it MUST send a new registration so the server can re-initiate observation requests. When a client deregisters or registers again, the server considers the previous observations as canceled.