Change Request

| Title: | MLP 3.5 5G Rel-16 Location | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA LOC |  |
| Doc to Change: | OMA-TS-MLP-V3_5-20181211-C |  |
| Submission Date: | 17 May 2020 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Åke Busin, Ericsson, ake.busin@ericsson.com |  |
| Replaces: | n/a |  |

Reason for Change

MLP 3.5 does not support new Rel-16 5G positioning methods defined by 3GPP. Further the new GNSS constellation NavIC (NAVigation with Indian Constellation) is not supported

Impact on Backward Compatibility

N/A

Impact on Other Specifications

SUP MLP 3.5

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Agree to the proposed changes and update MLP 3.5 accordingly.

Detailed Change Proposal

## 2.1 Normative References

| [22.071] | 3GPP TS 22.071 Release 10: "Location Services (LCS); Service description, Stage 1". URL:http://www.3gpp.org/ftp/Specs/latest/Rel-10/22_series/ |
| --- | --- |
| [23.271] | 3GPP TS 23.271 Release 10: "Functional stage 2 description of LCS" URL:http://www.3gpp.org/ftp/Specs/latest/Rel-10/23_series/ |
| [25.413] | 3GPP TS 25.413 Release 13: “UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling” |
| [29.171] | 3GPP TS 29.171 Release 13: “Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface” |
| [3GPP LTE] | 3GPP 36.331 "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification" URL:http://www.3gpp.org/ |
| [ASCII] | US-ASCII. Coded Character Set - 7-Bit American Standard Code for Information Interchange. Standard ANSI X3.4-1986, ANSI, 1986. |
| [IANA] | Internet Assigned Numbers Authority (IANA) URL:http://www.iana.org/ |
| [IOPPROC] | "OMA Interoperability Policy and Process", Version 1.10, Open Mobile AllianceTM. OMA-ORG-IOP-Process-V1_10, URL:http://www.openmobilealliance.org/ |
| [LPPe] | "LPP Extensions Specification", Open Mobile Alliance, OMA-TS-LPPe-V1_0 URL:http://www.openmobilealliance.org/ |
| [MLS AD] | "OMA Mobile Location Service Architecture".Version 1.4, Open Mobile Alliance. OMA-AD-MLS-V1_2, URL:http://www.openmobilealliance.org/ |
| [RFC 3986] | "Uniform Resource Identifier (URI): Generic Syntax ", January 2005, URL:http://www.ietf.org/rfc/rfc3986.txt |
| [RFC 4119] | "A Presence-based GEOPRIV Location Object Format", IETF, J. Peterson, December 2005. URL:http://www.ietf.org/rfc/rfc4119.txt |
| [RFC 5139] | "Revised Civic Location Format for Presence Information Data Format Location Object", IETF, M. Thomson et al, February 2008 URL:http://www.ietf.org/rfc/rfc5139.txt |
| [RFC 6848] | "Specifying Civic Address Extensions in the Presence Information Data Format Location Object (PIDF-LO)", IETF, J. Winterbottom et al, January 2013 URL:http://www.ietf.org/rfc/rfc6848.txt |
| [RFC2119] | "Key words for use in RFCs to Indicate Requirement Levels", IETF, S. Bradner. March 1997. URL:http://www.ietf.org/rfc/rfc2119.txt |
| [RFC2246] | "The TLS Protocol Version 1.0", IETF, T. Dierks, et al, January 1999. URL:http://www.ietf.org/rfc/rfc2246.txt |
| [RFC2616] | "Hypertext Transfer Protocol –HTTP/1.1", IETF, R. Fielding, et al, June 1999. URL:http://www.ietf.org/rfc/rfc2616.txt |
| [SUPL 3.0] | "User Plane Location Protocol", Open Mobile Alliance, OMA-TS-ULP-V3_0 URL:http://www.openmobilealliance.org/ |
| [XML-1.0] | "Extensible Markup Language (XML) 1.0" W3C Recommendation: URL:http://www.w3.org/TR/2000/REC-xml-20001006 |
| [3GPP NR] | 3GPP 38.331 "NR; Radio Resource Control (RRC) Protocol specification" URL:http://www.3gpp.org/ |
| [3GPP 38.413] | 3GPP TS 38.413, “NG-RAN; NG Application Protocol (NGAP)” URL:http://www.3GPP.org/ |
| [3GPP 38.305] | 3GPP TS 38.305, “NG-RAN; Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN” URL:http://www.3GPP.org/ |

## 3.3 Abbreviations

| AFLT | Advanced Forward Link Triangulation positioning method |
| --- | --- |
| A-GPS | Assisted GPS |
| ALN | Access Location Network |
| ANSI | American National Standards Institute |
| AoA | Angle of Arrival |
| AoD | Angle of Departure |
| APN | Access Point Name |
| ASID | Anonymous Subscriber Identity |
| ASP | Application Service Provider |
| BDS | BeiDou Navigation Satellite System |
| BT | Bluetooth |
| CRS | Coordinate Reference System |
| DL | DownLink |
| DMSH | Degrees Minutes Seconds Hemisphere |
| DTD | Document Type Definition |
| EFLT | Enhanced Forward Link Triangulation positioning method |
| E-OTD | Enhanced Observed Time Difference (E-OTD) |
| ESRK | Emergency Services Routing Key |
| GANSS | Galileo and Additional Global Navigation Satellite Systems |
| GEM | General Error Message |
| GLONASS | GLObal'naya NAvigatsionnaya Sputnikovaya Sistema |
| GML | Geography Markup Language |
| GMLC | Gateway Mobile Location Center |
| GMT | Greenwich Mean Time |
| GNSS | Global Navigation Satellite Systems |
| GPS | Global Positioning System |
| HLR | Home Location Register |
| HTTP | Hypertext Transfer Protocol |
| HTTPS | HTTP Secure |
| IMSI | International Mobile Station Identifier |
| LCS | Location Services |
| MBS | Metropolitan Beacon System |
| MLC | Mobile Location Center |
| MLP | Mobile Location Protocol |
| MPC | Mobile Positioning Center |
| MS | Mobile Station |
| MSID | Mobile Station Identifier |
| MSISDN | Mobile Station ISDN |
| NAI | Network Access Identifier |
| NR | New Radio |
| OTDOA | Observed Time Difference of Arrival |
| QZSS | Quasi-Zenith Satellite System |
| SBAS | Satellite Based Augmentation System |
| SOAP | Simple Object Access Protocol |
| SSL | Secure Socket Layer |
| TDOA | Time Difference of Arrival |
| TLS | Transport Layer Security |
| UL | UpLink |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |
| U-TDOA | Uplink Time Difference of Arrival |
| UTM | Universal Transverse Mercator |
| WAP | Wireless Application Protocol |
| WGS | World Geodetic System |
| WLAN | Wireless Local Area Network |
| WSP | Wireless Session Protocol |
| XML | Extensible Markup Language |

#### 5.2.2.3 Location Element Definitions

| <!-- MLP_LOC --> |  |  |
| --- | --- | --- |
| <!-- MLP V3.5 Document Type Definition Copyright Open Mobile Alliance Ltd., 2014 All rights reserved MLP is an XML language. Typical usage: <?xml version="1.0"?> <!DOCTYPE svc_xxx PUBLIC "-//OMA//DTD {abbrev x.y}//EN" "http://www.openmobilealliance.org/DTD/{filename}" [<?oma-{ref}-ver supported-versions="{versions}"?>]> <svc_xxx> ... </svc_xxx> Terms and conditions of use are available from the Open Mobile Alliance Ltd. web site at http://www.openmobilealliance.org/ --> |  |  |
| <!ELEMENT | pos | (msid, (pd ¦ location_uri ¦ poserr), gsm_net_param?, trans_id?,add_info?, positioning_data*)> |
| <!ATTLIST | pos |  |
|  | pos_method (CELL ¦ OTDOA ¦ GPS ¦ A-GPS ¦ GNSS ¦ A-GNSS ¦ E-OTD ¦ U-TDOA ¦ AFLT ¦ EFLT ¦ E-CID ¦ BARO ¦ BT ¦ MBS ¦ WLAN ¦ NR E-CID ¦ Multi-RTT ¦ DL-AoD ¦ DL-TDOA ¦ UL-TDOA ¦ UL-AoA ¦ UNKNOWN ¦ OTHER) | #IMPLIED |
|  | result_type (INTERMEDIATE ¦ FINAL) | "FINAL"> |
| <!ELEMENT | eme_pos | (msid+, (pd ¦ location_uri ¦ poserr)?, esrd?, esrk?, trans_id?, serving_cell?, target_serving_node?, location_id?, positioning_data*)> |
| <!ATTLIST | eme_pos |  |
|  | pos_method (CELL ¦ OTDOA ¦ GPS ¦ A-GPS ¦ GNSS ¦ A-GNSS ¦ E-OTD ¦ U-TDOA ¦ AFLT ¦ EFLT ¦ E-CID ¦ BARO ¦ BT ¦ MBS ¦ WLAN ¦ NR E-CID ¦ Multi-RTT ¦ DL-AoD ¦ DL-TDOA ¦ UL-TDOA ¦ UL-AoA ¦ UNKNOWN ¦ OTHER) | #IMPLIED |
|  | result_type (INTERMEDIATE ¦ FINAL) | "FINAL"> |
| <!ELEMENT | trl_pos | (msid?, reference_object*, (pd ¦ poserr ¦ time ¦ analytic_data), positioning_data*)> |
| <!ATTLIST | trl_pos |  |
|  | trl_trigger (PERIODIC ¦ MS_AVAIL ¦ CHANGE_AREA ¦ VELOCITY ¦ DISTANCE ¦ EQUIDISTANCE ¦ ALN_EVENT) | #REQUIRED |
|  | pos_method (CELL ¦ OTDOA ¦ GPS ¦ A-GPS ¦ GNSS ¦ A-GNSS ¦ E-OTD ¦ U-TDOA ¦ AFLT ¦ EFLT ¦ E-CID ¦ BARO ¦ BT ¦ MBS ¦ WLAN ¦ NR_E-CID ¦ Multi-RTT ¦ DL-AoD ¦ DL-TDOA ¦ UL-TDOA ¦ UL-AoA ¦ UNKNOWN ¦ OTHER) | #IMPLIED> |
| <!ELEMENT | positioning_data | EMPTY> |
| <!ATTLIST | positioning_data positioning_method (CELL ¦ OTDOA ¦ GPS ¦ A-GPS ¦ GNSS ¦ A-GNSS ¦ E-OTD ¦ U-TDOA ¦ AFLT ¦ EFLT ¦ E-CID ¦ BARO ¦ BT ¦ MBS ¦ WLAN ¦ NR_E-CID ¦ Multi-RTT ¦ DL-AoD ¦ DL-TDOA ¦ UL-TDOA ¦ UL-AoA ¦ OTHER ¦ UNKNOWN) | #IMPLIED |
|  | positioning_mode (UE-BASED ¦ UE-ASSISTED ¦ CONVENTIONAL) | #IMPLIED |
|  | ganss_id (GALILEO ¦ SBAS ¦ MODERNIZED-GPS ¦ QZSS ¦ GLONASS ¦ BDS ¦ NavIC ¦ UNKNOWN ¦ OTHER) | #IMPLIED> |
| <!ELEMENT | analytic_data | (time, num_of_ms, dwell_time?)> |
| <!ELEMENT | num_of_ms | (num_start?, num_entered, num_left, num_mean?, num_std_dev?)> |
| <!ELEMENT | dwell_time | (mean_dwell_time, dwell_time_std_dev?)> |
| <!ELEMENT | num_start | (#PCDATA)> |
| <!ELEMENT | num_entered | (#PCDATA)> |
| <!ELEMENT | num_left | (#PCDATA)> |
| <!ELEMENT | num_mean | (#PCDATA)> |
| <!ELEMENT | num_std_dev | (#PCDATA)> |
| <!ELEMENT | mean_dwell_time | (duration)> |
| <!ELEMENT | dwell_time_std_dev | (duration)> |
| <!ELEMENT | pd | (time, (shape ¦ civicloc ¦ (shape, civicloc) ¦ relativelocation), MapData?, (alt, alt_unc?)?, speed?, direction?, lev_conf?, qos_not_met?, MotionStateList?, (floor_number, floor_number_unc?)?, uncompensatedBarometricPressure?)> |
| <!ELEMENT | poserr | (result, add_info?, time)> |
| <!ELEMENT | relativelocation | ((Point ¦ CircularArea ¦ EllipticalArea), ReferencePoint?)> |
| <!ELEMENT | floor_number | (#PCDATA)> |
| <!ELEMENT | floor_number_unc | (#PCDATA)> |
| <!ELEMENT | MapData | (Base64Map ¦ Url)> |
| <!ELEMENT | Base64Map | (#PCDATA)> |
| <!ELEMENT | Url | (#PCDATA)> |
| <!ELEMENT | MapRequest | (Width?, Height?, Zoom?)> |
| <!ATTLIST | MapRequest |  |
|  | MapdataType (MAP ¦ URL) | "MAP" |
|  | MimeType (GIF ¦ JPEG ¦ PNG ¦ TIFF ) | "GIF"> |
| <!ELEMENT | Width | (#PCDATA)> |
| <!ELEMENT | Height | (#PCDATA)> |
| <!ELEMENT | Zoom | (#PCDATA)> |
| <!ATTLIST | Zoom |  |
|  | ZoomType (KILOMETER ¦ METER ¦ MILE ¦ INCH) | "KILOMETER"> |
| <!ELEMENT | time | (#PCDATA)> |
| <!ATTLIST | time |  |
|  | utc_off CDATA | "0000"> |
| <!ELEMENT | alt | (#PCDATA)> |
| <!ELEMENT | alt_unc | (#PCDATA)> |
| <!ELEMENT | civicloc | (civicloc_element+)> |
| <!ATTLIST | civicloc |  |
|  | xml:lang CDATA | #IMPLIED> |
| <!ELEMENT | civicloc_element | (#PCDATA)> |
| <!ATTLIST | civicloc_element |  |
|  | element_type (COUNTRY ¦ A1 ¦ A2 ¦ A3 ¦ A4 ¦ A5 ¦ A6 ¦ PRD ¦ POD ¦ STS ¦ HNO ¦ HNS ¦ LMK ¦ LOC ¦ FLR ¦ NAM ¦ PC ¦ BLD ¦ UNIT ¦ ROOM ¦ PLC ¦ PCN ¦ POBOX ¦ ADDCODE ¦ SEAT ¦ RD ¦ RDSEC ¦ RDBR ¦ RDSUBBR ¦ PRM ¦ POM ¦ PN ¦ MP ¦ STP ¦ HNP ¦ STPS ¦ LMKP ¦ VENUE_NAME ¦ VENUE_ID ¦ VENUE_SPECIFIC_NAME ¦ IANA) | #REQUIRED |
|  | xml:lang CDATA | #IMPLIED |
|  | iana_type CDATA | #IMPLIED> |
| <!ELEMENT | MotionStateList | (PrimaryMotionState, SecondaryMotionState*, Confidence)> |
| <!ELEMENT | PrimaryMotionState | (MotionState)> |
| <!ELEMENT | SecondaryMotionState | (MotionState)> |
| <!ELEMENT | MotionState | (#PCDATA)> |
| <!ELEMENT | Confidence | (#PCDATA)> |
| <!ELEMENT | qos_not_met | EMPTY> |
| <!ELEMENT | direction | (#PCDATA)> |
| <!ELEMENT | speed | (#PCDATA)> |
| <!ELEMENT | lev_conf | (#PCDATA)> |
| <!ELEMENT | geo_info | (CoordinateReferenceSystem)> |
| <!ATTLIST | geo_info |  |
|  | requested_positiondata ( SHAPE ¦CIVICLOC ¦ SHAPE_AND_CIVICLOC) | "SHAPE" |
|  | Strict (YES ¦ NO) | "YES"> |
| <!ELEMENT | CoordinateReferenceSystem | (Identifier)> |
| <!ELEMENT | Identifier | (code, codeSpace, edition)> |
| <!ELEMENT | code | (#PCDATA)> |
| <!ELEMENT | codeSpace | (#PCDATA)> |
| <!ELEMENT | edition | (#PCDATA)> |
| <!ELEMENT | service_coverage | ((cc, ndc*)+)> |
| <!ELEMENT | MotionStateRequest | EMPTY> |
| <!ELEMENT | serving_cell | (cgi ¦ sai ¦ (mcc, mnc, lte_ci) ¦) |
| <!ELEMENT | sai | (mcc, mnc, nr_ci))> (mcc, mnc, lac, sac)> |
| <!ELEMENT | sac | (#PCDATA)> |
| <!ELEMENT | lte_ci | (#PCDATA)> |
| <!ELEMENT | nr_ci | (#PCDATA)> |
| <!ELEMENT | location_uri | (#PCDATA)> |
| <!ELEMENT | location_id | (#PCDATA)> |
| <!ELEMENT | ReferencePoint | (Point?, civicloc?, floor_number?)> |
| <!ATTLIST | ReferencePoint |  |
|  | referencepoint_id CDATA | #REQUIRED |
|  | referencepoint_name CDATA | #IMPLIED> |
| <!ELEMENT | uncompensatedBarometricPressure | (#PCDATA)> |

#### 5.3.35.1 pos_method

| Description: |  |  |
| --- | --- | --- |
| Specifies the positioning method used to obtain the associated location estimate |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | CELL | Cell coverage based positioning method |
|  | OTDOA | Observed Time Difference of Arrival (OTDOA) positioning method |
|  | GPS | Global Positioning System (GPS) based positioning method |
|  | A-GPS | Assisted GPS based positioning method |
|  | GNSS | GNSS (GPS and GANSS) based positioning method |
|  | A-GNSS | Assisted GNSS (GPS and GANSS) based positioning method |
|  | E-OTD | Enhanced Observed Time Difference (E-OTD) positioning method |
|  | U-TDOA | Uplink Time Difference of Arrival (U-TDOA) positioning method |
|  | AFLT | Advanced Forward Link Triangulation positioning method |
|  | EFLT | Enhanced Forward Link Triangulation positioning method |
|  | E-CID | Enhancement Cell ID positioning method |
|  | BARO | Barometric Sensor positioning method |
|  | BT | Bluetooth positioning method |
|  | MBS | Metropolitan Beacon System positioning method |
|  | WLAN | WLAN positioning method |
|  | NR_E-CID | NR enhanced cell ID methods |
|  | Multi-RTT | Multi-Round Trip Time Positioning method |
|  | DL-AoD | Downlink Angle-of-Departure method |
|  | DL-TDOA | Downlink Time Difference of Arrival method |
|  | UL-TDOA | Uplink Time Difference of Arrival method |
|  | UL-AoA | Uplink Angle of Arrival method |
|  | UNKNOWN | Unknown positioning method |
|  | OTHER | Any other positioning method |
| Default value: |  |  |
| Example: | <eme_pos pos_method="A-GPS"> … </eme_pos> |  |
| Note: | The list of current values reflects the status quo described in 3GPP TS 23.271, V6.7.0, sect. 4.3 (plus A-GPS), [3GPP 38.305] and TIA-801. New values may be added as soon as the need arises. |  |

#### 5.3.93.1  pos_method

See section 5.3.35.1

#### 5.3.94.1 positioning_method

| Description: |  |  |
| --- | --- | --- |
| Specifies the positioning method used to obtain the associated location estimate |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | CELL | Cell coverage based positioning method |
|  | OTDOA | Observed Time Difference of Arrival (OTDOA) positioning method |
|  | GPS | Global Positioning System (GPS) based positioning method |
|  | A-GPS | Assisted GPS based positioning method |
|  | GNSS | GNSS (GPS and GANSS) based positioning method |
|  | A-GNSS | Assisted GNSS (GPS and GANSS) based positioning method |
|  | E-OTD | Enhanced Observed Time Difference (E-OTD) positioning method |
|  | U-TDOA | Uplink Time Difference of Arrival (U-TDOA) positioning method |
|  | AFLT | Advanced Forward Link Triangulation positioning method |
|  | EFLT | Enhanced Forward Link Triangulation positioning method |
|  | E-CID | Enhancement Cell ID positioning method |
|  | BARO | Barometric Sensor positioning method |
|  | BT | Bluetooth positioning method |
|  | MBS | Metropolitan Beacon System positioning method |
|  | WLAN | WLAN positioning method |
|  | NR_E-CID | NR enhanced cell ID methods |
|  | Multi-RTT | Multi-Round Trip Time Positioning method |
|  | DL-AoD | Downlink Angle-of-Departure method |
|  | DL-TDOA | Downlink Time Difference of Arrival method |
|  | UL-TDOA | Uplink Time Difference of Arrival method |
|  | UL-AoA | Uplink Angle of Arrival method |
|  | UNKNOWN | Unknown positioning method |
|  | OTHER | Any other positioning method |
| Default value: |  |  |
| Example: | <positioning_method> GNSS </positioning_method> |  |
| Note: |  |  |

#### 5.3.1.94.3 ganss_id

| Description: |  |  |
| --- | --- | --- |
| Specifies the GANSS identifier used to obtain the associated location estimate |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | GALILEO | Galileo positioning method |
|  | SBAS | Satellite Based Augmentation System positioning method |
|  | MODERNIZED-GPS | Modernized-GPS positioning method |
|  | QZSS | Quasi-Zenith Satellite System positioning method |
|  | GLONASS | GLObal'naya NAvigatsionnaya Sputnikovaya Sistema positioning method |
|  | BDS | BeiDou Navigation Satellite System positioning method |
|  | NavIC | NAVigation with Indian Constellation |
|  | UNKNOWN | Unknown GANSS positioning method |
|  | OTHER | Any other GANSS positioning method |
| Default value: |  |  |
| Example: | <ganss_id> Galileo </ganss_id> |  |
| Note: |  |  |

#### 5.3.137.1  pos_method

See section 5.3.35.1