Change Request

| Title: | SUP MLP 3.5 5G Rel-16 Location | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA LOC |  |
| Doc to Change: | OMA-SUP-DTD_MLP-V3_5-20181211-C |  |
| Submission Date: | 29 May 2020 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Åke Busin, Ericsson, ake.busin@ericsson.com |  |
| Replaces: | n/a |  |

Reason for Change

MLP 3.5 does not support new Rel-16 5G positioning methods defined by 3GPP. Further the new GNSS constellation NavIC (NAVigation with Indian Constellation) is not supported.

R01: “NR E-CID” corrected to “NR_E-CID” in some instances of DTDs.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

--

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Agree to the proposed changes and update SUP MLP 3.5 accordingly.

Detailed Change Proposal

!--

MLP DTD - Mobile Location Protocol

version - 3.5

date    - 11 Dec 2018

FILE INFORMATION

OMA Permanent Document

File: OMA-SUP-DTD_MLP-V3_5-20181211-C

Type: Text

============               UNCHANGED TEST OMITTED     ===========================

<!--   START OF   MLP_LOC  -->

<!ELEMENT	pos	(msid, (pd | location_uri |  poserr),

gsm_net_param?, trans_id? , add_info?, positioning_data*)>

<!ATTLIST	pos

pos_method (CELL | OTDOA | GPS | A-GPS | GNSS | A-GNSS | E-OTD | U-TDOA | AFLT | EFLT | E-CID | BARO | BT | MBS | WLAN | NR_E-CID | Multi-RTT | DL-AoD | DL-TDOA | UL-TDOA | UL-AoA | UNKNOWN | OTHER)	#IMPLIED

result_type (INTERMEDIATE | FINAL)	"FINAL">

<!ELEMENT	eme_pos	(msid+, (pd | location_uri | poserr)?, esrd?, esrk?, trans_id?, serving_cell?, target_serving_node?, location_id?, positioning_data*)>

<!ATTLIST	eme_pos

pos_method (CELL | OTDOA | GPS | A-GPS | GNSS | A-GNSS | E-OTD | U-TDOA | AFLT | EFLT | E-CID | BARO | BT | MBS | WLAN | NR_E-CID | Multi-RTT | DL-AoD | DL-TDOA | UL-TDOA | UL-AoA | UNKNOWN | OTHER)	#IMPLIED

result_type (INTERMEDIATE | FINAL)	"FINAL">

<!ELEMENT	trl_pos	(msid?, reference_object*, (pd | poserr | time | analytic_data), positioning_data*)>

<!ATTLIST	trl_pos

trl_trigger (PERIODIC | MS_AVAIL | CHANGE_AREA | VELOCITY | DISTANCE | EQUIDISTANCE | ALN_EVENT)	#REQUIRED

pos_method (CELL | OTDOA | GPS | A-GPS | GNSS | A-GNSS | E-OTD | U-TDOA | AFLT | EFLT | E-CID | BARO | BT | MBS | WLAN | NR_E-CID | Multi-RTT | DL-AoD | DL-TDOA | UL-TDOA | UL-AoA | UNKNOWN | OTHER)	#IMPLIED>

<!ELEMENT	positioning_data	EMPTY>

<!ATTLIST	positioning_data

positioning_method (CELL | OTDOA | GPS | A-GPS | GNSS | A-GNSS | E-OTD | U-TDOA | AFLT | EFLT | E-CID | BARO | BT | MBS | WLAN | NR_E-CID | Multi-RTT | DL-AoD | DL-TDOA | UL-TDOA | UL-AoA | OTHER | UNKNOWN)	#IMPLIED

positioning_mode (UE-BASED | UE-ASSISTED | CONVENTIONAL)	#IMPLIED

ganss_id (GALILEO | SBAS | MODERNIZED-GPS | QZSS | GLONASS | BDS | NavIC | UNKNOWN | OTHER)	#IMPLIED>

<!ELEMENT	analytic_data	(time, num_of_ms, dwell_time?)>

<!ELEMENT	num_of_ms	(num_start?, num_entered, num_left, num_mean?, num_std_dev?)>

<!ELEMENT	dwell_time	(mean_dwell_time, dwell_time_std_dev?)>

<!ELEMENT	num_start	(#PCDATA)>

<!ELEMENT	num_entered	(#PCDATA)>

<!ELEMENT	num_left	(#PCDATA)>

<!ELEMENT	num_mean	(#PCDATA)>

<!ELEMENT	num_std_dev	(#PCDATA)>

<!ELEMENT	mean_dwell_time	(duration)>

<!ELEMENT	dwell_time_std_dev	(duration)>

<!ELEMENT	pd	(time, (shape | civicloc | (shape, civicloc) | relativelocation), MapData?, (alt, alt_unc?)?, speed?, direction?, lev_conf?, qos_not_met?, MotionStateList?, (floor_number, floor_number_unc?)? , uncompensatedBarometricPressure?)>

<!ELEMENT	poserr	(result, add_info?, time)>

<!ELEMENT	relativelocation	((Point | CircularArea | EllipticalArea), ReferencePoint?)>

<!ELEMENT	floor_number	(#PCDATA)>

<!ELEMENT	floor_number_unc	(#PCDATA)>

<!ELEMENT	MapData	(Base64Map | Url)>

<!ELEMENT	Base64Map	(#PCDATA)>

<!ELEMENT	Url	(#PCDATA)>

<!ELEMENT	MapRequest	(Width?, Height?, Zoom?)>

<!ATTLIST	MapRequest

MapdataType (MAP | URL)	"MAP"

MimeType (GIF | JPEG |  PNG  | TIFF )	"GIF">

<!ELEMENT	Width	(#PCDATA)>

<!ELEMENT	Height	(#PCDATA)>

<!ELEMENT	Zoom	(#PCDATA)>

<!ATTLIST	Zoom

ZoomType (KILOMETER | METER | MILE | INCH)	"KILOMETER">

<!ELEMENT	time	(#PCDATA)>

<!ATTLIST	time

utc_off CDATA	"0000">

<!ELEMENT	alt	(#PCDATA)>

<!ELEMENT	alt_unc	(#PCDATA)>

<!ELEMENT	civicloc	(civicloc_element+)>

<!ATTLIST	civicloc

xml:lang CDATA	#IMPLIED>

<!ELEMENT	civicloc_element	(#PCDATA)>

<!ATTLIST	civicloc_element

element_type (COUNTRY | A1 | A2 | A3 | A4 | A5 | A6 | PRD | POD | STS | HNO | HNS | LMK | LOC | FLR | NAM | PC | BLD | UNIT | ROOM | PLC | PCN | POBOX | ADDCODE | SEAT | RD | RDSEC | RDBR | RDSUBBR | PRM | POM | PN | MP | STP | HNP | STPS | LMKP | VENUE_NAME |  VENUE_ID | VENUE_SPECIFIC_NAME | IANA)	#REQUIRED

xml:lang CDATA 	#IMPLIED

iana_type CDATA 	#IMPLIED>

<!ELEMENT	MotionStateList	(PrimaryMotionState, SecondaryMotionState*, Confidence)>

<!ELEMENT	PrimaryMotionState	(MotionState)>

<!ELEMENT	SecondaryMotionState	(MotionState)>

<!ELEMENT	MotionState	(#PCDATA)>

<!ELEMENT	Confidence	(#PCDATA)>

<!ELEMENT	qos_not_met	EMPTY>

<!ELEMENT	direction	(#PCDATA)>

<!ELEMENT	speed	(#PCDATA)>

<!ELEMENT	lev_conf	(#PCDATA)>

<!ELEMENT	geo_info	(CoordinateReferenceSystem)>

<!ATTLIST	geo_info

requested_positiondata ( SHAPE |CIVICLOC | SHAPE_AND_CIVICLOC)	"SHAPE"

Strict (YES | NO)	"YES">

<!ELEMENT	CoordinateReferenceSystem	(Identifier)>

<!ELEMENT	Identifier	(code, codeSpace, edition)>

<!ELEMENT	code	(#PCDATA)>

<!ELEMENT	codeSpace	(#PCDATA)>

<!ELEMENT	edition	(#PCDATA)>

<!ELEMENT	service_coverage	((cc, ndc*)+)>

<!ELEMENT	MotionStateRequest	EMPTY>

<!ELEMENT	serving_cell	(cgi | sai | (mcc, mnc, lte_ci)| (mcc, mnc, nr_ci))>

<!ELEMENT	sai	(mcc, mnc, lac, sac)>

<!ELEMENT	sac	(#PCDATA)>

<!ELEMENT	lte_ci	(#PCDATA)>

<!ELEMENT	nr_ci	(#PCDATA)>

<!ELEMENT	location_uri	(#PCDATA)>

<!ELEMENT	location_id	(#PCDATA)>

<!ELEMENT	ReferencePoint	(Point?, civicloc?, floor_number?)>

<!ATTLIST	ReferencePoint

referencepoint_id CDATA	#REQUIRED

referencepoint_name CDATA	#IMPLIED>

<!ELEMENT	uncompensatedBarometricPressure	(#PCDATA)>

<!-- END OF SECTION -->

============               UNCHANGED TEST OMITTED     ===========================