Change Request

| Title: | RLP 1.2 5G Rel-16 Location | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA LOC |  |
| Doc to Change: | OMA-TS-RLP-V1_2_1-20181211-A |  |
| Submission Date: | 17 May 2020 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Åke Busin, Ericsson, ake.busin@ericsson.com |  |
| Replaces: | n/a |  |

Reason for Change

RLP 1.2 does not support new Rel-16 5G positioning methods defined by 3GPP.

Impact on Backward Compatibility

N/A

Impact on Other Specifications

SUP RLP 1.2

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Agree to the proposed changes and update RLP 1.2 accordingly.

Detailed Change Proposal

## 3.3 Abbreviations

| A-GPS | Assisted GPS |
| --- | --- |
| ANSI | American National Standards Institute |
| AoA | Angle of Arrival |
| AoD | Angle of Departure |
| DL | DownLink |
| DTD | Document Type Definition |
| E-OTD | Enhanced Observed Time Difference (E-OTD) |
| GMLC | Gateway Mobile Location Center |
| GMT | Greenwich Mean Time |
| GPS | Global Positioning System |
| HTTP | Hypertext Transfer Protocol |
| HTTPS | HTTP Secure |
| LCS | Location Services |
| MLC | Mobile Location Center |
| MLP | Mobile Location Protocol |
| MPC | Mobile Positioning Center |
| MS | Mobile Station |
| MSID | Mobile Station Identifier |
| MSISDN | Mobile Station ISDN |
| NR | New Radio |
| OMA | Open Mobile Alliance |
| OTDOA | Observed Time Difference of Arrival |
| SLP | SUPL Location Platform |
| SSL | Secure Socket Layer |
| SUPL | Secure User Plane Location |
| TDOA | Time Difference of Arrival |
| TLS | Transport Layer Security |
| UL | UpLink |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |
| U-TDOA | Uplink Time Difference of Arrival |
| UTM | Universal Transverse Mercator |
| WAP | Wireless Application Protocol |
| WGS | World Geodetic System |
| XML | Extensible Markup Language |

### 7.2.3 Location Element Definitions

| <!-- RLP_LOC --> |  |  |
| --- | --- | --- |
| <!-- RLP V1.2 Document Type Definition Copyright Open Mobile Alliance Ltd., 2011 All rights reserved RLP is an XML language. Typical usage: <?xml version="1.0"?> <!DOCTYPE rlp_svc_xxx PUBLIC "-//OMA//DTD {abbrev x.y}//EN" "http://www.openmobilealliance.org/DTD/{filename}" [<?oma-{ref}-ver supported-versions="{versions}"?>]> <rlp_svc_xxx> ... </rlp_svc_xxx> Terms and conditions of use are available from the Open Mobile Alliance Ltd. web site at http://www.openmobilealliance.org/ --> |  |  |
| <!ELEMENT | pos | (msid, (pd ¦ poserr), add_info?)> |
| <!ATTLIST | pos |  |
|  | pos_method (CELL ¦ OTDOA ¦ GPS ¦ A-GPS ¦ GNSS ¦ A-GNSS ¦ E-OTD ¦ U-TDOA ¦ AFLT ¦ EFLT ¦ E-CID ¦ BARO ¦ BT ¦ MBS ¦ WLAN ¦ NR E-CID ¦ Multi-RTT ¦ DL-AoD ¦ DL-TDOA ¦ UL-TDOA ¦ UL-AoA ¦ UNKNOWN ¦ OTHER) | #IMPLIED |
|  | result_type (INTERMEDIATE ¦ FINAL) | "FINAL"> |
| <!ELEMENT | eme_pos | (msid, (pd ¦ poserr), esrd?, esrk?)> |
| <!ATTLIST | eme_pos |  |
|  | pos_method (CELL ¦ OTDOA ¦ GPS ¦ A-GPS ¦ GNSS ¦ A-GNSS ¦ E-OTD ¦ U-TDOA ¦ AFLT ¦ EFLT ¦ E-CID ¦ BARO ¦ BT ¦ MBS ¦ WLAN ¦ NR E-CID ¦ Multi-RTT ¦ DL-AoD ¦ DL-TDOA ¦ UL-TDOA ¦ UL-AoA ¦ UNKNOWN ¦ OTHER) | #IMPLIED |
|  | result_type (INTERMEDIATE ¦ FINAL) | "FINAL"> |
| <!ELEMENT | pd | (time, (shape ¦ civicloc ¦ (shape, civicloc)), (alt, alt_unc?)?, speed?, direction?, lev_conf?, qop_not_met? , MotionStateList?)> |
| <!ELEMENT | trl_pos | (msid, reference_object*, (pd ¦ poserr ¦ time))> |
| <!ATTLIST | trl_pos |  |
|  | trl_trigger (PERIODIC ¦ MS_AVAIL ¦ CHANGE_AREA ¦ VELOCITY ¦ DISTANCE ¦ EQUIDISTANCE) | #REQUIRED |
|  | pos_method (CELL ¦ OTDOA ¦ GPS ¦ A-GPS ¦ GNSS ¦ A-GNSS ¦ E-OTD ¦ U-TDOA ¦ AFLT ¦ EFLT ¦ E-CID ¦ BARO ¦ BT ¦ MBS ¦ WLAN ¦ NR E-CID ¦ Multi-RTT ¦ DL-AoD ¦ DL-TDOA ¦ UL-TDOA ¦ UL-AoA ¦ UNKNOWN ¦ OTHER) | #IMPLIED> |
| <!ELEMENT | poserr | (result, add_info?, time)> |
| <!ELEMENT | time | (#PCDATA)> |
| <!ATTLIST | time |  |
|  | utc_off CDATA | "0000"> |
| <!ELEMENT | alt | (#PCDATA)> |
| <!ELEMENT | alt_unc | (#PCDATA)> |
| <!ELEMENT | civicloc | (civicloc_element+)> |
| <!ATTLIST | civicloc |  |
|  | xml:lang CDATA | #IMPLIED> |
| <!ELEMENT | civicloc_element | (#PCDATA)> |
| <!ATTLIST | civicloc_element |  |
|  | element_type (COUNTRY ¦ A1 ¦ A2 ¦ A3 ¦ A4 ¦ A5 ¦ A6 ¦ PRD ¦ POD ¦ STS ¦ HNO ¦ HNS ¦ LMK ¦ LOC ¦ FLR ¦ NAM ¦ PC ¦ BLD ¦ UNIT ¦ ROOM ¦ PLC ¦ PCN ¦ POBOX ¦ ADDCODE ¦ SEAT ¦ RD ¦ RDSEC ¦ RDBR ¦ RDSUBBR ¦ PRM ¦ POM) | #REQUIRED |
|  | xml:lang CDATA | #IMPLIED> |
| <!ELEMENT | MotionStateList | (PrimaryMotionState, SecondaryMotionState*, Confidence)> |
| <!ELEMENT | PrimaryMotionState | (MotionState)> |
| <!ELEMENT | SecondaryMotionState | (MotionState)> |
| <!ELEMENT | MotionState | (#PCDATA)> |
| <!ELEMENT | Confidence | (#PCDATA)> |
| <!ELEMENT | qop_not_met | EMPTY> |
| <!ELEMENT | direction | (#PCDATA)> |
| <!ELEMENT | speed | (#PCDATA)> |
| <!ELEMENT | lev_conf | (#PCDATA)> |
| <!ELEMENT | geo_info | (CoordinateReferenceSystem)> |
| <!ATTLIST | geo_info |  |
|  | requested_positiondata (SHAPE ¦CIVICLOC ¦ SHAPE_AND_CIVICLOC) | "SHAPE" |
|  | Strict (YES ¦ NO) | "YES"> |
| <!ELEMENT | CoordinateReferenceSystem | (Identifier)> |
| <!ELEMENT | Identifier | (code, codeSpace, edition)> |
| <!ELEMENT | code | (#PCDATA)> |
| <!ELEMENT | codeSpace | (#PCDATA)> |
| <!ELEMENT | edition | (#PCDATA)> |
| <!ELEMENT | service_coverage | ((cc, ndc*)+)> |
| <!ELEMENT | MotionStateRequest | EMPTY> |

### 8.34.1 pos_method

| Description: |  |  |
| --- | --- | --- |
| Specifies the positioning method used to obtain the associated location estimate |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | CELL | Cell coverage based positioning method |
|  | OTDOA | Observed Time Difference of Arrival (OTDOA) positioning method |
|  | GPS | Global Positioning System (GPS) based positioning method |
|  | A-GPS | Assisted GPS based positioning method |
|  | E-OTD | Enhanced Observed Time Difference (E-OTD) positioning method |
|  | U-TDOA | Uplink Time Difference of Arrival (U-TDOA) positioning method |
|  | AFLT | Advanced Forward Link Triangulation positioning method |
|  | EFLT | Enhanced Forward Link Triangulation positioning method |
|  | E-CID | Enhancement Cell ID positioning method |
|  | BARO | Barometric Sensor positioning method |
|  | BT | Bluetooth positioning method |
|  | MBS | Metropolitan Beacon System positioning method |
|  | WLAN | WLAN positioning method |
|  | NR_E-CID | NR enhanced cell ID methods |
|  | Multi-RTT | Multi-Round Trip Time Positioning method |
|  | DL-AoD | Downlink Angle-of-Departure method |
|  | DL-TDOA | Downlink Time Difference of Arrival method |
|  | UL-TDOA | Uplink Time Difference of Arrival method |
|  | UL-AoA | Uplink Angle of Arrival method |
|  | UNKNOWN | Unknown positioning method |
|  | OTHER | Any other positioning method |
| Default value: |  |  |
| Example: | <pos pos_method="A-GPS"> …. </pos> |  |
| Note: |  |  |