# Change Request

| Title: | CR to Close OpenCMAPI RDRR Comments A050, A051 & A052 | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-RD-OpenCMAPI-V1_1-20130130-D |  |
| Submission Date: | 4st of February 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Alan Soloway, Qualcomm, asoloway@qti.qualcomm.com |  |
| Replaces: |  |  |

Reason for Change

This CR is proposing to close the following comments:

| A050 | 2012.12.13 | T | 6.19 CMAPI-P2P-xxx | Source: Qualcomm Form: doc#CD-OpenCMAPI-2012-0186 Comment: New Requirement Proposed: The OpenCMAPI Enabler SHALL be able to trigger the Local Device to announce its presence to Remote Device(s). | Status: OPEN Conf call 19/12/12: CR to be provided by Alan |
| --- | --- | --- | --- | --- | --- |
| A051 | 2012.12.13 | T | 6.19 CMAPI-P2P-xxx | Source: Qualcomm Form: doc#CD-OpenCMAPI-2012-0186 Comment: New Requirement Proposed: The OpenCMAPI Enabler SHALL be able to instruct the Local Device to reject a Remote Device from joining an existing group. | Status: OPEN Conf call 19/12/12: CR to be provided by Alan |
| A052 | 2012.12.13 | T | 6.19 CMAPI-P2P-xxx | Source: Qualcomm Form: doc#CD-OpenCMAPI-2012-0186 Comment: New Requirement Proposed: The OpenCMAPI Enabler SHALL be able to instruct the Local Device to be restricted from an existing group. | Status: OPEN Conf call 19/12/12: CR to be provided by Alan |

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The CD OpenCMAPI SWG should review and agree this CR and close the Comments A050, A051 & A052.

Detailed Change Proposal

Modify section 6.19

This section identifies the requirements for interfaces related to the management of P2P Direct connections between devices.

| Label | Description | Release |
| --- | --- | --- |
| CMAPI-P2P-001 | The OpenCMAPI Enabler SHALL be able to detect which P2P direct connection technology (ies) is/are supported if any. | 1.1 |
| CMAPI-P2P-002 | The OpenCMAPI Enabler SHALL provide the capability to activate or deactivate the P2P function in a P2P Direct enabled device. | 1.1 |
| CMAPI-P2P-003 | The OpenCMAPI Enabler SHALL provide the capability to allow or not allow the device to have a P2P connection simultaneously to a normal data connection using the same radio technology (e.g. using Wi-Fi Direct to exchange pictures with other devices while being connected to internet through Wi-Fi). | 1.1 |
| CMAPI-P2P-004 | The OpenCMAPI Enabler SHALL be able to offer registration and authorization capabilities to the applications in order to announce the P2P Direct services supported. | 1.1 |
| CMAPI-P2P-005 | The OpenCMAPI Enabler SHALL be able to trigger the Local Device to discover Remote Device(s). | 1.1 |
| CMAPI-P2P-006 | The OpenCMAPI Enabler SHALL be able to retrieve list of Remote Devices discovered by the Local Device. | 1.1 |
| CMAPI-P2P-007 | The OpenCMAPI Enabler SHOULD be able to trigger the Local Device to discover services supported by discovered Remote Device(s) (e.g. a printing service). | 1.1 |
| CMAPI-P2P-008 | The OpenCMAPI Enabler SHALL be able to enable the Local Device to create a new P2P Direct group with one or several Remote Device (s) (The group could be a simple instance group – one time or a persistent one). | 1.1 |
| CMAPI-P2P-009 | The OpenCMAPI Enabler SHALL be able to enable the Local Device to remove a P2P Direct group it previously created. | 1.1 |
| CMAPI-P2P-010 | The OpenCMAPI Enabler SHALL be able to enable a Local Device to be a member of several groups simultaneously. | 1.1 |
| CMAPI-P2P-011 | The OpenCMAPI Enabler SHALL be able to disable a Local Device to be a member of several groups simultaneously, providing that the Local Device is not in charge of any of these groups. | 1.1 |
| CMAPI-P2P-012 | The OpenCMAPI Enabler SHALL support the Local Device to be in charge of a group. | 1.1 |
| CMAPI-P2P-013 | The OpenCMAPI Enabler SHALL be able to enable the Local Device to remove a Remote Device from an existing group the Local Device owns. | 1.1 |
| CMAPI-P2P-014 | The OpenCMAPI Enabler SHALL be able to request the Local Device to establish or close an existing P2P Direct connection within the group. | 1.1 |
| CMAPI-P2P-015 | The OpenCMAPI Enabler SHALL be able to retrieve the status of the P2P Direct connection. | 1.1 |
| CMAPI-P2P-016 | The OpenCMAPI Enabler SHALL be able to request the Local Device to act as a relay to share its data connection with Remote Device members of the group (i.e. enable or disable concurrent operations). | 1.1 |
| CMAPI-P2P-017 | The OpenCMAPI Enabler SHALL enable the Local Device to invite a Remote Device to join an existing group. | 1.1 |
| CMAPI-P2P-018 | The OpenCMAPI Enabler SHALL be able to retrieve from the Local Device which P2P Direct enabled device(s) are in an existing group to which the Local Device is a member of. | 1.1 |
| CMAPI-P2P-019 | The OpenCMAPI Enabler SHALL be able to trigger the Local Device to announce its presence to Remote Device(s). | 1.1 |
| CMAPI-P2P-020 | The OpenCMAPI Enabler SHALL be able to instruct the Local Device to reject a Remote Device from joining an existing group. | 1.1 |
| CMAPI-P2P-021 | The OpenCMAPI Enabler SHALL be able to instruct the Local Device to be restricted from an existing group owned by a Remote Device. | 1.1 |
| CMAPI-P2P-022 | The OpenCMAPI Enabler SHALL be able to instruct the Local Device to reject an invitation to join an existing group. | 1.1 |

Table 32: P2P Direct Connection Management Functional Requirements