| # Open Connection Manager WebAPI |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.1 – 6 Jun 2013 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-OpenCMAPI_Web_V1_1-20130606-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2013 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope	5

2.	References	6

2.1	Normative References	6

2.2	Informative References	7

3.	Terminology and Conventions	8

3.1	Conventions	8

3.2	Definitions	8

3.3	Abbreviations	8

4.	Introduction	10

4.1	Version 1.0	10

4.2	Version 1.1	10

5.	Open Connection Manager Web Interface CMAPI-3	12

5.1	URL Variables Considerations	12

5.2	Example Level 2	12

5.2.1	Example Level 3	12

5.3	API Management	13

5.4	Device Discovery APIs	13

5.5	Cellular Network Management APIs	13

5.5.1	Resources Summary	13

5.5.2	Resource: CMAPI	13

5.6	Connection Management APIs	14

5.7	Network Management APIs	14

5.8	CDMA2000 APIs	14

5.9	Device Service APIs	14

5.10	PINs/PUKs Management APIs	14

5.11	UICC Management APIs	14

5.12	WLAN APIs	14

5.13	Statistics APIs	14

5.14	Information Status APIs	14

5.15	SMS Management APIs	14

5.16	USSD Management APIs	14

5.17	Contact Management APIs	14

5.18	GNSS APIs	14

5.19	Data Push Service Management APIs	14

5.20	P2P Direct Connection APIs	14

5.21	Router Management APIs	14

5.22	Callback APIs	14

Appendix A.	Change History (Informative)	15

A.1	Approved Version History	15

A.2	Draft/Candidate Version <current version> History	15

Appendix B.	Static Conformance Requirements (Normative)	16

B.1	SCR for XYZ Client	16

B.2	SCR for XYZ Server	16

Appendix C.	<Additional Information>	17

C.1	App Headers	17

C.1.1	More Headers	17

Figures

Figure 1: Example Figure	13

Tables

Table 1: Example Table	13

# Scope

This specification defines a RESTful Device API for the Open Connection Management using HTTP protocol bindings, based on [OpenCMAPI_TS], through which connection management services are made available to different applications. I.e. it and provides the CMAPI-3 interface (also called OpenCMAPI WebAPI) of to the architecture described in [OpenCMAPI-AD].

The specification addresses the requirements enumerated in [OpenCMAPI-RD].

# References

The policy for reference lists is:

1.	OMA documents listed should have at least one approved version – draft-only docs should not be referenced.  Exception exists for documents that will be approved with or after the referenced doc is approved (may be part of same enabler package).  In short – approved docs should not reference unapproved docs.

2.	When a reference is made to an OMA specification, then Open Mobile Alliance with the TM symbol (™) should be used in the description.

3.	The name + version (no date) for OMA specifications are generally sufficient – dates should be used only if there is a specific reason to limit the usage.

4.	For references to WAP Forum docs, dates should not be included as DID's for the old WAP Forum specifications are enough and the reference description should refer to WAP Forum™.

5.	References to other affiliate docs should similarly provide sufficient information to uniquely determine the needed document and should provide the appropriate source information.

6.	The URL for OMA material (new OMA and affiliate) should always be http://www.openmobilealliance.org (an exception is OMNA that is reached through http://www.openmobilealliance.org/tech/omna)

Models to use

[REFLABEL]	<General Model> “Ref Title”, Ref information (source, date, id),
URL:http//<ref-source>/

[OMADOC]	<OMA Model> “OMA Document Title”,{ Version x.y,} Open Mobile Alliance™, OMA-<docname>{-<version>}, URL:http//www.openmobilealliance.org/

If there are no entries in the table – enter ‘none’ to be clear.

DELETE THIS COMMENT

## Normative References

| [OpenCMAPI_TS] |  |  |  | “Open Connection Manager API”, Open Mobile Alliance™, OMA-TS-OpenCMAPI-V1_1, URL: http://www.openmobilealliance.org/ |  |
| --- | --- | --- | --- | --- | --- |
| [OpenCMAPI-AD] |  |  |  | “Open Connection Manager API Architecture”, Open Mobile Alliance™, OMA-AD-OpenCMAPI-V1_1, URL:http://www.openmobilealliance.org/ |  |
| [OpenCMAPI-RD] |  |  |  | “Open CM API Requirements”, Open Mobile Alliance™, OMA-RD-OpenCMAPI-V1_1, URL:http://www.openmobilealliance.org/ |  |
| [REST_DeviceAPI_Common] |  |  |  | “Common definitions for RESTful Device APIs”, Open Mobile Alliance™, OMA-TS-REST_DeviceAPI_Common-V1_0, URL: http://www.openmobilealliance.org/ |  |
| [REST_SUP_OPENCMAPI] |  |  |  | “XML schema for the RESTful Device API for Connection Manager APIs”, Open Mobile Alliance™, OMA-SUP-XSD_rest_netapi_opencmapi-V1_0, URL: http://www.openmobilealliance.org/ |  |
| [RFC2119] |  |  |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |  |
| [RFC2616] |  |  |  | “Hypertext Transfer Protocol -- HTTP/1.1”, R. Fielding et. al, January 1999, URL:http://www.ietf.org/rfc/rfc2616.txt |  |
| [RFC3966] |  |  |  | “The tel URI for Telephone Numbers”, H.Schulzrinne, December 2004, URL: http://www.ietf.org/rfc/rfc3966.txt |  |
| [RFC3986] |  |  |  | “Uniform Resource Identifier (URI): Generic Syntax”, R. Fielding et. al, January 2005, URL:http://www.ietf.org/rfc/rfc3986.txt |  |
| [RFC4627] |  |  |  | “The application/json Media Type for JavaScript Object Notation (JSON)”, D. Crockford, July 2006, URL: http://www.ietf.org/rfc/rfc4627.txt |  |
| [SCRRULES] |  |  |  | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL:http://www.openmobilealliance.org/ |  |
| [XMLSchema1] |  |  |  | W3C Recommendation, XML Schema Part 1: Structures Second Edition, URL: http://www.w3.org/TR/xmlschema-1/ |  |
| [XMLSchema2] |  |  |  | W3C Recommendation, XML Schema Part 2: Datatypes Second Edition, URL: http://www.w3.org/TR/xmlschema-2/ |  |
| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |  |  |  |
| [RFC4234] |  | “Augmented BNF for Syntax Specifications: ABNF”. D. Crocker, Ed., P. Overell. October 2005, URL:http://www.ietf.org/rfc/rfc4234.txt |  |  |  |
| [SCRRULES] |  | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL:http://www.openmobilealliance.org/ |  |  |  |

## Informative References

| [OMADICT] | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |
| --- | --- |
| [REST_WP] | “Guidelines for RESTful Device APIs”, Open Mobile Alliance™, OMA-WP-Guidelines_for_RESTful_Device_APIs, URL:http://www.openmobilealliance.org/ |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

<< If needed, describe or declare using appropriate normative references the additional conventions that are used.  DELETE THIS COMMENT >>

## Definitions

For the purpose of this TS, all definitions from the OMA Dictionary apply [OMADICT].

<< Add definitions in new rows of the following table as needed.  The following examples show how dictionary references should be made as well as locally defined terms.  This table should be maintained in sorted alphabetic order based on the labels of the terms.

Examples:

Entity	Use definition #1 from [OMADICT]

Interactive Service	Use definition from [OMADICT]

Local Term	The definition description would be presented directly

DELETE THIS COMMENT>>

| Long Polling | A variation of the traditional polling technique, where the server does not reply to a request unless a particular event, status or timeout has occurred. Once the server has sent a response, it closes the connection, and typically the client immediately sends a new request. This allows the emulation of an information push from a server to a client. |
| --- | --- |
| << Add/Remove definition rows to this table as needed - DELETE This Row >> |  |

## 3.3	Abbreviations

<< Add abbreviations as needed to the following table.  No special notation should be made regarding terms copied from the Dictionary.  This table should be maintained in alphabetic order.

DELETE THIS COMMENT >>

| ACR | Anonymous Customer Reference |
| --- | --- |
| API | Application Programming Interface |
| CRUD | Create, Read, Update, Delete |
| HTTP | HyperText Transfer Protocol |
| JSON | JavaScript Object Notation |
| MIME | Multipurpose Internet Mail Extensions |
| OMA | Open Mobile Alliance |
| REST | REpresentational State Transfer |
| SCR | Static Conformance Requirements |
| SIP | Session Initiation Protocol |
| TS | Technical Specification |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |
| WP | White Paper |
| XML | eXtensible Markup Language |
| XSD | XML Schema Definition |
| OMA | Open Mobile Alliance |
| << Add/Remove abbreviation rows to this table as needed - DELETE This Row>> |  |

# Introduction

<< From a market perspective...

What can you do with this specification?

What problem does this solve?

How can this specification be applied?

Consider the target audience and provide deployment examples as possible.

DELETE THIS COMMENT >>

The Technical Specification of the RESTful Device API for Open Connection Management contains HTTP protocol bindings for the [OpenCMAPI_TS] specification, using the REST architectural style. The specification provides resource definitions, the HTTP verbs applicable for each of these resources, and the element data structures, as well as support material including flow diagrams and examples using the supported message body format (i.e. JSON).

## Version 1.0

Void.

Version 1.1 is the first version which was produced of this document.

This section provides a high level, concise and informative description of the main functionality supported in the initial version of the specification.  The description should be brief, target length should be a few paragraphs.   When the enabler or reference release is finished, this description should be aligned with the final functionality.

DELETE THIS COMMENT

## Version 1.1

Version 1.1 of this specification supports the following operations:

- Version 1.1 of the RESTful Device API for Open CM API specification addresses the following aspects:

  - Security and concurrency control function, e.g. access control and authorization

  - Device Discovery & Device Handling

  - Device Services

  - Cellular Network Connection Management

  - PIN/PUK Management

  - Interaction with the UICC

  - WLAN connection management

  - Information Status handling

  - Statistics handling

  - GNSS handling

  - SMS&USSD management

  - Push Data service management

  - Callbacks & Registration/Deregistration to receive callbacks

  - Additional Information Status functions

  - Phone Book /Contacts management support

  - Support of Hotspot 2.0

  - Support of P2P (or D2D as known in 3GPP) Direct connection

  - Router Management support

This section should be included for each new major or minor version of the specification.

It should provide a high level, concise and informative description of the new or modified functionality introduced in this version of the specification, compared to the previous version.  The description should be brief, target length should be a few paragraphs.  When the enabler or reference release is finished, this description should be aligned with the final functionality differences.

DELETE THIS COMMENT

# Open Connection Manager Web Interface CMAPI-3

<<Sections for the normative specification text.  Fill in as needed.  The following validates the styles used for the headers.  DELETE THIS COMMENT >>

## URL Variables Considerations

The following applies to all resources defined in this specification regardless of the representation format (i.e. XML or JSON):

- Reserved characters in URL variables (parts of a URL denoted below by a name in curly brackets) MUST be percent-encoded according to [RFC3986]. Note that this always applies, no matter whether the URL is used as a Request URL or inside the representation of a resource (such as in “resourceURL” and “link” elements).

- If a user identifier (e.g. address, participantAddress, etc.) of type anyURI is in the form of an MSISDN, it MUST be defined as a global number according to [RFC3966] (e.g. tel:+19585550100). The use of characters other than digits and the leading “+” sign SHOULD be avoided in order to ensure uniqueness of the resource URL. This applies regardless of whether the user identifier appears in a URL variable or in a parameter in the body of an HTTP message.

- If an equipment identifier of type anyURI is in the form of a SIP URI, it MUST be defined according to [RFC3261].

- If a user identifier (e.g. address, userId, etc) of type anyURI is in the form of an Anonymous Customer Reference (ACR), it MUST be defined according to [IETF_ACR_draft], i.e. it MUST include the protocol prefix 'acr:' followed by the ACR.

  - The ACR ‘authorization’ is a supported reserved keyword, and MUST NOT be assigned as an ACR to any particular end user. See Error! Reference source not found. for details regarding the use of this reserved keyword.

The following request URL variables are common for all HTTP methods:

| Name | Description |
| --- | --- |
| serverRoot | Server base url: hostname+port+base path. Port and base path are OPTIONAL. Example: example.com/exampleAPI {cmapi.device¦localhost}??? |
| apiVersion | Version of the API client wants to use.The "apiVersion" URL variable SHALL have the value v1 to indicate that the API corresponds to this version of this specification. See [REST_Device_Common] which specifies the semantics of this variable. |

Table 1: URL variables

## Example Level 2

<text>

### Example Level 3

<text>

#### Example Level 4

<text>

Figure 1: Example Figure

|  | Column 1 | Column 2 |
| --- | --- | --- |
| Row 1 | Grid 1,1 data | Grid 1,2 data |
| Row 2 | Grid 2,1 data | Grid 2,2 data |

Table 2: Example Table

## API Management

## Device Discovery APIs

## Cellular Network Management APIs

### Resources Summary

### Resource: CMAPI

#### Additional request URL variables

#### GET

##### Example: example (Informative)

#### PUT

##### Example: example (Informative)

#### POST

##### Example: Example: example (Informative

#### DELETE

##### Example: Example: example (Informative

## Connection Management APIs

## Network Management APIs

## CDMA2000 APIs

## Device Service APIs

## PINs/PUKs Management APIs

## UICC Management APIs

## WLAN APIs

## Statistics APIs

## Information Status APIs

## SMS Management APIs

## USSD Management APIs

## Contact Management APIs

## GNSS APIs

## Data Push Service Management APIs

## P2P Direct Connection APIs

## Router Management APIs

## Callback APIs

Change History	(Informative)

<< The following is a model of a revision table.  DELETE THIS COMMENT >>

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No prior version –or- No previous version within OMA |

Draft/Candidate Version <current version> History

<< This section is available in pre-approved versions – it should be removed in the actual approved versions.  DELETE THIS COMMENT >>

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Versions OMA-xxyyz-V1_2 | 4 Jun 2013 | All | First baseline document |

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [SCRRULES].

The following is a model of a set of SCR tables.  DELETE THIS COMMENT

SCR for XYZ Client

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| XYZ-C-001-M | Something mandatory | Section x.y | (XYZ-C-004-O OR XYZ-C-003-M) AND XYZ-C-002-O |
| XYZ-C-002-O | Something optional | Section x.y |  |
| XYZ-C-003-M | Dependencies on ZYX | Section x.y | ZYX:MCF |
| XYZ-C-004-O | Dependencies on ZYX | Section x.y | ZYX:OCF |

SCR for XYZ Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| XYZ-S-001-M | Something mandatory | Section x.y | XYZ-S-004-O OR XYZ-S-002-O OR XYZ-S-003-M |
| XYZ-S-002-O | Something optional | Section x.y |  |
| XYZ-S-003-M | Dependencies on ZYX | Section x.y | ZYX:MSF |
| XYZ-S-004-O | Dependencies on ZYX | Section x.y | ZYX:OSF |

<Additional Information>

If needed, add annex to provide additional information to support the document.  In general, this information should be informative, as normative material should be contained in the primary body of the document.

Note that the styles for the headers in the appendix (App1, App2, App3) are different than the main body.  The use below is intended to validate the styles to be used.  Remove if not needed.

DELETE THIS COMMENT

App Headers

<More text>

More Headers

<More text>

Even More Headers

<More text>