# Change Request

| Title: | P2P Direct Mgt APIs part 2 | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_1-20130701-D |  |
| Submission Date: | 2nd Jul 2013 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Dieter Gludovacz, Deutsche Telekom AG, dieter.gludovacz@t-mobile.at Thierry Berisot, Deutsche Telekom AG, thierry.berisot@telekom.de |  |
| Replaces: | OMA-CD-OpenCMAPI-2013-0073R01-CR_P2P_Direct_Mgt_APIs_part_2 |  |

- Reason for Change

R02 reflects the comments made during the conf call the 24/07/2013.

R01 reflects the comments made during the conf call the 10/07/2013.

This CR addresses comments received during R&A by proposing additional functions and changes to the P2P direct Management APIs. Green marked functions in the table are changed or added in order to support respective requirements. Blue marked text needs more investigation.

• Missing a connection ACCEPT API – there doesn’t appear to be a way to alert an application of an incoming connection request and the discovery event the connect request is related to.

- Propose to add AcceptConnection()

• Missing a group JOIN ACCEPT API – the proposal does specify methods to send a group join invitation and reject invitations from a sender or receiver perspective but there doesn’t appear to be a method to positively accept an invitation on the receiver side.

- Propose to add AcceptInvitationToGroup()

• Missing a discovery MATCH API – doesn’t appear to have a way for the modem to alert an DeviceID/ServiceID/ServiceRecord (LTE-D expression container) of a discovery match

- Propose to add DiscoveryMatch() – shouldn’t this be a callback?

• Missing a discovery RESOLVE API – doesn’t define a way to resolve a ServiceRecord (LTE-D expression container) for metadata or connection info. The GetP2PInfo() API is only defined to get the local modem’s radio capabilities (not request remote service detail or capabilities)

- This needs further discussion, subject to a separate CR.

- The ServiceID & ServiceRecord structure proposal will be inadequate to support LTE-D expressions.

- e.g. A 32-bit dword ServiceID is too small to exchange an expression between the modem and app.

- For LTE-D, the app needs an API to pass a ‘double quadword’ 128 bits.

- There is no allowance for metadata in a ServiceRecord

- Propose to change add FFS

| Label | Description | supported by function |
| --- | --- | --- |
| CMAPI-P2P-001 | The OpenCMAPI Enabler SHALL be able to detect which P2P direct connection technology(ies) is/are supported if any. Do we need a method to detect the P2P direct technologies supported of the remote device as well? | GetP2PInfo() |
| CMAPI-P2P-002 | The OpenCMAPI Enabler SHALL provide the capability to activate or deactivate the P2P functions (Discovery & Connection) in a P2P Direct enabled device. | Enable/DisableDirectDiscovery() Enable/DisableDirectConnection() |
| CMAPI-P2P-003 | The OpenCMAPI Enabler SHALL provide the capability to allow or not allow the device to have a P2P connection simultaneously to a normal data connection using the same radio technology (e.g. using Wi-Fi Direct to exchange pictures with other devices while being connected to internet through Wi-Fi). | Allow/DisallowSimultaneousConnection()? |
| CMAPI-P2P-004 | The OpenCMAPI Enabler SHALL be able to offer registration and authorization capabilities to the applications in order to announce the P2P Direct services supported. Informational Note: The opportunity to set a filter of a list of services identifiers for the Applications to register specific service(s) will be detailed in the Technical Specification. Authorization needs more thoughts Registration needs more thougths | Announce() |
| CMAPI-P2P-005 | The OpenCMAPI Enabler SHALL be able to trigger the Local Device to discover Remote Device(s). | Monitor() |
| CMAPI-P2P-006 | The OpenCMAPI Enabler SHALL be able to retrieve the list of Remote Devices discovered by the Local Device. Callback Function? DiscoveryMatch() or so? -> to alert of a discovery match | DiscoveryMatch() ResolveDiscovery() |
| CMAPI-P2P-007 | The OpenCMAPI Enabler SHOULD be able to trigger the Local Device to discover services supported by discovered Remote Device(s) (e.g. a printing service). Informational Note: The opportunity to provide a list of services identifiers to the Applications to identify specific service(s) will be detailed in the Technical Specification. | Monitor() |
| CMAPI-P2P-008 | The OpenCMAPI Enabler SHALL be able to enable the Local Device to create a new P2P Direct group with one or several Remote Device (s) (The group could be a simple instance group – one time or a persistent one). Enable/disableGroupFunctions? | CreateGroup() |
| CMAPI-P2P-009 | The OpenCMAPI Enabler SHALL be able to enable the Local Device to remove a P2P Direct group it previously created. | RemoveGroup() |
| CMAPI-P2P-010 | The OpenCMAPI Enabler SHALL be able to enable a Local Device to be a member of several groups simultaneously. | EnableMembershipInSeveralGroups() |
| CMAPI-P2P-011 | The OpenCMAPI Enabler SHALL be able to disable a Local Device to be a member of several groups simultaneously, providing that the Local Device is not in charge of any of these groups. | DisableMembershipInSeveralGroups() |
| CMAPI-P2P-012 | The OpenCMAPI Enabler SHALL be able to enable the Local Device to remove a Remote Device from an existing group the Local Device owns. | RemoveDeviceFromGroup() |
| CMAPI-P2P-013 | The OpenCMAPI Enabler SHALL be able to request the Local Device to establish or close an existing P2P Direct connection within the group. | EstablishConnection(), (Connect() to be replaced) CloseConnection() AcceptConnection() RejectConnection() |
| CMAPI-P2P-014 | The OpenCMAPI Enabler SHALL be able to retrieve the status of the P2P Direct connection. | GetConnectionStatus() |
| CMAPI-P2P-015 | The OpenCMAPI Enabler SHALL be able to request the Local Device to act as a relay to share its data connection with Remote Device members of the group (i.e. enable or disable concurrent operations). | Enable/DisableRelay() |
| CMAPI-P2P-016 | The OpenCMAPI Enabler SHALL enable the Local Device to invite a Remote Device to join an existing group. What about a function that a remote device requests to join a group? | JoinGroup() (this is an invitation) |
| CMAPI-P2P-017 | The OpenCMAPI Enabler SHALL be able to retrieve from the Local Device which P2P Direct enabled device(s) are in an existing group to which the Local Device is a member of. | GetGroupInfo() |
| CMAPI-P2P-018 | The OpenCMAPI Enabler SHALL be able to trigger the Local Device to announce its presence to Remote Device(s). | Announce() |
| CMAPI-P2P-019 | The OpenCMAPI Enabler SHALL be able to instruct the Local Device to reject a Remote Device from joining an existing group. | RejectJoiningGroup() |
| CMAPI-P2P-020 | The OpenCMAPI Enabler SHALL be able to instruct the Local Device to be restricted from an existing group owned by a Remote Device. |  |
| CMAPI-P2P-021 | The OpenCMAPI Enabler SHALL be able to instruct the Local Device to reject an invitation to join an existing group. | RejectInvitationToGroup() AcceptInvitationToGroup() |

- Impact on Backward Compatibility

None

- Impact on Other Specifications

None

- Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

- Recommendation

The CD OpenCMAPI SWG should review and agree this CR.

- Detailed Change Proposal

- Add FFS and change type

### 6.2.x ServiceRecord

Definition ServiceRecord

This prototype defines a structure which describes a service record.

| Field Name | Type | Description |
| --- | --- | --- |
| serviceID | dqword | The identifier of a service |
| metadata | UTF8* | Metadata of the service. |

Editor’s note: FFS ->The ServiceID & ServiceRecord structure proposal will be inadequate to support LTE-D expressions

e.g. A 32-bit dword ServiceID is too small to exchange an expression between the modem and app.

o	For LTE-D, the app needs an API to pass a ‘double quadword’ 128 bits. 256bits? To be investigated to check if we can use byte array?

o	There is no allowance for metadata in a ServiceRecord

### 6.2.35 CallbackID

Definition CallbackID

This prototype defines an enumeration of callback ID to register to or unregister from.

| Field Name | Type | Description |
| --- | --- | --- |
| CallbackID | dword | Callback ID: - 0x00000001: Devices Detection Complete - 0x00000002: Device Changed - Device Addition and Removal - 0x00000003: GetNetworkList Async Complete - 0x00000004: Connect Complete - 0x00000005: Disconnect Complete - 0x00000006: Cancellation of connection Complete - 0x00000007: Session State Change - 0x00000008: Bearer Status Change - 0x00000009: Traffic Channel Dormancy - 0x0000000A: CDMA 2000 Activation State - 0x0000000B: Search WLAN Network Complete - 0x0000000C: Radio Power State Change - 0x0000000D: SetRadioState Async Complete - 0x0000000E: Roaming Status Change - 0x0000000F: Signal Strength Change - 0x00000010: GNSS State Change - 0X00000011: SMS Received - 0x00000012: SMS Received with the message - 0x00000013: Not used - 0x00000014: USSD Message Received - 0x00000015: QoS change - 0x00000016: RF Information change - 0x00000017: PIN PUK Status Change - 0x00000018: WLAN Scan complete - 0x00000019: WLAN New network available - 0x0000001A: WLAN Connection Status - 0X0000001B: PUSH message received - 0x0000001C: OMA DM Status Change - 0x0000001D: UICC ToolKit Proactive Command callback - 0x0000001E: UICC Device Terminal Profile callback - 0x0000001F: Verify PIN Needed - 0x00000020: Permitted Bearers Change - 0x00000021: Byte Count - 0x00000022: Connect Secondary PDP Context Complete - 0x00000023: Disconnect Secondary PDP Context Complete - 0x00000024: Cancellation of connection Secondary PDP Context Complete - 0x00000025: Incoming Voice Call - 0x00000026: P2P Discovery Match - 0x00000027: P2P Connection - 0x00000028: P2P Group Notification |

- Add P2P Direct Mgt functions

## P2P Direct Management APIs

Editor’s note: Error Codes values missing.

### 7.16.1 CMAPI_P2P_ResolveDiscovery()

The CMAPI_P2P_ResolveDiscovery() function is used to resolve a ServiceRecord (LTE-D expression container) for metadata and/or connection info.

Prototype

dword CMAPI_P2P_ResolveDiscovery(dword deviceID, ServiceRecord* pService, dword* pP2PInfo)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| pService | Input | The ServiceRecord to be resolved. |
| pP2PInfo | Output | Pointer to get the P2P direct connection technology type in bitmap. In the case of a device with multiple P2P direct connection technologies supported, there MAY be multiple settings returned. The bitmap definition follows the definition of P2PInfoType: - 0x00000000: None - 0x00000001: Wi-Fi Direct (could there be several versions) - 0x00000002: reserved for future use - 0x00000004: reserved for future use - 0x00000008: reserved for future use - 0x00000010: reserved for future use - 0x00010000: LTE Direct - 0x00020000: reserved for future use |

### 7.16.8 CMAPI_P2P_EstablishConnection()

The CMAPI_P2P_EstablishConnection() function is used to request the Local Device to establish a connection (P2P Direct connection or connection via network, subject to Service Provider policy) to a Remote Device or a group.

Editor’s note: This function requires future study.

Prototype

dword CMAPI_P2P_EstablishConnection(dword deviceID, DeviceRecord* pRemoteDevices, ServiceRecord* pServices, dword* connectionID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| pRemoteDevices | Input | Optional; List of Remote Devices |
| pServices | Input | List of Service Identifiers |
| connectionID | Output | The ID of the connection concerned |

### 7.16.8 CMAPI_P2P_RejectConnection()

The CMAPI_P2P_RejectConnection() function is used to reject an incoming connection request (P2P Direct connection or connection via network, subject to Service Provider policy)

Prototype

dword CMAPI_P2P_RejectConnection(dword deviceID, dword connectionID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| connectionID | Input | The ID of the connection concerned |

### 7.16.8 CMAPI_P2P_AcceptConnection()

The CMAPI_P2P_AcceptConnection() function is used to request an incoming connection request (P2P Direct connection or connection via network, subject to Service Provider policy) to a Remote Device or a group.

Editor’s note: FFS the way to alert an application of an incoming connection request and the discovery event the connect request is related to.

Prototype

dword CMAPI_P2P_AcceptConnection(dword deviceID, dword connectionID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| connectionID | Input | The ID of the connection concerned |

### 7.16.9 CMAPI_P2P_CloseConnection()

The CMAPI_P2P_CloseConnection() function is used to request the Local Device to close an exiting connection (P2P Direct connection or connection via network, subject to Service Provider policy) to a Remote Device or a group.

Prototype

dword CMAPI_P2P_CloseConnection (dword deviceID, dword connectionID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| connectionID | Input | The ID of the connection concerned |

### 7.16.10 CMAPI_P2P_AcceptInvitationToGroup()

The CMAPI_P2P_AcceptInvitationToGroup() function is used to positively accept an group join invitation on the receiver side.

Prototype

dword CMAPI_P2P_AcceptInvitationToGroup(dword deviceID, dword groupID, dword remoteDeviceID, dword invitationID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| groupID | Input | The ID of the group the Remote Device accepts to join. |
| remoteDeviceID | Input | The ID of the remote device accepting the invitation to this group. |
| invitationID | Input | The ID of this invitation |

### CMAPI_P2P_JoinGroup()

The CMAPI_P2P_JoinGroup() function is used to invite a Remote Device to join an existing group.

Prototype

dword CMAPI_P2P_JoinGroup (dword deviceID, dword groupID, dword remoteDeviceID, dword invitationID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| groupID | Input | The ID of the group the Remote Device is invited to join |
| remoteDeviceID | Input | The ID of the remote device being invited to this group. |
| invitationID | Input | The ID of this invitation |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. |
| 0X00000101 | The deviceID references a non-existing device or a device which is not open |
| 0X00000104 | The device does not contain hardware which supports this operation. |
| 0X00000130 | The device is not in a power state which allows this operation. |
| 0XF0000001 | The security request supplied when the API was opened does not grant privilege to access this functionality. You may close and reopen the API with updated credentials to perform this operation. |

### CMAPI_P2P_RejectInvitationToGroup()

The CMAPI_P2P_RejectInvitationToGroup() function is used to reject an invitation to join an existing group

Prototype

dword CMAPI_P2P_RejectInvitationToGroup (dword deviceID, dword groupID, dword remoteDeviceID, dword invitationID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| groupID | Input | The ID of the group the Local Device is rejecting to join |
| remoteDeviceID | Input | The ID of the device rejecting the invitation to join this group. |
| invitationID | Input | The ID of the invitation concerned. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. |
| 0X00000101 | The deviceID references a non-existing device or a device which is not open |
| 0X00000104 | The device does not contain hardware which supports this operation. |
| 0X00000130 | The device is not in a power state which allows this operation. |
| 0XF0000001 | The security request supplied when the API was opened does not grant privilege to access this functionality. You may close and reopen the API with updated credentials to perform this operation. |

### CMAPI_P2P_RejectJoiningGroup()

The CMAPI_P2P_RejectJoiningGroup() function is used to reject a Remote Device from joining to an existing group.

Prototype

dword CMAPI_P2P_RejectJoiningGroup (dword deviceID, dword groupID, dword remoteDeviceID, dword requestID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| groupID | Input | The ID of the group concerned. |
| remoteDeviceID | Input | The ID of the remote device being rejected to join this group. |
| requestID | Input | The ID of the request for joining the group. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. |
| 0X00000101 | The deviceID references a non-existing device or a device which is not open |
| 0X00000104 | The device does not contain hardware which supports this operation. |
| 0X00000130 | The device is not in a power state which allows this operation. |
| 0XF0000001 | The security request supplied when the API was opened does not grant privilege to access this functionality. You may close and reopen the API with updated credentials to perform this operation. |

### CMAPI_P2P_RequestToJoinGroup()

The CMAPI_P2P_RequestToJoinGroup() function is used to send a request for joining an existing group to the group owner.

Prototype

dword CMAPI_P2P_RequestToJoinGroup (dword deviceID, dword groupID, dword remoteDeviceID, dword requestID)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned. |
| groupID | Input | The ID of the group the device wants to join |
| remoteDeviceID | Input | The ID of the device being group owner. |
| requestID | Output | The ID of this request. |

- Add callback functions

### 8.3.x CMAPI_Callback_P2P_DiscoveryMatch()

The CMAPI_Callback_P2P_DiscoveryMatch() function is used to alert the Local Device of a DeviceID/ServiceID/ServiceRecord (LTE-D expression container) discovery match.

Prototype

dword CMAPI_Callback_P2P_DiscoveryMatch (CallbackStatus status, dword deviceID, dword remoteDeviceID, dqword serviceID, dword result)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| status | Input | The status of the callback. |
| deviceID | Input | The ID of the device concerned. |
| remoteDeviceID | Input | Remote Devices that match the criteria Editor’s note: FFS single or list? |
| serviceID | Input | Service Identifiers that match the criteria Editor’s note: FFS single or list? |
| result | Input | - 0x00000001: Device & Service matching criteria - 0x00000002: Device only matching criteria - 0x00000003: Service only matching criteria |

### 8.3.x CMAPI_Callback_P2P_Connection()

This callback shows the result of call made to CMAPI _P2P_EstablishConnection().

Prototype

dword CMAPI_P2P_Connection(CallbackStatus status, dword deviceID, dword remoteDeviceID, dqword serviceID, dword connectionID, dword result)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| status | Input | The status of the callback. |
| deviceID | Input | The ID of the device concerned. |
| remoteDeviceID | Input | The ID of the Remote Device concerned. |
| serviceID | Input | The ID of the service concerned. |
| connectionID | Input | The ID of the connection concerned. |
| result | Input | - 0x00000001: No answer - 0x00000002: Connection request accepted by remote device - 0x00000003: Connection request rejected by remote device - 0x00000004: Connection established |

### 8.3.x CMAPI_Callback_P2P_GroupNotification()

The CMAPI_Callback_P2P_GroupNotification() function is invoked as a result of a previous call to CMAPI_P2P_JoinGroup().

Prototype

dword CMAPI_Callback_P2P_GroupNotification (CallbackStatus status, dword deviceID, dword remoteDeviceID, dqword serviceID, dword result)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| status | Input | The status of the callback. |
| deviceID | Input | The ID of the device concerned. |
| remoteDeviceID | Input | The ID of the Remote Device concerned. |
| serviceID | Input | The ID of the service concerned. |
| result | Input | - 0x00000001: No answer - 0x00000002: Invitation accepted - 0x00000003: Invitation rejected |

- Add dqword definition

- dword will be used to denote 2 word values,

- qword will be used to denote 2 dword values,

- dqword will be used to denote 2 qword values,

- byte parameter [256] will indicate a 256 bytes long parameters,