Change Request

| Title: | CR_TS_Control_Plane_RFC3265_Updates_Pt4 | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA COM PCPS |  |
| Doc to Change: | OMA-PCPS-TS_Control_Plane-V1_0-20140525-D |  |
| Submission Date: | 25 May 2014 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Bob Fredericks – Motorola Solutions; |  |
| Replaces: | n/a |  |

Reason for Change

This change request updates the Control Plane TS with regard to references to obsolete RFC3265.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The PCPS group is recommended to agree to the changes and incorporate it into the corresponding document.

Detailed Change Proposal

- Updating obsolete normative reference RFC3265 to replacement RFC6665

From 7.3.3.1, pg. 256-257:

.

#### Subscribing to Participant Information

The PoC Server performing the Participating PoC Function MAY subscribe to the conference state event package to obtain information of the Participants of a PoC Session, e.g. for charging purposes.

When subscribing to the conference state event package, the PoC Server:

1. SHALL generate a SIP SUBSCRIBE request, using a new SIP-dialog when initiating a new subscription, according to rules and procedures of [RFC6665] and [RFC4575];

2. SHALL set the Request-URI of the SIP SUBSCRIBE request to PoC Session Identity;

3. SHOULD set the 'Expires' header to a value greater than zero.

4. SHALL include an Accept-Contact header with the PoC feature tag '+g.poc.talkburst' along with 'require' and 'explicit' parameters according to rules and procedures of [RFC3841] in an initial SIP SUBSCRIBE request;

NOTE 1:	When refreshing the subscription the parameters are specified here and in [RFC6665].

5. SHALL include a User-Agent header to indicate the OMA PoC release version of the PoC  Servers specified in subclause E.4.1 "Release version in User-agent and Server headers";

6. SHALL include the PoC feature tag '+g.poc.talkburst' in the Contact header;

7. SHALL include an Authenticated Originator's PoC Address with the address of the PoC Server;

8. SHALL include a Target-Dialog header as specified in [RFC4538]; and,

9. SHALL send the SIP SUBSCRIBE request towards the PoC Server performing the Controlling PoC Function according to rules and procedures of the SIP/IP Core.

Upon receiving a SIP 200 "OK" or a SIP 202 "Accepted" response to the SIP SUBSCRIBE request the PoC Server:

1. SHALL handle the request according to rules and procedures of [RFC3261].

Upon receiving an incoming SIP NOTIFY request that is part of the same SIP dialog as the previously sent SIP SUBSCRIBE request the PoC Server:

1. SHALL handle the request according to rules and procedures of [RFC6665] and [RFC4575].

When needed the PoC Server SHALL terminate the subscription and indicate it terminated according to rules and procedures of [RFC6665].

NOTE 2:	The contents of the SIP NOTIFY request body is specified in subclause 7.2.1.11.2 "Generating a SIP NOTIFY request".

.

.

From 7.4.2.1, pg. 267-268:

#### Reception of the SIP REGISTER request

Upon receiving a SIP REGISTER request the PoC Server:

1. SHALL generate a SIP 200 "OK" response according to rules and procedures of [RFC3261]; and,

2. SHALL send the SIP 200 "OK" response according to rules and procedures of SIP/IP Core.

If the PoC Server does not already subscribe to the "reg" event package for the PoC Address received in the SIP REGISTER request the PoC Server:

1. SHALL generate a SIP SUBSCRIBE request according to rules and procedures of [RFC6665] and [RFC3680];

2. SHALL set the Request-URI of the SIP SUBSCRIBE request to the PoC Address received in the SIP REGISTER request;

3. SHALL set the Authenticated Originator's PoC Address to a SIP URI of the PoC Server;

4. MAY set a expiration timer in Expires header, if set, the value of the timer SHALL be set according to rules and procedures of [RFC3903], in the same range as the registration timer recommended by SIP/IP Core; and,

5. SHALL send the SIP SUBSCRIBE request towards the SIP/IP Core using a new SIP dialog, according to rules and procedures of the SIP/IP Core.

.

.

From 7.4.2.2, pg. 268:

.

.

#### Reception of the SIP NOTIFY request

Upon receiving a SIP NOTIFY request from SIP/IP Core as the result of the "reg" event subscription the PoC Server:

- 1. SHALL generate a SIP 200 "OK" response according to rules and procedures of [RFC6665] and [RFC3680];

- 2. SHALL send the SIP 200 "OK" response according to rules and procedures of SIP/IP Core;

- 3. SHALL handle the received registration information as specified in subclause 5.7A "PoC Session association"; and,

- 4. SHALL terminate the subscription as specified in [RFC6665] and [RFC3680] if there is no cached registration information any longer for this PoC User.

When the SIP/IP Core corresponds with 3GPP/3GPP2 IMS, the PoC Server SHALL use 3GPP/3GPP2 IMS mechanisms, according to rules and procedures of [3GPP TS 24.229] / [3GPP2 X.S0013.4] with the clarifications given in this subclause.

.

.