UE PoC Box 1-1 PoC Session establishment On-demand Session

Originating flow – PoC Session Establishment with a UE PoC Box

The signaling sequences in this subclause show the establishment of the 1-1 PoC Session with a UE-PoC Box using on-demand signaling in the originating side. As a prerequisite for the on-demand signaling, PoC Client needs to be registered.

This flow shows the signaling sequence for the Confirmed indication case when the terminating side answers manually and SIP 200 "OK" response is not sent before the SIP 200 "OK" response from the terminating side is received.

QoE Profiles feature is used in this flow. PoC Client A requests Professional QoE Profile for the PoC Session and for his Local QoE Profile.

The flow is as shown in Figure 9 "1-1 PoC Session establishment on the originating side using on-demand signaling, establishment with a UE PoC Box ".

Figure 39: 1-1 PoC Session establishment on the originating side using on-demand signaling, establishment with a UE PoC Box

The steps of the flow are as follows:

1.	SIP INVITE request (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP INVITE request to SIP/IP Core A.

| Request-URI | sip:PoCConferenceFactoryURI.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Preferred-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=msg-taker;require;explicit |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA@networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst |
| Supported: | Timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Require: | recipient-list-invite |
| Content-Type: | multipart/mixed |
| Accept-Language: | en |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| m= | audio 3456 RTP/AVP 97 |
| a= | poc-qoe: professional |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:aa |
| XML MIME BODY |  |
| Content-Type: | application/resource-lists+xml |
| Content-Disposition: | recipient-list |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

2.	SIP 100 "Trying" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A sends a SIP 100 "Trying" response to PoC Client A.

3.	SIP INVITE request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP INVITE request to the PoC Server A.

| Request-URI | sip:PoCConferenceFactoryURI.networkA.net;gr |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=msg-taker;require;explicit |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Privacy: | id |
| Contact: | <sip:PoC-UserA.networkA.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst |
| Supported: | timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,OPTIONS,MESSAGE |
| Require: | recipient-list-invite |
| Content-Type: | multipart/mixed |
| Accept-Language: | en |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 5555::aaa:bbb:ccc:ddd |
| a= | poc-qoe:professional |
| m= | audio 3456 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | rtcp:5560 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:aa |
| m= | application 2000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:aa |
| XML MIME BODY |  |
| Content-Type: | application/resource-lists+xml |
| Content-Disposition: | recipient-list |
|  | <?xml version="1.0" encoding="UTF-8"?> |
|  | <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

4.	SIP 100 "Trying" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 100 "Trying" response to the SIP/IP Core A.

5.	PoC Server A sends SIP INVITE requests towards the Invited PoC Clients

The PoC Server A sends SIP INVITE requests to the Invited PoC Client.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=msg-taker;require;explicit |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Supported: | 100rel,timer,norefersub |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:ll |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

6.	PoC Server A receives SIP 200 "OK" response

The first SIP 200 "OK" response is received from the terminating PoC Network.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Accept-Language: | en |
| Content-Type: | application/sdp |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 97 |
| a= | rtpmap:97 AMR |
| a= | Rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:uu |

7.	SIP 200 "OK" response (from PoC Server A to SIP/IP Core A)

The PoC Server A sends a SIP 200 "OK" response to SIP/IP Core A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net> |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uac |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device";isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE, REFER,MESSAGE,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 57787 RTP/AVP 97 |
| i= | speech |
| a= | label:xxx |
| a= | rtpmap:97 AMR |
| a= | Rtcp:57000 |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:xxx |

A.	MBCP Media Burst Granted (PoC Server A to PoC Client A)

The PoC Server A sends MBCP Media Burst Granted message to PoC Client A.

8.	SIP 200 "OK" response (from SIP/IP Core A to PoC Client A)

The SIP/IP Core A forwards the SIP 200 "OK" response to PoC Client A.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | <sip:PoCConferenceFactoryURI.networkA.net> |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uac |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerA.networkA.net;gr;session=1-1>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device";isfocus |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE, REFER,MESSAGE,OPTIONS |
| Supported: | norefersub |
| SDP PARAMETERS |  |
| c= | IN IP6 57777::eee:fff:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 57787 RTP/AVP 97 |
| i= | speech |
| a= | label:xxx |
| a= | rtpmap:97 AMR |
| a= | Rtcp:57000 |
| m= | application 57790 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:xxx |

9.	SIP ACK request (from PoC Client A to SIP/IP Core A)

The PoC Client A sends a SIP ACK request to SIP/IP Core A.

B.	PoC Client sends RTP Media to PoC Server A

The PoC Client A sends RTP Media packets to PoC Server A.

10.	SIP ACK request (from SIP/IP Core A to PoC Server A)

The SIP/IP Core A forwards the SIP ACK request to PoC Server A.

Terminating flow – UE PoC Box Accepts the PoC Session

PoC Server X invites the UE PoC Box B to a PoC Session by sending a SIP INVITE request to the terminating SIP/IP Core.

The signaling sequences in this subclause show the establishment of the 1-1 PoC Session using on-demand signaling in the terminating side when the PoC User has explicitly requested to reach a PoC Box and a UE PoC Box is selected. As a prerequisite for the on-demand signaling, the PoC Client needs to be registered.

This scenario applies for the On-demand PoC Session case.

QoE Profiles feature is used in this flow, QoE Profile assigned to the PoC Session is Professional QoE Profile and PoC Client B is subscribed and selects Premium as Local QoE Profile.

This flow shows the signaling sequence for the session where the PoC User has explicitly requested to reach a PoC Box  and a UE PoC Box is selected. The flow is as shown in Figure 10 " PoC Session invitation with UE PoC Box ".

Figure 40 : PoC Session invitation with UE PoC Box

The steps of the flow are as follows:

1.	Session invitation has been received to invite PoC Box

2.	SIP INVITE request (from the PoC Server X to SIP/IP Core X)

PoC Server X sends the SIP INVITE request to SIP/IP Core X.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=msg-taker;require;explicit |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkbust; isfocus |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professsional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:ll |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

3.	SIP INVITE request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=msg-taker;require;explicit |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professsional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:ll kk |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

4.	SIP INVITE request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP INVITE request to PoC Server B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=msg-taker;require;explicit |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | poc-qoe:professional |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:ll |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

5.	SIP INVITE request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=1-1>;+g.poc.talkburst;isfocus |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Manual;Require |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:al |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

6.	SIP INVITE request (from SIP/IP Core B to UE PoC Box B)

SIP/IP Core B forwards the SIP INVITE request to UE PoC Box B.

| Request-URI | sip: UE-PoC-BoxB@PoCClientB.networkB.net |
| --- | --- |
| SIP HEADERS |  |
| Accept-Contact: | *;+g.poc.talkburst;sip.automata;sip.actor=principal;sip.description="poc recording device";require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=1-1>; +g.poc.talkburst;isfocus |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Manual;Require |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Message from Alice |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:al |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

7.	SIP 200 "OK" response (from UE PoC Box B to SIP/IP Core B)

UE PoC Box B sends a SIP 200 "OK" response to SIP/IP Core B. PoC User B is not allowed to use Professional QoE Profile because he is subscribed to Premium QoE Profile.

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-uebox/OMA2.0 Acme-Talk5000/v1.01 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 7557::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 75575 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:7500 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 7559 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:bl |

8.	SIP 200 "OK" response (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP 200 "OK" response to PoC Server B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-uebox/OMA2.0 Acme-Talk5000/v1.01 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 72557::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 75575 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:7500 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 72559 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:bl |

9.	SIP 200 "OK" response (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 200 "OK" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:uu |

10.	SIP 200 "OK" response (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 200 "OK" response to SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:uu |

11.	SIP 200 "OK" response (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 200 "OK" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:uu |

A.	MBCP Media Burst Taken message (from PoC Server X to PoC Server B).

The MBCP Media Burst Taken message is sent to inform the UE PoC Box B that PoC Client A has been granted to send a Media Burst. The message contains the identity of the PoC User that has been granted to send a Media Burst.

B.	MBCP Media Burst Taken message (from PoC Server B to UE PoC Box B).

The MBCP Media Burst Taken message is sent to inform the UE PoC Box B that PoC Client A has been granted to send a Media Burst. The message contains the identity of the PoC User that has been granted to send a Media Burst.

12.	SIP ACK request (from PoC Server X to SIP/IP Core X)

PoC Server X sends a SIP ACK request to SIP/IP Core X.

13.	SIP ACK request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP ACK request to SIP/IP Core B.

14.	SIP ACK request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP ACK request to PoC Server B.

15.	SIP ACK request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP ACK request to SIP/IP Core B.

16.	SIP ACK request (from SIP/IP Core B to UE PoC Box B)

SIP/IP Core B forwards the  SIP ACK request to UE PoC BoxB.

Terminating flow – PoC User directs UE PoC Box to accept the PoC Session

PoC Server X invites the PoC Client B to a PoC Session by sending a SIP INVITE request to the terminating SIP/IP Core.

The signaling sequences in this subclause show the establishment of the 1-1 PoC Session using on-demand signaling in the terminating side. As a prerequisite for the on-demand signaling, the PoC Client needs to be registered.

This scenario applies for the On-demand Session case.

QoE Profiles feature is used in this flow, QoE Profile assigned to the PoC Session is Professional QoE Profile and PoC Client B is subscribed and selects Premium as Local QoE Profile.

This flow shows the signaling sequence for the manual answer case when the PoC User  directs the UE PoC Box to accept the PoC Session. In this example the UE PoC Box only accepts PoC Speech. The flow is as shown in Figure 10 "PoC Session invitation where PoC User directs UE PoC Box to accept the PoC Session".

Figure 41: PoC Session invitation where PoC User directs UE PoC Box to accept the PoC Session

The steps of the flow are as follows:

1.	Session invitation has been received to invite PoC Client

2.	SIP INVITE request (from the PoC Server X to SIP/IP Core X)

PoC Server X sends the SIP INVITE request to SIP/IP Core X.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkbust; isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professsional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

3.	SIP INVITE request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| a= | poc-qoe:professsional |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

4.	SIP INVITE request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP INVITE request to PoC Server B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Referred-By: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerX.networkX.net;gr;session=1-1>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | 100rel,norefersub,timer |
| Session-Expires: | 1800 |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| SDP PARAMETERS |  |
| Content-Type: | application/sdp |
| c= | IN IP6 50555::ccc:ddd:aaa:bbb |
| m= | audio 53456 RTP/AVP 97 98 |
| a= | poc-qoe:professional |
| a= | rtpmap:97 AMR |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:53080 |
| i= | speech |
| a= | label:ll |
| a= | a=upcc:0 |
| m= | Application 50000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:ll kk |
| m= | video 7566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:kk |
| a= | a=upcc:0 |
| m= | message 54321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[50555::ccc:ddd:aaa:bbb]:54321/ksdgfjhafdga45sdf2a;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: Content-Disposition: | text/directory;profile="vcard";charset=UTF-8 "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:PoC-UserB@networkB.com" cc:copyControl="to" cc:anonymize="true"/> </list> </resource-lists> |

5.	SIP INVITE request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP INVITE request to SIP/IP Core B.

| Request-URI | sip:PoC-UserB@networkB.net |
| --- | --- |
| SIP HEADERS |  |
| P-Asserted-Identity: | "PoC User A" <sip:PoC-UserA@networkA.net> |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Privacy: | id |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=1-1>;+g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Manual;Require |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:al ak |
| m= | video 47566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:ak |
| a= | a=upcc:0 |
| m= | message 44321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[60333::ddd: ccc:aaa:bbb]:44321/gsgsdfg4;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

6.	SIP INVITE request (from SIP/IP Core B to PoC Client B)

SIP/IP Core B forwards the SIP INVITE request to PoC Client B.

| Request-URI | sip:PoC-ClientB@PoCClientB.networkB.net |
| --- | --- |
| SIP HEADERS |  |
| Accept-Contact: | *;+g.poc.talkburst; require;explicit |
| User-Agent: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC_ServerB.networkB.net;gr;session=1-1>; +g.poc.talkburst;isfocus;+g.poc.discretemedia |
| Supported: | norefersub,timer |
| Session-Expires: | 1800 |
| Answer-Mode: | Manual;Require |
| Allow: | INVITE,ACK,CANCEL,BYE,PRACK,UPDATE,REFER,MESSAGE,OPTIONS |
| Subject: | Let's talk |
| Alert-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myFavouriteRingingTone.wav> |
| Call-Info: | <http://publicweb.networkB.net/users/PoC-UserA@networkA.net/myOwnPicture.jpg>;purpose=icon |
| SDP PARAMETERS |  |
| c= | IN IP6 60333::ddd: ccc:aaa:bbb |
| a= | poc-qoe:professional |
| m= | audio 63776 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:63090 |
| i= | speech |
| a= | label:al |
| m= | Application 60000 udp TBCP |
| a= | fmtp:TBCP queuing=1; tb_priority=2; timestamp=1;multimedia=1;local_grant=1 |
| a= | floorid:0 mstrm:al ak |
| m= | video 47566 RTP/AVP 99 |
| a= | rtpmap:99 MP4V-ES |
| a= | label:ak |
| a= | a=upcc:0 |
| m= | message 44321 TCP/MSRP * |
| a= | accept-types:message/cpim application/vnd.oma.poc.final-report+xml application/vnd.oma.poc.detailed-progress-report+xml application/vnd.oma.poc.optimized-progress-report+xml |
| a= | accept-wrapped-types:text/plain |
| a= | path:msrp://[60333::ddd: ccc:aaa:bbb]:44321/gsgsdfg4;tcp |
| a= | max-size:100000000 |
| Included Media Content |  |
| Content-Type: | text/directory;profile="vcard";charset=UTF-8 |
| Content-Disposition: | "attachment" |
|  | begin:vcard fn:John P. Smith n:Smith;John;Peter;Dr.;MBA EMAIL:TYPE=internet:POC-UserB@networkA.net version:3.0 end:vcard |
| Invited parties identity information |  |
| Content-Disposition: | recipient-list-history;handling="optional" |
| Content-Type: | application/resource-lists+xml |
|  | <?xml version="1.0" encoding="UTF-8"?> <resource-lists xmlns="urn:ietf:params:xml:ns:resource-lists" xmlns:cc="urn:ietf:params:xml:ns:copycontrol" > <list> <entry uri="sip:anonymous@anonymous.invalid" cc:copyControl="to" cc:count="2"/> </list> </resource-lists> |

7.	SIP 180 "Ringing" response (from PoC Client B to SIP/IP Core B)

PoC Client B sends a SIP 180 "Ringing" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |

8.	SIP 180 "Ringing" response (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP 180 "Ringing" response to PoC Server B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-client/OMA2.0 Acme-Talk5000/v1.01 |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>"; +g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |

9.	SIP 180 "Ringing" response (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 180 "Ringing" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,PRACK, UPDATE,MESSAGE,OPTIONS |

10.	SIP 180 "Ringing" response (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 180 "Ringing" response to SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.ne;grt>;+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,PRACK,UPDATE,MESSAGE,OPTIONS |

11.	SIP 180 "Ringing" response (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 180 "Ringing" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Server: | PoC-serv/OMA2.0 |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr>;+g.poc.talkburst;+g.poc.fdcfo |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,PRACK,UPDATE,MESSAGE,OPTIONS |

12.	SIP 200 "OK" response (from UE PoC Box B to SIP/IP Core B)

The PoC User B directs the UE PoC Box to accept the PoC Session. The UE PoC Box  B sends a SIP 200 "OK" response to SIP/IP Core B. In this example the UE PoC Box only accepts PoC Speech. PoC User B is not allowed to use Professional QoE Profile because he is subscribed to Premium QoE Profile.

| SIP HEADERS |  |
| --- | --- |
| P-Preferred-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-uebox/OMA2.0 Acme-Talk5000/v1.01 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-UserB@networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 75555::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 7557 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:7500 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 7559 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:bl |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

13.	SIP 200 "OK" response (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP 200 "OK" response to PoC Server B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-uebox/OMA2.0 Acme-Talk5000/v1.01 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-UserB.networkB.net;gr=urn:uuid:00000000-0000-1000-8000-000A95A0E128>;+sip.instance="< urn:gsma:imei:90420156-025763-0>";+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Allow: | INVITE,ACK,CANCEL,BYE,REFER,MESSAGE,OPTIONS |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 75555::eee:aaa:fff:bbb |
| a= | poc-qoe:premium |
| m= | audio 7557 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:7559 |
| i= | speech |
| a= | label:bl |
| b= | AS:25.4 |
| m= | application 75590 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:bl |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

14.	SIP 200 "OK" response (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP 200 "OK" response to SIP/IP Core B.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:uu |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

15.	SIP 200 "OK" response (from SIP/IP Core B to SIP/IP Core X)

SIP/IP Core B forwards the SIP 200 "OK" response to SIP/IP Core X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:uu |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

16.	SIP 200 "OK" response (from SIP/IP Core X to PoC Server X)

SIP/IP Core X forwards the SIP 200 "OK" response to PoC Server X.

| SIP HEADERS |  |
| --- | --- |
| P-Asserted-Identity: | "PoC User B" <sip:PoC-UserB@networkB.net> |
| Privacy: | id |
| Require: | timer |
| Server: | PoC-serv/OMA2.0 |
| Session-Expires: | 1800;refresher=uas |
| Contact: | <sip:PoC-SessionABCDEF@PoC-ServerB.networkB.net;gr;b2bua>;+g.poc.talkburst; sip.automata;sip.actor=principal;sip.description="poc recording device" |
| Accept-Language: | en |
| SDP PARAMETERS |  |
| c= | IN IP6 99055::fff:aaa:eee:bbb |
| a= | poc-qoe:premium |
| m= | audio 59006 RTP/AVP 98 |
| a= | rtpmap:98 EVRC/8000 |
| a= | rtcp:50000 |
| i= | speech |
| b= | AS:25.4 |
| a= | label:uu |
| a= | a=upcc:0 |
| m= | application 50000 udp TBCP |
| a= | fmtp:TBCP multimedia=1 |
| a= | floorid:0 mstrm:uu |
| m= | video 0 RTP/AVP 99 |
| m= | message 0 TCP/MSRP * |

A.	MBCP Media Burst Taken message (from PoC Server X to PoC Server B).

The MBCP Media Burst Taken message is sent to inform the UE PoC Box B that PoC Client A has been granted to send a Media Burst. The message contains the identity of the PoC User that has been granted to send a Media Burst.

B.	MBCP Media Burst Taken message (from PoC Server B to UE PoC Box B).

The MBCP Media Burst Taken message is sent to inform the UE PoC Box B that PoC Client A has been granted to send a Media Burst. The message contains the identity of the PoC User that has been granted to send a Media Burst.

17.	SIP ACK request (from PoC Server X to SIP/IP Core X)

PoC Server X sends a SIP ACK request to SIP/IP Core X.

19.	SIP ACK request (from SIP/IP Core X to SIP/IP Core B)

SIP/IP Core X forwards the SIP ACK request to SIP/IP Core B.

19.	SIP ACK request (from SIP/IP Core B to PoC Server B)

SIP/IP Core B forwards the SIP ACK request to PoC Server B.

20.	SIP ACK request (from PoC Server B to SIP/IP Core B)

PoC Server B sends a SIP ACK request to SIP/IP Core B.

21.	SIP ACK request (from SIP/IP Core B to UE PoC Box B)

SIP/IP Core B forwards the  SIP ACK request to UE PoC BoxB.