| # OMA Device Management Standardized Objects |  |  |
| --- | --- | --- |
| Approved Version 1.3 – 24 May 2016 |  |  |
| Open Mobile Alliance |  |  |
| OMA-TS-DM_StdObj-V1_3-20160524-A |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2016 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope	4

2.	References	5

2.1	Normative References	5

2.2	Informative References	5

3.	Terminology and Conventions	6

3.1	Conventions	6

3.2	Definitions	6

3.3	Abbreviations	6

4.	Introduction	7

5.	Standardized Objects	8

5.1	Management Objects	8

5.2	Management objects standardized by other organizations	8

5.3	The OMA DM management objects	8

5.3.1	The DM Account management object	8

5.3.2	The DevInfo management object	16

5.3.3	The DevDetail management object	20

5.3.4	Inbox	23

Appendix A.	Change History (Informative)	25

A.1	Approved Version History	25

Appendix B.	Static Conformance Requirements (Normative)	26

B.1	SCR for DM Client	26

B.2	SCR for DM Server	26

Appendix C.	Mapping of Device Management parameters	27

Figures

Figure 1: The DM Account Management Object	9

Figure 2: The DevInfo management object	16

Figure 3: The DevDetail management object	20

Tables

Table 1: AAuthLevel Values	13

Table 2: AAuthType Values	14

# Scope

This document defines a set of management objects. Some of these are mandatory for all OMA DM compliant devices and others are optional. The objects are defined using the OMA DM Device Description Framework.

# References

## Normative References

| [DevDetailDDF] | “OMA Device Management Detail Information Management Object DDF, Version 1.3”. Open Mobile Alliance. OMA-SUP-MO_DM_DevDetail-V1_3. URL:http://www.openmobilealliance.org |
| --- | --- |
| [DevInfoDDF] | “OMA Device Management Information Management Object DDF, Version 1.3”. Open Mobile Alliance. OMA-SUP-MO_DM_DevInfo-V1_3. URL:http://www.openmobilealliance.org |
| [DMAccDDF] | “OMA Device Management Account Management Object DDF, Version 1.3”. Open Mobile Alliance. OMA-SUP-MO_DM_DMAcc-V1_3. URL:http://www.openmobilealliance.org |
| [DMBOOT] | “OMA Device Management Bootstrap, Version 1.3”. Open Mobile Alliance. OMA-TS-DM_Bootstrap-V1_3. URL:http://www.openmobilealliance.org |
| [DMDICT] | "OMA Device Management Dictionary", Draft Version 1.0, Open Mobile Alliance™, URL:http://www.openmobilealliance.org/ |
| [DMNOTI] | “OMA Device Management Notification Initiated Session, Version 1.3”. Open Mobile Alliance.OMA-TS-DM_Notification-V1_3. URL:http://www.openmobilealliance.org |
| [DMPRO] | “OMA Device Management Protocol, Version 1.3”. Open Mobile Alliance. OMA-TS-DM_Protocol-V1_3. URL:http://www.openmobilealliance.org |
| [DMREPPRO] | “OMA Device Management Representation Protocol, Version 1.3”. Open Mobile Alliance. OMA-TS-DM_RepPro-V1_3, URL:http://www.openmobilealliance.org |
| [DMSEC] | “OMA Device Management Security, Version 1.3”. Open Mobile Alliance. OMA-TS-DM_Security-V1_3. URL:http://www.openmobilealliance.org |
| [DMTND] | “OMA Device Management Tree and Description, Version 1.3”. Open Mobile Alliance. OMA-TS-DM_TND-V1_3. URL:http://www.openmobilealliance.org |
| [DMTNDS] | “OMA Device Management Tree and Description Serialization, Version 1.3”. Open Mobile Alliance. OMA-TS-DM_TNDS-V1_3. URL:http://www.openmobilealliance.org |
| [ERELDCP] | “Enabler Release Definition for OMA Client Provisioning Specifications, version 1.1”. Open Mobile Alliance. OMA-ERELD-ClientProvisioning-V1_1. URL:http://www.openmobilealliance.org |
| [RFC1766] | “Tags for the Identification of Languages”. H. Alvestrand. March 1995. URL:http://www.ietf.org/rfc/rfc1766.txt |
| [RFC2119] | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |
| [SCRRULES] | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL:http://www.openmobilealliance.org/ |
| [w7] | “OMA w7 Application Characteristic for DM Version 1.0”. Open Mobile Alliance. OMA-w7-Application-Characteristic-for-DM-V1_0. URL:http://www.openmobilealliance.org |

## Informative References

None.

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

Any reference to components of the DTD's or XML snippets is specified in this typeface.

## Definitions

Kindly consult [DMDICT] for all definitions used in this document.

## Abbreviations

Kindly consult [DMDICT] for all abbreviations used in this document.

# Introduction

Other OMA DM specifications define the syntax and semantics of the OMA DM protocol. However, the usefulness of such a protocol would be limited if the managed entities in devices required different data formats and displayed different behaviors. To avoid this situation this specification defines a number of mandatory management objects for various uses in devices.

# Standardized Objects

## Management Objects

This specification uses the graphical notation as defined in [DMTND]. All the MO defined in this specification uses the extended syntax in the DDF Files as defined in [DMTND].

## Management objects standardized by other organizations

OMA DM has been designed so that existing management objects can be managed. These existing management objects have typically already been standardized by other standards organizations.

## The OMA DM management objects

Clients implementing OMA DM MUST support DM Account management object, DevInfo management object, DevDetail management object and Inbox management object. OMA DM servers MUST support all four management objects as well.

| Management Object | Client Support | Server Support | Description |
| --- | --- | --- | --- |
| DMAcc | MUST | MUST | Settings for the DM client in a managed device. |
| DevInfo | MUST | MUST | Device information for the OMA DM server. Sent from the client to the server. |
| DevDetail | MUST | MUST | General device information that benefits from standardization. |
| Inbox | MUST | MUST | Reserved URI where the device SHOULD use the management object identifier to identify the absolute URI. |

The difference between DevInfo and DevDetail is that the DevInfo parameters are needed by the management server for problem free operation of the OMA DM protocol. The DevInfo object is sent from client to server in the beginning of every session.

DevDetail contains other device specific parameters that benefits from being standardized and mandatory. The only difference is that these parameters are not sent from client to server automatically. Instead, these parameters are managed by servers as any other parameters and can be manipulated using OMA DM commands.

### The DM Account management object

The management object is used to manage settings for OMA DM protocol.

Management object identifier: urn:oma:mo:oma-dm-dmacc:1.1

Figure 1: The DM Account Management Object

Parameters are also described in Device Management Application Characteristic registration document [w7] which is used as a part of OMA Client Provisioning specifications [ERELDCP]. General mapping rules of OMA Client Provisioning parameters are described in [DMBOOT]. When the DM Account parameters are derived from OMA Client Provisioning w7 document, see more information about parameter mapping in Appendix C.

The complete DDF description of this management object can be found in [DMAccDDF].

The DM Account Managed Object shown in Figure 1 may be located anywhere in the DM Tree. While there may be business cases for locating the DM Account Managed Object in a location other than at the root of the DM Tree, for most device management scenarios locating the DM Account Managed Object at the root of the DM Tree will meet the needs of the industry. Therefore it is recommended the DM Account Managed Object be located in the DM Tree as the URI ‘DMAcc’.

An example for DM account URI under a fixed location is ‘DMAcc/<x>/…..’.

The optional DMAcc node is an interior node acts as a placeholder for one or more DM Accounts as defined below:

- Occurrence: ZeroOrOne

- Format: Node

- Access Types: Get

- Values: N/A

| .../<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for one or more accounts or for a fixed node. Management Object Identifier for the DMAcc MO MUST be: “urn:oma:mo:oma-dm-dmacc:1.1”. |  |  |  |  |

| AppID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | chr | Get |  |
|  | This optional node specifies the application ID for device management account object. The value of this node, if present, MUST be ‘w7’. |  |  |  |  |

| ServerID |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node specifies a server identifier for management server used in the management session. |  |  |  |  |

| Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node specifies user displayable name for the management server. |  |  |  |  |

| PrefConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node specifies a reference to preferred connectivity. It is expected that either a URI to proxy or NAP MO is specified, but other, implementation-specific connectoids MAY be referenced. |  |  |  |  |

| ToConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get |  |
|  | This interior node is used to allow application to refer to a collection of connectivity definitions. Several connectoids MAY be listed for a given application under this interior node. |  |  |  |  |

| ToConRef/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for one or more connectivity parameters. |  |  |  |  |

| ToConRef/<x>/ConRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node indicates the linkage to connectivity parameters, specified either as an URI to an MO or as an implementation-specific identifier. |  |  |  |  |

| AppAddrPref |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | chr | Get |  |
|  | This node specifies a reference to preferred Management Server address. The preferred Management Server address is specified as a “AppAddr/<X>/Addr” URI in the node. The DM Client SHOULD connect to the address referenced from this node, but the DM Client MAY access to another Management Server address which is specified in the “AppAddr/<X>” in case of the preferred Management Server address is invalid or not available. |  |  |  |  |

| AppAddr |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node is used to specify multiple Management Server addresses. |  |  |  |  |

| AppAddr/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for separating one or more Server Addresses. |  |  |  |  |

| AppAddr/<x>/Addr |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node specifies a Management Server address dependent upon AddrType. |  |  |  |  |

| AppAddr/<x>/AddrType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node specifies a Management Server address type. Valid values are: “URI”, “IPv4” or “IPv6”. |  |  |  |  |

| AppAddr/<x>/Port |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get |  |
|  | This interior node acts as a placeholder for aggregating one or more Port settings. |  |  |  |  |

| AppAddr/<x>/Port/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for aggregating one or more Port settings. |  |  |  |  |

| AppAddr/<x>/Port/<x>/PortNbr |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node specifies port number. The port number MUST be a decimal number and must fit within the range of a 16 bit unsigned integer. |  |  |  |  |

| AAuthPref |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This is a string-valued node whose possible value is exactly one of the names of the various possible authentication types (AAuthType values). E.g. "DIGEST". If this node is present, the client SHOULD use this authentication type when connecting to the server. The use of this node is intended to reduce the number of round trips between client and server that would be caused by authentication challenges. If the value is empty, the default behaviour is to indicate the authentication mechanism negotiated in the previous session if one exists. See <x>/AppAuth/<x>/AAuthTypes in this section for possible values of this node. |  |  |  |  |

| AppAuth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | node | Get |  |
|  | This node specifies authentication information. |  |  |  |  |

| AppAuth/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get |  |
|  | This interior node acts as a placeholder for one or more authentication settings. To ensure against misalignment of credentials with their correct Authentication Level (<X>/AppAuth/<X>/AAuthLevel) and avoid unnecessary processing within Device, the node name used for this node SHOULD be the value of AAuthLevel node under it, For example: <NodeName>AppAuth</NodeName> <NodeName>CLCRED</NodeName> <NodeName>AAuthLevel</NodeName> <Value>CLCRED</Value> |  |  |  |  |

Note that this interior node SHOULD be appeared only once for each same AAuthLevel value.

| AppAuth/<x>/AAuthLevel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node specifies the authentication level. Values: Status; Interpretation. CLCRED; Optional; Credentials DM Client uses to authenticate itself to the OMA DM Server at the DM protocol level.. SRVCRED; Optional; Credentials DM Server uses to authenticate itself to the OMA DM Client at the DM protocol level.. NOTICRED; Optional; Credentials DM Server uses to authenticate itself to the OMA DM Client at the DM Notification message level. MACCRED; Optional; Credentials for MAC authentication. See ‘Transport Neutral Integrity’ section of [DMSEC] for more detailed information. NOTE: If this AAuthLevel is selected, only HMAC is valid value for AAuthType.. OBEX; Optional; Credentials for OBEX authentication. NOTE: If this AAuthLevel is selected only HTTP-BASIC, HTTP-DIGEST and TRANSPORT are valid values for AAuthType.. HTTP; Optional; Credentials for HTTP (/WSP) authentication. NOTE: If this AAuthLevel is selected only HTTP-BASIC, HTTP-DIGEST and TRANSPORT are valid values for AAuthType.. Table 1: AAuthLevel Values |

| AppAuth/<x>/AAuthType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node specifies the authentication type. Values: Status; Interpretation. HTTP-BASIC; Optional; HTTP basic authentication done according to RFC 2617.. HTTP-DIGEST; Optional; HTTP digest authentication done according to RFC 2617.. BASIC; Optional; DM 'syncml:auth-basic' authentication as specified in [DMSEC].. DIGEST; Optional; DM 'syncml:auth-md5' authentication as specified in [DMSEC].. DIGEST-SHA256; Optional; DM’syncml:auth-sha256’ authentication as specified in [DMSEC], or digest calculation on DM Notification as specified in [DMNOTI] if AuthLevel is ‘NOTICRED’. HMAC; Optional; DM 'syncml:auth-MAC' authentication as specified in [DMSEC].. TRANSPORT; Optional; Secure Transport authentication is used. Transport layer authentication is beyond the scope of OMA DM Security.. Table 2: AAuthType Values |

| AppAuth/<x>/AAuthName |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node specifies the authentication name. |  |  |  |  |

| AppAuth/<x>/AAuthSecret |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | chr | Get |  |
|  | This node specifies the authentication secret. |  |  |  |  |

| AppAuth/<x>/AAuthData |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bin | No Get |  |
|  | This node specifies the authentication data relating to the AAuthType. |  |  |  |  |

| NoAutoInitialSession |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | bool | Get |  |
|  | If the NoAutoInitialSession leaf node is set to true, following completion of the DM Bootstrap operation, the DM Client MUST NOT attempt an untriggered connection to the DM Server. If set to false or omitted, the DM Client MUST conform to the normal bootstrap connection rule. |  |  |  |  |

| NotifyAddr |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | This interior node is to store contact address information for receiving DM Notification. |  |  |  |  |

| NotifyAddr/PreferredRef |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | This node indicates the URI to locate the preferred contact address for receiving DM Notification. |  |  |  |  |

| NotifyAddr/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrMore | node | Get |  |
|  | This interior node is an interior node to store contact address information for receiving DM Notification. |  |  |  |  |

| NotifyAddr/<x>/Transport |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Indicates the transport service to deliver the DM Notification message. The DM Client MUST support one of the following values. But the possible value for Transport node is not limited to pre-defined values. Value; Description. OMA-Push; OMA-Push transport is used.. UDP; UDP transport is used.. |

| NotifyAddr/<x>/Address |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Indicates the address to reach the DM Client for sending DM Notification message. |  |  |  |  |

| NotifyAddr/<x>/Port |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | ZeroOrOne | int | Get |  |
|  | Indicates the port number to reach the DM Client for sending DM Notification message if it is necessary for the transport. |  |  |  |  |

| Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | This interior node is where child nodes pertaining to specialized DM Server behaviour and the vendor specific information about device management application are placed (vendor meaning application vendor, device vendor, OS vendor etc.). Use of this sub tree can be mandated by other enablers or external standards. Vendor specific information MUST be identified by a vendor specific name. The tree structure under the vendor identified is not defined and can therefore include a non-standard sub-tree. |  |  |  |  |

### The DevInfo management object

Management object identifier: urn:oma:mo:oma-dm-devinfo:1.1

DevInfo MO MUST be associated with the following fixed URI: "./DevInfo".

The following figure shows an overview of the DevInfo management object.

Figure 2: The DevInfo management object

The nodes making up DevInfo have the following meanings:

| DevInfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This required interior node specifies the unique object id of the DevInfo management object. Management Object Identifier for the DevInfo MO MUST be: “urn:oma:mo:oma-dm-devinfo:1.1”. |  |  |  |  |

| DevInfo/Bearer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | An optional, interior node in which items related to the bearer (CDMA, etc.) are stored. Use of this sub tree can be mandated by other standards. |  |  |  |  |

| DevInfo/DevId |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Status | Occurrence | Format | Min. Access Types |  |  |
|  |  | Required | One | chr | Get |  |  |
|  | A unique identifier for the device. This value SHOULD be globally unique and MUST be formatted as a URN as specified for the <LocURI> element in [DMREPPRO]. |  |  |  |  |  |

| DevInfo/Man |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | The manufacturer identifier. |  |  |  |  |

| DevInfo/Mod |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | A model identifier (manufacturer specified string). |  |  |  |  |

| DevInfo/DmV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | OMA device management client version identifier (manufacturer specified string). |  |  |  |  |

| DevInfo/Lang |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | The current language setting of the device. The syntax of the language tags and their use are defined in [RFC1766]. Language codes are defined by ISO in the standard ISO639-2. |  |  |  |  |

| DevInfo/Roaming |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node indicated the current roaming status for the current DM session. The following values are valid: Value; Description. 0; Current DM session is not over a roaming connection.. 1; Current DM session is over a roaming connection. 2; It is unknown if the current DM Session is over a roaming connection.. |

| DevInfo/CBT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | int | Get |  |
|  | This node provides bearer type information over which the DM session is currently being carried. The content of this node is an integer with the value in range from 0 to 255, and currently the following values are allocated for different bearer types. For the bearer types not covered in this version of specification the value ‘0’ (Other Bearer Type) MUST be used. Bearer; Technology; Value. Other Bearer Type; 0. 3GPP Circuit Switched Bearer; 1. 3GPP Packet Switched Bearer; GERAN; 2. UTRAN / HSPA; 3. LTE; 4. LTE-Advanced; 5. I-WLAN 3GPP IP Access; 6. Other; 7. 3GPP2 CDMA Packet Data Bearer; 1X / HRPD; 8. UMB; 9. Other; 10. WLAN; 11. DSL; 12. WiMAX; 13. Bluetooth; 14. Ethernet; 15. IrDA; 16. LOCAL; DM Server on Smart Card; 17. |

| DevInfo/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | An optional, interior node, designating the only branch of the DevInfo sub tree into which extensions can be added, permanently or dynamically. |  |  |  |  |

The complete DDF description of this management object can be found in [DevInfoDDF].

### The DevDetail management object

Management object identifier: urn:oma:mo:oma-dm-devdetail:1.1

DevDetail MO MUST be associated with the following fixed URI: "./DevDetail".

The following figure shows an overview of the DevDetail management object.

Figure 3: The DevDetail management object

The nodes making up DevDetail have the following meanings:

| DevDetail |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node specifies the unique object id of the DevDetail management object. Management Object Identifier for the DevDetail MO MUST be: “urn:oma:mo:oma-dm-devdetail:1.1”. |  |  |  |  |

| DevDetail/Bearer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | One | node | Get |  |
|  | An optional, interior node, designating a branch of the DevDetail sub tree into which items related to the bearer (CDMA, etc.) are stored. Use of this sub tree can be mandated by other standards. |  |  |  |  |

| DevDetail/URI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node holds URI related information. |  |  |  |  |

| DevDetail/URI/MaxDepth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Specifies the maximum depth of the management tree supported by the device. The maximum depth of the tree is defined as the maximum number of URI segments that the device supports. The value is a 16 bit, unsigned integer encoded as a numerical string. The value ‘0’ means that the device supports a tree of ‘unlimited’ depth. |  |  |  |  |

| DevDetail/URI/MaxTotLen |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Specifies the maximum total length of any URI used to address a node or node property. The maximum total length of a URI is defined as the largest total number of characters making up the URI which the device can support. Note that depending on the character set this might not be the same as the number of bytes. The value is a 16 bit, unsigned integer encoded as a numerical string. The value ‘0’ means that the device supports URI of ‘unlimited’ length. |  |  |  |  |

| DevDetail/URI/MaxSegLen |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Specifies the maximum total length of any URI segment in a URI used to address a node or node property. The maximum total length of a URI segment is defined as the largest number of characters which the device can support in a single URI segment. Note that depending on the used character set this might not be the same as the number of bytes. The value is a 16 bit, unsigned integer encoded as a numerical string. The value ‘0’ means that the device supports URI segments of ‘unlimited’ length. |  |  |  |  |

| DevDetail/DevType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Device type, for example PDA, pager, or phone. |  |  |  |  |

| DevDetail/OEM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Original Equipment Manufacturer of the device. |  |  |  |  |

| DevDetail/FwV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Firmware version of the device. |  |  |  |  |

| DevDetail/SwV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Software version of the device. |  |  |  |  |

| DevDetail/HwV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get |  |
|  | Hardware version of the device. |  |  |  |  |

| DevDetail/LrgObj |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the OMA DM Large Object Handling specification, as defined in [DMPRO]. |  |  |  |  |

| DevDetail/SessionlessRpt |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | This node indicates whether or not the DM Client supports the sessionless reporting feature. |  |  |  |  |

| DevDetail/UserInteraction |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | node | Get |  |
|  | This interior node is a placeholder for the user interaction leaf nodes. |  |  |  |  |

| DevDetail/UserInteraction/UserInput |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the User Input Alert, as defined in [DMPRO]. |  |  |  |  |

| DevDetail/UserInteraction/Display |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Display Alert, as defined in [DMPRO]. |  |  |  |  |

| DevDetail/UserInteraction/Confirm |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Confirmation Alert, as defined in [DMPRO]. |  |  |  |  |

| DevDetail/UserInteraction/Choice |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Choice Alert, as defined in [DMPRO]. |  |  |  |  |

| DevDetail/UserInteraction/ProgressNotify |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | bool | Get |  |
|  | Indicates whether the device supports the Progress Notification Alert, as defined in [DMPRO]. |  |  |  |  |

| DevDetail/Ext |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Optional | ZeroOrOne | node | Get |  |
|  | An optional, interior node, designating the only branch of the DevDetail sub tree into which extensions can be added, permanently or dynamically. |  |  |  |  |

It is RECOMMENDED that the combination of HwV, SwV, FwV, Man, Mod, and OEM provide a unique signature identifying the specific version of software, thus providing a means for other implementations to make special provisions based on that identification.

The complete DDF description of this management object can be found in [DevDetailDDF].

### Inbox

Management object identifier: urn:oma:mo:oma-dm-inbox:1.0

Inbox is designed to be used when the DM Server wants the DM Client to choose where to create a management object in the management tree. To have the DM Client perform this action, the DM Server will add a MO using TNDS to the ‘Inbox’ URI, and the DM Client MUST add this MO into the DM tree using a location of DM Client’s choice.

For example, an operator would put a DMAcc and some connectivity MO into a single TNDS object, which in turn would be the data for an Add operation in a Bootstrap Message.  This Bootstrap Message could then be provided via WAP Push or via a smartcard to a device for the DM Client to process.  Since the DM Server may not know precisely where the DMAcc needs to reside, it will use the Inbox for the destination of the Add. How the DM Client will decide the location of the MO in the DM Tree is up to the DM Client, but might be chosen by the MOID of the new MO.
Inbox MUST be associated with the following fixed URI:  "Inbox"
Inbox MUST be supported by the DM Client.

DM Clients MUST only permit the Add operation on “Inbox”.  A DM Client MUST return the status code “Command not allowed” (405) in response to any command other than Add with targets “Inbox”.
Inbox MUST support the ACL Runtime Property. The Inbox ACL property MUST be used to set access rights to DM Servers that are allowed to use this feature.

Inbox MUST only be used with TNDS objects [DMTNDS] - non-TNDS objects MUST be rejected with "Unsupported media type or format." (415).

Inbox MUST NOT support any child nodes.  Commands that address child nodes of Inbox MUST be rejected with "Forbidden" (403).

The DM Client MUST copy the location of the new MO into the TargetRef element [DMREPPRO] in the returned Status command.

An example that illustrates the usage of the Inbox functionality is provided below

<Add> <CmdID>4</CmdID> <Item> <Target> <LocURI>Inbox</LocURI> </Target> <Meta> <Format xmlns=’syncml:metinf’>xml</Format> <Type xmlns=’syncml:metinf’> application/vnd.syncml.dmtnds+xml </Type> </Meta> <Data> ... TNDS encoded Object ... </Data> </Item> </Add>

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| OMA-TS-DM_StdObj-V1_3-20160524-A | 24 May 2016 | Status changed to Approved by TP TP Ref # OMA-TP-2016-0041R01-INP_DM_V1_3_ERP_for_final_Approval |

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [SCRRULES].

SCR for DM Client

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| SCR-DM-STDOBJ-C-001-M | Support of DevInfo object | Section 5.3.2 |  |
| SCR-DM-STDOBJ-C-002-M | Support of DevDetail Object | Section 5.3.3 |  |
| SCR-DM-STDOBJ-C-003-M | Support of DM Account Object | Section 5.3.1 |  |
| SCR-DM-STDOBJ-C-004-M | Support of Inbox Object | Section 5.3.4 |  |

SCR for DM Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| SCR-DM-STDOBJ-S-001-M | Support of DevInfo object | Section 5.3.2 |  |
| SCR-DM-STDOBJ-S-002-M | Support of DevDetail Object | Section 5.3.3 |  |
| SCR-DM-STDOBJ-S-003-M | Support of DM Account Object | Section 5.3.1 |  |
| SCR-DM-STDOBJ-S-004-M | Support of Inbox Object | Section 5.3.4 |  |

Mapping of Device Management parameters

In the below table the Device Management Account Management Object and Device Management Provisioning Content Application characteristic (w7) parameter correspondence is shown.

| DEVICE MANAGEMENT ACCOUNT MANAGEMENT OBJECT | DM PROVISIONING CONTENT APPLICATION CHARACTERISTIC (w7) |
| --- | --- |
| AppID | APPID |
| ServerID | PROVIDER-ID |
| Name | NAME |
| PrefConRef, if multiple ToConRef/<X>/ConRef | TO-PROXY |
| PrefConRef, if multiple ToConRef/<X>/ConRef | TO-NAPID |
| AppAddrPref | N/A |
| AppAddr/<X>/Addr | APPADDR/ADDR |
| AppAddr/<X>/AddrType | APPADDR/ADDRTYPE |
| AppAddr/<X>/Port/<X>/PortNbr | APPADDR/PORT/PORTNBR |
| AAuthPref | N/A |
| AppAuth/<X>/AAuthLevel | APPAUTH/AAUTHLEVEL |
| AppAuth/<X>/AAuthType | APPAUTH/AAUTHTYPE |
| AppAuth/<X>/AAuthName | APPAUTH/AAUTHNAME |
| AppAuth/<X>/AAuthSecret | APPAUTH/AAUTHSECRET |
| AppAuth/<X>/AAuthData | APPAUTH/AAUTHDATA |

The following diagram shows how information from the provisioning content and the w7 characteristic are mapped to the management tree.

Requirements for DM client when it converts the w7 APPLICATION characteristic to the management tree:

- DM Client MUST assign a unique name for the <X> (DMAcc Interior node) as specified in Section 5.3.2 in [DMBOOT]. Management server can modify this node name in some subsequent DM session.

- The DM Client MUST grant Get, Replace and Delete ACL rights to the specified ServerId for the <X> (DMAcc Interior node) as specified in Section 5.3.4 in [DMBOOT]. The provisioning server MAY modify this ACL to provide broader or narrower access in a subsequent DM session.

The values of each leaf in the DMAcc object is derived from a w7 APPLICATION characteristic as follows:

- AppID – takes the value of the APPLICATION/APPID = w7.

- ServerID – takes the value of APPLICATION/PROVIDER-ID

- Name – takes the value of APPLICATION/NAME

- PrefConRef – client assigned reference to connectoid, e.g. Connectivity MO or connection information maintained outside of the management tree, for example as specified within PXLOGICAL and NAPDEF.

- ToConRef/<X>/ConRef - client assigned name, MO pointer or may be left empty by the DM client to use connection information maintained outside of the management tree, for example as specified within PXLOGICAL and NAPDEF.

- AppAddr/<X>/Addr – takes the value of APPLICATION/APPADDR/ADDR

- AppAddr/<X>/AddrType - takes the value of APPLICATION/APPADDR/ADDRTYPE

- AppAddr/<X>/Port/<X>/PortNbr – takes the value of APPLICATION/APPADDR/PORT/PORTNBR

- AppAuth/<X>/AAuthLevel – correspondence to APPLICATION/APPAUTH/AAUTHLEVEL values is as follows:

| w7 APPLICATION/APPAUTH/AAUTHLEVEL | DMAcc AppAuth/<x>/AAuthLevel |
| --- | --- |
| APPSRV | CLCRED |
| CLIENT | SRVCRED |
| OBEX | OBEX |

- AppAuth/<X>/AAuthType – takes the value of APPLICATION/APPAUTH/AAUTHTYPE

- AppAuth/<X>/AAuthName – takes the value of APPLICATION/APPAUTH/AAUTHNAME

- AppAuth/<X>/AAuthSecret – takes the value of APPLICATION/APPAUTH/AAUTHSECRET

- AppAuth/<X>/AAuthData – takes the value of APPLICATION/APPAUTH/AAUTHDATA