# Change Request

| Title: | Resource Model | Public OMA Confidential |
| --- | --- | --- |
| To: | DM WG |  |
| Doc to Change: | OMA-TS-LightweightM2M-V1_0_0-20120904-D.doc |  |
| Submission Date: | 14 Sept. 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Zach Shelby, Sensinode, zach@sensinode.com Yawen Niu, CMSS, niuyawen@chinamobile.com Linyi Tian, Huawei,, tianlinyi@huawei.com |  |
| Replaces: | n/a |  |

Reason for Change

This contribution defines the Resource and Object Model and Identifiers for LWM2M along with examples.

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

DM WG to review and discuss this proposed CR

Detailed Change Proposal

Identifiers and Resources

This section defines the identifiers and resource model for the LWM2M Enabler.

Resource Model

The LWM2M Enabler defines a simple resource model where each piece of information made available by a LWM2M Client is a Resource, and Resources are logically organized into Objects. Figure 1 illustrates this structure, and the relationship between resources, objects and LWM2M Client. A LWM2M Client may have any number of Resources, each of which belongs to an Object.

Figure 1 Relationship between LWM2M Client, Object and Resources

Editor’s Note: Change Device to LWM2M Client.

Editor’t Note: Show the access control and logical operations in a seperate diagram.

Editor’s Node:: show multiple instances of resource in this diagram.

Resources are defined per Object, and each resource is given an identifier within that Object. Resources are accessed directly by the L2M2M Server. Each resource is defined to have one or more Operations that it supports. A Resource MAY contain multiple instances of that Resource.

An Object defines a grouping of Resources, for example the Firmware Update Object would contain all the Resources used for firmware update purposes. Each Object is assigned a unique OMA Management Object ID and corresponding ID Index, which may be used to perform Group Operations on the set of Resources associated with that Object.

Both Objects and Resources are extensible, and new ones may be added.

Identifiers

Four identifiers are defined by the LWM2M Enabler: Endpoint Name, Human Readable Object URN, Object ID and Resource ID. These identifiers are defined in Table 1.

Table 1 Definition of LWM2M terms

| Identifier | Semantics | Description |
| --- | --- | --- |
| Endpoint Name | String (max 63 bytes) | Uniquely identifies the LWM2M Client on one LWM2M Server. Provided to the Server during Registration For example the IMEI, serial number or a logical name of the device. |
| Human Readable Object URN | URN for the OMA Management Object | Assigned by the object specification |
| Object ID | 16-bit unsigned integer in ASCII | Short integer ID, assigned by OMA |
| Resource ID | 16-bit unsigned integer in ASCII | Short integer ID, assigned by the object specification |

Resource Data Format

Two data formats are defined by the LWM2M Enabler. A plain text format is used for all singular Resources. The value of the resource is simply represented as a string. This data format has a Media Type of text/plain.

For requests to Objects or Resources with multiple instances, a simple JSON format is used where a set of values is represented. Each entry of the JSON format is a key:value pair, where the key is the Resource ID for requests to an Object or the instance number for requests to a Resource with multiple instances. This data format has a Media Type of application/json. The format follows the following syntax:

{

"<Object Index or Resource ID>": {

"<ID or Instance>":"<Value>",

"<ID or Instance>":"<Value>",

"<ID or Instance>":"<Value>"

}

}

Editor’s Note: Separate it into two examples (fetch all by object id, fetch instances of resources.

Example Objects and Resources

This section lists example Objects and Resources that LWM2M could define. The actual objects will be defined in an object specification.

| Example Object | Human Readable Object URN Editor Note: If this is not needed, then we remove it. | Example Object ID | Example Resource | Example Resource ID | Logical Operations |
| --- | --- | --- | --- | --- | --- |
| Bootstrapping | urn:oma:mo:lwm2m2:bootstrapping | 1 |  |  |  |
|  |  |  | Server Location | 0 | R, W |
|  |  |  | Endpoint Name | 1 | R |
| Firmware Update | urn:oma:mo:lwm2m2:firmware | 2 |  |  |  |
|  |  |  | Firmware | 0 | R, W |
|  |  |  | Firmware Name | 1 | R |
|  |  |  | Last Updated | 2 | R |
|  |  |  | Version | 3 | R, W |
|  |  |  | Size | 4 | R, W |
|  |  |  | Do Upgrade | 5 | E |
| Device Info | urn:oma:mo:lwm2m2:device | 3 |  |  |  |
|  |  |  | Manufacturer | 0 |  |
|  |  |  | Model Number | 1 |  |
|  |  |  | Serial Number | 2 |  |
|  |  |  | Device ID | 3 |  |
| Location | urn:oma:mo:lwm2m2:location | 4 |  |  |  |
|  |  |  | GPS Location | 0 |  |
|  |  |  | GPS Fix | 1 |  |
|  |  |  | APN Location | 2 |  |
|  |  |  | X,Y Location | 3 |  |
|  |  |  | Update Period | 4 |  |
| Bearer Info | urn:oma:mo:lwm2m2:bearer | … |  |  |  |

Editor’s Note: Add an example about how to access multiple instances of resource.

Editor’s Note: Reserve range for OMA, other SDO, vendors for object ID registration.

Editor’s Note: How to reuse the existing management objects need further study.

For example a request to Resource 0 from Object 3 would return the following plain text payload:

Open Mobile Alliance

For example a request to Object 3 in the example would return the following JSON payload:

{

"3": {

"0":"Open Mobile Alliance",

"1":"LWM2M v1.0",

"2":"9347112"

}

}