| # Lightweight M2M – Binary App Data Container |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.0.1 – 13 Feb 2019 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-LWM2M_BinaryAppDataContainer-V1_0_1-20190213-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  The information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

THIS DOCUMENT IS PROVIDED ON AN "AS IS" "AS AVAILABLE" AND "WITH ALL FAULTS" BASIS.

© 2019 Open Mobile Alliance.

Used with the permission of the Open Mobile Alliance under the terms set forth above.
Contents

1.	Scope	4

2.	References	5

2.1	Normative References	5

2.2	Informative References	5

3.	Terminology and Conventions	6

3.1	Conventions	6

3.2	Definitions	6

3.3	Abbreviations	6

4.	Introduction	7

4.1	Version 1.0	7

5.	Application Data Container Use cases	8

5.1	Architecture	8

5.2	LwM2M Client Reports Application Data	8

5.3	LwM2M Server Sends Application Data	9

6.	LwM2M Object: Application Data Container	11

Appendix A.	Change History (Informative)	12

A.1	Approved Version History	12

A.2	Draft Version 1.0.1 History	12

Appendix B.	Example LwM2M Client (Informative)	13

Figures

Figure 1: Application data transfer by LwM2M architecture	8

Figure 2: LwM2M Client Reports Application Data	8

Figure 3: LwM2M Server Sends the Application Data	9

Tables

Table 1: Object Instances of the example	13

Table 2: BinaryAppDataContainer Object Instance [0]	13

Table 3: BinaryAppDataContainer Object Instance [1]	13

# Scope

This document defines an Object to be used to transfer Application Data with the Lightweight M2M enabler in order to manage application service data on the device.

# References

## Normative References

| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |  |
| --- | --- | --- | --- |
| [RFC4234] |  | “Augmented BNF for Syntax Specifications: ABNF”. D. Crocker, Ed., P. Overell. October 2005, URL:http://www.ietf.org/rfc/rfc4234.txt |  |
| [SCRRULES] |  | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL:http://www.openmobilealliance.org/ |  |

## Informative References

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

| BinaryAppDataCont | Application Data Container |
| --- | --- |

## Abbreviations

| OMA | Open Mobile Alliance |
| --- | --- |
| PDN | Packet data network |
| PS | Packet switched |

# Introduction

LwM2M is designed to support both device management (DM) and service enablement (SE). OMA only specify LwM2M objects for DM, while external organizations may specify and register LwM2M objects for SE (e.g. IPSO objects).  However, domain specific applications/devices (e.g. water meter) may not want to expose their data models as LwM2M objects due to security concern or integration cost with legacy systems. Instead, an opaque data blob transfer is often used.

Current practices rely on vendor-specific extension of LwM2M objects to solve the problem above, and this is a well-supported feature of LwM2M’s tooling and infrastructure but for the support of legacy systems, or where the formatting of data is to remain opaque, it is desirable to define a standardized, generic means to transfer application-specific data using LwM2M protocol. Please note that this object should ONLY be used in those circumstances where the use of an existing or proprietary LwM2M object is unacceptable.

## Version 1.0

Version 1.0 of the specification covers:

1) LwM2M Client reports application data to LwM2M Server.2) LwM2M Server sends application data to LwM2M Client.

# Application Data Container Use cases

## Architecture

. . .

. . .

MIoT-Device

LwM2M Client

(modem/chipset)

LwM2M Server

AppClient1

AppServer1

LwM2M interface

AppClient2

AppClientN

。

。

。

AppServer2

AppServerN

Figure 1: Application data transfer by LwM2M architecture

It is desirable to use a standardized, generic means to transfer application-specific data by LwM2M protocol which supports transferring service data transparently between Application Client and Application Server.

LwM2M Client can communicate with Application client by AT command or API interface. LwM2M Server can communicate with Application Server by standard interface. Both interfaces are out of scope of LwM2M specification.

## LwM2M Client Reports Application Data

Device

LwM2M Client

(modem/chipset)

LwM2M Server

App

AppServer

1 Observe /19/X/0 token=12xxx

3

4 Notify /19/X/0  token=12xxx

content

5

2  ACK

Figure 2: LwM2M Client Reports Application Data

Figure 2 shows an example exchange where the Client reports an Application Data.

Step 1: The LwM2M Server sends the Observe request to LwM2M Client by instant X of Object 19. The instant number is specified in section 6.

Step 2: The LwM2M Client sends the ACK to the server.

Step 3: The Application Client sends the application data to LwM2M Client.

Note: The interface of Step 3 between application client and LwM2M Client is out of the scope of LwM2M specification, eg, AT command or API.

Step 4: The LwM2M Client sends the Notify message to the LwM2M Server.

Step 5: The LwM2M Server sends the application data the Application Server.

Note: The interface of Step 5 between application server and LwM2M server is out of the scope of LwM2M specification, eg, API.

## LwM2M Server Sends Application Data

Device

LwM2M Client

(modem/chipset)

LwM2M Server

App1

AppServer

1 post command

2 Write /19/X/0 token=13XXX

3

4

5  2.04 token=13XXX

6

Figure 3: LwM2M Server Sends the Application Data

Figure 3 shows an example exchange where the Server sends an Application Data message to the Client, eg, “TURN ON” command.

Step 1: The Application Server sends the message to LwM2M Server.

Note: The interface of Step 1 between application server and LwM2M server is out of the scope of LwM2M specification, eg, API.

Step 2: The LwM2M Server sends the Write request to LwM2M Client by instant X of Object 19. The instant number is specified in section 6.

Step 3: The LwM2M Client sends the data to the Application Client.

Step 4: The Application Client sends the application data to LwM2M Client.

Note: The interface of Step 3 and 4 between application client and LwM2M Client are out of the scope of LwM2M specification, eg, AT command or API.

Step 5: The LwM2M Client sends the response message to the LwM2M Server.

Step6: The LwM2M Server sends the application data the Application Server.

Note: The interface of Step 6 between application server and LwM2M server is out of the scope of LwM2M specification, eg, API.

# LwM2M Object: Application Data Container

Description

This LwM2M Objects provides the application service data related to a LwM2M Server, eg. Water meter data. There are several methods to create instance to indicate the message direction based on the negotiation between Application and LwM2M. The Client and Server should negotiate the instance(s) used to exchange the data. For example: - Using a single instance for both directions communication, from Client to Server and from Server to Client. - Using an instance for communication from Client to Server and another one for communication from Server to Client - Using several instances

Object definition

Name; Object ID; Instances; Mandatory; Object URN. BinaryAppDataContainer; 19; Multiple; Optional; urn:oma:lwm2m:oma:19. Object version; LwM2M Version. 1.0; 1.0.

Resource definitions

ID; Name; Operations; Instances; Mandatory; Type; Range or Enumeration; Units; Description. 0; Data; RW; Multiple; Mandatory; Opaque; Indicates the application data content. 1; Data Priority; RW; Single; Optional; Integer; 1 bytes; Indicates the Application data priority: 0:Immediate 1:BestEffort 2:Latest 3-100: Reserved for future use. 101-254: Proprietary mode.. 2; Data Creation Time; RW; Single; Optional; Time; Indicates the Datainstance creation timestamp.. 3; Data Description; RW; Single; Optional; String; 32 bytes; Indicates the data description. e.g. "meter reading". 4; Data Format; RW; Single; Optional; String; 32 bytes; Indicates the format of the Application Data. e.g. YG-Meter-Water-Reading UTF8-string. 5; App ID; RW; Single; Optional; Integer; 2 bytes; Indicates the destination Application ID..

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| OMA-TS-LWM2M_BinaryAppDataContainer-V1_0-20180615-A | 15 Jun 2018 | Status changed to Approved by DM Doc Ref # OMA-DM&SE-2018-0061-INP_LWM2M_APPDATA_V1_0_ERP_for_final_Approval |

Draft Version 1.0.1 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| OMA-TS-LWM2M_BinaryAppDataContainer-V1_0 | 15 Jan 2019 | 6, Appendix B | Incorporated CR: OMA-DM-LightweightM2M-2019-0003-CR_Bug_Fix_for_BinaryAppDataContainer |
|  | 23 Jan 2019 | 6 | Incorporated CR: OMA-DM-LightweightM2M-2019-0005-CR_Bug_Fix_for_BinaryAppDataContainer.docx |
|  | 13 Feb 2019 | 4 | Incorporated CR: OMA-IPSO-2019-0014-CR_Editorials_for_BinaryAppDataContainer |

Example LwM2M Client (Informative)

Instance 0 is used to send application data from LwM2M client to LwM2M server. Instance 1 is used to send application data from LwM2M server to LwM2M client.

| Object | Object ID | Object Instance ID |
| --- | --- | --- |
| BinaryAppDataContainer Object[0] | 19 | 0 |
| BinaryAppDataContainer Object[1] | 19 | 1 |

Table 1: Object Instances of the example

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Data | 0 | 0 | InNlcnZpY2VJZCI6Ik1ldGVyIiwNCiJzZXJ2aWNlRGF0YSI6ew0KImN1cnJlbnRSZWFkaW5nIjoiNDYuMyIsDQoic2lnbmFsU3RyZW5ndGgiOjE2LA0KImRhaWx5QWN0aXZpdHlUaW1lIjo1NzA2DQo= | In this example, the Message Data is an encoded version of data for a water meter reading, including: - Current reading - Signal strength - temperature |
| Data Priority | 1 |  | 0 |  |
| DataCreation Time | 2 |  | 1367491215 | May 2nd, 2013 at 11:42 AM GMT |
| Data Description | 3 |  | “MeterReading” |  |

Table 2: BinaryAppDataContainer Object Instance [0]

| Resource Name | Resource ID | Resource Instance ID | Value | Notes |
| --- | --- | --- | --- | --- |
| Data | 0 | 0 | InNlcnZpY2VJZCI6IldhdGVyTWV0ZXIiLA0KImNtZCI6IlNFVF9URU1QRVJBVFVSRV9SRUFEX1BFUklPRCIsDQoicGFyYXMiOnsNCiJ2YWx1ZSI6NA0KICAgIH0sDQoNCg0K | In this example, the Message Data is an encoded version of data for a water meter reading, including: - Command ( “Set Temperature Read Period”) - Parameters associated with the command |
| Data Priority | 1 |  | 1 |  |
| DataCreation Time | 2 |  | 1367491215 | May 2nd, 2013 at 11:42 AM GMT |
| Data Description | 3 |  | “ReadPeriodSet” |  |

Table 3: BinaryAppDataContainer Object Instance [1]