Hello Andreas, Hello Charles

Please have a look on the CR OMA-LOC-2004-0312-AD-Corrections-and-Consistency.doc

In this change request I also did some changes to section 6.7.1.1. Maybe we can discuss our two CRs and make only one CR out of it.

My suggestion for compromissing section 6.7.1.1 :

SUPL_RESPONSE

Your CR 330: For SET Initiated applications the SUPL RESPONSE message is used by the SLP as a response to the SUPL START message

My CR 312: The SUPL RESPONSE message is used by the SLP in response to a  SUPL START message in a SET initiated immediate location request. This message MAY skipped if a computed position (using information transported in the SUPL START message, that meets the requested QoS) is already available for the SET

CR Compromise:

The SUPL RESPONSE message is used by the SLP as a response to the SUPL START message in a SET initiated location request. This message MAY skipped if a computed position (using information transported in the SUPL START message, that meets the requested QoS) is already available for the SET

Remarks :

… initiated location request … seems for me better wording to describe the state when this message is used as … initiated applications … as you wrote.

The intention behind the second sentence “This message MAY skipped …” is to state explicitly in an early paragraph that this message may be skipped under some special circumstances.

I also did some consistency arrangements of paragraphs for the skipping of this message in section 6.9. See my CR for this.

SUPL_END

CR 330: You moved the SUPL_END message to the Position Determination part. I know why but I think the main reason for SUPL_END is to end an existing SUPL session not to transport position determination information. Also if the position determination part is skipped, some message is needed to end the session. My suggestions is to let the SUPL_END message where it was previously and move it back to Service Management.

I know that as the AD evolves the SUPL_END is transporting - lets say “janus-faced” - session handling AND location information.

SUPL_POS_INIT

I support your CR here.

Kind regards

Stefan Meister, SIEMENS AG