Input Contribution

| Title: | Allowing Supported Shapes to be specified over MLP | Public OMA Confidential |
| --- | --- | --- |
| To: | OMA LOC |  |
| Submission Date: | 23 Sep 2013 |  |
| Source: | Khiem Tran, TCS Khitran[at]telecomsys[dot]com |  |
| Attachments: | n/a | Public OMA Confidential |
|  |  | Public OMA Confidential |
|  |  | Public OMA Confidential |
|  |  | Public OMA Confidential |
| Replaces: | N/A |  |

Reason for Contribution

Different applications require locations in different formats. For example, some applications may only support circles; while others can handle more complex shapes such as arc bands and polygons. Conversion between these shapes is often lossy and so should be done as little as possible.

There is currently support in RLP for indicating the list of shapes that a requesting entity would like to receive, but no similar support in MLP. It is therefore proposed to apply the same mechanism used in RLP to the MLP protocol. It is also proposed to allow the requesting entity to indicate an order of preference for the list of supported shapes, by assigning q-values to each of the supported shapes.

Summary of Contribution

This proposal requires updates to MLP3.5 and the latest version of RLP to make both of them consistent. MLP3.5 is currently covered by MLS1.4, but there is no open WID for RLP at  present.

Detailed Proposal

In RLP, the rlp header has an optional supported_shapes parameter:

| <!ELEMENT | rlp_hdr | (locationserver, client?, serving_node_action?, supported_shapes?, net_param?)> |
| --- | --- | --- |

This parameter has attributes for each of the possible shapes, with a value of “YES” indicating that the shape is supported:

| <!ELEMENT | supported_shapes | EMPTY> |
| --- | --- | --- |
| <!ATTLIST | supported_shapes |  |
|  | Point (NO ¦ YES) | "NO" |
|  | LineString (NO ¦ YES) | "NO" |
|  | Polygon (NO ¦ YES) | "NO" |
|  | Box (NO ¦ YES) | "NO" |
|  | CircularArea (NO ¦ YES) | "NO" |
|  | CircularArcArea (NO ¦ YES) | "NO" |
|  | EllipticalArea (NO ¦ YES) | "NO" |
|  | MultiLineString (NO ¦ YES) | "NO" |
|  | MultiPoint (NO ¦ YES) | "NO" |
|  | MultiPolygon (NO ¦ YES) | "NO" |
|  | LinearRing (NO ¦ YES) | "NO" |
|  | Altitude (NO ¦ YES) | "NO"> |

The proposal is to add a similar supported_shapes parameter to the mlp header; to allow a requesting entity to indicate the shapes that it can support.

| <!ELEMENT | hdr | ((sessionid ¦ (client , sessionid?)), subclient*, requestor?, supported_shapes?)> |
| --- | --- | --- |
| <!ATTLIST | hdr |  |
|  | ver CDATA | #FIXED "3.5.0"> |

In addition, it is proposed to replace the YES/NO values of each supported shape attribute with a numerical value indicating the order of preference of each of the supported shapes. Each attribute would have a decimal value ranging from 0 to 1, with 0 meaning unsupported and 1 meaning most preferred.

| <!ELEMENT | supported_shapes | EMPTY> |
| --- | --- | --- |
| <!ATTLIST | supported_shapes |  |
|  | Point CDATA | "0" |
|  | LineString CDATA | "0” |
|  | Polygon CDATA | "0" |
|  | Box CDATA | "0" |
|  | CircularArea CDATA | "0" |
|  | CircularArcArea CDATA | "0" |
|  | EllipticalArea CDATA | "0" |
|  | MultiLineString CDATA | "0" |
|  | MultiPoint CDATA | "0" |
|  | MultiPolygon CDATA | "0" |
|  | LinearRing CDATA | "0" |
|  | Altitude CDATA | "0"> |

As an example, if a location client included a Polygon attribute with a value of 1; a CircularArea attribute with a value of 0.5; and an EllipticalArea with a value of 0.5, this would mean “Give me a polygon if you can; otherwise a circle or an ellipse are equally acceptable.”

For consistency, this change would also be applied to RLP. We’ll need to choose an appropriate version to make this change, as RLP in not currently included in MLS1.4 and there is no open WID for RLP work.

An additional result code would also be needed to indicate that the server did not support any of the requested shapes.

### Result codes

This table defines the result codes that indicate the result of the request or individual positioning. The error codes are divided in ranges:

| 0 | - | 99 | Location server specific errors |
| --- | --- | --- | --- |
| 100 | - | 199 | Request specific errors |
| 200 | - | 299 | Network specific errors |
| 300 | - | 499 | Reserved for future use |
| 500 | - | 599 | Vendor specific errors |
| 600 | - | 699 | MLS Client specific errors |

Note: For privacy reasons it might be needed to not report certain specific errors. In this case it is up to the implementation or configuration of the location server which errors will be reported.

| Resid | Slogan | Description |
| --- | --- | --- |
| 0 | OK | No error occurred while processing the request. |
| 1 | SYSTEM FAILURE | The request can not be handled because of a general problem in the location server. |
| 2 | UNSPECIFIED ERROR | An unspecified error used in case none of the other errors apply. This can also be used in case privacy issues prevent certain errors from being presented. |
| 3 | UNAUTHORIZED APPLICATION | The requesting location-based application is not allowed to access the location server or a wrong password has been supplied. |
| 4 | UNKNOWN SUBSCRIBER | Unknown subscriber. The user is unknown, i.e. no such subscription exists. |
| 5 | ABSENT SUBSCRIBER | Absent subscriber. The user is currently not reachable. |
| 6 | POSITION METHOD FAILURE | Position method failure. The location service fails to obtain the user’s position. The exact cause may be indicated in ADD_INFO by the inclusion of an event code (A, B, C etc.) from the list below: A: Target does not support SUPL. B: SUPL Positioning Failure—Target does not support requested service. For example a SUPL 1.0 device doesn’t support periodic trigger service. C: SUPL Positioning Failure—Target fails to deliver Cell Info. D: SUPL Positioning Failure—both Cell ID location and GNSS positioning fail. |
| 7 | TIMEOUT | Timer expiry for the requested event trigger |
| 101 | CONGESTION IN LOCATION SERVER | The request can not be handled due to congestion in the location server. |
| 103 | UNSUPPORTED VERSION | The Location server does not support the indicated protocol version. |
| 104 | TOO MANY POSITION ITEMS | Too many position items have been specified in the request. |
| 105 | FORMAT ERROR | A protocol element in the request has invalid format. The invalid element is indicated in ADD_INFO. |
| 106 | SYNTAX ERROR | The position request has invalid syntax. Details may be indicated in ADD_INFO. |
| 107 | PROTOCOL ELEMENT NOT SUPPORTED | A protocol element specified in the position request is not supported by the Location Server, or the position result is not supported by the LCS Client. The element is indicated in ADD_INFO. |
| 108 | SERVICE NOT SUPPORTED | The requested service is not supported in the Location Server. The service is indicated in ADD_INFO. |
| 109 | PROTOCOL ELEMENT ATTRIBUTE NOT SUPPORTED | A protocol element attribute is not supported in the Location Server. The attribute is indicated in ADD_INFO. |
| 110 | INVALID PROTOCOL ELEMENT VALUE | A protocol element in the request has an invalid value. The element is indicated in ADD_INFO. |
| 111 | INVALID PROTOCOL ELEMENT ATTRIBUTE VALUE | A protocol element attribute in the request has a wrong value. The element is indicated in ADD_INFO. |
| 112 | PROTOCOL ELEMENT VALUE NOT SUPPORTED | A specific value of a protocol element is not supported in the Location Server. The element and value are indicated in ADD_INFO. |
| 113 | PROTOCOL ELEMENT ATTRIBUTE VALUE NOT SUPPORTED | A specific value of a protocol element attribute is not supported in the Location Server. The attribute and value are indicated in ADD_INFO. |
| 114 | CANCELLATION OF TRIGGERED LOCATION REQUEST | The requested triggered location report is cancelled. |
| 115 | INVALID MSID IN TLRSR | One or more msid elements in the Triggered Location Reporting Stop Request are not valid to the Location Server. |
| 116 | TLRSR FOR INDIVIDUAL TARGET NOT SUPPORTED | The function of stopping triggered location reporting for individual target(s) is not supported in Location Server. |
| 117 | REQUESTED SHAPE TYPES NOT SUPPORTED | The Location Server could not provide any of the requested shape types. |
| 201 | QOP NOT ATTAINABLE | The requested QoP cannot be provided. The exact QoP parameter which cannot be provided, i.e. accuracy, response time or max_loc_age, and value are indicated in ADD_INFO. |
| 202 | POSITIONING NOT ALLOWED | The subscriber does not allow the application to position him/her for whatever reason (privacy settings in location server, LCS privacy class). |
| 203 | CONGESTION IN MOBILE NETWORK | The request can not be handled due to congestion in the mobile network. |
| 204 | DISALLOWED BY LOCAL REGULATIONS | The location request is disallowed by local regulatory requirements. |
| 207 | MISCONFIGURATION OF LOCATION SERVER | The location server is not completely configured to be able to calculate a position. |
| 208 | TARGET MOVED TO NEW MSC/SGSN | The triggered Location Request has been aborted due to that target has moved to another MSC/SGSN. This result code shall only be used towards The Home Location Server. Restrictions: - This code SHALL only be used in RLP. - This result code shall only be used towards The Home Location Server. |
| 500 -599 |  | Vendor specific errors |
| 601 | STANDARD LOCATION REPORT SERVICE NOT SUPPORTED | The MLS Client does not support the standard location report service. |
| 602 | MLS CLIENT ERROR | An error occurred in the MLS Client. |
| 603 | STANDARD LOCATION REPORT SERVICE NOT ACCEPTED | The standard location report was not accepted by the MLS Client. |
| 604 | SUBSCRIBER IN STANDARD LOCATION REPORT SERVICE NOT VALID | The subscriber in the Standard Location Report is not valid to the MLS Client. |
| 605 | INVALID SERVICE ID IN STANDARD LOCATION REPORT SERVICE | The service identity in the Standard Location Report is not valid to the MLS Client. |

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

Discuss and agree on way forward.