| RESTful Network API for Anonymous Customer Reference Management |  |  |  |
| --- | --- | --- | --- |
| Draft Version 1.0 – 19 Feb 2013 |  |  |  |
| Open Mobile Alliance |  |  |  |
| OMA-TS-REST_NetAPI_ACR-V1_0-20130219-D |  |  |  |

Use of this document is subject to all of the terms and conditions of the Use Agreement located at http://www.openmobilealliance.org/UseAgreement.html.

Unless this document is clearly designated as an approved specification, this document is a work in process, is not an approved Open Mobile Alliance™ specification, and is subject to revision or removal without notice.

You may use this document or any part of the document for internal or educational purposes only, provided you do not modify, edit or take out of context the information in this document in any manner.  Information contained in this document may be used, at your sole risk, for any purposes.  You may not use this document in any other manner without the prior written permission of the Open Mobile Alliance.  The Open Mobile Alliance authorizes you to copy this document, provided that you retain all copyright and other proprietary notices contained in the original materials on any copies of the materials and that you comply strictly with these terms.  This copyright permission does not constitute an endorsement of the products or services.  The Open Mobile Alliance assumes no responsibility for errors or omissions in this document.

Each Open Mobile Alliance member has agreed to use reasonable endeavors to inform the Open Mobile Alliance in a timely manner of Essential IPR as it becomes aware that the Essential IPR is related to the prepared or published specification.  However, the members do not have an obligation to conduct IPR searches.  The declared Essential IPR is publicly available to members and non-members of the Open Mobile Alliance and may be found on the “OMA IPR Declarations” list at http://www.openmobilealliance.org/ipr.html.  The Open Mobile Alliance has not conducted an independent IPR review of this document and the information contained herein, and makes no representations or warranties regarding third party IPR, including without limitation patents, copyrights or trade secret rights.  This document may contain inventions for which you must obtain licenses from third parties before making, using or selling the inventions.  Defined terms above are set forth in the schedule to the Open Mobile Alliance Application Form.

NO REPRESENTATIONS OR WARRANTIES (WHETHER EXPRESS OR IMPLIED) ARE MADE BY THE OPEN MOBILE ALLIANCE OR ANY OPEN MOBILE ALLIANCE MEMBER OR ITS AFFILIATES REGARDING ANY OF THE IPR’S REPRESENTED ON THE “OMA IPR DECLARATIONS” LIST, INCLUDING, BUT NOT LIMITED TO THE ACCURACY, COMPLETENESS, VALIDITY OR RELEVANCE OF THE INFORMATION OR WHETHER OR NOT SUCH RIGHTS ARE ESSENTIAL OR NON-ESSENTIAL.

THE OPEN MOBILE ALLIANCE IS NOT LIABLE FOR AND HEREBY DISCLAIMS ANY DIRECT, INDIRECT, PUNITIVE, SPECIAL, INCIDENTAL, CONSEQUENTIAL, OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE USE OF DOCUMENTS AND THE INFORMATION CONTAINED IN THE DOCUMENTS.

© 2013 Open Mobile Alliance Ltd.  All Rights Reserved.
Used with the permission of the Open Mobile Alliance Ltd. under the terms set forth above.

Contents

1.	Scope	6

2.	References	7

2.1	Normative References	7

2.2	Informative References	7

3.	Terminology and Conventions	8

3.1	Conventions	8

3.2	Definitions	8

3.3	3.3 Abbreviations	8

4.	Introduction	9

4.1	Version 1.0	9

5.	Anonymous Customer Reference Management API definition	10

5.1	Resources Summary	10

5.2	Data Types	13

5.2.1	XML Namespaces	13

5.2.2	Structures	13

5.2.2.1	Type: AcrList	13

5.2.2.2	Type: Acr	13

5.2.2.3	Type: Status	15

5.2.3	Enumerations	15

5.2.3.1	Enumeration: AcrStatus	15

5.2.4	Values of the Link “rel” attribute	15

5.3	Sequence Diagrams	15

5.3.1	Operations on Anonymous Customer References	15

6.	Detailed specification of the resources	18

6.1	Resource: End User’s Application	18

6.1.1	Request URL variables	18

6.1.2	Response Codes and Error Handling	19

6.1.3	GET	19

6.1.3.1	Example 1: Retrieve the issued ACR using “auth” keyword (Informative)	19

6.1.3.1.1	Request	19

6.1.3.1.2	Response	19

6.1.3.2	Example 2: Retrieve the issued ACR using MSISDN  (Informative)	19

6.1.3.2.1	Request	19

6.1.3.2.2	Response	20

6.1.3.3	Example 3: Retrieve a non-existent ACR using MSISDN  (Informative)	20

6.1.3.3.1	Request	20

6.1.3.3.2	Response	20

6.1.4	PUT	20

6.1.5	POST	21

6.1.5.1	Example 1: Create an ACR using “auth” keyword;  expiry value “0001-01-01T00:00:00” is included in OAuth access token (Informative)	21

6.1.5.1.1	Request	21

6.1.5.1.2	Response	21

6.1.5.2	Example 2: Create an ACR using MSISDN  (Informative)	21

6.1.5.2.1	Request	21

6.1.5.2.2	Response	22

6.1.5.3	Example 3: Create an ACR over mobile network authenticated connection, response with a location of created resource (Informative)	22

6.1.5.3.1	Request	22

6.1.5.3.2	Response	22

6.1.5.4	Example 4: Create ACR over mobile network authenticated connection, response with creation failure due to an existing expired ACR  (Informative)	23

6.1.5.4.1	Request	23

6.1.5.4.2	Response	23

6.1.5.5	Example 5: Create an ACR over mobile network authenticated connection, response with a copy of created resource (Informative)	23

6.1.5.5.1	Request	23

6.1.5.5.2	Response	24

6.1.6	DELETE	24

6.2	Resource: Anonymous Customer Reference	24

6.2.1	Request URL variables	24

6.2.2	Response Codes and Error Handling	24

6.2.3	GET	24

6.2.3.1	Example 1: Retrieve ACR data  (Informative)	25

6.2.3.1.1	Request	25

6.2.3.1.2	Response	25

6.2.4	PUT	25

6.2.5	POST	25

6.2.6	DELETE	25

6.2.6.1	Example 1: Remove an ACR (Informative)	25

6.2.6.1.1	Request	25

6.2.6.1.2	Response	25

6.3	Resource: ACR Status	25

6.3.1	Request URL variables	26

6.3.2	Response Codes and Error Handling	26

6.3.3	GET	26

6.3.3.1	Example 1: Read the Status of an ACR using “auth” keyword  (Informative)	26

6.3.3.1.1	Request	26

6.3.3.1.2	Response	26

6.3.3.2	Example 2: Read the Status of a non-existent ACR using “auth” keyword (Informative)	27

6.3.3.2.1	Request	27

6.3.3.2.2	Response	27

6.3.4	PUT	27

6.3.4.1	Example 1: Refresh an expired ACR by using “auth” keyword  (Informative)	27

6.3.4.1.1	Request	27

6.3.4.1.2	Response	27

6.3.4.2	Example 2: Refreshing a revoked ACR is rejected (Informative)	28

6.3.4.2.1	Request	28

6.3.4.2.2	Response	28

6.3.5	POST	28

6.3.6	DELETE	29

7.	Fault definitions	30

7.1	Service Exceptions	30

7.1.1	SVC1005: ACR creation failed. Unknown userId	30

7.1.2	SVC1006: ACR not found	30

7.2	Policy Exceptions	30

7.2.1	POL1024: ACR creation failed. An active ACR already exists	30

7.2.2	POL1025: ACR creation failed. An expired ACR already exists	31

7.2.3	POL1026: Creation of Static ACR is not supported	31

7.2.4	POL1027: Revoked ACR can no longer be used or refreshed	31

7.2.5	POL1028: Expired ACR may not be used unless refreshed	31

Appendix A.	Change History (Informative)	33

A.1	Approved Version History	33

A.2	Draft/Candidate Version 1.0 History	33

Appendix B.	Static Conformance Requirements (Normative)	34

B.1	SCR for REST.ACRMgnt Server	34

B.1.1	SCR for REST. ACRMgt.UsersApp Server	34

B.1.2	SCR for REST. ACRMgt.ACR Server	34

B.1.3	SCR for REST. ACRMgt.ACR.Status Server	34

Appendix C.	Application/x-www-form-urlencoded Request Format for POST Operations (Normative)	35

Appendix D.	JSON examples  (Informative)	36

D.1	Retrieve the issued ACR using “auth” keyword (section 6.1.3.1)	36

D.2	Retrieve the issued ACR using MSISDN (section 6.1.3.2)	36

D.3	Create an ACR using “auth” keyword (section 6.1.4.1)	37

D.4	Create an ACR using MSISDN (section 6.1.4.2)	37

D.5	Create an ACR over mobile network authenticated connection (section 6.1.4.3)	38

D.6	Retrieve ACR data (section 6.2.3.1)	39

D.7	Remove an ACR (section 6.2.6.1)	39

D.8	Read the Status of an ACR using “auth” keyword (section 6.3.3.1)	39

D.9	Refresh an expired ACR by using “auth” keyword (section 6.3.5.1)	40

Appendix E.	Operations mapping to a pre-existing baseline specification (Informative)	41

Appendix F.	Light-weight resources (Informative)	42

Appendix G.	Authorization aspects (Normative)	43

G.1	Use with OMA Authorization Framework for Network APIs	43

G.1.1	Scope values	43

G.1.1.1	Definitions	43

G.1.1.2	Downscoping	43

G.1.1.3	Mapping with resources and methods	43

G.1.2	Use of ‘acr:auth’	44

Figures

Figure 1 Resource structure defined by this specification	11

Figure 2 Management of own service capabilities	16

Tables

Table 1: Scope values for RESTful Anonymous Customer Reference Management API	33

Table 2: Required scope values for: Management of Anonymous Customer Reference	34

# Scope

This specification defines a RESTful API for Anonymous Customer Reference Management using HTTP protocol bindings.

# References

## Normative References

| [Autho4API_10] |  | “Authorization Framework for Network APIs”, Open Mobile Alliance™, OMA-ER-Autho4API-V1_0, URL: http://www.openmobilealliance.org/ |  |
| --- | --- | --- | --- |
| [IETF_ACR_draft] |  | “The acr URI for anonymous users”, S.Jakobsson, K.Smith, January 2010, URL: http://tools.ietf.org/html/draft-uri-acr-extension-04 |  |
| [RD-REST_NetAPI_ACR] |  | “OMA RESTful Network API for Anonymous Customer Reference Management Requirements”, Open Mobile Alliance™, OMA-RD-REST_NetAPI_ACR-V1_0, URL: http://www.openmobilealliance.org/ |  |
| [REST_NetAPI_Common] |  | “Common definitions for RESTful Network APIs”, Open Mobile Alliance™, OMA-TS-REST_NetAPI_Common-V1_0, URL: http://www.openmobilealliance.org/ |  |
| [REST_SUP_ACR] |  | “XML schema for the RESTful Network API for Anonymous Customer Reference Management”, Open Mobile Alliance™, OMA-SUP-XSD_rest_netapi_acr-V1_0, URL: http://www.openmobilealliance.org/ |  |
| [RFC2119] |  | “Key words for use in RFCs to Indicate Requirement Levels”, S. Bradner, March 1997, URL:http://www.ietf.org/rfc/rfc2119.txt |  |
| [RFC2616] |  | “Hypertext Transfer Protocol -- HTTP/1.1”, R. Fielding et. al, January 1999, URL:http://www.ietf.org/rfc/rfc2616.txt |  |
| [RFC3261] |  | “SIP: Session Initiation Protocol”, J. Rosenberg, et. Al, June 2002, URL: http://www.ietf.org/rfc/rfc3261.txt |  |
| [RFC3966] |  | “The tel URI for Telephone Numbers”, H.Schulzrinne, December 2004, URL: http://www.ietf.org/rfc/rfc3966.txt |  |
| [RFC3986] |  | “Uniform Resource Identifier (URI): Generic Syntax”, R. Fielding et. al, January 2005, URL:http://www.ietf.org/rfc/rfc3986.txt |  |
| [RFC4627] |  | “The application/json Media Type for JavaScript Object Notation (JSON)”, D. Crockford, July 2006, URL: http://www.ietf.org/rfc/rfc4627.txt |  |
| [SCRRULES] |  | “SCR Rules and Procedures”, Open Mobile Alliance™, OMA-ORG-SCR_Rules_and_Procedures, URL: http://www.openmobilealliance.org/ |  |
| [XMLSchema1] |  | W3C Recommendation, XML Schema Part 1: Structures Second Edition, URL: http://www.w3.org/TR/xmlschema-1/ |  |
| [XMLSchema2] |  | W3C Recommendation, XML Schema Part 2: Datatypes Second Edition, URL: http://www.w3.org/TR/xmlschema-2/ |  |

## Informative References

| [OMADICT] |  | “Dictionary for OMA Specifications”, Version 2.9, Open Mobile Alliance™, OMA-ORG-Dictionary-V2_9, URL:http://www.openmobilealliance.org/ |  |
| --- | --- | --- | --- |
| [REST_WP] |  | “Guidelines for RESTful Network APIs”, Open Mobile Alliance™, OMA-WP-Guidelines_for_RESTful_Network_APIs, URL:http://www.openmobilealliance.org/ |  |

# Terminology and Conventions

## Conventions

The key words “MUST”, “MUST NOT”, “REQUIRED”, “SHALL”, “SHALL NOT”, “SHOULD”, “SHOULD NOT”, “RECOMMENDED”, “MAY”, and “OPTIONAL” in this document are to be interpreted as described in [RFC2119].

All sections and appendixes, except “Scope” and “Introduction”, are normative, unless they are explicitly indicated to be informative.

## Definitions

For the purpose of this TS, all definitions from the OMA Dictionary apply [OMADICT].

## 3.3	Abbreviations

| ACR | Anonymous Customer Reference |
| --- | --- |
| API | Application Programming Interface |
| APP | Application |
| HTTP | HyperText Transfer Protocol |
| JSON | JavaScript Object Notation |
| MIME | Multipurpose Internet Mail Extensions |
| MSISDN | Mobile Subscriber ISDN Number |
| OMA | Open Mobile Alliance |
| REST | REpresentational State Transfer |
| SIP | Session Initiation Protocol |
| SCR | Static Conformance Requirements |
| TS | Technical Specification |
| URI | Uniform Resource Identifier |
| URL | Uniform Resource Locator |
| WP | White Paper |
| XML | eXtensible Markup Language |
| XSD | XML Schema Definition |

# Introduction

The Technical Specification of the RESTful Network API for Anonymous Customer Reference Management contains HTTP protocol bindings based on the requirements for Anonymous Customer Reference Management defined in [RD-REST_NetAPI_ACR], using the REST architectural style. The specification provides resource definitions, the HTTP verbs applicable for each of these resources, and the element data structures, as well as support material including flow diagrams and examples using the various supported message body formats (i.e. XML, JSON).

## Version 1.0

Version 1.0 of this specification supports the following operations:

- Create an Anonymous Customer Reference (ACR)

- Query the status of an ACR

- Refresh an ACR

In addition, this specification provides:

- Support for scope values used with authorization framework defined in [Autho4API_10]

- Support for Anonymous Customer Reference (ACR) as an end user identifier

- Support for “acr:auth” as a reserved keyword in a resource URL variable that identifies an end user

# Anonymous Customer Reference Management API definition

This section is organized to support a comprehensive understanding of the Anonymous Customer Reference (ACR) Management API design. It specifies the definition of all resources, definition of all data structures, and definitions of all operations permitted on the specified resources.

Common data types, naming conventions, fault definitions and namespaces are defined in [REST_NetAPI_Common].

The remainder of this document is structured as follows:

Section 5 starts with a diagram representing the resources hierarchy followed by a table listing all the resources (and their URL) used by this API, along with the data structure and the supported HTTP verbs (section 5.1). What follows are the data structures (section 5.2). A sample of typical use cases is included in section 5.3, described as high level flow diagrams.

Section 6 contains detailed specification for each of the resources. Each such subsection defines the resource, the request URL variables that are common for all HTTP methods, and the supported HTTP verbs. For each supported HTTP verb, a description of the functionality is provided, along with an example of a request and an example of a response. For each unsupported HTTP verb, the returned HTTP error status is specified, as well as what should be returned in the Allow header.

All examples in section 60 use XML as the format for the message body. JSON examples are provided in Appendix D.

Section 7 contains fault definition details such as Service Exceptions and Policy Exceptions.

Appendix B provides the Static Conformance Requirements (SCR).

Appendix E provides the operations mapping to a pre-existing baseline specification, where applicable.

Appendix F provides a list of all light-weight resources, where applicable.

Appendix G defines authorization aspects to control access to the resources defined in this specification.

Note: Throughout this document client and application can be used interchangeably.

## Resources Summary

This section summarizes all the resources used by the RESTful Network API for Anonymous Customer Reference Management.

The "apiVersion" URL variable SHALL have the value "v1" to indicate that the API corresponds to this version of the specification. See [REST_NetAPI_Common] which specifies the semantics of this variable.

Figure 1 Resource structure defined by this specification

The following tables give a detailed overview of the resources defined in this specification, the data type of their representation and the allowed HTTP methods.

Purpose: Operations on Anonymous Customer Reference

| # Resource | # URL Base URL: http://{serverRoot}/acrmanagement/{apiVersion}/{userId} | # Data Structures | # HTTP verbs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GET | PUT | POST | DELETE |
| End user’s application | /application | AcrList (used for GET response) Acr (used for POST) | Retrieves the issued ACR for the given end user identified by the {userId} in the context of a given application. | No | Creates an ACR for the user identified by the {userId}. | no |
| Anonymous customer reference | /application/{ACR} | Acr (used for GET response) | Retrieves the ACR data. | No | no | Remove an ACR. |
| ACR status | /application/{ACR}/status | Status | Retrieves the ACR status. | Refresh an “expired” ACR. (set the ACR status to “Valid”) | no | no |

## Data Types

### XML Namespaces

The XML namespace for the Anonymous Customer Reference Management data types is:

urn:oma:xml:rest:netapi:acrmanagement:1

The 'xsd' namespace prefix is used in the present document to refer to the XML Schema data types defined in XML Schema [XMLSchema1, XMLSchema2]. The 'common' namespace prefix is used in the present document to refer to the data types defined in [REST_NetAPI_Common]. The use of namespace prefixes such as 'xsd' is not semantically significant.

The XML schema for the data structures defined in the section below is given in [REST_SUP_ACR].

### Structures

The subsections of this section define the data structures used in the Anonymous Customer Reference Management API.

Some of the structures can be instantiated as so-called root elements.

#### Type: AcrList

This type represents a list of Anonymous Customer References.

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| acr | Acr [0..unbounded] | Yes | A list of ACRs. |
| resourceURL | xsd:anyURI | No | Self referring URL. |

A root element named acrList of type AcrList is allowed in response bodies.

#### Type: Acr

This type represents an individual Anonymous Customer Reference and associated metadata.

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| value | xsd:anyURI | Yes | The string comprising the ACR, as per [IETF_ACR_draft]. It MUST be included in responses. “Dyna” and “Stat” are the values of the type field in the ACR value as per [IETF_ACR_draft]. A static ACR once created would never expire. It stays valid until it is revoked by either the user or the server. On the other hand a dynamic ACR is valid only for a certain period of time (as authorized by the user, requested by the application or determined by the server policy), after which it goes into an “expired” status. An expired ACR would need to be refreshed before it can be used again. An “expired” ACR after being dormant for a period of time may be removed by the server. Once a dormant-expired dynamic ACR is removed by the server, the application would need to request the creation of a brand new ACR (hence the name Dynamic as the ACR value may change if not used/refreshed in a timely manner). The value of expiry parameter as explained below determines whether the ACR is a dynamic or static ACR. |
| acrStatus | AcrStatus | Yes | Status of the ACR. It MUST be included in responses, if the ACR exists. |
| expiry | xsd:dateTime | Yes | The time at which the ACR will expire. This parameter MUST be included in responses if ACR is dynamic. However, this parameter may be excluded from responses for static (non-temporary) ACRs which means that the ACR shall not expire. If expiry parameter for static ACR is included in the responses, it shall have the value of “0001-01-01T00:00:00”. For Dynamic ACRs expiry date/time may be requested or determined in the following ways: - user authorizing a expiry date/time through OAuth access token which may be present in the request header (see [Autho4API_10]). - application indicating a expiry date/time in the API request via this (“expiry”) parameter. - server’s policy. In API requests if OAuth access token specifies an expiry date/time for the ACR then it has precedence over the application’s requested expiry date/time (i.e. “expiry” parameter in the request is ignored). Requested expiry date/time may be overridden by the server. As a result, the actual ACR expiry date/time is returned in the responses to the ACR creation requests. Expiry date/time value of “0001-01-01T00:00:00” in the requests (either through OAuth access token or the “expiry” parameter), would mean the request is for a Static ACR (i.e. the ACR should never expire). An expired ACR needs to be refreshed before it can be used again. How long an expired ACR would be kept around on the server’s side before it is removed is determined by server’s policy and out side of the scope of this document. |
| resourceURL | xsd:anyURI | Yes | Self referring URL. The resourceURL SHALL NOT be included in POST requests by the client, but MUST be included in POST requests representing notifications by the server to the client, when a complete representation of the resource is embedded in the notification. The resourceURL MUST also be included in responses to any HTTP method that returns an entity body, and in PUT requests. |

A root element named acr of type Acr is allowed in request and/or response bodies.

#### Type: Status

This type represents the status of an Anonymous Customer Reference.

| Element | Type | Optional | Description |
| --- | --- | --- | --- |
| acrStatus | AcrStatus | Yes | Status of the ACR.It MUST be included in responses, if the ACR exists. |
| resourceURL | xsd:anyURI | No | Self referring URL |

A root element named status of type Status is allowed in request and/or response bodies.

### Enumerations

The subsections of this section define the enumerations used in the Anonymous Customer Reference Management API.

#### Enumeration: AcrStatus

This enumeration defines possible values to describe the status of the validity of an ACR.

| Enumeration | Description |
| --- | --- |
| Valid | Indicates that the ACR is recognised by the server and may be used as the userId part within other requests managed by that server. |
| Revoked | The ACR has been revoked by the server, and may not be used as the userId part of other requests. A new ACR must be obtained for that MSISDN/User. |
| Expired | The ACR has expired, and a request can be made to refresh it. An expired ACR will have to be refreshed (see section 6.3.5) before it can be used as part of any other API request. |

### Values of the Link “rel” attribute

This specification does not define any elements of type Link.

## Sequence Diagrams

The following subsections describe the resources, methods and steps involved in typical scenarios.

### Operations on Anonymous Customer References

This figure below shows a scenario for retrieving, issuing and managing ACRs and ACR data.

The resources:

- To retrieve the issued ACR for the given end user identified by the {userId} in the context of a given application, read resource under

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application

- To issue (create) an ACR for the user identified by the {userId}, create resource under

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application

- To retrieve ACR data , read resource under

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application/{ACR}

- To remove an ACR, delete resource under

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application/{ACR}

- To retrieve the ACR status, read resource under

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application/{ACR}/status

- To refresh an “expired” ACR (i.e. set the ACR status to “valid”), update resource under

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application/{ACR}/status

Figure 2 Management of own service capabilities

- An application requests the ACR for the given user using GET method and receives the already issued application-specific ACR.

- An application requests creation of an application-specific ACR for the given user identified by the userId using POST method and receives the resulting ACR.

- An application requests the ACR data (value, status, expiry) using GET method and receives the ACR data.

- An application deletes (removes) an ACR with data by using DELETE method and receives response with the result of operation.

- An application requests the status of an ACR using GET method and receives the actual status of the ACR.

- An application refreshes an “expired” ACR (i.e. sets the ACR status to “valid”) by using PUT method and receives response with updated ACR status.

# Detailed specification of the resources

The ACR Management API amongst other operations enables an application to create an ACR resource which in fact is an application-specific user identity of the user on whose behalf the application is acting. To create an ACR, the user’s identity has to be known to the server through {userId} part of the resource URL. The following values are possible for the {userId}:

- an identifier such as MSISDN

- the “acr:auth” keyword.

When the {userId} is set to “acr:auth” keyword, it implies user’s identity is known either through the OAuth access token (see [Autho4API_10]) available in the HTTP Authorization header of the API request or if the  user’s device is connected to the mobile network then  MSISDN is known by the network and it is not necessary to be passed explicitly by the application in the API request.

The following applies to all resources defined in this specification regardless of the representation format (i.e. XML and JSON):

- Reserved characters in URL variables (parts of a URL denoted below by a name in curly brackets) MUST be percent-encoded according to [RFC3986]. Note that this always applies, no matter whether the URL is used as a Request URL or inside the representation of a resource (such as in “resourceURL” and “link” elements).

- If a user identifier (e.g. address, userId, etc) of type anyURI is in the form of an MSISDN, it MUST be defined as a global number according to [RFC3966] (e.g. tel:+19585550100). The use of characters other than digits and the leading “+” sign SHOULD be avoided in order to ensure uniqueness of the resource URL. This applies regardless of whether the user identifier appears in a URL variable or in a parameter in the body of an HTTP message.

- If a user identifier (e.g. address, userId, etc) of type anyURI is in the form of a SIP URI, it MUST be defined according to [RFC3261].

- If a user identifier (e.g. address, userId, etc) of type anyURI is in the form of an Anonymous Customer Reference (ACR), it MUST be defined according to [IETF_ACR_draft], i.e. it MUST include the protocol prefix 'acr:' followed by the ACR.

  - The ACR ‘auth’ is a supported reserved keyword, and MUST NOT be assigned as an ACR to any particular end user. See G.1.2  for details regarding the use of this reserved keyword.

- For requests and responses that have a body, the following applies: in the requests received, the server SHALL support JSON and XML encoding of the parameters in the body. The Server SHALL return either JSON or XML encoded parameters in the response body, according to the result of the content type negotiation as specified in [REST_NetAPI_Common]. In notifications to the Client, the server SHALL use either XML or JSON encoding, depending on which format the client has specified in the related subscription. The generation and handling of the JSON representations SHALL follow the rules for JSON encoding in HTTP Requests/Responses as specified in [REST_NetAPI_Common].

## Resource: End User’s Application

The resource used is:

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application

This resource is used for retrieving an already issued application-specific ACR and for creating an application-specific ACR.

### Request URL variables

The following request URL variables are common for all HTTP commands:

| Name | Description |
| --- | --- |
| serverRoot | Server base url: hostname+port+base path. Port and base path are OPTIONAL. Example: example.com/exampleAPI |
| apiVersion | Version of the API client wants to use. The value of this variable is defined in section 5.1 |
| userId | User identifier. Examples: tel:+19585550100, acr:auth |

See section 6 for a statement on the escaping of reserved characters in URL variables.

### Response Codes and Error Handling

For HTTP response codes, see [REST_NetAPI_Common].

For Policy Exception and Service Exception fault codes applicable to Anonymous Customer Reference Management, see section 7.

### GET

This operation is used for retrieving an already issued application-specific ACR. List returned may contain more than one ACR. For instance the list may contain a revoked ACR and an active ACR. It is assumed that the server knows how to determine which ACRs belong to a given application and appropriately list them in the response, through methods which are beyond the scope of this document (e.g. using the access token provided in the HTTP request header – see [Autho4API_10]).

As shown in the example below, GET operation enables a 3rd-party application the retrieval of an application-specific ACR over the RESTful API. Please note that, to support certain use cases, it might also be possible for the service provider to push an already created and valid ACR to the 3rd-party application, e.g. via HTTP header enrichment techniques (i.e. by injecting additional HTTP header fields carrying an ACR into the user’s traffic). HTTP header enrichment with ACR is beyond the scope of this document.

#### Example 1: Retrieve the issued ACR using “auth” keyword	(Informative)

##### Request

GET /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Host: example.com Accept: application/xml Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903

##### Response

HTTP/1.1 200 OK Date: Thu, 04 Oct 2012 02:51:59 GMT Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <cr:acrList xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <acr> <value>acr:abc123;ncc=23415;type=Stat</value> <acrStatus>Valid</acrStatus> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth /application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DStat</resourceURL> </acr> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application</resourceURL> </cr:acrList>

#### Example 2: Retrieve the issued ACR using MSISDN 	(Informative)

##### Request

GET /exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application HTTP/1.1 Host: example.com Accept: application/xml

##### Response

HTTP/1.1 200 OK Date: Thu, 04 Oct 2012 02:51:59 GMT Content-Type: application/xml Content-Length: nnnn <?xml version="1.0" encoding="UTF-8"?> <cr:acrList xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <acr> <value>acr:abc123;ncc=23415;type=Dyna</value> <acrStatus>Valid</acrStatus> <expiry>2013-10-26T21:32:52</expiry> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna</resourceURL> </acr> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application</resourceURL> </cr:acrList>

#### Example 3: Retrieve a non-existent ACR using MSISDN 	(Informative)

##### Request

GET /exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application HTTP/1.1 Host: example.com Accept: application/xml

##### Response

HTTP/1.1 404 Not Found Content-Type: application/xml Content-Length: nnnn Date: Thu, 04 Oct 2012 02:51:59 GMT <?xml version="1.0" encoding="UTF-8"?> <common:requestError xmlns:common="urn:oma:xml:rest:netapi:common:1"> <serviceException> <messageId>SVC1006</messageId> <text>ACR not found</text> </serviceException> </common:requestError>

### PUT

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: GET, POST’ field in the response as per section 14.7 of [RFC2616].

### POST

This operation is used for creating an ACR. The “expiry” value which may be provided by the OAuth access token in the HTTP header (see [Autho4API_10]) or the body of the POST, would determine the type of the ACR (Dynamic or Static) requested.

Please note that, the inclusion of “expiry” value as part of the OAuth access token in order to ensure user’s authorization  (as shown in Example 1 below) would require the usage of “subscoping” technique as defined in [Autho4API_10]). Please also note that, expiry value  may also be present in the request body (as shown in example 1 below) as per aforementioned “subscoping” technique however, as stated in section 5.2.2.2 the value of expiry value in the OAuth access token takes precedence (if one is available).

#### Example 1: Create an ACR using “auth” keyword;	 expiry value “0001-01-01T00:00:00” is included in OAuth access token (Informative)

##### Request

POST /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Accept: application/xml Content-Length: nnnn Content-Type: application/xml Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <expiry>0001-01-01T00:00:00</expiry> </cr:acr>

##### Response

HTTP/1.1 201 Created Location: http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DStat Content-Type: application/xml Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <value>acr:abc123;ncc=23415;type=Stat</value> <acrStatus>Valid</acrStatus> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DStat</resourceURL> </cr:acr>

#### Example 2: Create an ACR using MSISDN 	(Informative)

##### Request

POST /exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application HTTP/1.1 Accept: application/xml Content-Length: nnnn Content-Type: application/xml Host: example.com <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <expiry>2013-10-26T21:32:52</expiry> </cr:acr>

##### Response

HTTP/1.1 201 Created Location: http://example.com/exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3B ncc%3D23415%3Btype%3DDyna Content-Type: application/xml Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <value>acr:abc123;ncc=23415;type=Dyna</value> <acrStatus>Valid</acrStatus> <expiry>2013-10-26T21:32:52</expiry> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/tel%2B%3A4479901234567/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna</resourceURL> </cr:acr>

#### Example 3: Create an ACR over mobile network authenticated connection, response with a location of created resource	(Informative)

##### Request

POST /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Accept: application/xml Content-Length: nnnn Content-Type: application/xml Host: example.com <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <expiry>2013-10-26T21:32:52</expiry> </cr:acr>

##### Response

HTTP/1.1 201 Created Location: http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna Content-Type: application/xml Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT <?xml version="1.0" encoding="UTF-8"?> <common:resourceReference xmlns:common="urn:oma:xml:rest:netapi:common:1"> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna</resourceURL> </common:resourceReference>

#### Example 4: Create ACR over mobile network authenticated connection, response with creation failure due to an existing expired ACR	 (Informative)

##### Request

POST /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Accept: application/xml Content-Length: nnnn Content-Type: application/xml Host: example.com <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <expiry>2013-10-26T21:32:52</expiry> </cr:acr>

##### Response

HTTP/1.1 403 Forbidden Content-Type: application/xml Content-Length: nnnn Date: Thu, 04 Oct 2012 02:51:59 GMT <?xml version="1.0" encoding="UTF-8"?> <common:requestError xmlns:common="urn:oma:xml:rest:netapi:common:1"> <policyException> <messageId>POL1025</messageId> <text>An expired ACR, abc123;ncc=23415;type=Dyna, already exists which needs to be refreshed prior to usage</text> </policyException> </common:requestError>

#### Example 5: Create an ACR over mobile network authenticated connection, response with a copy of created resource	(Informative)

##### Request

POST /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Accept: application/xml Content-Length: nnnn Content-Type: application/xml Host: example.com <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <expiry>2013-10-26T21:32:52</expiry> </cr:acr>

##### Response

HTTP/1.1 201 Created Location: http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna Content-Type: application/xml Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <value>acr:abc123;ncc=23415;type=Dyna</value> <acrStatus>Valid</acrStatus> <expiry>2013-10-26T21:32:52</expiry> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna</resourceURL> </cr:acr>

### DELETE

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: GET, POST’ field in the response as per section 14.7 of [RFC2616].

## Resource: Anonymous Customer Reference

The resource used is:

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application/{ACR}

This resource is used for retrieving ACR data and for removing an ACR.

### Request URL variables

The following request URL variables are common for all HTTP commands:

| Name | Description |
| --- | --- |
| serverRoot | Server base url: hostname+port+base path. Port and base path are OPTIONAL. Example: example.com/exampleAPI |
| apiVersion | Version of the API client wants to use. The value of this variable is defined in section 5.1 |
| userId | User identifier. Examples: tel:+19585550100, acr:auth |
| ACR | The string comprising the ACR, as per [IETF_ACR_draft] |

See section 6 for a statement on the escaping of reserved characters in URL variables.

### Response Codes and Error Handling

For HTTP response codes, see [REST_NetAPI_Common].

For Policy Exception and Service Exception fault codes applicable to Anonymous Customer Reference Management, see section 7.

### GET

This operation is used for retrieving ACR data.

#### Example 1: Retrieve ACR data 	(Informative)

##### Request

GET /exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna HTTP/1.1 Accept: application/xml Host: example.com

##### Response

HTTP/1.1 200 OK Content-Type: application/xml Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT <?xml version="1.0" encoding="UTF-8"?> <cr:acr xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <value>acr:abc123;ncc=23415;type=Dyna</value> <acrStatus>Valid</acrStatus> <expiry>2013-10-26T21:32:52</expiry> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna</resourceURL> </cr:acr>

### POST

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET, DELETE]’ field in the response as per section 14.7 of [RFC2616].

### PUT

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET, DELETE]’ field in the response as per section 14.7 of [RFC2616].

### POST

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET, DELETE]’ field in the response as per section 14.7 of [RFC2616].

### DELETE

This operation is used for removing an ACR.

#### Example 1: Remove an ACR	(Informative)

##### Request

DELETE /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DStat HTTP/1.1 Host: example.com Accept: application/xml

##### Response

HTTP/1.1 204 No Content Date: Thu, 26 Oct 2012 21:32:52 GMT

## Resource: ACR Status

The resource used is:

http://{serverRoot}/acrmanagement/{apiVersion}/{userId}/application/{ACR}/status

This resource is used for retrieving and refreshing the status of an ACR.

### Request URL variables

The following request URL variables are common for all HTTP commands:

| Name | Description |
| --- | --- |
| serverRoot | Server base url: hostname+port+base path. Port and base path are OPTIONAL. Example: example.com/exampleAPI |
| apiVersion | Version of the API client wants to use. The value of this variable is defined in section 5.1 |
| userId | User identifier. Examples: tel:+19585550100, acr:auth |
| ACR | The string comprising the ACR, as per [IETF_ACR_draft] |

See section 6 for a statement on the escaping of reserved characters in URL variables.

### Response Codes and Error Handling

For HTTP response codes, see [REST_NetAPI_Common].

For Policy Exception and Service Exception fault codes applicable to Anonymous Customer Reference Management, see section 7.

### GET

This operation is used for retrieving the status of an ACR.

#### Example 1: Read the Status of an ACR using “auth” keyword 	(Informative)

##### Request

GET /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%Btype%3DStat/status HTTP/1.1 Accept: application/xml Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903

##### Response

HTTP/1.1 200 OK Content-Type: application/xml Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT <?xml version="1.0" encoding="UTF-8"?> <cr:status xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <acrStatus>Valid</acrStatus> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna/status</resourceURL> </cr:status>

#### Example 2: Read the Status of a non-existent ACR using “auth” keyword	(Informative)

##### Request

GET /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%Btype%3DStat/status HTTP/1.1 Accept: application/xml Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903

##### Response

HTTP/1.1 404 Not Found Content-Type: application/xml Content-Length: nnnn Date: Thu, 04 Oct 2012 02:51:59 GMT <?xml version="1.0" encoding="UTF-8"?> <common:requestError xmlns:common="urn:oma:xml:rest:netapi:common:1"> <serviceException> <messageId>SVC1006</messageId> <text>ACR not found</text> </serviceException> </common:requestError>

### POST

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET, PUT]’ field in the response as per section 14.7 of [RFC2616].

### PUT

This operation is used for refreshing an expired ACR.

#### Example 1: Refresh an expired ACR by using “auth” keyword	 (Informative)

##### Request

PUT /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna/status HTTP/1.1 Accept: application/xml Content-Length: nnnn Content-Type: application/xml Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 <?xml version="1.0" encoding="UTF-8"?> <cr:status xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <acrStatus>Valid</acrStatus> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna/status</resourceURL> </cr:status>

##### Response

HTTP/1.1 200 OK Content-Type: application/xml Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT <?xml version="1.0" encoding="UTF-8"?> <cr:status xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <acrStatus>Valid</acrStatus> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna/status</resourceURL> </cr:status>

#### Example 2: Refreshing a revoked ACR is rejected	(Informative)

##### Request

PUT /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna/status HTTP/1.1 Accept: application/xml Content-Length: nnnn Content-Type: application/xml Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 <?xml version="1.0" encoding="UTF-8"?> <cr:status xmlns:cr="urn:oma:xml:rest:netapi:acrmanagement:1"> <acrStatus>Valid</acrStatus> <resourceURL>http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DDyna/status</resourceURL> </cr:status>

##### Response

HTTP/1.1 403 Forbidden Content-Type: application/xml Content-Length: nnnn Date: Thu, 04 Oct 2012 02:51:59 GMT <?xml version="1.0" encoding="UTF-8"?> <common:requestError xmlns:common="urn:oma:xml:rest:netapi:common:1"> <policyException> <messageId>POL1027</messageId> <text>ACR, abc123;ncc=23415;type=Dyna, is revoked. A new ACR is required to be created</text> </policyException> </common:requestError>

### POST

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET, PUT]’ field in the response as per section 14.7 of [RFC2616].

### DELETE

Method not allowed by the resource. The returned HTTP error status is 405. The server should also include the ‘Allow: [GET, PUT]’ field in the response as per section 14.7 of [RFC2616].

# Fault definitions

## Service Exceptions

For common Service Exceptions refer to [REST_NetAPI_Common].

The following additional Service Exception codes are defined for the RESTful Anonymous Customer Reference Management API.

### SVC1005: ACR creation failed. Unknown userId

| Name | Description |
| --- | --- |
| MessageID | SVC1005 |
| Text | ACR creation operation failed. Unknown userId |
| Variables | None |
| HTTP status code(s) | 403 Forbidden |

### SVC1006: ACR not found

| Name | Description |
| --- | --- |
| MessageID | SVC1006 |
| Text | ACR not found |
| Variables | None |
| HTTP status code(s) | 404 Not Found |

## Policy Exceptions

For common Policy Exceptions refer to [REST_NetAPI_Common].

The following additional Policy Exception codes are defined for the RESTful Anonymous Customer Reference Management API.

### POL1024: ACR creation failed. An active ACR already exists

| Name | Description |
| --- | --- |
| MessageID | POL1024 |
| Text | An active ACR, %1, already exists |
| Variables | %1 – ACR value |
| HTTP status code(s) | 403 Forbidden |

### POL1025: ACR creation failed. An expired ACR already exists

| Name | Description |
| --- | --- |
| MessageID | POL1025 |
| Text | An expired ACR, %1, already exists which needs to be refreshed prior to usage |
| Variables | %1 – ACR value |
| HTTP status code(s) | 403 Forbidden |

### POL1026: Creation of Static ACR is not supported

| Name | Description |
| --- | --- |
| MessageID | POL1026 |
| Text | Creation of Static ACR is not supported |
| Variables | None |
| HTTP status code(s) | 403 Forbidden |

### POL1027: Revoked ACR can no longer be used or refreshed

| Name | Description |
| --- | --- |
| MessageID | POL1027 |
| Text | ACR, %1, is revoked. A new ACR is required to be created. |
| Variables | %1 – ACR value |
| HTTP status code(s) | 403 Forbidden |

### POL1028: Expired ACR may not be used unless refreshed

| Name | Description |
| --- | --- |
| MessageID | POL1028 |
| Text | ACR, %1, is expired. It is required to be refreshed before it is used. |
| Variables | %1 – ACR value |
| HTTP status code(s) | 403 Forbidden |

Change History	(Informative)

Approved Version History

| Reference | Date | Description |
| --- | --- | --- |
| n/a | n/a | No previous version within OMA |

Draft/Candidate Version 1.0 History

| Document Identifier | Date | Sections | Description |
| --- | --- | --- | --- |
| Draft Version: REST_NetAPI_ACR-V1_0 | 13 Mar 2012 | all | Initial baseline. Incorporates input to committee: OMA-ARC-2012-0060-INP_BaselineREST_NetAPI_ACR_TS |
|  | 17 Jul 2012 | 5, 7 | Incorporated: OMA-ARC-REST-ACR-2012-0004-CR_TS_section5_section7 |
|  | 24 Sep 2012 | 5.1, 5.2 | Incorporated: OMA-ARC-REST-ACR-2012-0008R01-CR_resource_summary_section OMA-ARC-REST-ACR-2012-0009R02-CR_Data_types_section |
|  | 8 Oct 2012 | 5.3 | Incorporated: OMA-ARC-REST-ACR-2012-0010R01-CR_Sequence_flows |
|  | 22 Oct 2012 | many | Incorporated: OMA-ARC-REST-ACR-2012-0011R01-CR_Detailed_specification with additional comments included received in R&A. |
|  | 31 Oct 2012 | 6, App B, App G | Incorporated: OMA-ARC-REST-ACR-2012-0012R01-CR_SCRs_and_editorials |
|  | 13 Nov 2012 | many | Incorporated: OMA-ARC-REST-ACR-2012-0017R02-CR_acr_auth_replacing_me_keyword |
|  | 31 Jan 2013 | Many | Incorporated : OMA-ARC-REST-ACR-2013-0001-CR_fixing_TS_based_on_CONRR OMA-ARC-REST-ACR-2013-0004-CR_CONRR_fix_B051_B052_JSON_examples_in_TS |
|  | 09 Feb 2013 | Many | Incorporated: OMA-ARC-REST-ACR-2013-0006-CR_fixing_TS_based_on_CONRR File format corrected from docx to doc |
|  | 19 Feb 2013 | Many | Incorporated: OMA-ARC-REST-ACR-2013-0012-CR_Resolving_XML_validation_issues OMA-ARC-REST-ACR-2013-0011-CR_fixing_TS_based_on_remaining_CONRR_comments |

Static Conformance Requirements	(Normative)

The notation used in this appendix is specified in [SCRRULES].

SCR for REST.ACRMgnt Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| REST-ACRMGNT-SUPPORT-S-001-M | Support for the RESTful Anonymous Customer Reference Management API | 5, 6 |  |
| REST-ACRMGNT-SUPPORT-S-002-M | Support for the XML request & response format | 6 |  |
| REST-ACRMGNT-SUPPORT-S-003-M | Support for the JSON request & response format | 6 |  |

SCR for REST. ACRMgt.UsersApp Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| REST- ACRMGNT-USERSAPP-S-001-M | Support for creation of an ACR and retrieval of an issued ACR | 6.1 |  |
| REST- ACRMGNT-USERSAPP-S-002-M | Retrieve the issued ACR - GET | 6.1.3 |  |
| REST- ACRMGNT-USERSAPP-S-003-M | Create an ACR – POST (XML or JSON) | 6.1.5 |  |

SCR for REST. ACRMgt.ACR Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| REST- ACRMGNT-ACR-S-001-M | Support for removal of an ACR and retrieval of ACR data | 6.2 |  |
| REST- ACRMGNT-ACR-S-002-M | Retrieve ACR data - GET | 6.2.3 |  |
| REST- ACRMGNT-ACR-S-003-M | Remove an ACR – DELETE | 6.2.6 |  |

SCR for REST. ACRMgt.ACR.Status Server

| Item | Function | Reference | Requirement |
| --- | --- | --- | --- |
| REST- ACRMGNT-ACRSTATUS-S-001-M | Support for refreshing expired ACRs and reading the status of an ACR | 6.3 |  |
| REST- ACRMGNT-ACRSTATUS-S-002-M | Read the status of an ACR – GET | 6.3.3 |  |
| REST- ACRMGNT-ACRSTATUS-S-003-M | Refresh an expired ACR – PUT | 6.3.4 |  |

Application/x-www-form-urlencoded Request Format for POST Operations	(Normative)

This specification does not define any API request based on application/x-www-form-urlencoded MIME type.

JSON examples 	(Informative)

JSON (JavaScript Object Notation) is a light-weight, text-based, language-independent data interchange format. It provides a simple means to represent basic name-value pairs, arrays and objects. JSON is relatively trivial to parse and evaluate using standard JavaScript libraries, and hence is suited for REST invocations from browsers or other processors with JavaScript engines. Further information on JSON can be found at [RFC4627].

The following examples show the request and response for various operations using the JSON data format. The examples follow the XML to JSON serialization rules in [REST_NetAPI_Common]. A JSON response can be obtained by using the content type negotiation mechanism specified in [REST_NetAPI_Common].

For full details on the operations themselves please refer to the section number indicated.

Retrieve the issued ACR using “auth” keyword (section 6.1.3.1)

Request:

GET /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Host: example.com Accept: application/json Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903

Response:

HTTP/1.1 200 OK Date: Thu, 04 Oct 2012 02:51:59 GMT Content-Type: application/json Content-Length: nnnn {"acrList": { "acr": { "acrStatus": "Valid", "expiry": "2013-10-26T21:32:52", "resourceURL": "exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3Bncc%3D23415%3Btype%3DStat", "value": "acr:abc123;ncc=23415;type=Stat" }, "resourceURL": "exampleAPI/acrmanagement/v1/acr%3Aauth/application" }}

Retrieve the issued ACR using MSISDN (section 6.1.3.2)

Request:

GET /exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application HTTP/1.1 Host: example.com Accept: application/json

Response:

HTTP/1.1 200 OK Date: Thu, 04 Oct 2012 02:51:59 GMT Content-Type: application/json Content-Length: nnnn {"acrList": { "acr": { "acrStatus": "Valid", "expiry": "2013-10-26T21:32:52", "resourceURL": "exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123;ncc=23415;type=Dyna", "value": "acr:abc123;ncc=23415;type=Dyna" }, "resourceURL": "exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application" }}

Create an ACR using “auth” keyword (section 6.1.4.1)

Request:

POST /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Accept: application/json Content-Length: nnnn Content-Type: application/json Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 {"acr": "\n"}

Response:

HTTP/1.1 201 Created Location: http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc=23415%3Btype=Stat Content-Type: application/json Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT {"acr": { "acrStatus": "Valid", "resourceURL": "exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc=23415%3Btype=Stat", "value": "acr:abc123;ncc=23415;type=Stat " }}

Create an ACR using MSISDN (section 6.1.4.2)

Request:

POST /exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application HTTP/1.1 Accept: application/json Content-Length: nnnn Content-Type: application/json Host: example.com {"acr": {"expiry": "2013-10-26T21:32:52"}}

Response:

HTTP/1.1 201 Created Location: http://example.com/exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3B ncc=23415%3Btype=Dyna Content-Type: application/json Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT {"acr": { "acrStatus": "Valid", "expiry": "2013-10-26T21:32:52", "resourceURL": "exampleAPI/acrmanagement/v1/tel%2B%3A4479901234567/application/acr%3Aabc123%3Bncc=23415%3Btype=Dyna", "value": "acr:abc123;ncc=23415;type=Dyna " }}

Create an ACR over mobile network authenticated connection (section 6.1.4.3)

Request:

POST /exampleAPI/acrmanagement/v1/acr%3Aauth/application HTTP/1.1 Accept: application/json Content-Length: nnnn Content-Type: application/json Host: example.com {"acr": {"expiry": "2013-10-26T21:32:52"}}

Response:

HTTP/1.1 201 Created Location: http://example.com/exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc=23415%3Btype=Dyna Content-Type: application/json Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT {"acr": { "acrStatus": "Valid", "expiry": "2013-10-26T21:32:52", "resourceURL": "exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc=23415%3Btype=Dyna", "value": "acr:abc123;ncc=23415;type=Dyna" }}

Retrieve ACR data (section 6.2.3.1)

Request:

GET /exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3Bncc=23415%3Btype=Dyna HTTP/1.1 Accept: application/json Host: example.com

Response:

HTTP/1.1 200 OK Content-Type: application/json Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT {"acr": { "acrStatus": "Valid", "expiry": "2013-10-26T21:32:52", "resourceURL": "exampleAPI/acrmanagement/v1/tel%3A%2B4479901234567/application/acr%3Aabc123%3Bncc=23415%3Btype=Dyna", "value": "acr:abc123;ncc=23415;type=Dyna" }}

Remove an ACR (section 6.2.6.1)

Request:

DELETE /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc=23415%3Btype=Stat HTTP/1.1 Host: example.com Accept: application/json

Response:

HTTP/1.1 204 No Content Date: Thu, 26 Oct 2012 21:32:52 GMT

Read the Status of an ACR using “auth” keyword (section 6.3.3.1)

Request:

GET /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123%3Bncc=23415%Btype=Stat/status HTTP/1.1 Accept: application/json Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903

Response:

HTTP/1.1 200 OK Content-Type: application/json Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT {"acr": { "acrStatus": "Valid", "value": "acr:abc123;ncc=23415;type=Stat" }}

Refresh an expired ACR by using “auth” keyword (section 6.3.5.1)

Request:

PUT /exampleAPI/acrmanagement/v1/acr%3Aauth/application/acr%3Aabc123;ncc=23415%3Btype=Dyna HTTP/1.1 Accept: application/json Content-Length: nnnn Content-Type: application/json Host: example.com Authorization: BEARER 08776724-6d0d-4aa6-a404-2bc19b5cf903 {"acr": { "acrStatus": "Valid", "expiry": "2013-10-26T21:32:52", "value": "acr:abc123;ncc=23415;type=Dyna" }}

Response:

HTTP/1.1 200 OK Content-Type: application/json Content-Length: nnnn Date: Thu, 26 Oct 2012 21:32:52 GMT {"acr": { "acrStatus": "Valid", "expiry": "2013-10-26T21:32:52", "value": "acr:abc123;ncc=23415;type=Dyna" }}

Operations mapping to a pre-existing baseline specification	(Informative)

As this specification does not have a baseline specification, this appendix is empty.

Light-weight resources	(Informative)

As this version of the specification does not define any light-weight resources, this appendix is empty.

Authorization aspects	(Normative)

This appendix specifies how to use the RESTful Anonymous Customer Reference Management API in combination with some authorization frameworks.

Use with OMA Authorization Framework for Network APIs

The RESTful Anonymous Customer Reference Management API MAY support the authorization framework defined in [Autho4API_10].

A RESTful Anonymous Customer Reference Management API supporting [Autho4API_10]:

SHALL conform to section D.1 of [REST_NetAPI_Common];

SHALL conform to this section G.1.

Scope values

Definitions

In compliance with [Autho4API_10], an authorization server serving clients requests for getting authorized access to the resources exposed by the RESTful Anonymous Customer Reference Management API:

SHALL support the scope values defined in the table below;

MAY support scope values not defined in this specification.

| Scope value | Description | For one-time access token |
| --- | --- | --- |
| oma_rest_acrm.all_{apiVersion} | Provide access to all defined operations on the resources in this version of the API. The {apiVersion} part of this identifier SHALL have the same value as the “apiVersion” URL variable which is defined in section 5.1. This scope value is the union of the other scope values listed in next rows of this table. | No |

Table 1: Scope values for RESTful Anonymous Customer Reference Management API

Downscoping

This version of the specification does not define any scope values to be used for downscoping purposes by the server and/or the resource owner.

Mapping with resources and methods

Tables in this section specify how the scope values defined in section G.1.1.1for the RESTful Anonymous Customer Reference Management API map to the REST resources and methods of this API. In these tables, the root “oma_rest_acrm.” of scope values is omitted for readability reasons.

| Resource | URL Base URL: http://{serverRoot}/acrmanagement/{apiVersion}{userId} | Section refe-rence | HTTP verbs |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GET | PUT | POST | DELETE |
| End User’s Application | /application | 6.1 | all_{apiVersion} | n/a | all_{apiVersion} | n/a |
| Anonymous Customer Reference | /application/{ACR} | 6.2 | all_{apiVersion} | n/a | n/a | all_{apiVersion} |
| ACR Status | /application/{ACR}/status | 6.3 | all_{apiVersion} | all_{apiVersion} | n/a | n/a |

Table 2: Required scope values for: Management of Anonymous Customer Reference

Use of ‘acr:auth’

This section specifies the use of ‘acr:auth’ in place of an end user identifier in a resource URL path.

An ‘acr’ URI of the form ‘acr:auth’, where ‘auth’ is a reserved keyword MAY be used to avoid exposing a real end user identifier in the resource URL path.

A client MAY use ‘acr:auth’ in a resource URL in place of the{userId} resource URL variable in the resource URL path, when the RESTful Anonymous Customer Reference Management API is used in combination with [Autho4API_10].

In the case the RESTful Anonymous Customer Reference Management API supports [Autho4API_10], the server:

- SHALL accept ‘acr:auth’ as a valid value for the resource URL variable {userId}.

SHALL conform to [REST_NetAPI_Common] section 5.8.1.1 regarding the processing of ‘acr:auth’