# Change Request

| Title: | The input for some PIN Mgmt APIs | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_0-20110914-D |  |
| Submission Date: | Sep 21 2011 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Xinmiao Chang, Huawei Technologies Co., Ltd., changxm@huawei.com Bo Hu, China Unicom, hubo68@chinaunicom.cn |  |
| Replaces: | OMA-CD-OpenCMAPI-2011-0016 |  |

Reason for Change

R01 is based on the comments received from the Vancouver meeting.

This contribution provides the APIs to implement following requirements in RD.

| Label | Description | Release |
| --- | --- | --- |
| CMAPI-PIN-001 | The OpenCMAPI Enabler SHALL support Personal Identification Number (PINs) and Personal Unblocking Key (PUKs) management feature | 1.0 |
| CMAPI-PIN-002 | The OpenCMAPI Enabler SHALL provide APIs for: - Enabling and disabling PIN protection - And the corresponding answers from the SIM/R-UIM/NAA on UICC | 1.0 |
| CMAPI-PIN-003 | The OpenCMAPI Enabler SHALL provide APIs for: - Verifying a PIN - And the corresponding answers from the SIM/R-UIM/NAA on UICC | 1.0 |
| CMAPI-PIN-004 | The OpenCMAPI Enabler SHALL provide APIs for: - Unblocking a PIN - And the corresponding answers from the SIM/R-UIM/NAA on UICC | 1.0 |
| CMAPI-PIN-005 | The OpenCMAPI Enabler SHALL provide APIs for: - Changing a PIN - And the corresponding answers from the SIM/R-UIM/NAA on UICC | 1.0 |
| CMAPI-PIN-006 | The OpenCMAPI Enabler SHALL provide APIs for: - Checking current PIN status (ready, not initialised, enabled, disabled, blocked, permanently blocked , unblocked) - And the corresponding answers from the SIM/R-UIM/NAA on UICC | 1.0 |
| CMAPI-PIN-007 | The OpenCMAPI Enabler SHALL verify that the CM application provides only digits 0-9 for the PINs/PUKs entry. Any other values shall be prohibited. | 1.0 |
| CMAPI-PIN-008 | The OpenCMAPI Enabler SHALL verify that the PIN provided by the CM application is any four to eight digits number before sending it to the SIM/R-UIM/NAA on UICC. | 1.0 |

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The CD OpenCMAPI SWG should review and agree this CR.

Detailed Change Proposal

Add the definitions for the following APIs

5.  Application Programming Interfaces (APIs) Design

5.X   PINs/PUKs Management  APIs

5.X.Y CMAPI_DevSrv_EnablePin()

The CMAPI_DevSrv_EnablePin() function is used to enable PIN protection.

Prototype

unsigned long CMAPI_DevSrv_EnablePin (byte btType, const char * pszPinCode )

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| byType | Input | The type of the PIN: 0—PIN, 1—PIN2 |
| pszPinCode | Input | PIN code, value '0' ~ '9', 4-8 digit length. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |
| 0x02 | Wrong PIN (note : or using a specific pin_get_status function after fatal error…) |
| 0x03 | PUK needed (note : or using a specific pin_get_status function after fatal error…) |
| 0x04 | Invalid parameter(s) (note : or using a specific pin_get_status function after fatal error…) |

5.X.Y+1 CMAPI_DevSrv_DisablePin()

The CMAPI_DevSrv_DisablePin() function is used to disable PIN protection.

Prototype

unsigned long CMAPI_DevSrv_DisablePin (byte btType, const char * pszPinCode )

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| byType | Input | The type of PIN: 0—PIN, 1—PIN2 |
| pszPinCode | Input | PIN code, value '0' ~ '9', 4-8 digit length. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y+2 CMAPI_DevSrv_VerifyPin()

The CMAPI_DevSrv_VerifyPin() function is used to verify a PIN.

Prototype

unsigned long CMAPI_DevSrv_VerifyPin (byte btType, const char * pszPinCode, byte * pbtRetry )

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| byType | Input | The type of PIN: 0—PIN, 1—PIN2 |
| pszPinCode | Input | PIN code, value '0' ~ '9', 4-8 digit length. |
| pbtRetry | Input | Times to retry PIN code. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y+3 CMAPI_DevSrv_UnblockPin()

The CMAPI_DevSrv_UnblockPin() function is used to unblock a PIN.

Prototype

unsigned long CMAPI_DevSrv_UnblockPin (byte btType, const char * pszPuk, const char * pszNewPinCode, byte * pbtRetry )

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| byType | Input | The type of PUK: 0—PUK, 1—PUK2 |
| pszPuk | Input | PUK code, value '0' ~ '9', 8 digit length. |
| pszPinCode | Input | New PIN code, value '0' ~ '9', 4-8 digit length. |
| pbtRetry | Input | Times to retry PIN code. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y+4 CMAPI_DevSrv_ChangePin()

The CMAPI_DevSrv_ChangePin() function is used to change a PIN.

Prototype

unsigned long CMAPI_DevSrv_ChangePin (byte btType, const char * pszOldPinCode, const char * pszNewPinCode)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| byType | Input | The type of PUK: 0—PUK, 1—PUK2 |
| pszOldPinCode | Input | Old PIN code, value '0' ~ '9', 4-8 digit length. |
| pszNewPinCode | Input | New PIN code, value '0' ~ '9', 4-8 digit length. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |