# Change Request

| Title: | Recommendation for Mobile IP API | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_0-20110923-D |  |
| Submission Date: | Sep 26 2011 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Mark Staskauskas, Qualcomm Inc., mstaskau@qualcomm.com Chris Heistad, Smith Micro Software, Inc., cheistad@smithmicro.com |  |
| Replaces: | n/a |  |

Reason for Change

This contribution provides the APIs to implement following requirements in the OpenCMAPI RD.

6.3.4

| Label | Description | Release |
| --- | --- | --- |
| CMAPI-MIP-001 | The OpenCMAPI Enabler SHALL support the mechanism to enable and disable the device’s Mobile IP functionality. | 1.0 |
| CMAPI-MIP-002 | The OpenCMAPI Enabler SHALL be able to get and set the Mobile IP configuration, which includes - Home IPv4/IPv6 address - Primary and secondary Home Agent IP addresses - Reverse tunnelling status (enabled/disabled) - NAI - Security parameters and key strings | 1.0 |
| CMAPI-MIP-003 | The OpenCMAPI Enabler SHALL be able to get and set the following Mobile IP parameters: - Mobile IP mode (on, preferred or off) - Registration retry attempt limit and interval - Re-registration period - Authentication method | 1.0 |

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

The CD OpenCMAPI SWG should review and agree this CR.

Detailed Change Proposal

Add the definitions for the following APIs

5.  Application Programming Interfaces (APIs) Design

5.X   Mobile IP APIs

5.X.Y CMAPI_MobileIP_SetMobileIPState()

The CMAPI_MobileIP_SetMobileIPState () function is used to set the current Mobile IP state of the device.

Prototype

unsigned long CMAPI_MobileIP_SetMobileIPState (unsigned long mode)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| mode | Input | The new setting of the device’s Mobile IP mode: |
|  |  | - 0x00000000: Mobile IP off (simple IP only) |
|  |  | - 0x00000001: Mobile IP preferred |
|  |  | - 0x00000002: Mobile IP only |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_GetMobileIPState()

The CMAPI_MobileIP_GetMobileIPState () function is used to retrieve the current Mobile IP state of the device.

Prototype

unsigned long CMAPI_ CMAPI_MobileIP_GetMobileIPState (unsigned long *pMode)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| *pMode | Output | Pointer to the current setting of the device’s Mobile IP mode: |
|  |  | - 0x00000000: Mobile IP off (simple IP only) |
|  |  | - 0x00000001: Mobile IP preferred |
|  |  | - 0x00000002: Mobile IP only |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_SetActiveMobileIPProfile()

The CMAPI_MobileIP_SetActiveMobileIPProfile () function is used to set the index of the Mobile IP profile that the device will use.  There can be several Mobile IP profiles configured on the device, each of which is identified by a unique index.

Prototype

unsigned long CMAPI_MobileIP_SetActiveMobileIPProfile (string *pSPC, byte index)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| *pSPC | Input | Pointer to a string containing the Service Programming Code (SPC). |
| index | Input | Index of the mobile IP profile that will be made the active one. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_GetActiveMobileIPProfile()

The CMAPI_MobileIP_GetActiveMobileIPProfile () function is used to retrieve the index of the Mobile IP profile that the device is currently using.

Prototype

unsigned long CMAPI_MobileIP_GetActiveMobileIPProfile (byte *pIndex)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| *pIndex | Output | Pointer to the index of the currently active mobile IP profile. |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_SetMobileIPProfile()

The CMAPI_MobileIP_SetMobileIPProfile () function is used to configure the contents of a Mobile IP profile on the device.  The function takes as arguments the index of the Mobile IP profile that will be modified and the profile values that will be set by the function.

Prototype

unsigned long CMAPI_MobileIP_SetMobileIPProfile (string *pSPC, byte index, byte *pEnabled, unsigned long *pAddress, unsigned long *pPriHA, unsigned long *pSecHA, byte *pRevTunn, string *pNAI, unsigned long *pHASPI, unsigned long *pAAASPI, string *pMNHA, string *pMNAAA)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| *pSPC | Input | Pointer to a string containing the Service Programming Code (SPC). |
| index | Input | Index of the mobile IP profile that is being set with this function. |
| *pEnabled | Input | (Optional) Enable profile: 0x00: No (disable), any other value: Yes (enable) |
| *pAddress | Input | (Optional) Home IPv4 address |
| *pPriHA | Input | (Optional) Primary Home Agent IPv4 address |
| *pSecHA | Input | (Optional) Secondary Home Agent IPv4 address |
| *pRevTunn | Input | (Optional) Reverse tunneling mode: 0x00: No (Disabled), any other value: Enabled |
| *pNAI | Input | (Optional) Network Access Identifier string |
| *pHASPI | Input | (Optional) Home Agent Security Parameter Index |
| *pAAASPI | Input | (Optional) AAA server Security Parameter Index |
| *pMNHA | Input | (Optional) MN-HA key string |
| *pMNAAA | Input | (Optional) AAA key string |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_GetMobileIPProfile()

The CMAPI_MobileIP_GetMobileIPProfile () function is used to retrieve the contents of a Mobile IP profile on the device.  The function takes as arguments the index of the Mobile IP profile that will be retrieved and the profile values that will be returned by the function.

Prototype

unsigned long CMAPI_MobileIP_GetMobileIPProfile (byte index, byte *pEnabled, unsigned long *pAddress, unsigned long *pPriHA, unsigned long *pSecHA, byte *pRevTunn, byte naiSize, string *pNAI, unsigned long *pHASPI, unsigned long *pAAASPI, unsigned long *pHAState, unsigned long *pAAAState)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| index | Input | Index of the mobile IP profile that is being set with this function. |
| *pEnabled | Output | Profile status: 0x00: Disabled; 0x01: Enabled; 0xFF: Unknown |
| *pAddress | Output | Home IPv4 address (0xFFFFFFFF: Unknown) |
| *pPriHA | Output | Primary Home Agent IPv4 address (0xFFFFFFFF: Unknown) |
| *pSecHA | Output | Secondary Home Agent IPv4 address (0xFFFFFFFF: Unknown) |
| *pRevTunn | Output | Reverse tunneling status: 0x00: Disabled; 0x01: Enabled; 0xFF: Unknown |
| naiSize | Input | Maximum number of characters in the NAI string |
| *pNAI | Output | Network Access Identifier string |
| *pHASPI | Output | Home Agent Security Parameter Index (0xFFFFFFFF: Unknown) |
| *pAAASPI | Output | AAA server Security Parameter Index (0xFFFFFFFF: Unknown) |
| *pHAState | Output | Home Agent Key state: |
|  |  | - 0x00000000: Unset |
|  |  | - 0x00000001: Set, default value |
|  |  | - 0x00000002: Set, non-default value |
|  |  | - 0xFFFFFFFF: Unknown |
| *pAAAState | Output | AAA Key state: |
|  |  | - 0x00000000: Unset |
|  |  | - 0x00000001: Set, default value |
|  |  | - 0x00000002: Set, non-default value |
|  |  | - 0xFFFFFFFF: Unknown |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_SetMobileIPParameters()

The CMAPI_MobileIP_SetMobileIPParameters () function is used to set various parameters that configure the behavior of the device’s Mobile IP client.

Prototype

unsigned long CMAPI_MobileIP_SetMobileIPParameters (string *pSPC, unsigned long *pMode, byte *pRetryLimit, byte *pRetryInterval, byte *pReRegPeriod, byte *pReRegTraffic, byte *pHAAuthenticator, byte *pHA2002bis)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| *pSPC | Input | Service Programming Code (SPC) |
| *pMode | Input | (Optional) Mobile IP mode: |
|  |  | - 0x00000000: Mobile IP off (simple IP only) |
|  |  | - 0x00000001: Mobile IP preferred |
|  |  | - 0x00000002: Mobile IP only |
| *pRetryLimit | Input | (Optional) Mobile IP registration attempt retry limit |
| *pRetryInterval | Input | (Optional) Mobile IP registration attempt retry interval (i.e. time between registration attempts) in minutes |
| *pReRegPeriod | Input | (Optional) Mobile IP re-registration period (time after which current registration expires) in minutes |
| *pReRegTraffic | Input | (Optional) Determines whether to re-register only if there has been data traffic since last registration (0x00: Disabled; any other value: Enabled) |
| *pHAAuthenticator | Input | (Optional) State of MH-HA authenticator calculator (0x00: Disabled; any other value: Enabled) |
| *pHA2002bis | Input | (Optional) Determines whether to use RFC2002bis authentication instead of RFC2002 (0x00: RFC2002; any other value: RFC2002bis) |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_GetMobileIPParameters()

The CMAPI_MobileIP_GetMobileIPParameters () function is used to retrieve the current values of the parameters that configure the behavior of the device’s Mobile IP client.

Prototype

unsigned long CMAPI_MobileIP_GetMobileIPParameters (unsigned long *pMode, byte *pRetryLimit, byte *pRetryInterval, byte *pReRegPeriod, byte *pReRegTraffic, byte *pHAAuthenticator, byte *pHA2002bis)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| *pMode | Output | Mobile IP mode: |
|  |  | - 0x00000000: Mobile IP off (simple IP only) |
|  |  | - 0x00000001: Mobile IP preferred |
|  |  | - 0x00000002: Mobile IP only - 0xFFFFFFFF: Unknown |
| *pRetryLimit | Output | Mobile IP registration attempt retry limit (0xFF if unknown) |
| *pRetryInterval | Output | Mobile IP registration attempt retry interval (i.e. time between registration attempts) in minutes (0xFF if unknown) |
| *pReRegPeriod | Output | Mobile IP re-registration period (time after which current registration expires) in minutes (0xFF if unknown) |
| *pReRegTraffic | Output | Determines whether to re-register only if there has been data traffic since last registration (0x00: Disabled; 0x01: Enabled; 0xFF: Unknown) |
| *pHAAuthenticator | Output | State of MH-HA authenticator calculator (0x00: Disabled; 0x01: Enabled; 0xFF: Unknown) |
| *pHA2002bis | Output | (Optional) Determines whether to use RFC2002bis authentication instead of RFC2002 (0x00: Disabled; 0x01: Enabled; 0xFF: Unknown) |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |

5.X.Y CMAPI_MobileIP_GetLastMobileIPError()

The CMAPI_MobileIP_GetLastMobileIPError () function is used to retrieve the last Mobile IP error that occurred (refer to RFC2002 for a list of error codes).

Prototype

unsigned long CMAPI_ CMAPI_MobileIP_GetLastMobileIPError (unsigned long *pError)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| *pError | Output | Pointer to the most recent Mobile IP error code: |
|  |  | - 0x00000000: Success |
|  |  | - Any other value: Error code as defined in [RFC2002] |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00 | The function succeeded. |
| 0X01 | A fatal error has occurred. Consult the logger for details. |