Change Request

| Title: | UICC De-Personalization APIs | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_0-20120125-D.doc |  |
| Submission Date: | 1 February 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Alan Soloway, Qualcomm, asoloway@qualcomm.com |  |
| Replaces: | n/a |  |

Reason for Change

This document provides additional API function specification for controlling UICC de-personalization.

The following requirement has not been met by the Technical Specification OMA-TS-OpenCMAPI-V1_0-20120125-D:

| CMAPI-UICC-002 | The OpenCMAPI Enabler SHALL provide APIs for managing the de personalisation cycle using the CCK (Corporate Control Key), NCK (Network Control Key), NSCK (Network Subset Control Key), PCK (Personalisation Control Key), SPCK (Service Provider Control Key) as defined in [3GPP TS 22.022] and the CCK (Corporate Control Key), HNCK (HRPD Network Control Key), NCK1 (Network Type 1 Control Key), NCK2 (Network Type 2 Control Key), PCK (Personalization Control Key), SPCK (Service Provider Control Key) as defined in [3GPP2 C.S0068] | 1.0 |
| --- | --- | --- |

This has been noted as part of the Consistency Review with the following comment in OMA-TS-OpenCMAPI-V1_0-20120125-D:

Editor Note: Requirements UICC-002, UICC-003, UICC-004 & UICC-011 are not covered – need to be addressed

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

We recommend to review the input and to agree on the suggested API change.

Detailed Change Proposal

Modify the Open CM API Technical Specification

## UICC Management APIs

## CMAPI_UICC_GetControlKeyStatus()

The CMAPI_ UICC_GetControlKeyStatus() function is used to get the specified UICC de-personalization control key status.

Prototype

dword CMAPI_UICC_GetControlKeyStatus (dword controlKeyID, dword* controlKeyStatus, dword* verifyRetriesLeft, dword* unblockRetriesLeft, dword* blockingStatus)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| controlKeyID | Input | The ID of the specified Control Key: - 0x00000000: Network Personalization - 0x00000001: Network Subset Personalization - 0x00000002: Service Provider Personalization - 0x00000003: Corporate Personalization - 0x00000004: UIM Personalization - 0x00000005: Network Type 1 Personalization - 0x00000006: Network Type 2 Personalization - 0x00000007: HRPD Personalization - 0x00000008: Service Provider Personalization (3GPP2) - 0x00000009: Corporate Personalization (3GPP2) - 0x0000000A: R-UIM Personalization |
| controlKeyStatus | Output | Control Key Status: - 0x00000000: Deactivated - 0x00000001: Activated - 0x00000002: Blocked |
| verifyRetriesLeft | Output | The number of retries left after which the control key will be blocked |
| unblockRetriesLeft | Output | The number of unblock retries left after which the control key will be permanently blocked |
| blockingStatus | Output | Indicates whether the control key affects normal device operation: - 0x00000000: No - 0x00000001: Yes - 0x00000002: Undefined |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. Consult the logger for details. |

## CMAPI_UICC_DeactivateControlKey()

The CMAPI_ UICC_DeactivateControlKey() function is used to deactivate the specified UICC de-personalization control key.  Activation of the control key is performed outside the control of the Open CM API.

Prototype

dword CMAPI_UICC_DeactivateControlKey (dword controlKeyID, string controlKeyValue, dword* verifyRetriesLeft)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| controlKeyID | Input | The ID of the specified Control Key: - 0x00000000: Network Personalization - 0x00000001: Network Subset Personalization - 0x00000002: Service Provider Personalization - 0x00000003: Corporate Personalization - 0x00000004: UIM Personalization - 0x00000005: Network Type 1 Personalization - 0x00000006: Network Type 2 Personalization - 0x00000007: HRPD Personalization - 0x00000008: Service Provider Personalization (3GPP2) - 0x00000009: Corporate Personalization (3GPP2) - 0x0000000A: R-UIM Personalization |
| controlKeyValue | Input | Control Key de-personalization string (Maximum 8 characters) |
| verifyRetriesLeft | Output | The number of retries left after which the control key will be blocked |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. Consult the logger for details. |

## CMAPI_UICC_UnblockControlKey()

The CMAPI_ UICC_UnblockControlKey() function is used to unblock the specified UICC de-personalization control key.

Prototype

dword CMAPI_UICC_UnblockControlKey (dword controlKeyID, string controlKeyUnblockValue, dword* unblockRetriesLeft)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| controlKeyID | Input | The ID of the specified Control Key: - 0x00000000: Network Personalization - 0x00000001: Network Subset Personalization - 0x00000002: Service Provider Personalization - 0x00000003: Corporate Personalization - 0x00000004: UIM Personalization - 0x00000005: Network Type 1 Personalization - 0x00000006: Network Type 2 Personalization - 0x00000007: HRPD Personalization - 0x00000008: Service Provider Personalization (3GPP2) - 0x00000009: Corporate Personalization (3GPP2) - 0x0000000A: R-UIM Personalization |
| controlKeyUnblockValue | Input | Control Key unblock string (Maximum 8 characters) |
| unblockRetriesLeft | Output | The number of unblock retries left after which the control key will be permanently blocked |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. Consult the logger for details. |