Change Request

| Title: | UICC De-Personalization APIs | Public OMA Confidential |
| --- | --- | --- |
| To: | CD OpenCMAPI |  |
| Doc to Change: | OMA-TS-OpenCMAPI-V1_0-20120213-D.doc |  |
| Submission Date: | 15 February 2012 |  |
| Classification: | 0: New Functionality 1: Major Change 2: Bug Fix 3: Editorial |  |
| Source: | Alan Soloway, Qualcomm, asoloway@qualcomm.com Patrice Beaudou, Gemalto, patrice.beaudou@gemalto.com |  |
| Replaces: | OMA-CD-OpenCMAPI-2012-0016-CR_UICC_DePersonalization_API |  |

Reason for Change

This document provides additional API function specification for controlling Mobile Equipment (device) de-personalization.

The following requirement has not been met by the Technical Specification OMA-TS-OpenCMAPI-V1_0-20120125-D:

| CMAPI-UICC-002 | The OpenCMAPI Enabler SHALL provide APIs for managing the de personalisation cycle using the CCK (Corporate Control Key), NCK (Network Control Key), NSCK (Network Subset Control Key), PCK (Personalisation Control Key), SPCK (Service Provider Control Key) as defined in [3GPP TS 22.022] and the CCK (Corporate Control Key), HNCK (HRPD Network Control Key), NCK1 (Network Type 1 Control Key), NCK2 (Network Type 2 Control Key), PCK (Personalization Control Key), SPCK (Service Provider Control Key) as defined in [3GPP2 C.S0068] | 1.0 |
| --- | --- | --- |

This has been noted as part of the Consistency Review with the following comment in OMA-TS-OpenCMAPI-V1_0-20120125-D:

Editor Note: Requirements UICC-002, UICC-003, UICC-004 & UICC-011 are not covered – need to be addressed

Impact on Backward Compatibility

None

Impact on Other Specifications

None

Intellectual Property Rights

Members and their Affiliates (collectively, "Members") agree to use their reasonable endeavours to inform timely the Open Mobile Alliance of Essential IPR as they become aware that the Essential IPR is related to the prepared or published Specification.  This obligation does not imply an obligation on Members to conduct IPR searches.  This duty is contained in the Open Mobile Alliance application form to which each Member's attention is drawn.  Members shall submit to the General Manager of Operations of OMA the IPR Statement and the IPR Licensing Declaration.  These forms are available from OMA or online at the OMA website at www.openmobilealliance.org.

Recommendation

We recommend to review the input and to agree on the suggested API change.

Detailed Change Proposal

Modify the Open CM API Technical Specification

## Device Service APIs

## 5.11.15	CMAPI_DevSrv_GetControlKeyStatus()

The CMAPI_DevSrv_GetControlKeyStatus() function is used to get the specified Mobile Equipment (device) de-personalization control key status.

Prototype

dword CMAPI_DevSrv_GetControlKeyStatus (dword deviceID, dword systemID, dword controlKeyID, dword* controlKeyStatus, dword* verifyRetriesLeft, dword* unblockRetriesLeft)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned |
| systemID | Input | The radio system, either GPP or 3GPP2, to which the function apply when the device is a multi-mode device. - 0x00000000: 3GPP - 0x00000001: 3GPP2 |
| controlKeyID | Input | The ID of the specified Control Key: - 0x00000000: Network Control Key (NCK, only applicable for 3GPP systems) - 0x00000001: Network Subset Control Key (NSCK, only applicable for 3GPP systems) - 0x00000002: Service Provider Control Key (SPCK) - 0x00000003: Corporate Control Key (CCK) - 0x00000004: Personalization Control Key (PCK) - 0x00000005: Network Type 1 Control Key (NCK1, only applicable for 3GPP2 systems) - 0x00000006: Network Type 2 Control Key (NCK2, only applicable for 3GPP2 system) - 0x00000007: HRPD Network Control Key (HNCK, only applicable for 3GPP2 systems) See [3GPP TS 22.022] and [3GPP2 C.S0068] for Control Keys definition and procedures. |
| controlKeyStatus | Output | Control Key Status: - 0x00000000: Deactivated - 0x00000001: Activated - 0x00000002: Blocked |
| verifyRetriesLeft | Output | The number of retries left after which the control key will be blocked |
| unblockRetriesLeft | Output | The number of unblock retries left after which the control key will be permanently blocked |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. Consult the logger for details. |
| 0X00000002 | Control Key not supported by this system (when an ID of a 3GPP2 only Control Key is sent to a 3GPP system device or when an ID of a 3GPP only Control Key is sent to a 3GPP2 system device). |

## 5.11.16	CMAPI_DevSrv_DeactivateControlKey()

The CMAPI_DevSrv_DeactivateControlKey() function is used to deactivate the specified Mobile Equipment (device) de-personalization control key.  Activation of the control key is performed outside the control of the Open CM API.

Prototype

dword CMAPI_DevSrv_DeactivateControlKey (dword deviceID, dword systemID, dword controlKeyID, string controlKeyValue, dword* verifyRetriesLeft)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned |
| systemID | Input | The radio system, either GPP or 3GPP2, to which the function apply when the device is a multi-mode device. - 0x00000000: 3GPP - 0x00000001: 3GPP2 |
| controlKeyID | Input | The ID of the specified Control Key: - 0x00000000: Network Control Key (NCK, only applicable for 3GPP systems) - 0x00000001: Network Subset Control Key (NSCK, only applicable for 3GPP systems) - 0x00000002: Service Provider Control Key (SPCK) - 0x00000003: Corporate Control Key (CCK) - 0x00000004: Personalization Control Key (PCK) - 0x00000005: Network Type 1 Control Key (NCK1, only applicable for 3GPP2 systems) - 0x00000006: Network Type 2 Control Key (NCK2, only applicable for 3GPP2 system) - 0x00000007: HRPD Network Control Key (HNCK, only applicable for 3GPP2 systems) See [3GPP TS 22.022] and [3GPP2 C.S0068] for Control Keys definition and procedures. |
| controlKeyValue | Input | Control Key de-personalization decimal string (Maximum 16 digits) |
| verifyRetriesLeft | Output | The number of retries left after which the control key will be blocked |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. Consult the logger for details. |
| 0X00000002 | Control Key not supported by this system (when an ID of a 3GPP2 only Control Key is sent to a 3GPP system device or when an ID of a 3GPP only Control Key is sent to a 3GPP2 system device). |

## 5.11.17	CMAPI_DevSrv_UnblockControlKey()

The CMAPI_DevSrv_UnblockControlKey() function is used to unblock the specified Mobile Equipment (device) de-personalization control key.

Prototype

dword CMAPI_DevSrv_UnblockControlKey (dword deviceID, dword systemID, dword controlKeyID, string controlKeyUnblockValue, dword* unblockRetriesLeft)

| Parameters |  |  |
| --- | --- | --- |
| Field Name | Mode | Description |
| deviceID | Input | The ID of the device concerned |
| systemID | Input | The radio system either GPP or 3GPP2 to which the function apply when the device is a multi-mode device. - 0x00000000: 3GPP - 0x00000001: 3GPP2 |
| controlKeyID | Input | The ID of the specified Control Key: - 0x00000000: Network Control Key (NCK, only applicable for 3GPP systems) - 0x00000001: Network Subset Control Key (NSCK, only applicable for 3GPP systems) - 0x00000002: Service Provider Control Key (SPCK) - 0x00000003: Corporate Control Key (CCK) - 0x00000004: Personalization Control Key (PCK) - 0x00000005: Network Type 1 Control Key (NCK1, only applicable for 3GPP2 systems) - 0x00000006: Network Type 2 Control Key (NCK2, only applicable for 3GPP2 system) - 0x00000007: HRPD Network Control Key (HNCK, only applicable for 3GPP2 systems) See [3GPP TS 22.022] and [3GPP2 C.S0068] for Control Keys definition and procedures. |
| controlKeyUnblockValue | Input | Control Key unblock decimal string (Maximum 16 digits) |
| unblockRetriesLeft | Output | The number of unblock retries left after which the control key will be permanently blocked |

| Return Values |  |
| --- | --- |
| Value | Description |
| 0X00000000 | The function succeeded. |
| 0X00000001 | A fatal error has occurred. Consult the logger for details. |
| 0X00000002 | Control Key not supported by this system (when an ID of a 3GPP2 only Control Key is sent to a 3GPP system device or when an ID of a 3GPP only Control Key is sent to a 3GPP2 system device). |